| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, Korea, 20-28 January 2011 | Document: JCTVC-D044-rev1 WG11 Number: m18791 |
| --- | --- |

| Title: | Pulse code modulation mode for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-VC] |  |  |
| Purpose: | [Proposal] |  |  |
| Author(s) or Contact(s): | Keiichi Chono Kenta Senzaki Hirofumi Aoki Junji Tajime Yuzo Senda 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 431 7595 chono@ct.jp.nec.com k-senzaki@bp.jp.nec.com hao@bu.jp.nec.com tajime@ak.jp.nec.com y-senda@bc.jp.nec.com |
| Source: | [Information and Media Processing Labs, NEC Corporation] |  |  |

_____________________________

# Abstract

This contribution presents the preliminary results of Pulse Code Modulation (PCM) mode encoding in HEVC codec, in which new single-bit syntax is introduced in PU header in order to signal the use of PCM coding in its associated Intra CU. Experimental results for common test sequences show average BD-rate losses of 0.01% (Y), 0.02% (U), 0.02% (V) for all intra high efficiency setting, 1.43% (Y), 1.18 % (U), 1.16 % (V) for all intra low complexity setting, 0.01% (Y), 0.01% (U), 0.11% (V) for random access high efficiency setting, 0.43% (Y), 0.53% (U), 0.51% (V) for random access low complexity setting, 0.01% (Y), -0.29% (U), -0.02% (V) for low delay high efficiency setting, and 0.15% (Y), 0.19% (U), 0.24% (V) for low delay low complexity setting. Additional experimental results for a synthesized sequence demonstrate that the use of PCM mode avoids yielding the number of bits which is prohibitively greater than that of input video data and reduces decode time significantly. It is proposed that the concept of PCM mode coding is adopted in HM ver.2 and that its design is further studied.

- Introduction

The PCM mode coding in the H.264/MPEG-4 AVC standard transmits sample values of its associated MB without prediction, transform coding, and entropy coding, and thus it allows encoders to adjust the number of bits of MB to a predetermined value or less without complicated computation. Since HM ver.1 does not have means which plays a role similar to the PCM mode coding, it would be desirable to adopt the use of PCM mode in HM ver. 2.

The purpose of this contribution is to present preliminary results of PCM mode coding in TMuC0.9 and to discuss further study items on the PCM design of HEVC. The high-level description of PCM in TMuC0.9 and its simulation results are described in the following sections.

# High-level description of PCM mode encoding

2.1. PCM mode encoding in the H.264/MPEG-4 AVC standard

The use of PCM mode coding is indicated by a MB mode syntax, and when PCM mode coding is used, the associated MB’s sample values are transmitted without prediction, transform coding, and entropy coding. Note that when entropy coding is CABAC, BAC is terminated and initialized prior to transmitting sample values.

2.2. Proposed PCM mode encoding in HM

The use of PCM mode coding is indicated by a new single-bit syntax, pcm_flag, which is conditionally signaled in the PU header layer when the associated CU is Intra. If pcm_flag presents and its value is one, the associated Intra CU is encoded by PCM mode coding; otherwise the associated Intra CU is encoded by intra prediction.

Entropy coding of pcm_flag for CABAC is the same as that of end_of_slice_flag, and therefore BAC is terminated and initialized prior to transmitting sample values when pcm_flag=1 is transmitted. On the other hand, no entropy coding is applied to pcm_flag for LCEC, that is, fixed-length one bit is transmitted. Obviously the signaling method of pcm_flag for LCEC is not efficient; some sophisticated signaling methods can be further studied. For example, pcm_flag can be jointly encoded with PU’s prediction mode or partition size information.

PCM sample values are transmitted in a way similar to that of the H.264/MPEG-4 AVC standard except that those values are quantized to the original bit-depth prior to the transmission when IBDI is used. The LSBs of samples extended by IBDI do not have any information about their original sample values and increase the amount of PCM data significantly. Therefore the quantization is applied when IBDI is used. The benefits of the quantization are demonstrated in 3.3.

The description of the proposed syntax and semantics is shown in appendix.

- Simulation

3.1. Experimental setup

PCM mode coding described in 2.2 is implemented into the TMuC0.9. 64-bit encoder and decoder executables were generated by using Microsoft Visual C++ 2008. They ran on Windows-based computing platforms listed below. Note that as inferred from the list, CPU automated over-clocking happened for some results. Individual encode and decode times can be significantly varied.

| Computing platform | Usage |
| --- | --- |
| Windows 7 64-bit Professional Intel Xeon 3.33GHz and Memory 32GB | Up to 12 processes for encoding and decoding ran at the same time. |

Simulation results for common test sequences were conducted according to JCTVC-C500 [1] and described in 3.2. Since PCM mode is not selected in the common test sequences with the common coding conditions, additional simulation results for a synthesized sequence with low QP values are described in 3.3.

3.2. Results for common test sequences

An attached three Excel sheet, JCTVC-D044-NEC-PCM-results.xls, shows the detailed results of PCM mode encoding. Table 1 shows its summary.

Table 1: Summary of PCM results.

The followings are observed from the Table 1:

- Entropy coding of pcm_flag for CABAC does not increase BD-rates significantly in all coding cases, as expected from the fact that its MPS probability is about to 0.99;

- Entropy coding of pcm_flag for LCEC moderately increases BD-rates in All intra coding case, whereas it slightly increases BD-rates in other cases (up to 0.43% on average).

The observations suggest that efficient pcm_flag signaling methods for LCEC are further studied.

3.3. Additional results for synthesized noisy sequence

PCM mode is not selected in the common test sequences with the common coding conditions. Therefore this subsection provides additional results for a synthesized sequence depicted in Figure 1. The synthesized sequence is Gaussian noise, which is difficult to compress by using prediction, transform coding, and entropy encoding. Some videos can partially contain textures whose nature is similar to Gaussian noise.

Figure 1: Synthesized sequence, Sandstorms, (416x240 30Hz 30frames)

Table 2 and Table 3 show high efficiency intra coding results for Sandstorms by using QP values 12, 17, 22, 27, 32, and 37. “Comp. Ratio” is the ratio between the data sizes of input video and its compressed bitsream. “Relative dec.” is the ratio between the decode times of TMuC and Proposal (expressed in percentage). From the tables, it is observed that Proposal (PCM mode encoding) avoids yielding the number of bits which is significantly greater than that of input video for QP=12. In addition, the tables show that Proposal saves its decode time significantly for QP=12 and 17 by restricting the amount of entropy-coded bits.

Table 2: High efficiency all intra results. Note that “99.99” in Y psnr means that decode video is loss less.

Table 3: Low complexity all intra results. Note that “99.99” in Y psnr means that decode video is loss less.

Next, the benefit of applying the quantization to IBDI-extended PCM samples is demonstrated. Table 4 compares two PCM mode encoding methods: one (denoted by “Proposal w/o quantization”) does not apply the quantization to IBDI-extended PCM samples prior to their transmission; other does apply the quantization to those. QP=12 and 17 results show that the quantization avoids yielding the number of bits which is prohibitively greater than that of input video data and that w/o quantization yields the great number of bits which is roughly equivalent to the size of 12-bit version of the input video.

Table 4: High efficiency all intra results. Note that “99.99” in Y psnr means that decode video is loss less.

- Conclusions

This contribution has presented the preliminary results of PCM mode encoding in HEVC codec. Simulation results demonstrated that:

- Proposed entropy coding of pcm_flag for CABAC affects BD-rates negligibly;

- Proposed entropy coding of pcm_flag for LCEC moderately increases BD-rates in All intra coding case;

- Applying quantization to IBDI-extended PCM samples is efficient to avoid yielding the number of bits which is prohibitively greater than that of input video data.

It is recommended that the concept of PCM mode coding is adopted in HM ver.2 and that its implementation, especially entropy coding of pcm_flag for LCEC, is further improved.

- Reference

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-C500, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010.

# Patent rights declaration(s)

NEC Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix: Proposed syntax and semantics

(The modified portion of JCT-VC B205 is highlighted with red colour.)

| coding_unit( x0, y0, currCodingUnitSize ) { | C | Descriptor |
| --- | --- | --- |
| if( x0+currCodingUnitSize < PicWidthInSamplesL && y0+currCodingUnitSize < PicHeightInSamplesL && |  |  |
| currCodingUnitSize > MinCodingUnitSize ) |  |  |
| split_coding_unit_flag | 2 | u(1) ¦ ae(v) |
| if( split_coding_unit_flag ) { |  |  |
| splitCodingUnitSize = currCodingUnitSize >> 1 |  |  |
| x1 = x0 + splitCodingUnitSize |  |  |
| y1 = y0 + splitCodingUnitSize |  |  |
| coding_unit( x0, y0, splitCodingUnitSize ) | 2 ¦ 3 ¦ 4 |  |
| if( x1 < PicWidthInSamplesL ) |  |  |
| coding_unit( x1, y0, splitCodingUnitSize ) | 2 ¦ 3 ¦ 4 |  |
| if( y1 < PicHeightInSamplesL ) |  |  |
| coding_unit( x0, y1, splitCodingUnitSize ) | 2 ¦ 3 ¦ 4 |  |
| if( x1 < PicWidthInSamplesL && y1 < PicHeightInSamplesL ) |  |  |
| coding_unit( x1, y1, splitCodingUnitSize ) | 2 ¦ 3 ¦ 4 |  |
| } else { |  |  |
| prediction_unit( x0, y0, currCodingUnitSize ) | 2 |  |
| if( PredMode == I_PCM ) { |  |  |
| while( !byte_aligned( ) ) |  |  |
| pcm_alignment_zero_bit | 3 | f(1) |
| for( i = 0; i < currCodingUnitSize* currCodingUnitSize; i++ ) |  |  |
| pcm_sample_luma[ i ] | 3 | u(v) |
| for( i = 0; i < currCodingUnitSize* currCodingUnitSize/2; i++ ) |  |  |
| pcm_sample_chroma[ i ] | 3 | u(v) |
| } |  |  |
| if( PredMode != MODE_SKIP ¦¦ !(PredMode == MODE_INTRA && planar_flag == 1) ) |  |  |
| if( entropy_coding_mode_flag ) { |  |  |
| transform_unit_tree( x0, y0, currCodingUnitSize, 0 ) | 3 ¦ 4 |  |
| transform_unit_coeff( x0, y0, currCodingUnitSize, 0, 0 ) | 3 ¦ 4 |  |
| transform_unit_coeff( x0, y0, currCodingUnitSize, 0, 1 ) | 3 ¦ 4 |  |
| transform_unit_coeff( x0, y0, currCodingUnitSize, 0, 2 ) | 3 ¦ 4 |  |
| } else |  |  |
| transform_unit_vlc( x0, y0, currCodingUnitSize ) | 3 ¦ 4 |  |
| } |  |  |
| } |  |  |

| prediction_unit( x0, y0, currPredUnitSize ) { | C | Descriptor |
| --- | --- | --- |
| if( slice_type != I ) |  |  |
| skip_flag |  |  |
| if( skip_flag ) { |  |  |
| if( mv_competition_flag ) { |  |  |
| if( inter_pred_idc != Pred_L1 && NumMVPCand( L0 ) > 1 ) |  |  |
| mvp_idx_l0 | 2 | ue(v) ¦ ae(v) |
| if( inter_pred_idc != Pred_L0 && NumMVPCand( L1 ) > 1 ) |  |  |
| mvp_idx_l1 | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| } |  |  |
| else { |  |  |
| if(!entropy_coding_mode_flag) |  |  |
| mode_table_idx |  | vlc(n,v) |
| else { |  |  |
| if( slice_type != I ) |  |  |
| pred_mode | 2 | u(1) ¦ ae(v) |
| } |  |  |
| if( PredMode == MODE_INTRA ) { |  |  |
| pcm_flag | 2 | u(1) ¦ ae(v) |
| if(PredMode != I_PCM){ |  |  |
| planar_flag | 2 | u(1) ¦ ae(v) |
| if(planar_flag) { |  |  |
| planar_delta_y = getPlanarDelta() |  |  |
| planar_delta_uv_present_flag | 2 | u(1) ¦ ae(v) |
| if(planar_delta_uv_present_flag) { |  |  |
| planar_delta_u = getPlanarDelta() |  |  |
| planar_delta_v = getPlanarDelta() |  |  |
| } |  |  |
| } else { |  |  |
| if(entropy_coding_mode_flag) |  |  |
| intra_split_flag | 2 | ae(v) |
| combined_intra_pred_flag | 2 | u(1) ¦ ae(v) |
| for( i = 0; i < ( intra_split_flag ? 4 : 1 ); i++ ) { |  |  |
| prev_intra_luma_pred_ flag | 2 | u(1) ¦ ae(v) |
| if( !prev_intra_pred_luma_flag ) |  |  |
| rem_intra_luma_pred_mode | 2 | ue(v) ¦ ae(v) |
| if( adaptive_intra_smoothing_flag ) |  |  |
| intra_luma_filt_flag | 2 | u(1) ¦ ae(v) |
| } |  |  |
| } |  |  |
| if( chroma_format_idc != 0 ) |  |  |
| intra_chroma_pred_mode | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| else if( PredMode == MODE_INTER ) { |  |  |
| if(entropy_coding_mode_flag) |  |  |
| inter_partitioning_idc | 2 | ue(v) ¦ ae(v) |
| for( i= 0; i < NumPuParts( inter_partitioning_idc ); i++ ) { |  |  |
| if( NumMergeCandidates > 0 ) { |  |  |
| merge_flag[ i ] | 2 | ue(1) ¦ ae(v) |
| if( merge_flag[ i ] && NumMergeCandidates > 1 ) |  |  |
| merge_left_flag[ i ] | 2 | ue(1) ¦ ae(v) |
| } |  |  |
| if( !merge_flag[i] ) { |  |  |
| if( slice_type == B ) |  |  |
| inter_pred_idc[ i ] | 2 | ue(v) ¦ ae(v) |
| if( inter_pred_idc[ i ] != Pred_L1 ) { |  |  |
| if( entropy_coding_mode_flag ) { |  |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |  |
| ref_idx_l0[ i ] | 2 | ue(v) ¦ ae(v) |
| mvres_l0 [ i ] | 2 | ue(1) ¦ ae(v) |
| } |  |  |
| else { |  |  |
| ref_idx_mvres_l0[ i ] | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| mvd_l0[ i ][ 0 ] | 2 | se(v) ¦ ae(v) |
| mvd_l0[ i ][ 1 ] | 2 | se(v) ¦ ae(v) |
| if( mv_competition_flag && NumMVPCand( L0, i ) > 1 ) |  |  |
| mvp_idx_l0[ i ] | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| if( inter_pred_idc[ i ] != Pred_L0 ) { |  |  |
| if( entropy_coding_mode_flag ) { |  |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |  |
| ref_idx_l1[ i ] | 2 | ue(v) ¦ ae(v) |
| mvres_l1 [ i ] | 2 | ue(1) ¦ ae(v) |
| } |  |  |
| else { |  |  |
| ref_idx_mvres_l1[ i ] | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| mvd_l1[ i ][ 0 ] | 2 | se(v) ¦ ae(v) |
| mvd_l1[ i ][ 1 ] | 2 | se(v) ¦ ae(v) |
| if( mv_competition_flag && NumMVPCand( L1, i ) > 1 ) |  |  |
| mvp_idx_l1[ i ] | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| else if( PredMode == MODE_DIRECT ) { |  |  |
| if( slice_type == B ) |  |  |
| inter_pred_idc | 2 | ue(v) ¦ ae(v) |
| if( mv_competition_flag ) { |  |  |
| if( inter_pred_idc != Pred_L1 && NumMVPCand( L0 ) > 1 ) |  |  |
| mvp_idx_l0 | 2 | ue(v) ¦ ae(v) |
| if( inter_pred_idc != Pred_L0 && NumMVPCand( L1 ) > 1 ) |  |  |
| mvp_idx_l1 | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| } |  |  |
| } |  |  |

pcm_alignment_zero_bit is a bit equal to 0.

pcm_sample_luma[ i ] is a sample value. The pcm_sample_luma[ i ] values represent coded luma sample values in the raster scan within the coding unit. The number of bits used to represent each of these samples is BitDepthY. Each of coded luma sample values is reconstructed by (pcm_sample_luma[ i ] << increased_bit_depth_luma).

pcm_sample_chroma[ i ] is a sample value. The first currCodingUnitSize* currCodingUnitSize/4 pcm_sample_chroma[ i ] values represent coded Cb sample values in the raster scan within the coding unit and the remaining currCodingUnitSize* currCodingUnitSize/4 pcm_sample_chroma[ i ] values represent coded Cr sample values in the raster scan within the coding unit. The number of bits used to represent each of these samples is BitDepthC. Each of coded chroma sample values is reconstructed by (pcm_sample_chroma[ i ] << increased_bit_depth_chroma).

pcm_flag specifies if associated intra coding unit is coded by I_PCM: If the pcm_flag value is equal to 1, associated intra coding unit is coded by I_PCM; otherwise the pcm_flag velue is equal to 0 or the pcm_flag syntax is not present, associated intra coding unit is coded by MODE_INTRA.