| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, Korea, 20-28 January 2011 | Document: JCTVC-D045-rev1 WG11 Number: m18792 |
| --- | --- |

| Title: | Rounding-error conscious memory compression method for IBDI |  |  |
| --- | --- | --- | --- |
| Status: | [Input Document to JCT-VC] |  |  |
| Purpose: | [Proposal] |  |  |
| Author(s) or Contact(s): | Keiichi Chono Kenta Senzaki Hirofumi Aoki Junji Tajime Yuzo Senda 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 431 7595 chono@ct.jp.nec.com k-senzaki@bp.jp.nec.com hao@bu.jp.nec.com tajime@ak.jp.nec.com y-senda@bc.jp.nec.com |
| Source: | [Information and Media Processing Labs, NEC Corporation] |  |  |

_____________________________

# Abstract

This contribution proposes a rounding-error conscious memory compression method for IBDI, in which bit depth reduction based on fixed rounding is introduced in prediction loop and an enhanced Sum of Squared Errors (SSE) computation is conducted by encoder. Simulation results show that in video coding systems based on in-loop fixed rounding, the enhanced SSE computation improves BD-rate by 0.4%(Y), -0.2%(U), and 0.2%(V) for high efficiency random access setting, and 3.8%(Y), 0.9%(U), and 2.5%(V) for high efficiency low delay setting. Simulation results also demonstrates that IBDI with the rounding-error conscious memory compression attains BD-rate improvements by 0.7%(Y), 4.3%(U), and 5.5%(V) for random access setting, and 1.4%(Y), 5.6%(U), and 6.4%(V) for low delay setting, without increasing reference picture memory bandwidth.

It is suggested that:

- The rounding-error conscious memory compression method is considered as one of the operation points of HEVC codec when IBDI is used;

- The rounding-error conscious memory compression method is used in a potential core experiments on reference picture memory compression technologies.

- Introduction

One of the reference pictures memory compression AHG tasks is to investigate anomalous coding efficiency loss caused by fixed rounding of extended bit depth values for reference picture storage, which is pointed out by Toshiba in JCTVC-C076 [1]. Toshiba [2] and NEC had been investigated the problem independently and it has been found that Sum of Squared Errors (SSE) computation of the current TMuC software is unconscious with quantization error associated with the fixed rounding of extended bit depth values. This contribution proposes an enhanced SSE computation method for IBDI rounding-error conscious memory compression and evaluates its performance in terms of BD-rates and memory bandwidth.

- Improved SSE computation method for IBDI rounding-error conscious memory compression

Figure 1 depicts an encoder block diagram of current TMuC software in which the bit-depth of input video frame is extended prior to encoding and its reconstructed video frame is used in prediction loop. Since quantization error associated with fixed rounding of extended bit-depth values is not introduced in prediction loop, current TMuC software computes SSE values for mode decision, ALF on/off decision, etc. as follows:

SSE(src,rec,Δ)=(src[i,j]−rec[i,j])/Δ)2					(1)

where src and rec are bit-depth extended input and reconstructed signals, respectively, and ∆ is step size associated with the extended bit-depth values.

Figure 1	Encoder block diagram w/o IBDI fixed rounding

When fixed rounding of extended bit-depth values (hereafter, hereafter referred to as IBDI fixed rounding) is introduced into prediction loop for reference picture buffer size reduction as depicted in Figure 2, it is obvious that SSE computation of eq. (1) is undesirable; transmitting motion-compensated prediction residual to be discarded by IBDI fixed rounding, for example, does not improve the quality of reconstructed video frames which are used for encoding subsequent frames. Therefore SSE values must be computed in a way that is conscious with quantization error associated with IBDI fixed rounding as follows;

SSE(src,rec,Δ)=(src[i,j]Δ+0.5−rec[i,j]Δ+0.5)2				(2)

Figure 2	Encoder block diagram with IBDI fixed rounding

- Simulation

The following coding cases were tested according to JCTVC-C500 [3] by using Windows-based computing platforms listed below:

Fixed-Rounding: TMuC0.9 with IBDI fixed rounding

IBDI-Off: Anchor without IBDI

Proposal: Fixed-Rounding with the enhanced SSE computation (eq. (2))

| Computing platform | Usage |
| --- | --- |
| Windows 7 64-bit Professional Intel Xeon 3.33GHz and Memory 32GB | Up to 12 processes for encoding and decoding ran at the same time. |

Attached three Excel sheets, NEC-Anchor-vs-FixedRounding.xls, NEC-Anchor-vs-noIBDI.xls, and NEC-Anchor-vs-Proposal.xls, show the detailed results of Fixed-Rounding, IBDI-Off, and Proposal, respectively. Table 1 shows the summary of those results.

Table 1 and Table2 show that:

- Anomalous coding efficiency losses appear in Fixed-Rounding for Class E sequences, as observed in JCTVC-C76, whereas such anomalous coding efficiency losses do not appear in Proposal;

- Coding efficiency loss of Proposal is smaller than that of IBDI-Off in all cases.

Table 1		Summary of results. RA and LD denote random-access and low-delay, respectively.

The first observation verifies that the proposed SSE is essential in video coding systems based on IBDI fixed rounding. As shown in Table A in Appendix, the proposed SSE improves BD-rate by 0.4%(Y), -0.2%(U), and 0.2%(V) for high efficiency random access setting, and 3.8%(Y), 0.9%(U), and 2.5%(V) for high efficiency low delay setting.

The second observation and the fact that the reference picture memory bandwidth of Proposal is equivalent to that of IBDI-Off demonstrate that using IBDI with the rounding-error conscious memory compression is one of the good operation points of HEVC codec in terms of the balance between BD-rate and reference picture memory bandwidth. As shown in Table B in Appendix, it can attain BD-rate improvements by 0.7%(Y), 4.3%(U), and 5.5%(V) for random access setting, and 1.4%(Y), 5.6%(U), and 6.4%(V) for low delay setting, without increasing the reference picture memory bandwidth.

- 4 Conclusions

This contribution has presented a rounding-error conscious memory compression method for IBDI and verified its effectiveness in terms of the balance of BD-rates and reference picture memory bandwidth.

It is recommended that:

- The rounding-error conscious memory compression method is considered as one of the operation points of HEVC codec when IBDI is used;

- The rounding-error conscious memory compression method is used in a potential core experiments on reference picture memory compression technologies.

- Reference

- T. Chujoh, “TE2: Summary of TE2 on IBDI and memory compression,” JCT-VC document JCTVC-C076, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd meeting, Guangzhou, October 2010.

- T. Chujoh and T. Yamakage, "Adaptive scaling for bit depth compression on IBDI,” JCT-VC document JCTVC-D152, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 4th meeting, Guangzhou, Daegu, Korea, 20-28 January 2011.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-C500, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010.

# Patent rights declaration(s)

NEC Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

- Appendix: Summaries of BD-rates

Table A: BD-rates of Fixed-Rounding vs. Proposal

Table B: BD-rates of IBDI-Off vs. Proposal