| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D046 WG11 Number: m18793 |
| --- | --- |

| Title: | CE7: Mode-Dependent Transforms for Block-based Intra Coding from Institute for Infocomm Research |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chuohao Yeo Yih Han Tan Zhengguo Li Susanto Rahardja 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 chyeo@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

To simplify the operation of Mode Dependent Directional Transform (MDDT), a Mode-Dependent Fast Separable KLT for Block-based Intra Coding was previously introduced that requires only two transform matrices: a DCT and a DST (Odd Type-3 DST). The 4x4 DST also has a structure that can be exploited to reduce the operation count of the transform operation. This contribution provides test results as stipulated in CE7. Experimental results show that the proposed technique matches the performance of using trained KLTs even though the approach requires no training, and has lower operation count and storage costs.

- Introduction

In MDDT, separable transforms are used to approximate an ideal non-separable directional transform [1]. Specifically, if X is an N x N block of pixels, then its 2D transform coefficients are given by:

Y=CmXRmT

The subscript m in Cm and Rm denotes the dependence of the column and row transforms on the prediction mode. Note that in H.264, Cm = Rm = M, where M is an integer Discrete Cosine Transform (DCT) transform matrix. In the Mode-Dependent Directional Transform (MDDT) scheme, Karhunen-Loève Transforms (KLT) are used for Cm and Rm and are computed by performing SVD on residual blocks of prediction mode m collected from training video sequences.

In a previous proposal, we have presented a technique for reducing the computational and storage complexity of MDDT by requiring only two transforms [2]: DCT and an Odd Type-3 Discrete Sine Transform (ODST-3):

KijN=22N+1sin(2i−1)jπ2N+1   ,   1≤i,j≤N

To handle the unified intra prediction modes [3] as adopted in HM1 [4], we have also derived a mapping of intra prediction modes to row/column transforms [5]. This is summarized in Table 1.

In this contribution, we provide detailed experimental results using the test conditions in CE7 [6], according to the common test conditions [7], for our proposed scheme. Note that in this proposal, only the 4x4 and 8x8 transforms for intra prediction residuals are modified, while scanning of coefficients for coding remains unchanged, i.e., defaulting to HM1.

Table 1. Choice of transforms for various intra prediction modes

| Intra Prediction Mode | Column | Row |
| --- | --- | --- |
| DC | EDCT-2 | EDCT-2 |
| VER-8 to VER-1 | ODST-3 | ODST-3 |
| VER to VER+8 | ODST-3 | EDCT-2 |
| HOR-7 to HOR-1 | ODST-3 | ODST-3 |
| HOR to HOR+8 | EDCT-2 | ODST-3 |

# Simulation Results

The test conditions as specified in CE7 [6] are followed. The implementation is on TMuC 0.9. Tests were conducted using the two intra configurations, i.e., high-efficiency and low-complexity, and the two random access configurations [7].

We compare our proposed scheme to the HM1 anchor. For comparison purposes, we also provide results using the trained KLT matrices provided by Qualcomm’s implementation, but without the adaptive scanning of coefficients. This was done using TMuC 0.9 code, with QC_MDDT set to 1, but with the source code handling adaptive scanning of transform coefficients disabled.

## RD results

Table 2 shows the BD-Rate [8] results for our proposed scheme, while Table 3 shows the BD-Rate results for Qualcomm’s trained KLTs, for various coding configurations. Here, the proposed technique closely matches the RD performance of the trained KLTs, but requires less storage and computational complexity.

Table 2. BD-Rate results for proposed set of transforms

|  | Intra |  |  |  |  | Intra LoCo |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -2.2 |  | -2.5 |  | -2.6 | -3.6 |  | -1.5 | -1.0 |  |
| Class B | -0.9 |  | -1.1 |  | -1.1 | -1.7 |  | -1.1 | -0.9 |  |
| Class C | -1.2 |  | -1.0 |  | -1.0 | -1.9 |  | -1.1 | -1.1 |  |
| Class D | -1.3 |  | -1.1 |  | -1.0 | -1.9 |  | -1.0 | -1.0 |  |
| Class E | -1.5 |  | -1.7 |  | -1.5 | -2.7 |  | -0.4 | -0.9 |  |
| All | -1.3 |  | -1.4 |  | -1.3 | -2.2 |  | -1.0 | -1.0 |  |
| Enc Time[%] | 101% |  |  |  |  | 103% |  |  |  |  |
| Dec Time[%] | 101% |  |  |  |  | 104% |  |  |  |  |
|  | Random access |  |  |  |  | Random access LoCo |  |  |  |  |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -0.9 |  | -0.5 |  | -0.2 | -1.2 |  | -0.2 | 0.1 |  |
| Class B | -0.5 |  | -0.3 |  | -0.4 | -0.6 |  | -0.3 | -0.3 |  |
| Class C | -0.6 |  | -0.3 |  | -0.4 | -0.6 |  | -0.3 | -0.3 |  |
| Class D | -0.6 |  | -0.3 |  | -0.1 | -0.5 |  | -0.2 | -0.2 |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| All | -0.6 |  | -0.3 |  | -0.3 | -0.7 |  | -0.2 | -0.2 |  |
| Enc Time[%] | 100% |  |  |  |  | 100% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  | 101% |  |  |  |  |

Table 3. Results for using trained Qualcomm's trained KLT in TMuC 0.9

|  | Intra |  |  |  |  | Intra LoCo |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -2.3 |  | -2.4 | -2.5 |  | -3.5 |  | -1.6 | -1.2 |  |
| Class B | -1.0 |  | -1.2 | -1.2 |  | -1.9 |  | -1.3 | -1.1 |  |
| Class C | -1.2 |  | -1.1 | -1.1 |  | -2.0 |  | -1.3 | -1.3 |  |
| Class D | -1.4 |  | -1.2 | -1.2 |  | -2.0 |  | -1.2 | -1.2 |  |
| Class E | -1.6 |  | -1.7 | -1.6 |  | -2.9 |  | -0.6 | -1.1 |  |
| All | -1.4 |  | -1.4 | -1.4 |  | -2.3 |  | -1.2 | -1.1 |  |
| Enc Time[%] | 101% |  |  |  |  | 105% |  |  |  |  |
| Dec Time[%] | 102% |  |  |  |  | 106% |  |  |  |  |
|  | Random access |  |  |  |  | Random access LoCo |  |  |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -1.0 |  | -0.4 | -0.2 |  | -1.3 |  | -0.2 | 0.1 |  |
| Class B | -0.6 |  | -0.3 | -0.3 |  | -0.8 |  | -0.3 | -0.3 |  |
| Class C | -0.7 |  | -0.4 | -0.5 |  | -0.7 |  | -0.4 | -0.3 |  |
| Class D | -0.7 |  | -0.1 | -0.2 |  | -0.6 |  | -0.3 | -0.3 |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| All | -0.7 |  | -0.3 | -0.3 |  | -0.8 |  | -0.3 | -0.2 |  |
| Enc Time[%] | 100% |  |  |  |  | 100% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  | 101% |  |  |  |  |

More detailed coding results can be found in the Excel attachments.

## Qualitative results

We have not noticed any items of note qualitatively.

## Complexity

Our implementation of the 4-point ODST-3 requires only 8 multiplies and 10 adds [2], as compared to 16 multiples and 12 adds for a full matrix multiply. In addition, when the DCT is used, the operations count can be further reduced to that of a fast DCT. Our implementation of the 8-point ODST-3 still requires 64 multiplies and 56 adds as in a full matrix multiply.

Our proposal addresses reducing the number of transforms, and we require storing just two transforms: the DCT and the ODST-3.

There are no significant differences in encoding and decoding run-time.

## Training

Since our KLT is derived analytically, no training is required.

# Conclusion

This contribution presents CE7 results for our earlier proposal on simplifying MDDT [2]. The proposed technique reduces the computational and storage complexity of MDDT by using only two transforms: DCT and ODST-3. Experimental results show that coding gains are provided at little additional complexity compared to the HM1 reference, and that there is no significant loss in coding efficiency compared to using trained KLTs. We recommend adopting this proposal into HM.

# References

[1] Y. Ye and M. Karczewicz, “Improved Intra Coding,” ITU-T Q.6/SG11 VCEG, VCEG-AG11, Shenzhen, China, Oct 2007.

[2] C. Yeo, Y. H. Tan, Z. Li and S. Rahardja, “Mode-Dependent Fast Separable KLT for Block-based Intra Coding,” JCTVC-B024, Geneva, Switzerland, Jul 2010.

[3] J.-H. Min, S. Lee, I.-K. Kim, W.-J. Han, J. Lainema, K. Ugur, “Unification of the Directional Intra Prediction Methods in TMuC,” JCTVC-B100, Geneva, Switzerland, Jul 2010.

[4] T. K. Tan et al., “Summary of HEVC working draft 1 and HEVC test model (HM),” JCTVC-C405, Guangzhou, China, Oct 2010.

[5] C. Yeo, Y. H. Tan, Z. Li and S. Rahardja, “Choice of transforms in MDDT for Unified Intra Prediction,” JCTVC-C039, Guangzhou, China, Oct 2010.

[6] R. Cohen et al., “CE7: Alternative Transforms,” JCTVC-B507, Guangzhou, China, Oct 2010.

[7] Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-C500, Guangzhou, China, Oct 2010.

[8] Gisle Bjontegaard, “Calculation of Average PSNR Differences between RD curves,” ITU-T SC16/Q6, VCEG-M33, Austin, USA, Apr 2001.

# Patent rights declaration(s)

Institute for Infocomm Research may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

For avoidance of doubt, positive BD-Rate implies that the proposed method uses more rate, while a negative BD-Rate implies that the proposed method improves coding performance.