| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D048 WG11 Number: m18795 |
| --- | --- |

| Title: | Low-Complexity 4-point Integer Discrete Sine Transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chuohao Yeo Yih Han Tan Zhengguo Li 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 chyeo@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

In this proposal, a method for performing a multiplier-less 4-point integer discrete sine transform is presented. In previous proposals, it has been shown that the mode-dependent directional transform can be carried out using an appropriate combination of discrete cosine transform and discrete sine transform. However, the discrete sine transform is carried out using fixed-point arithmetic. Here, an integer approximation of the discrete sine transform that is exactly orthogonal is presented. Furthermore, the resulting integer transform can be implemented without any multiplications. Experimental results show that the proposed integer transform has compression performance that is similar to the higher precision fixed-point arithmetic implementation.

- Introduction

In MDDT, separable transforms are used to approximate an ideal non-separable directional transform [1]. Specifically, if X is an N x N block of pixels, then its 2D transform coefficients are given by:

Y=CmXRmT

The subscript m in Cm and Rm denotes the dependence of the column and row transforms on the prediction mode. Note that in H.264, Cm = Rm = M, where M is an integer Discrete Cosine Transform (DCT) transform matrix. In the Mode-Dependent Directional Transform (MDDT) scheme, Karhunen-Loève Transforms (KLT) are used for Cm and Rm and are computed by performing SVD on residual blocks of prediction mode m collected from training video sequences.

In a previous proposal, we have presented a technique for reducing the computational and storage complexity of MDDT by requiring only two transforms [2]: DCT and an Odd Type-3 Discrete Sine Transform (ODST-3):

KijN=22N+1sin(2i−1)jπ2N+1   ,   1≤i,j≤N

To handle the unified intra prediction modes [3] as adopted in HM1 [4], we have also derived a mapping of intra prediction modes to row/column transforms [5]. This is summarized in Table 1.

Table 1. Choice of transforms for various intra prediction modes

| Intra Prediction Mode | Column | Row |
| --- | --- | --- |
| DC | EDCT-2 | EDCT-2 |
| VER-8 to VER-1 | ODST-3 | ODST-3 |
| VER to VER+8 | ODST-3 | EDCT-2 |
| HOR-7 to HOR-1 | ODST-3 | ODST-3 |
| HOR to HOR+8 | EDCT-2 | ODST-3 |

In the literature, while fast methods exist for computing the even discrete sine transform (EDST) [7], there is no published fast algorithm for ODST. One possible reason is that up to now, the ODST has not seen many applications [9]. Another possible reason is that even when N is a power of 2, the implicit periodic extension is 4N+2, which is not a power of 2.

Nevertheless, for N=4, there is a way to avoid a full matrix multiply that performs the ODST-3 with 8 multiplies and 10 adds [2]. In contrast, a full matrix multiply requires 16 multiplies and 12 adds, while a fast EDCT-2 requires 4 multiplies and 8 adds.

Suppose the following 4-point ODST-3 is desired to be computed:

Where the notation  is used to denote a NxN matrix with the (i,j) entry being given by . One can easily work out that:

where . Ignoring the scale factor, the forward 4-point ODST-3 can be carried out using the following steps [2]:

# Proposed Approach

In the TMuC MDDT implementation [1], fixed-point arithmetic (with 7 bits of fractional accuracy) is used to implement the transform. This means that the actual implemented integer transform is not exactly orthogonal. When the transform is not exactly orthogonal, there can be distortion after performing the forward transform followed by the backward transform even without any quantization of the transform coefficients.

In this proposal, an integer approximation of the 4-point ODST-3 that is exactly orthogonal is presented. Consider the following matrix:

In fact, any factor in the range of [11.43, 12.83] could be used to produce the same transform matrix. It can be easily verified that  is orthogonal. Furthermore, each transform coefficient is at most the sum of two powers of 2, which means that the transform can be efficiently implemented with just shifts and adds:

Here, shifts operations are denoted by “<<”. A total of 6 shifts and 15 adds are needed to compute the forward transform.

The backward transform is simply , and it is easily verified that the following steps performs the backward transform:

Compared to an implementation with 7-bit of fractional accuracy which increases the input dynamic range by 8 bits, the implementation above only increases the input dynamic range by about 5 bits.

In practice, the quantization and de-quantization processes typically handle the scaling that is introduced in the forward and backward transform [10]. Further details regarding the quantization scaling matrices used are provided in the Appendix.

# Simulation Results

The test conditions as specified in CE7 [6] are followed. The implementation is on TMuC 0.9. Tests were conducted using the two intra configurations, i.e., high-efficiency and low-complexity, and the two random access configurations [7].

We compare our proposed scheme to the HM1 anchor. For comparison purposes, we also provide results using the trained KLT matrices provided by Qualcomm’s implementation, but without the adaptive scanning of coefficients. This was done using TMuC 0.9 code, with QC_MDDT set to 1, but with the source code handling adaptive scanning of transform coefficients disabled.  We also show results for the 7-bit fractional accurate versions of DCT/DST [2].

## RD results

Table 2 shows the BD-Rate [8] results for our proposed scheme, while Table 3 and Table 4 show the BD-Rate results for the 7-bit fractional accurate versions of DCT/DST [2,11] and Qualcomm’s trained KLTs, for various coding configurations [1] respectively.

Table 2. Results using proposed low-complexity orthogonal transform

|  | Intra |  |  |  |  | Intra LoCo |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -2.2 |  | -2.4 | -2.4 |  | -3.0 |  | -1.6 | -1.2 |  |
| Class B | -0.9 |  | -1.1 | -1.1 |  | -1.5 |  | -1.2 | -1.0 |  |
| Class C | -1.2 |  | -1.0 | -1.1 |  | -1.8 |  | -1.2 | -1.2 |  |
| Class D | -1.3 |  | -1.0 | -1.0 |  | -1.8 |  | -1.1 | -1.1 |  |
| Class E | -1.5 |  | -1.8 | -1.7 |  | -2.5 |  | -0.7 | -1.1 |  |
| All | -1.3 |  | -1.3 | -1.3 |  | -2.0 |  | -1.1 | -1.1 |  |
| Enc Time[%] | 102% |  |  |  |  | 105% |  |  |  |  |
| Dec Time[%] | 101% |  |  |  |  | 104% |  |  |  |  |
|  | Random access |  |  |  |  | Random access LoCo |  |  |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -0.9 |  | -0.6 | -0.3 |  | -1.1 |  | -0.2 | -0.1 |  |
| Class B | -0.5 |  | -0.4 | -0.5 |  | -0.6 |  | -0.3 | -0.3 |  |
| Class C | -0.7 |  | -0.6 | -0.6 |  | -0.6 |  | -0.5 | -0.4 |  |
| Class D | -0.6 |  | -0.1 | 0.0 |  | -0.5 |  | -0.4 | -0.2 |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| All | -0.6 |  | -0.4 | -0.4 |  | -0.6 |  | -0.4 | -0.3 |  |
| Enc Time[%] | 101% |  |  |  |  | 101% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  | 100% |  |  |  |  |

Table 3. BD-Rate results for 8-bit fractional accurate versions of DCT/DST [2,11]

|  | Intra |  |  |  |  | Intra LoCo |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -2.2 |  | -2.5 |  | -2.6 | -3.6 |  | -1.5 | -1.0 |  |
| Class B | -0.9 |  | -1.1 |  | -1.1 | -1.7 |  | -1.1 | -0.9 |  |
| Class C | -1.2 |  | -1.0 |  | -1.0 | -1.9 |  | -1.1 | -1.1 |  |
| Class D | -1.3 |  | -1.1 |  | -1.0 | -1.9 |  | -1.0 | -1.0 |  |
| Class E | -1.5 |  | -1.7 |  | -1.5 | -2.7 |  | -0.4 | -0.9 |  |
| All | -1.3 |  | -1.4 |  | -1.3 | -2.2 |  | -1.0 | -1.0 |  |
| Enc Time[%] | 101% |  |  |  |  | 103% |  |  |  |  |
| Dec Time[%] | 101% |  |  |  |  | 104% |  |  |  |  |
|  | Random access |  |  |  |  | Random access LoCo |  |  |  |  |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -0.9 |  | -0.5 |  | -0.2 | -1.2 |  | -0.2 | 0.1 |  |
| Class B | -0.5 |  | -0.3 |  | -0.4 | -0.6 |  | -0.3 | -0.3 |  |
| Class C | -0.6 |  | -0.3 |  | -0.4 | -0.6 |  | -0.3 | -0.3 |  |
| Class D | -0.6 |  | -0.3 |  | -0.1 | -0.5 |  | -0.2 | -0.2 |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| All | -0.6 |  | -0.3 |  | -0.3 | -0.7 |  | -0.2 | -0.2 |  |
| Enc Time[%] | 100% |  |  |  |  | 100% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  | 101% |  |  |  |  |

Table 4. Results for using trained Qualcomm's trained KLT in TMuC 0.9 [1]

|  | Intra |  |  |  |  | Intra LoCo |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -2.3 |  | -2.4 | -2.5 |  | -3.5 |  | -1.6 | -1.2 |  |
| Class B | -1.0 |  | -1.2 | -1.2 |  | -1.9 |  | -1.3 | -1.1 |  |
| Class C | -1.2 |  | -1.1 | -1.1 |  | -2.0 |  | -1.3 | -1.3 |  |
| Class D | -1.4 |  | -1.2 | -1.2 |  | -2.0 |  | -1.2 | -1.2 |  |
| Class E | -1.6 |  | -1.7 | -1.6 |  | -2.9 |  | -0.6 | -1.1 |  |
| All | -1.4 |  | -1.4 | -1.4 |  | -2.3 |  | -1.2 | -1.1 |  |
| Enc Time[%] | 101% |  |  |  |  | 105% |  |  |  |  |
| Dec Time[%] | 102% |  |  |  |  | 106% |  |  |  |  |
|  | Random access |  |  |  |  | Random access LoCo |  |  |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -1.0 |  | -0.4 | -0.2 |  | -1.3 |  | -0.2 | 0.1 |  |
| Class B | -0.6 |  | -0.3 | -0.3 |  | -0.8 |  | -0.3 | -0.3 |  |
| Class C | -0.7 |  | -0.4 | -0.5 |  | -0.7 |  | -0.4 | -0.3 |  |
| Class D | -0.7 |  | -0.1 | -0.2 |  | -0.6 |  | -0.3 | -0.3 |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| All | -0.7 |  | -0.3 | -0.3 |  | -0.8 |  | -0.3 | -0.2 |  |
| Enc Time[%] | 100% |  |  |  |  | 100% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  | 101% |  |  |  |  |

The results show that the compared to the 7-bit accurate versions of the 4-point DST, there is little loss in coding performance when the proposed low-complexity integer DST is used. Similarly, there is also a small decrease in coding performance compared to the trained KLTs provided by Qualcomm.

Furthermore, we also show results in Table 5 when the proposed orthogonal DST is combined with mode-dependent simplified scans (MDSS) [12] for the high-efficiency cases. For comparison, we also show results for the 7-bit accurate DCT/DST with MDSS, and Qualcomm’s MDDT.

Table 5. Results comparing proposed orthogonal DST+MDSS with DCT/DST+MDSS and QC MDDT

|  | Proposed Integer DST + MDSS |  |  | DCT/DST + MDSS |  |  | QC MDDT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Intra |  |  | Intra |  |  | Intra |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.5 | -2.9 | -2.8 | -2.5 | -3.0 | -2.9 | -2.8 | -3.0 | -2.9 |
| Class B | -1.3 | -1.7 | -1.7 | -1.3 | -1.7 | -1.7 | -1.5 | -1.8 | -1.8 |
| Class C | -2.0 | -1.9 | -1.9 | -2.0 | -1.8 | -1.8 | -2.1 | -2.0 | -2.0 |
| Class D | -2.1 | -1.7 | -1.8 | -2.1 | -1.8 | -1.8 | -2.1 | -2.0 | -1.9 |
| Class E | -2.7 | -2.9 | -2.6 | -2.7 | -2.9 | -2.6 | -2.7 | -2.7 | -2.5 |
| All | -2.0 | -2.1 | -2.0 | -2.0 | -2.1 | -2.0 | -2.1 | -2.2 | -2.1 |
| Enc Time[%] | 108% |  |  | 106% |  |  | 106% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  | 223% |  |  |
|  | Random access |  |  | Random access |  |  | Random access |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.1 | -0.7 | -0.5 | -1.1 | -0.5 | -0.3 | -1.2 | -0.6 | -0.2 |
| Class B | -0.7 | -0.5 | -0.6 | -0.7 | -0.4 | -0.3 | -0.8 | -0.5 | -0.4 |
| Class C | -0.9 | -0.7 | -0.6 | -1.0 | -0.7 | -0.5 | -1.0 | -0.6 | -0.7 |
| Class D | -0.9 | -0.7 | -0.5 | -0.9 | -0.6 | -0.3 | -0.9 | -0.5 | -0.5 |
| Class E |  |  |  |  |  |  |  |  |  |
| All | -0.9 | -0.6 | -0.6 | -0.9 | -0.6 | -0.4 | -0.9 | -0.5 | -0.5 |
| Enc Time[%] | 102% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  | 322% |  |  |

The results show that all 3 methods have very similar coding performance gains. However, the proposed low-complexity orthogonal ODST-3 with MDSS incurs no significant increase in decoding time, and increases the dynamic range less than the 7-bit fractional accurate ODST-3.

More detailed coding results can be found in the Excel attachments.

## Qualitative results

We have not noticed any items of note qualitatively.

## Complexity

Our proposed 4-point ODST-3 requires only 6 shifts and 15 adds, compared to the previous implementation which required 8 multiplies and 10 adds [2]. A full matrix multiply requires 16 multiples and 12 adds.

Our 8-point ODST-3 still requires 64 multiplies and 56 adds as in a full matrix multiply.

In addition to the integer DCT, only one additional transform (the ODST-3) is required for each block size.

There are no significant differences in encoding and decoding run-time.

## Training

Since our KLT is derived analytically, no training is required.

# Conclusion

This contribution presents experimental results for our proposal on providing a low-complexity and orthogonal integer approximation of ODST-3. Experimental results show that coding gains are provided at little additional complexity compared to the HM1 reference, and that there is no significant loss in coding efficiency compared to using trained KLTs or using 7-bit fractional accurate version of the ODST-3. We recommend further study of this proposal for adoption into HM.

# References

[1] Y. Ye and M. Karczewicz, “Improved Intra Coding,” ITU-T Q.6/SG11 VCEG, VCEG-AG11, Shenzhen, China, Oct 2007.

[2] C. Yeo, Y. H. Tan, Z. Li and S. Rahardja, “Mode-Dependent Fast Separable KLT for Block-based Intra Coding,” JCTVC-B024, Geneva, Switzerland, Jul 2010.

[3] J.-H. Min, S. Lee, I.-K. Kim, W.-J. Han, J. Lainema, K. Ugur, “Unification of the Directional Intra Prediction Methods in TMuC,” JCTVC-B100, Geneva, Switzerland, Jul 2010.

[4] T. K. Tan et al., “Summary of HEVC working draft 1 and HEVC test model (HM),” JCTVC-C405, Guangzhou, China, Oct 2010.

[5] C. Yeo, Y. H. Tan, Z. Li and S. Rahardja, “Choice of transforms in MDDT for Unified Intra Prediction,” JCTVC-C039, Guangzhou, China, Oct 2010.

[6] R. Cohen et al., “CE7: Alternative Transforms,” JCTVC-B507, Guangzhou, China, Oct 2010.

[7] Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-C500, Guangzhou, China, Oct 2010.

[8] Gisle Bjontegaard, “Calculation of Average PSNR Differences between RD curves,” ITU-T SC16/Q6, VCEG-M33, Austin, USA, Apr 2001.

[9] Z. Wang, “Fast Discrete Sine Transform Algorithms,” in Elsevier Signal Processing, no. 19, pp. 91–102, 1990.

[10] H. S. Malvar, A. Hallapuro, M. Karczewicz, and L. Kerofsky, “Low-complexity transform and quantization in H.264/AVC,” in IEEE CSVT, Vol. 13, No. 7, pp. 598-603, Jul. 2003.

[11] C. Yeo, Y. H. Tan, Z. Li and S. Rahardja, “CE7: Mode-Dependent Transforms for Block-based Intra Coding from Institute for Infocomm Research,” JCTVC-D046, Daegu, Korea, Jan 2011.

[12] C. Yeo, Y. H. Tan, and Z. Li, “Mode-Dependent Coefficient Scanning for Intra Prediction Residual Coding,” JCTVC-D049, Daegu, Korea, Jan 2011.

# Appendix I – Quantization issues

Assume that the following 2-D 4x4 transform has been carried out:

Thus, Y(i,j) contains the transform coefficients. Here, Cm and Rm would be either the integer cosine transform used in H.264/AVC [10] or the integer ODST-3 presented above.

Quantization is performed using the following formula [10]:

If QP (0-51) is the quantization parameter used,  and  . Also, A(C,R,Q_M,I,j) is a scaling factor that depends on the row transform used (R), column transform used (C), QM, and the location of the coefficient (i,j). f is a parameter that controls that size of the quantization deadzone. QS(C,R) is the number of bits to be shifted down by when performing quantization and depends on the column and row transform used. Thus, the quantization process does not require any division, and all the scaling that is required by the transform is absorbed into A(.).

Similarly, de-quantization is performed using the following [10]:

Here, B(C,R,QM,i,j) is a scaling factor used for de-quantization. The process is still not complete; after the inverse transform is performed, an additional bitshift of DQS(C,R) is needed.

The table below shows the values used for QS(.) and DQS(.). Note that for the case where the DCT is used for both row and column, it defaults to the H.264/AVC choices.

| Column Transform | Row Transform | QS(C,R) | DQS(C,R) |
| --- | --- | --- | --- |
| DCT | DCT | 15 | 6 |
| DCT | DST | 17 | 9 |
| DST | DCT | 17 | 9 |
| DST | DST | 19 | 12 |

The pseudo-code below shows the values used for A(.) and B(.). Again, for the case of DCT being used as the row and column transforms, the values default to those in H.264/AVC.

int A[4][6][16] =

{ //     Row   Col

// 0 - DCT x DCT (QuantPrec = 15)

{

{

13107,  8066, 13107,  8066,

8066,  5243,  8066,  5243,

13107,  8066, 13107,  8066,

8066,  5243,  8066,  5243,

},

{

11916,  7490, 11916,  7490,

7490,  4660,  7490,  4660,

11916,  7490, 11916,  7490,

7490,  4660,  7490,  4660,

},

{

10082,  6554, 10082,  6554,

6554,  4194,  6554,  4194,

10082,  6554, 10082,  6554,

6554,  4194,  6554,  4194,

},

{

9362,  5825,  9362,  5825,

5825,  3647,  5825,  3647,

9362,  5825,  9362,  5825,

5825,  3647,  5825,  3647,

},

{

8192,  5243,  8192,  5243,

5243,  3355,  5243,  3355,

8192,  5243,  8192,  5243,

5243,  3355,  5243,  3355,

},

{

7282,  4559,  7282,  4559,

4559,  2893,  4559,  2893,

7282,  4559,  7282,  4559,

4559,  2893,  4559,  2893,

},

},

// 1 - DCT x DST (QuantPrec = 15+2)

{

{

8779,  5371,  8779,  5371,

8779,  5371,  8779,  5371,

8779,  5371,  8779,  5371,

8779,  5371,  8779,  5371,

},

{

7609,  5072,  7609,  5072,

7609,  5072,  7609,  5072,

7609,  5072,  7609,  5072,

7609,  5072,  7609,  5072,

},

{

6714,  4348,  6714,  4348,

6714,  4348,  6714,  4348,

6714,  4348,  6714,  4348,

6714,  4348,  6714,  4348,

},

{

6007,  3970,  6007,  3970,

6007,  3970,  6007,  3970,

6007,  3970,  6007,  3970,

6007,  3970,  6007,  3970,

},

{

5435,  3382,  5435,  3382,

5435,  3382,  5435,  3382,

5435,  3382,  5435,  3382,

5435,  3382,  5435,  3382,

},

{

4755,  3043,  4755,  3043,

4755,  3043,  4755,  3043,

4755,  3043,  4755,  3043,

4755,  3043,  4755,  3043,

},

},

// 2 - DST x DCT (QuantPrec = 15+2)

{

{

8779,  8779,  8779,  8779,

5371,  5371,  5371,  5371,

8779,  8779,  8779,  8779,

5371,  5371,  5371,  5371,

},

{

7609,  7609,  7609,  7609,

5072,  5072,  5072,  5072,

7609,  7609,  7609,  7609,

5072,  5072,  5072,  5072,

},

{

6714,  6714,  6714,  6714,

4348,  4348,  4348,  4348,

6714,  6714,  6714,  6714,

4348,  4348,  4348,  4348,

},

{

6007,  6007,  6007,  6007,

3970,  3970,  3970,  3970,

6007,  6007,  6007,  6007,

3970,  3970,  3970,  3970,

},

{

5435,  5435,  5435,  5435,

3382,  3382,  3382,  3382,

5435,  5435,  5435,  5435,

3382,  3382,  3382,  3382,

},

{

4755,  4755,  4755,  4755,

3043,  3043,  3043,  3043,

4755,  4755,  4755,  4755,

3043,  3043,  3043,  3043,

},

},

// 3 - DST x DST (QuantPrec = 15+4)

{

{

5846,  5846,  5846,  5846,

5846,  5846,  5846,  5846,

5846,  5846,  5846,  5846,

5846,  5846,  5846,  5846,

},

{

5230,  5230,  5230,  5230,

5230,  5230,  5230,  5230,

5230,  5230,  5230,  5230,

5230,  5230,  5230,  5230,

},

{

4517,  4517,  4517,  4517,

4517,  4517,  4517,  4517,

4517,  4517,  4517,  4517,

4517,  4517,  4517,  4517,

},

{

4141,  4141,  4141,  4141,

4141,  4141,  4141,  4141,

4141,  4141,  4141,  4141,

4141,  4141,  4141,  4141,

},

{

3549,  3549,  3549,  3549,

3549,  3549,  3549,  3549,

3549,  3549,  3549,  3549,

3549,  3549,  3549,  3549,

},

{

3206,  3206,  3206,  3206,

3206,  3206,  3206,  3206,

3206,  3206,  3206,  3206,

3206,  3206,  3206,  3206,

},

},

};

int B[4][6][16] =

{ //     Row   Col

// 0 - DCT x DCT (DequantPrec = 6)

{

{

10, 13, 10, 13,

13, 16, 13, 16,

10, 13, 10, 13,

13, 16, 13, 16,

},

{

11, 14, 11, 14,

14, 18, 14, 18,

11, 14, 11, 14,

14, 18, 14, 18,

},

{

13, 16, 13, 16,

16, 20, 16, 20,

13, 16, 13, 16,

16, 20, 16, 20,

},

{

14, 18, 14, 18,

18, 23, 18, 23,

14, 18, 14, 18,

18, 23, 18, 23,

},

{

16, 20, 16, 20,

20, 25, 20, 25,

16, 20, 16, 20,

20, 25, 20, 25,

},

{

18, 23, 18, 23,

23, 29, 23, 29,

18, 23, 18, 23,

23, 29, 23, 29,

},

},

// 1 - DCT x DST (DequantPrec = 6+3)

{

{

13,  17,  13,  17,

13,  17,  13,  17,

13,  17,  13,  17,

13,  17,  13,  17,

},

{

15,  18,  15,  18,

15,  18,  15,  18,

15,  18,  15,  18,

15,  18,  15,  18,

},

{

17,  21,  17,  21,

17,  21,  17,  21,

17,  21,  17,  21,

17,  21,  17,  21,

},

{

19,  23,  19,  23,

19,  23,  19,  23,

19,  23,  19,  23,

19,  23,  19,  23,

},

{

21,  27,  21,  27,

21,  27,  21,  27,

21,  27,  21,  27,

21,  27,  21,  27,

},

{

24,  30,  24,  30,

24,  30,  24,  30,

24,  30,  24,  30,

24,  30,  24,  30,

},

},

// 2 - DST x DCT (DequantPrec = 6+3)

{

{

13,  13,  13,  13,

17,  17,  17,  17,

13,  13,  13,  13,

17,  17,  17,  17,

},

{

15,  15,  15,  15,

18,  18,  18,  18,

15,  15,  15,  15,

18,  18,  18,  18,

},

{

17,  17,  17,  17,

21,  21,  21,  21,

17,  17,  17,  17,

21,  21,  21,  21,

},

{

19,  19,  19,  19,

23,  23,  23,  23,

19,  19,  19,  19,

23,  23,  23,  23,

},

{

21,  21,  21,  21,

27,  27,  27,  27,

21,  21,  21,  21,

27,  27,  27,  27,

},

{

24,  24,  24,  24,

30,  30,  30,  30,

24,  24,  24,  24,

30,  30,  30,  30,

},

},

// 3 - DST x DST (DequantPrec = 6+6)

{

{

17,  17,  17,  17,

17,  17,  17,  17,

17,  17,  17,  17,

17,  17,  17,  17,

},

{

19,  19,  19,  19,

19,  19,  19,  19,

19,  19,  19,  19,

19,  19,  19,  19,

},

{

22,  22,  22,  22,

22,  22,  22,  22,

22,  22,  22,  22,

22,  22,  22,  22,

},

{

24,  24,  24,  24,

24,  24,  24,  24,

24,  24,  24,  24,

24,  24,  24,  24,

},

{

28,  28,  28,  28,

28,  28,  28,  28,

28,  28,  28,  28,

28,  28,  28,  28,

},

{

31,  31,  31,  31,

31,  31,  31,  31,

31,  31,  31,  31,

31,  31,  31,  31,

},

},

};

# Patent rights declaration(s)

Institute for Infocomm Research may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

For avoidance of doubt, positive BD-Rate implies that the proposed method uses more rate, while a negative BD-Rate implies that the proposed method improves coding performance.