| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D049 WG11 Number: m18796 |
| --- | --- |

| Title: | Mode-Dependent Coefficient Scanning for Intra Prediction Residual Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chuohao Yeo Yih Han Tan Zhengguo Li 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 chyeo@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

In this proposal, a method for the scanning of intra prediction residual coefficients is presented to improve rate-distortion performance of intra coding. While prior work in mode-dependent coefficient scanning has shown similar gains, they greatly increase the decoding complexity, since the scans are adaptive and residual coefficients statistics have to be update as each block is decoded. Furthermore, due to the arbitrary scan orders that are used, parallelization of the coding process may be difficult. The method in this contribution attempts to overcome these difficulties by using a simplified set of scans that lend themselves well to parallelization, and requires no statistics updating. Experimental results show that the proposed method has similar compression performance to adaptive scans while requiring much less decoding complexity.

- Introduction

In HM1, two entropy coders can be used [1]. One is an arithmetic coding based Context-based Adaptive Binary Arithmetic Coder (CABAC) [2][3], while the other is a variable length coding (VLC) based Low-Complexity Entropy Coder (LCEC) [4]. This contribution is targeted at CABAC and the transform coefficient coding presented in [3], which has been adopted in HM1, but can also be adapted for LCEC.

Within CABAC, entropy coding of transform coefficients takes place in two stages. In the first stage, the significance map, which signals where non-zero coefficients within block are located, is coded. In the second stage, the values of the non-zero transform coefficients are coded. Figure 1 below illustrates this.

Figure 1. Illustration of entropy coding of transform coefficients in CABAC

Coding of the significance map proceeds by going over each coefficient, and signaling whether it is significant or not. If it is signaled to be significant, then a second flag is coded to signal if it is the last significant coefficient. If it is, then coding of the significance map stops, since the rest of the coefficients is implied to be zero. Therefore, it is beneficial to scan from the coefficient most likely to be non-zero to the coefficient least likely to be non-zero, since this would avoid coding unnecessary “zero coefficient” flags.

The mechanism for doing so in CABAC is the following [3]. Scanning proceeds diagonal by diagonal, from the top-left to the bottom-right, as shown in the figure below. Within each diagonal, the scan can proceed towards the bottom-left (“down-left”), or towards the top-right (“up-right”). The actual choice of direction is adaptive: after coding each diagonal, the number of significant coefficients already coded in the upper-right half and the number of significant coefficients already coded in the lower-left half is compared. If the former is larger, then the scan direction for the next diagonal is down-left, and if the latter is larger, then the scan direction for the next diagonal is up-right. If they are the same, then the previous scan direction is retained.

| - Scan order for significance map | - Down-Left scan | - Up-Right scan |
| --- | --- | --- |

Figure 2. Scanning used for coding significance map. (a) Scanning proceeds diagonal by diagonal from the top-left (1) to the bottom-right (7); (b) A “Down-Left” scan; (c) An “Up-Right” scan.

There are two main issues with this approach. First, two counters are needed to keep track of the number of significant coefficients in the upper-right half and the lower-left half, and at the end of coding each diagonal, a decision needs to be made as to which scan direction is used next. This increases decoding complexity. Second, due to the context modelling used for coding the significance flag for each coefficient, there are some difficulties in parallelizing coding of the scans [5].

# Related Work

To improve coding efficiency, Ye and Karczewicz have proposed using mode-dependent adaptive scan orders [6]. The method has two main parts. First, the scan order used to code the significance map depends on the intra prediction mode that has been signalled. In other words, instead of zig-zag scans or the scan described above, an arbitrary and different scan is adopted for each prediction mode. Second, the scan order is adaptive; during encoding and decoding, the frequency of non-zero coefficients at each block location is tracked, and is used to update the scan order after encoding/decoding each block. Figure 3 gives an example of an arbitrary scan order and its corresponding frequency statistics.

Figure 3. Illustration of adaptive scan order.

Because this method aims to scan coefficients from largest to smallest based on collected statistics, it is able to improve coding performance, as many zero coefficients can avoid being signaled when coding the significance map.

However, this method has three main issues. First, collecting the frequency statistics and updating the scan order on a per-block basis can drastically increase decoding complexity, since sorting of the frequency to derive the scan order needs to be done. Second, the resulting arbitrary scan order makes it difficult to parallelize the coding operations. Third, a large amount of memory is needed to store the initial scan statistics, as well as the derived scan order, especially for large block sizes.

On a slightly different note, to allow better parallelization, Sze and Budagavi have proposed using diagonal scans in the same direction to enable wave-front scanning [5]. This also avoids the need to maintain counters as in [3] to switch between scan directions. Their experimental results indicate that doing so does not significantly impact coding performance.

# Proposed Approach

In this contribution, the use of fixed mode-dependent scan orders for intra prediction residual coding is proposed. The set of scans to be considered consists of “up-right” scan, “down-left” scan, “vertical” scan and “horizontal” scan, as illustrated in Figure 4 below. This will be referred to this as Mode-Dependent Simplified Scans (MDSS).

Figure 4. Scans used in the proposed method.

The scans to be used depend on both the intra prediction mode and the transform block size. The table below summarizes the choice of scans. In the table, the prediction modes shown are: “DC” – DC prediction, “VER+x” – vertical prediction with offset x, and “HOR+x” – horizontal prediction with offset x. This is illustrated in Figure 5. The set of choices indicated in the table is one possible implementation, but other combinations are also possible.

Table 1. Choice of scan orders

| Intra Prediction Mode | N = 4 | N = 8 | N = 16 | N = 32 |
| --- | --- | --- | --- | --- |
| DC | DL | DL | DL | DL |
| VER-8 | UR | UR | UR | UR |
| VER-7 to VER-5 | DL | DL | DL | DL |
| VER-4 to VER+4 | H | H | DL | DL |
| VER+5 to VER+8 | DL | DL | DL | DL |
| HOR-7 to HOR-5 | UR | UR | UR | UR |
| HOR-4 to HOR+4 | V | V | UR | UR |
| HOR+5 to HOR+8 | UR | UR | UR | UR |

Source Pixels

Reference    Pixels

VER+8

VER-8

VER

HOR+8

HOR-7

HOR

Figure 5. Illustration of intra prediction modes

As in [6], the scan order to be used would depend on the intra prediction mode that is used, but unlike [6], there is no updating of the scans, so no statistics collection or re-sorting is necessary. Similarly, no counters need to be kept to decide on the direction of each diagonal scan, unlike in [3].  Furthermore, a small set of scans are used, all of which are easy to implement directly, so there is no need to store large tables indicating the positions of the scan orders or the coefficient statistics needed to derive the scan order. Therefore, the proposed method adds very little additional complexity.

In addition, because the scans are all in the same directions, it is possible to do a wave-front scan, as suggested in [5], which allows for better parallelization.

If LCEC is used, these set of scans can be used to determine how the coefficients are scanned (in reverse order) when they are coded.

# Simulation Results

The implementation is on TMuC 0.9. Tests were conducted using the two intra configurations, i.e., high-efficiency and low-complexity, and the two random access configurations from the common coding configurations [7].

We compare our proposed scheme to the HM1 anchor. For comparison purposes, we also provide results using Qualcomm’s adaptive scan order for intra coding. This was done using TMuC 0.9 code, with QC_MDDT set to 1, but with the use of KLT disabled.

## RD results

Table 2 shows the BD-Rate [8] results for our proposed scheme, while Table 3 shows the BD-Rate results for the Qualcomm’s adaptive scan order [6], for various coding configurations.

Table 2. Results using proposed Mode-Dependent Simplified Scans (MDSS)

|  | Intra |  |  |  |  | Intra LoCo |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -0.4 |  | -0.2 | -0.1 |  | 0.9 |  | 0.7 | 0.6 |  |
| Class B | -0.5 |  | -0.4 | -0.4 |  | 0.2 |  | 0.1 | -0.1 |  |
| Class C | -1.1 |  | -0.6 | -0.7 |  | -0.3 |  | -0.4 | -0.5 |  |
| Class D | -1.0 |  | -0.7 | -0.7 |  | -0.1 |  | -0.3 | -0.4 |  |
| Class E | -1.4 |  | -0.8 | -0.7 |  | 0.1 |  | -0.2 | -0.1 |  |
| All | -0.9 |  | -0.6 | -0.5 |  | 0.1 |  | -0.1 | -0.2 |  |
| Enc Time[%] | 106% |  |  |  |  | 101% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  | 99% |  |  |  |  |
|  | Random access |  |  |  |  | Random access LoCo |  |  |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -0.2 |  | 0.1 | 0.4 |  | 0.4 |  | 0.4 | 0.3 |  |
| Class B | -0.3 |  | 0.0 | 0.1 |  | 0.1 |  | 0.2 | 0.0 |  |
| Class C | -0.4 |  | -0.2 | -0.2 |  | -0.1 |  | -0.1 | -0.1 |  |
| Class D | -0.3 |  | -0.4 | -0.1 |  | -0.1 |  | -0.3 | 0.1 |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| All | -0.3 |  | -0.1 | 0.0 |  | 0.0 |  | 0.0 | 0.1 |  |
| Enc Time[%] | 101% |  |  |  |  | 100% |  |  |  |  |
| Dec Time[%] | 101% |  |  |  |  | 100% |  |  |  |  |

Table 3. Results for using trained Qualcomm's adaptive scan order in TMuC 0.9 [6]

|  | Intra |  |  |  |  | Intra LoCo |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -0.4 |  | -0.3 | -0.3 |  | 0.2 |  | -0.3 | -0.4 |  |
| Class B | -0.5 |  | -0.5 | -0.4 |  | -0.9 |  | -1.4 | -1.6 |  |
| Class C | -1.0 |  | -0.8 | -0.8 |  | -0.8 |  | -1.1 | -1.1 |  |
| Class D | -0.9 |  | -0.7 | -0.7 |  | -0.7 |  | -1.0 | -1.1 |  |
| Class E | -1.2 |  | -0.8 | -0.7 |  | -1.0 |  | -1.2 | -1.0 |  |
| All | -0.8 |  | -0.6 | -0.6 |  | -0.7 |  | -1.1 | -1.2 |  |
| Enc Time[%] | 106% |  |  |  |  | 106% |  |  |  |  |
| Dec Time[%] | 220% |  |  |  |  | 368% |  |  |  |  |
|  | Random access |  |  |  |  | Random access LoCo |  |  |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -0.2 |  | 0.1 | 0.3 |  | 0.0 |  | 0.0 | -0.2 |  |
| Class B | -0.3 |  | -0.1 | -0.3 |  | -0.5 |  | -0.5 | -0.7 |  |
| Class C | -0.3 |  | -0.3 | -0.3 |  | -0.4 |  | -0.4 | -0.4 |  |
| Class D | -0.3 |  | -0.2 | -0.1 |  | -0.2 |  | -0.2 | -0.4 |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| All | -0.3 |  | -0.1 | -0.2 |  | -0.3 |  | -0.3 | -0.5 |  |
| Enc Time[%] | 101% |  |  |  |  | 101% |  |  |  |  |
| Dec Time[%] | 320% |  |  |  |  | 557% |  |  |  |  |

The results show that for the high-efficiency cases that use CABAC, the proposed MDSS performs just as well as Qualcomm’s adaptive scan order, but without the increased decoding complexity.

For the low-complexity cases that use LCEC, the proposed MDSS is not as effective. We are still investigating how to further improve this.

Furthermore, we provide experimental results when MDDT is used with our DCT/ODST-3 combination [9] for the transforms, and with MDSS for the coefficient scanning, and compare that to Qualcomm’s MDDT [6] implementation in TMuC 0.9. These results are for the intra high-efficiency and random access high-efficiency coding configurations.

Table 3. Results comparing DCT/ODST-3 + MDSS and Qualcomm's MDDT

|  | I2R DCT/ODST-3 + MDSS |  |  | QC MDDT |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Intra |  |  | Intra |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.5 | -3.0 | -2.9 | -2.8 | -3.0 | -2.9 |
| Class B | -1.3 | -1.7 | -1.7 | -1.5 | -1.8 | -1.8 |
| Class C | -2.0 | -1.8 | -1.8 | -2.1 | -2.0 | -2.0 |
| Class D | -2.1 | -1.8 | -1.8 | -2.1 | -2.0 | -1.9 |
| Class E | -2.7 | -2.9 | -2.6 | -2.7 | -2.7 | -2.5 |
| All | -2.0 | -2.1 | -2.0 | -2.1 | -2.2 | -2.1 |
| Enc Time[%] | 106% |  |  | 106% |  |  |
| Dec Time[%] | 102% |  |  | 223% |  |  |
|  | Random access |  |  | Random access |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.1 | -0.5 | -0.3 | -1.2 | -0.6 | -0.2 |
| Class B | -0.7 | -0.4 | -0.3 | -0.8 | -0.5 | -0.4 |
| Class C | -1.0 | -0.7 | -0.5 | -1.0 | -0.6 | -0.7 |
| Class D | -0.9 | -0.6 | -0.3 | -0.9 | -0.5 | -0.5 |
| Class E |  |  |  |  |  |  |
| All | -0.9 | -0.6 | -0.4 | -0.9 | -0.5 | -0.5 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 322% |  |  |

These results show our combination of DCT/ODST-3 and simplified coefficient scans matches the performance of MDDT but with no significant increase in decoding complexity compared to the HM1 anchors.

More detailed coding results can be found in the Excel attachments.

# Conclusion

This contribution presents experimental results for our proposal on providing a low-complexity mode dependent scans for intra coding. The proposed technique uses a set of coefficient scans consisting of diagonal, horizontal and vertical scans. The scan to be used depends on the intra prediction mode and the block size. Experimental results show that in the high-efficiency cases where CABAC is used, MDSS provides the same coding gains as Qualcomm’s adaptive scan order but without the additional decoding complexity. We recommend adopting this proposal into HM for coefficient coding when CABAC is used, and to further study it for coefficient coding when LCEC is used.

# References

[1] T. K. Tan, G. J. Sullivan, and J.-R. Ohm, “Summary of HEVC working draft 1 and HEVC test model (HM),” in JCTVC-C405, Guangzhou, China, Oct. 2010.

[2] D. Marpe, H. Schwarz, and T. Wiegand, “Context-Based Adaptive Binary Arithmetic Coding in the H.264/AVC Video Compression Standard,” in IEEE CSVT, Vol. 13, No. 7, pp. 620-636, Jul. 2003.

[3] M. Winken et al., “Description of video coding technology proposal by Fraunhofer HHI,” in JCTVC-A116, Dresden, Germany, Apr. 2010.

[4] K. Ugur, K. R. Andersson, and A. Fuldseth, “Description of video coding technology proposal by Tandberg, Nokia, Ericsson,” in JCTVC-A119, Dresden, Germany, Apr. 2010.

[5] V. Sze, and M. Budagavi, “Parallelization of HHI_TRANSFORM_CODING,” in JCTVC-C227, Guangzhou, China, Oct. 2010.

[6] Y. Ye and M. Karczewicz, “Improved H.264 intra coding based on bi-directional intra prediction, directional transform, and adaptive coefficient scanning,” in IEEE ICIP, 2008.

[7] F. Bossen, “Common test conditions and software reference configurations,” in JCTVC-C500, Guangzhou, China, Oct. 2010.

[8] G. Bjøntegaard, “Calculation of Average PSNR Differences between RD curves,” in ITU-T SC16/Q6, VCEG-M33, Austin, USA, Jan. 2004.

[9] C. Yeo, Y. H. Tan, Z. Li and S. Rahardja, “CE7: Mode-Dependent Transforms for Block-based Intra Coding from Institute for Infocomm Research,” in JCTVC-D046, Daegu, Korea, Jan. 2010.

# Patent rights declaration(s)

Institute for Infocomm Research may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

For avoidance of doubt, positive BD-Rate implies that the proposed method uses more rate, while a negative BD-Rate implies that the proposed method improves coding performance.