| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-C052 WG11 Number: m18799 |
| --- | --- |

| Title: | Updated evaluation result of proposed syntax and semantics for very low delay coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui, Junpei Koyama, and Akira Nakagawa 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki JAPAN | Tel: Email: | +81-44-754-2639 kazui.kimihiko@jp.fujitsu.com koyama.junpei@jp.fujitsu.com anaka@jp.fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

# Abstract

This contribution reports the additional test result of our proposed scheme for very low delay coding of less than 1 frame period [1][2] compared with the another possible scheme based on AVC.

The software used for this test is based on TMuC 0.9.

The coding gain is 6.0% (64x64 LCU) and 6.3% (32x32 LCU).

- Introduction

Low delay coding of less than 1 frame period is highly required in real-time bi-directional video transmission application such as TV conference. We think the HEVC standard shall meet this requirement.

We proposed the scheme which minimizes the loss of coding efficiency in very low delay coding of less than one frame period at the 2nd JCT-VC meeting [1]. We showed the coding gain of the proposed scheme using TMuC0.7 software at the 3rd JCT-VC meeting [2]. The coding gain of our proposed scheme compared with the conventional slice insertion scheme is 9.0% (64x64 CU size) and 15.9% (32x32 CU size) in BD-Bitrate.

This contribution reports the additional test result of our proposed scheme based on TMuC 0.9 compared with the another possible scheme based on AVC.

- Test condition

## Tested schemes

The evaluated schemes are summarized in Table 1. The sizes of a LCU are 64x64 (which is according to the "low delay, high efficiency" setting of the common test condition described in JCTVC-B300 [2]) and 32x32.

Table 1  Tested schemes

|  | Scheme | Detail |
| --- | --- | --- |
| 1 | "Proposed" | The proposed scheme as described in JCTVC-B031. The area boundary whose position is described in a slice header circulates from the left to the right of a picture. See Figure 1 for detail. In this implementation, the unit of the position of the region boundary is the width of a LCU. Other parameters are identical to those in the "low delay, high efficiency" setting. |
| 2 | "No filter" (For comparison) | The "No filter" scheme is the another possible scheme for realizing very low delay coding based on AVC. The conventional scheme compared in [2] inserts multiple slices in one picture in order to prevent the contamination of a refreshed MB from non-refreshed MBs through the deblocking filter process and the adaptive in-loop filter process. The “No filter” scheme turns off deblocking filter and adaptive loop filter in all MBs instead of inserting multiple slices. (See Figure 2) The same restriction on MV search range as applied to the conventional scheme compared in [2] is also applied. A motion vector from a refreshed MB to the area that contains a non-refreshed pixel in a reference picture is prohibited. In addition, the directions of intra prediction for a refreshed MB is restricted so that a non-refreshed pixel in the same picture is not used in the intra prediction process. |

Figure 1 The position of the region boundary in the proposed scheme

Figure 2  The “No filter” scheme which turns deblocking filter and adaptive loop filter off.
The position of the region boundary in each picture coincides with that for the test of the proposed scheme.

## Test sequence

All sequences in Class B, Class C and Class D described in the common test condition are used for the evaluation. The length of each sequence is set equal to (<a multiple of refresh period> + 1). The latter number (i.e. “1”) corresponds to the first IDR picture.

Table 2  Tested sequences

| Class | Sequence (frame rate) | Length | Refresh period |
| --- | --- | --- | --- |
| B | Kimono (24) ParkScene (24) Cactus (50) BQTerrace (60) BasketBallDrive (50) | 181 (24p) 481 (50p/60p) | 60 (for 32x32 LCTB) 30 (for 64x64 LCTB) |
| C | RaceHorsesC (30) BQMall (60) PartyScene (50) BasketballDrill (50) | 209 | 26 (for 32x32 LCTB) 13 (for 64x64 LCTB) |
| D | BasketballPass (50) BQSquare (60) BlowingBubbles (50) RaceHorses (30) | 497 (50p/60p) 295 (30p) | 13 (for 32x32 LCTB) 7 (for 64x64 LCTB) |

# Result

The coding gain of the proposed scheme from the "No filter" scheme is summarized in this clause. Please refer to the attached Excel file for detail.

Table 3 and Table 4 show the coding gains of the proposed scheme from the “No filter” scheme when the sizes of a LCU are 64x64 and 32x32, respectively.

Table 3  Coding gain of the proposed scheme from the “No filter” scheme (LCU size is 64x64)

| Class | Sequence | VS. "No filter" |  |
| --- | --- | --- | --- |
|  |  | BD-PSNR | BD-Bitrate |
| B | Kimono | 0.505 | -13.983 |
|  | ParkScene | 0.154 | -4.504 |
|  | Cactus | 0.197 | -7.464 |
|  | BQTerrace | 0.196 | -8.555 |
|  | BasketballDrive | 0.228 | -9.040 |
|  | Average | 0.256 | -8.709 |
| C | BasketballDrill | 0.245 | -5.595 |
|  | BQMall | 0.239 | -5.270 |
|  | PartyScene | 0.137 | -2.638 |
|  | RaceHorsesC | 0.279 | -6.241 |
|  | Average | 0.225 | -4.936 |
| D | BasketballPass | 0.269 | -5.038 |
|  | BQSquare | 0.123 | -2.349 |
|  | BlowingBubbles | 0.080 | -1.665 |
|  | RaceHorses | 0.270 | -5.270 |
|  | Average | 0.185 | -3.580 |
| Average |  | 0.225 | -5.970 |

Table 4  The coding gain of the proposed scheme from the “No filter” scheme (LCU size is 32x32)

| Class | Sequence | VS. "No filter" |  |
| --- | --- | --- | --- |
|  |  | BD-PSNR | BD-Bitrate |
| B | Kimono | 0.502 | -13.835 |
|  | ParkScene | 0.159 | -4.655 |
|  | Cactus | 0.177 | -6.781 |
|  | BQTerrace | 0.214 | -9.465 |
|  | BasketballDrive | 0.228 | -8.989 |
|  | Average | 0.256 | -8.745 |
| C | BasketballDrill | 0.274 | -6.283 |
|  | BQMall | 0.258 | -5.793 |
|  | PartyScene | 0.167 | -3.268 |
|  | RaceHorses | 0.288 | -6.549 |
|  | Average | 0.247 | -5.473 |
| D | BasketballPass | 0.283 | -5.368 |
|  | BQSquare | 0.174 | -3.627 |
|  | BlowingBubbles | 0.105 | -2.249 |
|  | RaceHorses | 0.283 | -5.590 |
|  | Average | 0.211 | -4.209 |
| Average |  | 0.239 | -6.342 |

# Conclusion and recommendation

We tested the coding gain of our proposed scheme which minimizes the loss of coding efficiency in very low delay coding compared with the another conventional scheme that disables deblocking filter and adaptive loop filter in all MBs in a picture.

The coding gain is 6.0% (64x64 LCU) and 6.3% (32x32 LCU).

We request to start the discussion of the support of very low delay coding in HEVC.

References

[1] 2nd JCT-VC Meeting, “Proposal of requirement on very low delay coding”, JCTVC-B031, July 2010, Geneva CH

[2] 2nd JCT-VC Meeting, “Common test conditions and software reference configurations”, JCTVC-B300, July 2010, Geneva CH

# Patent rights declaration(s)

FUJITSU LIMITED may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).