| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D053 WG11 Number: m18800 |
| --- | --- |

| Title: | Draft description of proposed syntax and semantics for very low delay coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui, Junpei Koyama, and Akira Nakagawa 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki JAPAN | Tel: Email: | +81-44-754-2639 kazui.kimihiko@jp.fujitsu.com koyama.junpei@jp.fujitsu.com anaka@jp.fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

# Abstract

We proposed the scheme which minimizes the loss of coding efficiency in very low delay coding of less than one frame period at the 2nd JCT-VC meeting.

The detail of the proposed high-level syntax and semantics is described in this contribution. The detail of the new buffer model proposed at the 3rd meeting is also described.

- Introduction

Low delay coding of less than 1 frame period is highly required in real-time bi-directional video transmission application such as TV conference. We think the HEVC standard shall meet this requirement.

We proposed the scheme which minimizes the loss of coding efficiency in very low delay coding of less than one frame period at the 2nd JCT-VC meeting [1]. The proposed scheme contains new high-level syntax and semantics.

The detail of the proposed high-level syntax and semantics is described in this contribution. The detail of the new buffer model proposed at the 3rd meeting [2] is also described.

The high-level syntax and semantics described in this contribution is based on the description in JCTVC-B205_draft007 [3]. Since JCTVC-B205_draft007 is not stable yet, only the summary of the proposed semantics for deblocking filter and adaptive in-loop filter is shown.

# Proposed syntax

The proposed new syntax is highlighted with yellow color.

## Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | C |  | Descriptor |  |
| --- | --- | --- | --- | --- |
| profile_idc | 0 |  | u(8) |  |
| reserved_zero_8bits /* equal to 0 */ | 0 |  | u(8) |  |
| level_idc | 0 |  | u(8) |  |
| seq_parameter_set_id | 0 |  | ue(v) |  |
| bit_depth_luma_minus8 | 0 |  | ue(v) |  |
| bit_depth_chroma_minus8 | 0 |  | ue(v) |  |
| increased_bit_depth_luma | 0 |  | ue(v) |  |
| ine_bit_depth_chroma | 0 |  | ue(v) |  |
| log2_max_frame_num_minus4 | 0 |  | ue(v) |  |
| pic_order_cnt_type | 0 |  | ue(v) |  |
| if( pic_order_cnt_type = = 0 ) |  |  |  |  |
| log2_max_pic_order_cnt_lsb_minus4 | 0 |  | ue(v) |  |
| else if( pic_order_cnt_type = = 1 ) { |  |  |  |  |
| delta_pic_order_always_zero_flag | 0 |  | u(1) |  |
| offset_for_non_ref_pic | 0 |  | se(v) |  |
| num_ref_frames_in_pic_order_cnt_cycle | 0 |  | ue(v) |  |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |  |  |  |
| offset_for_ref_frame[ i ] | 0 |  | se(v) |  |
| } |  |  |  |  |
| max_num_ref_frames | 0 |  | ue(v) |  |
| gaps_in_frame_num_value_allowed_flag | 0 |  | u(1) |  |
| max_coding_unit_width |  | 0 |  | ue(v) |
| max_coding_unit_height |  | 0 |  | ue(v) |
| max_coding_unit_hierarchy_depth_correct |  | 0 |  | u(1) |
| log2_min_transform_unit_size_minus2 |  | 0 |  | ue(v) |
| if( log2_min_transform_unit_size_minus2 < 4 ) |  |  |  |  |
| max_transform_unit_hierarchy_depth | 0 |  | ue(v) |  |
| pic_height_in_luma_samples | 0 |  | u(16) |  |
| numExtraFilters | 0 |  | ue(v) |  |
| for(i=0; i< numExtraFilters; i++){ |  |  |  |  |
| log2_filterCoeffPrecision | 0 |  | ue(v) |  |
| halfNumTap | 0 |  | ue(v) |  |
| for(j=0;j<(3*halfNumTap); j++){ |  |  |  |  |
| filterCoef[i][j] | 0 |  | i(v) |  |
| } |  |  |  |  |
| } |  |  |  |  |
| adaptive_intra_smoothing_flag | 0 |  | u(1) |  |
| inter_merge_flag | 0 |  | u(1) |  |
| interleaved_motion_vector_predictor_flag | 0 |  | u(1) |  |
| pbic_flag | 0 |  | u(1) |  |
| region_based_refresh_flag | 0 |  | u(1) |  |
| rbsp_trailing_bits( ) | 0 |  |  |  |
| } |  |  |  |  |

## Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| pic_parameter_set_id | 1 | ue(v) |
| seq_parameter_set_id | 1 | ue(v) |
| entropy_coding_mode_flag | 1 | u(1) |
| num_ref_idx_l0_default_active_minus1 | 1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | 1 | ue(v) |
| pic_init_qp_minus26 /* relative to 26 */ | 1 | se(v) |
| constrained_intra_pred_flag | 1 | u(1) |
| for(i=0;i<15; i++){ |  |  |
| numAllowedFilters[i] | 1 | ue(v) |
| for(j=0;j<numAllowedFilters;j++){ |  |  |
| filtIdx[i][j] | 1 | ue(v) |
| } |  |  |
| } |  |  |
| if(region_based_refresh_flag){ |  |  |
| refresh_direction | 1 | ue(v) |
| boundary_position_precision | 1 | ue(v) |
| } |  |  |
| rbsp_trailing_bits( ) | 1 |  |
| } |  |  |

## Slice header syntax

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| first_lctb_in_slice | 2 | ue(v) |
| slice_type | 2 | ue(v) |
| pic_parameter_set_id | 2 | ue(v) |
| frame_num | 2 | u(v) |
| if( IdrPicFlag ) |  |  |
| idr_pic_id | 2 | ue(v) |
| pic_order_cnt_lsb | 2 | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |  |
| num_ref_idx_active_override_flag | 2 | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |  |
| num_ref_idx_l0_active_minus1 | 2 | ue(v) |
| if( slice_type = = B ) |  |  |
| num_ref_idx_l1_active_minus1 | 2 | ue(v) |
| } |  |  |
| } |  |  |
| ref_pic_list_modification( ) |  |  |
| if( nal_ref_idc != 0 ) |  |  |
| dec_ref_pic_marking( ) | 2 |  |
| if( entropy_coding_mode_flag ) { |  |  |
| pipe_multi_codeword_flag | 2 | u(1) |
| if( !pipe_multi_codeword_flag ) |  |  |
| pipe_max_delay_shift_6 | 2 | ue(v) |
| else |  |  |
| balanced_cpus | 2 | u(8) |
| if( slice_type != I ) |  |  |
| cabac_init_idc | 2 | ue(v) |
| } |  |  |
| slice_qp_delta | 2 | se(v) |
| alf_param() |  |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |  |
| mc_interpolation_idc | 2 | ue(v) |
| mv_competition_flag | 2 | u(1) |
| if ( mv_competition_flag ) { |  |  |
| mv_competition_temporal_flag | 2 | u(1) |
| } |  |  |
| } |  |  |
| if ( slice_type = = B && mv_competition_flag) |  |  |
| collocated_from_l0_flag | 2 | u(1) |
| sifo_param() |  |  |
| edge_based_prediction_flag | 2 | u(1) |
| if( edge_prediction_ipd_flag = = 1 ) |  |  |
| threshold_edge | 2 | u(8) |
| if(region_based_refresh_flag){ |  |  |
| boundary_position | 2 | ue(v) |
| } |  |  |
| } |  |  |

## SEI payload syntax

| sei_payload( payloadType, payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| if( payloadType = = 0 ) |  |  |
| buffering_period( payloadSize ) | 5 |  |
| ... |  |  |
| else if( payloadType = = 6 ) |  |  |
| recovery_point( payloadSize ) | 5 |  |
| .... |  |  |

## Buffering period SEI message syntax

| buffering_period( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| seq_parameter_set_id | 5 | ue(v) |
| if( NalHrdBpPresentFlag ) { |  |  |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |  |
| initial_cpb_removal_delay[ SchedSelIdx ] | 5 | u(v) |
| initial_cpb_removal_delay_offset[ SchedSelIdx ] | 5 | u(v) |
| } |  |  |
| } |  |  |
| if( VclHrdBpPresentFlag ) { |  |  |
| for( SchedSelIdx = 0; SchedSelIdx <= cpb_cnt_minus1; SchedSelIdx++ ) { |  |  |
| initial_cpb_removal_delay[ SchedSelIdx ] | 5 | u(v) |
| initial_cpb_removal_delay_offset[ SchedSelIdx ] | 5 | u(v) |
| } |  |  |
| } |  |  |
| if( NalHrdBpPresentFlag ¦¦ VclHrdBpPresentFlag) { |  |  |
| num_groups_in_picture_minus1 | 5 | ue(v) |
| } |  |  |
| } |  |  |

## Recovery point SEI message syntax

| recovery_point( payloadSize ) { | C | Descriptor |
| --- | --- | --- |
| recovery_frame_cnt | 5 | ue(v) |
| exact_match_flag | 5 | u(1) |
| broken_link_flag | 5 | u(1) |
| changing_slice_group_idc | 5 | u(2) |
| if(region_based_refresh_flag){ |  |  |
| region_index | 5 | ue(v) |
| } |  |  |
| } |  |  |

# Proposed semantics

## Sequence parameter set RBSP semantics

region_based_refresh_flag equal to 0 specifies that there is no syntax element relating to the region-based refresh mode in the picture_parameter_set_rbsp() syntax structure and in the slice_header() syntax structure. region_based _refresh_flag equal to 1 specifies that there are syntax elements relating to the region-based refresh mode in the picture_parameter_set_rbsp() syntax structure and in the slice_header() syntax structure.

When region_based _refresh_flag is equal to 1, the region-based refresh mode is enabled and the following decoding processes are changed.

- Derivation process for neighbouring largest coding units (excluding parsing process)

- Derivation process for neighbouring coding units (excluding parsing process)

- In-loop filter process

- Deblocking filter process

- Interpolation methods

## Picture parameter set RBSP semantics

region_direction specifies the direction of the movement of the region boundary when the region-based refresh mode is enabled.

Table X-X – Meaning of region_direction

| - region_direction | - Meaning |
| --- | --- |
| - 0 | - Top to Bottom |
| - 1 | - Bottom to Top |
| - 2 | - Left to Right |
| - 3 | - Right to Left |
| - 4 | - Raster Scan |
| - Other | - Reserved |

boundary_position_precision specifies the unit of the position of the region boundary when the region-based refresh mode is enabled. When region_direction is not equal to 4, boundary_position_precision shall be the multiple of SuSize.

## Slice header semantics

boundary_position specifies the position of the region boundary (in unit of boundary_position_precision ) of the picture which contains this slice_header() syntax structure when the region-based refresh mode is enabled.

## Buffering period SEI message semantics

num_groups_in_picture_minus1 plus 1 specifies the number of the group in a picture in the buffer model of the hypothetical reference decoder. num_groups_in_picture_minus1 shall satisfies the following equation.

(PicWidthInLCUs * PicHeightInLCUs) %  (num_groups_in_picture_minus1 + 1) = 0

## Recovery point SEI message semantics

region_index specifies the region index of the refreshed area RegionIndexA. The region index of the non-refreshed area RegionIndexB is equal to ( region_index - 1).

region_index associated with a recovery point is applied to all pictures between the recovery point and the picture just before the next recovery point in the decoding order.

region_index is set 1 at an IDR picture. region_index shall be increased 1 when a Recovery point SEI appears in the bitstream.

## Region-related semantics

The process specified in this subclause is applied only when the region-based refresh mode is enabled

### Deviation process of refreshed area and non-refreshed area

Each picture contains a refreshed area and a non-refreshed area. The refreshed area and the non-refreshed area cover whole area of the picture and are not overlapped each other.

The refreshed area and the non-refreshed area are specified according to region_direction, boundary_position_precision, and boundary_position as follows.

Variable BoundaryPosition is calculated as follows.

BoundaryPosition = boundary_position * boundary_position_precision

When region_direction is equal to 0, the refreshed area covers the area that contains luminance samples at (x, y) | {x = [0, BoundaryPosition - 1], y = [0, PicHeightInSamplesL - 1]} and the non-refreshed area covers the area that contains luminance samples at (x, y) | {x = [BoundaryPosition, PicWidthInSamplesL - 1], y = [0, PicHeightInSamplesL - 1]}.

Fig. X-X Definition of refreshed area and non-refreshed area when region_direction is equal to 0

When region_direction is equal to 1, the refreshed area covers the area that contains luminance samples at (x, y) | { x = [BoundaryPosition, PicWidthInSamplesL - 1], y = [0, PicHeightInSamplesL - 1]} and the non-refreshed area covers the area that contains luminance samples at (x, y) | { x = [0, BoundaryPosition - 1], y = [0, PicHeightInSamplesL - 1]}.

Fig. X-X Definition of refreshed area and non-refreshed area when region_direction is equal to 1

When region_direction is equal to 2, the refreshed area covers the area that contains luminance samples at (x, y) | { x = [0, PicWidthInSamplesL - 1], y = [0, BoundaryPosition - 1]} and the non-refreshed area covers the area that contains luminance samples at (x, y) | { x = [0, PicWidthInSamplesL - 1], y = [BoundaryPosition, PicHeightInSamplesL - 1]}.

Fig. X-X Definition of refreshed area and non-refreshed area when region_direction is equal to 2

When region_direction is equal to 3, the refreshed area covers the area that contains luminance samples at (x, y) | { x = [0, PicWidthInSamplesL - 1], y = [BoundaryPosition - 1, PicHeightInSamplesL - 1]} and the non-refreshed area covers the area that contains luminance samples at (x, y) | { x = [0, PicWidthInSamplesL - 1], y = [0, BoundaryPosition - 1]}.

Fig. X-X Definition of refreshed area and non-refreshed area when region_direction is equal to 3

When region_direction is equal to 4, the refreshed area and the non-refreshed area are defined depending on BoundaryPosition.

First, variable TmpX is calculated as follows.

TmpX = (BoundaryPosition % PicWidthInLCUs ) * LcuSize

When TmpX is equal to 0, the refreshed area covers the area that contains luminance samples at (x, y) | { x = [0, PicWidthInSamplesL - 1], y = [0, TmpY1 - 1]} and the non-refreshed area covers the area that contains luminance samples at (x, y) | { x = [0, PicWidthInSamplesL - 1], y = [TmpY1, PicHeightInSamplesL - 1]}, where variable TmpY1 is equal to {((BoundaryPosition  /  PicWidthInLCUs) + 1) * LcuSize}.

Otherwise (i.e. TmpX is not equal to 0), the refreshed area covers the area that contains luminance samples at (x, y) | { x = [0, PicWidthInSamplesL - 1], y = [0, TmpY - 1]} and at (x, y) | { (x, y) | x = [0, TmpX - 1], y = [TmpY, TmpY2 - 1] }, and the non-refreshed area covers the area that contains luminance samples at (x, y) | { [0, PicWidthInSamplesL - 1], y = [TmpY2, PicHeightInSamplesL - 1] } and at (x, y) | { x = [TmpX, PicWidthInSamplesL - 1], y = [TmpY, TmpY2 - 1] }, where variable TmpY1 is equal to {((BoundaryPosition  /  PicWidthInLCUs) + 1) * LcuSize} and variable TmpY2 is equal to { min((TmpY1 + LcuSize), PicHeightInSamplesL) }.

Fig. X-X Definition of refreshed area and non-refreshed area when region_direction is equal to 4 and (TmpX == 0)

Fig. X-X Definition of refreshed area and non-refreshed area when region_direction is equal to 4 and (TmpX != 0)

### Deviation process of region index of coding unit and prediction unit

Let (X0, Y0) be the location of the upper-left luminance sample of a CU (or a PU).

A CU (or a PU) belongs to a refreshed area when (X0, Y0) is inside the refreshed area of the current picture. A CU (or a PU) belongs to a non-refreshed area when (X0, Y0) is inside the non-refreshed area of the current picture. The region index of the CU (or the PU or a location (X, Y) ) is equal to the region index of its belonging area.

Function getRegionIdxForPixel( p, x, y ) is used in the following explanation. getRegionIdxForPixel( p, x, y ) returns the region index of a location (X, Y) in picture p.

The following functions are also defined.

getRegionIdxForLCU(p, lcuAddr) = getRegionIdxForPixel(p, lcuX0, lcuY0)

getRegionIdxForCU(p, lcuAddr, cuIdx) = getRegionIdxForPixel(p, cuX0, cuY0)

getRegionIdxForPU(p, lcuAddr, puIdx) = getRegionIdxForPixel(p, puX0, puY0)

where,

(lcuX0, lcuY0): the location of the upper-left luminance sample of a LCU lcuAddr.

(cuX0, cuY0): the location of the upper-left luminance sample of a CU cuAddr.

(puX0, puY0): the location of the upper-left luminance sample of a PU puAddr.

Fig. X-X Illustrated example of CUs/PUs and their belonging areas

## Derivation process of the availability for coding units

Modified parts are highlighted with yellow color.

Inputs to this process are:

- a current coding unit index CurrCuIdx and the corresponding largest coding unit address CurrLcuAddr

- a referring coding unit index cuIdx and corresponding largest coding unit address lcuAddr

- a current picture p

Output of this process is the availability of the referring coding unit.

The referring coding unit is marked as available, unless any of the following conditions is true in which case the referring coding unit marked as not available:

- lcuAddr < 0

- lcuAddr > CurrLcuAddr

- the LCU address lcuAddr belongs to a different slice than the current slice

- (lcuAddr  !=  CurrLcuAddr ) && (ConvertRasterToZscan( cuIdx ) >  ConvertRasterToZscan( CurrCuIdx ))

- ((region_based_refresh_flag == 1) && (! EntropyDecodingProcess) && (getRegionIdxForCU(p, lcuAddr, cuIdx) < getRegionIdxForCU(p, CurrLcuAddr, CurrCuIdx)))

Variable EntropyDecodingProcess is equal to 1 when this deviation process is invoked for the entropy decoding process. Otherwise, EntropyDecodingProcess is equal to 0.

The prediction unit and the partition inside a coding unit index cuIdx also have same availability of the coding unit index cuIdx.

## Luma sample interpolation process

The interpolation process defined in subclause 6.3.2.2.1 is modified as follows.

A CU index CurrCuIdx and a LCU addres CurrLcuAddr of the current prediction unit partition are added as argments.

Subclause 6.3.2.2.1 is modified as follows.

Inputs to this process are

- a luma location in full-sample units (xIntL, yIntL),

- a luma location offset in fractional-sample units (xFracL, yFracyL), and

- the luma sample array of the selected reference picture refPicLXL

Outputs of this process is a predicted luma sample value predPartLXL[ xL, yL ].

When region_based_refresh_flag is equal to 1, the luma sample array used in the following interpolation process is defined as follows.

- When RegionIdxBRef is equal to ( getRegionIdxForCU(CurrPic, CurrCuIdx, CurrLcuAddr ) ), the luma sample array used in the following interpolation process is the luma sample array of the selected reference picture refPicLXL.

- Otherwise, the luma sample array used in the following interpolation process is p'[x, y].

p'[x, y] is obtained as follows.

When refresh_direction is equal to 0,

p'[x, y] = p[x, y]                    y < BoundaryPosition

p'[x, y] = p[x, BoundaryPosition]      Otherwise

When refresh_direction is equal to 1,

p'[x, y] = p[x, y]                    y >= BoundaryPosition

p'[x, y] = p[x, BoundaryPosition]      Otherwise

When refresh_direction is equal to 2,

p'[x, y] = p[x, y]                    x < BoundaryPosition

p'[x, y] = p[BoundaryPosition, y]      Otherwise

When refresh_direction is equal to 3,

p'[x, y] = p[x, y]                    x >= BoundaryPosition

p'[x, y] = p[BoundaryPosition, y]      Otherwise

When refresh_direction is equal to 4,

When (BoundaryPosition % PicWidthInLCUs) is equal to 0,

TmpY1 = ((BoundaryPosition  /  PicWidthInLCUs) + 1) * LcuSize

p'[x, y] = p[x, y]           y < TmpY1

p'[x, y] = p[x, TmpY1]      Otherwise

Otherwise,

TmpX = (BoundaryPosition % PicWidthInLCUs ) * LcuSize

TmpY2 = min((TmpY1 + LcuSize), PicHeightInSamplesL)

p'[x, y] = p[x, TmpY2 - 1]           (x < TmpX) && (y >= TmpY2)

p'[x, y] = p[TmpX - 1, y]            (x >= TmpX) && (y >= TmpY1) && (y < TmpY2)

p'[x, y] = p'[x, TmpY2 - 1]            (x >= TmpX) && (y >= TmpY2)

p'[x, y] = p[x, y]      Otherwise

where p[x, y] is the the luma sample array of the selected reference picture refPicLXL, and BoundaryPosition is equal to BoundaryPosition of the selected reference picture refPicLXL.

## Chroma sample interpolation process

Similar process as defined in the previous subclause is applied for chroma sample interpolation process. When chroma format is 4:2:0, PicWidthInSamplesL and PicHeightInSamplesL are replaced with PicWidthInSamplesC and PicHeightInSamplesC, respectively. Also, LcuSize, TmpX, TmpY1, and TmpY2 are divided by 2.

## Deblocking filter process

New variables condLeftEdge and condTopEdge are defined in subclause 6.5.1.

condLeftEdge is derived as follows.

- If (region_based_refresh_flag != 1) condLeftEdge = 0

- Otherwise

- If getRegionIdxForCU(CurrPic, cuAddrA, lcuAddrA) < getRegionIdxForCU(CurrPic, CurrCuAddr, CurrLcuAddr), condLeftEdge is 1

- Otherwise, condLeftEdge is 0

condTopEdge is derived as follows.

- If (region_based_refresh_flag != 1) condTopEdge = 0

- Otherwise

- If getRegionIdxForCU(CurrPic, cuAddrB, lcuAddrB) < getRegionIdxForCU(CurrPic, CurrCuAddr, CurrLcuAddr), condTopEdge is 1

- Otherwise, condTopEdge is 0

When region_based_refresh_flag is equal to 1, the following restriction is applied to the deblocking filtering process of the luminance component for the leftmost vertical edge. Similar restriction is applied to the deblocking filtering process of the chrominance component for the leftmost vertical edge.

- If ((condLeftEdge == 1) && ((refresh_direction == 0) || (refresh_direction == 4))) is true, pji remain unchanged.

- If ((condLeftEdge == 1) && (refresh_direction == 3)) is true, qji remain unchanged.

When region_based_refresh_flag is equal to 1, the following restriction is applied to the deblocking filtering process of the luminance component for the topmost horizontal edge. Similar restriction is applied to the deblocking filtering process of the chrominance component for the topmost horizontal edge.

- If ((condTopEdge == 1) && ((refresh_direction == 2) || (refresh_direction == 4))) is true, pji remain unchanged.

- If ((condTopEdge == 1) && (refresh_direction == 3)) is true, qji remain unchanged.

Fig. X-X Illustrated explanation of deblocking filering process at region boundary when refresh_direction is 0

## In-loop filter process

When region_based _refresh_flag is equal to 1, argument S’L[ x, y ] as the input to the process defined in subclause 6.5.2.2 and 6.5.2.3 is replaced with extS’L[ x, y ].

extS’L[ x, y ] is defined as follows.

- If RegionIndexA is equal to getRegionIdx(CurrPic, CurrCuAddr, CurrLcuAddr)), extS’L[ x, y ] is derived as follows.

- Otherwise, extS’L[ x, y ] = S’L[ x, y ]

When refresh_direction is equal to 0,

extS’L[ x, y ] = S’L[ x, y ]                   y < BoundaryPosition

extS’L [x, y] = S’L [x, BoundaryPosition]      Otherwise

When refresh_direction is equal to 1,

extS’L [x, y] = S’L [x, y]                    y >= BoundaryPosition

extS’L [x, y] = S’L [x, BoundaryPosition]      Otherwise

When refresh_direction is equal to 2,

extS’L [x, y] = S’L [x, y]                    x < BoundaryPosition

extS’L [x, y] = S’L [BoundaryPosition, y]      Otherwise

When refresh_direction is equal to 3,

extS’L [x, y] = S’L [x, y]                    x >= BoundaryPosition

extS’L [x, y] = S’L [BoundaryPosition, y]      Otherwise

When refresh_direction is equal to 4,

When (BoundaryPosition % PicWidthInLCUs) is equal to 0,

TmpY1 = ((BoundaryPosition  /  PicWidthInLCUs) + 1) * LcuSize

extS’L [x, y] = S’L [x, y]           y < TmpY1

extS’L [x, y] = S’L [x, TmpY1]      Otherwise

Otherwise,

TmpX = (BoundaryPosition % PicWidthInLCUs ) * LcuSize

TmpY2 = min((TmpY1 + LcuSize), PicHeightInSamplesL)

extS’L [x, y] = S’L [x, TmpY2 - 1]           (x < TmpX) && (y >= TmpY2)

extS’L [x, y] = S’L [TmpX - 1, y]            (x >= TmpX) && (y >= TmpY1) && (y < TmpY2)

extS’L [x, y] = extS’L [x, TmpY2 - 1]            (x >= TmpX) && (y >= TmpY2)

extS’L [x, y] = S’L [x, y]      Otherwise

where BoundaryPosition is equal to BoundaryPosition of the current picture.

Similar process is applied to S’Cb and S’Cr. When chroma format is 4:2:0, PicWidthInSamplesL and PicHeightInSamplesL are replaced with PicWidthInSamplesC and PicHeightInSamplesC, respectively. Also, LcuSize, TmpX, TmpY1, and TmpY2 are divided by 2.

Fig. X-X Illustrated explanation of in-loop filter process at region boundary when refresh_direction is 0

# New buffer model for very low delay coding

## Timing of coded group of LCUs removal

A group of LCU g( m ) is the m-th group in a coded picture. The number of groups of LCUs is equal to ( num_groups_in_picture_minus1 + 1 ).

Each group of LCU contains equal number of LCUs NumLCUs. NumLCUs is equal to ( PicSizeInLCUs / ( num_groups_in_picture_minus1 + 1 ) ). g( m ) contains LCUs whose addresses are from (NumLCUs * m) to ( (NumLCUs * (m + 1) - 1 )).

The nominal removal time of the m-th group of LCUs in the n-th access unit, tr,n( n, m ), is defined as follows.

tr,n ( n, m ) = tr,n ( n - 1 ) + (tr,n ( n ) - tr,n ( n - 1 ) ) * m / ( num_groups_in_picture_minus1 + 1 )

The last bit of coded g( m ) shall arrive at the CPB of a decoder before tr,n ( n, m ).

## Definition of last bit of coded group of LCUs

The last bit of a coded group of LCUs is not obvious because the boundary of a group of LCUs does not always coincide with the boundary of a slice. The last bit of a coded group of LCUs is defined in the following subclauses.

### LCEC case

When entropy_coding_mode_flag is equal to 0, the last bit of a coded group of LCUs is defined as follows.

- When a coded group of LCUs is the last coded group of LCUs in a picture, the first bit of rbps_slice_trailing_bits() which follows the coded group of LCUs is the last bit of the coded group of LCUs.

- Otherwise, when there is a slice_header() syntax structure between a coded group of LCUs and the next coded group of LCUs, the first bit of rbps_slice_trailing_bits() which follows the coded group of LCUs is the last bit of the coded group of LCUs.

- Otherwise, the last bit of the last coded LCU in a coded group of LCUs is the last bit of the coded group of LCUs.

Note: bits belongs to a coded LCU in a bitstream are from the bit corresponding to split_coding_unit_flag of depth 0 of the LCU, to the bit just before the bit corresponding to split_coding_unit_flag of depth 0 of the next LCU.

### CABAC case

When entropy_coding_mode_flag is equal to 1, the last bit of a coded group of LCUs is defined as follows.

- The last bit of bit sequence which corresponds to end_of_slice_flag of the last coded LCU in a coded group of LCUs is the last bit of the coded group of LCUs.

In the case of CABAC, there is no one-to-one mapping between a bitstream and a sequence of bins as the output of the CABAC decoding process. One single bit in a bitstream may correspond to more than two bins, and vice versa.

The last bit in a bitstream which corresponds to bin X is defined as follows.

- When bin X is reconstructed through the arithmetic decoding process of bit A in a bitstream, then bit A is the last bit in a bitstream which corresponds to bin X.

The figure below illustratively explains the definition the last bit in a bitstream which corresponds to a bin X.

A black circle in the figure show the relationship between a bit in a bitstream and a bin as the output of the arithmetic decoding process of the bitstream. For example, the first bin is generated as the result of the arithmetic decoding process of the first bit and the second bit in the bitstream.

As shown in the figure, the last bin (i.e. a bin corresponding to end_of_slice_flag ) of the 0-th LCU is generated as the result of the arithmetic decoding process of the third bit in the bitstream. That is, the last bit of the 0-th coded LCU is the third bit in the bitstream. Similarly, the last bins of the first LCU, the second LCU, and the third LCU are generated as the result of the arithmetic decoding process of the 11th bit in the bitstream. That is, the last bit of the first coded LCU, the last bit of the second coded LCU, and the last bit of the third coded LCU are the 11th bit in the bitstream.

Fig. X-X Illustrated explanation of the relationship between last bin of a LCU and corresponding bits in a bitstream (CABAC case)

### PIPE case

When PIPE is used as an entropy coder, the last bit of a coded group of LCUs is defined as follows.

- Let a group of V2V words W be the set of V2V words which correspond to each bin of all coded LCUs in a coded group of LCUs.

- Let a V2V word w be the last V2V word in W which appears at latest in a bitstream

- The last bit of w is the last bit of the coded group of LCUs.

The last bit in a V2V word which corresponds to bin X is defined as follows.

- When bin X is reconstructed through the PIPE decoding process of a V2V word w in a bitstream, then the last bit of w is the last bit in a bitstream which corresponds to bin X.

The figure below illustratively explains the definition the last bit in a bitstream which corresponds to a bin X.

A black circle in the figure show the relationship between a bit in a bitstream and a bin as the output of the PIPE decoding process of the bitstream. For example, [0, 3]-th bits are corresponding to one V2V word. And the 0-th bin, the fifth bin, and the seventh bin are generated through the PIPE decoding process of the V2V word.

As shown in the figure, all bins in the 0-th LCU and all bins in the first LCU are generated as the result of the PIPE decoding process of the V2V word consists of the [0, 3]-th bit, the V2V word consists of the [4, 6]-th bit, and the V2V word consists of the [7, 8]-th bit. That is, the eighth bit, the last bit of the V2V word which appears at latest among three V2V words, is the last bit of the 0-th LCU and the last bit of the first LCU. Similarly, all bins in the third LCU are generated as the result of the PIPE decoding process of the V2V word consists of the [4, 6]-th bit, and the V2V word consists of the [9, 14]-th bit. That is, the 14-th bit is the last bit of the third LCU.

Fig. X-X Illustrated explanation of the relationship between last bin of a LCU and corresponding bits in a bitstream (PIPE case)

# Conclusion and recommendation

The detail of the proposed high-level syntax and semantics at the 2nd meeting, and the detail of the new buffer model proposed at the 3rd meeting are explained.

Please not that the description in this contribution is based the description in JCTVC-B205_draft007. More concrete description based on the WD document will be provided at the future meeting.

References

[1] 2nd JCT-VC Meeting, “Proposal of requirement on very low delay coding”, JCTVC-B031, July 2010, Geneva CH

[2] 3rd JCT-VC Meeting, “Evaluation result of JCTVC-B031”, JCTVC-C021, October 2010, Guangzhou, CN

[3] 2nd JCT-VC Meeting, “Test Mode under Consideration”, JCTVC-B205(draft007), July 2010, Geneva CH

# Patent rights declaration(s)

FUJITSU LIMITED may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).