| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D054 WG11 Number: m18801 |
| --- | --- |

| Title: | Benefit of the new syntax and semantics for very low delay coding in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui, Junpei Koyama, and Akira Nakagawa 4-1-1 Kamikodanaka, Nakahara-ku 211-8588 Kawasaki JAPAN | Tel: Email: | +81-44-754-2639 kazui.kimihiko@jp.fujitsu.com koyama.junpei@jp.fujitsu.com anaka@jp.fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

# Abstract

We proposed the scheme which minimizes the loss of coding efficiency in very low delay coding of less than one frame period at the 2nd JCT-VC meeting. We also proposed the new buffer model that realizes interoperable very low delay coding at the 3rd JCT-VC meeting.

This contribution describes the feature of our proposal and its merit to HEVC.

- Introduction

Low delay coding of less than 1 frame period is highly required in real-time bi-directional video transmission application such as TV conference. We think the HEVC standard shall meet this requirement.

We proposed the scheme which minimizes the loss of coding efficiency in very low delay coding of less than one frame period at the 2nd JCT-VC meeting [1]. We showed the coding gain of the proposed scheme using TMuC0.7 software at the 3rd JCT-VC meeting [2]. In [2] we also proposed the new buffer model that realizes interoperable very low delay coding.

In this contribution we first summarize the background of our proposal, that is the loss of coding efficiency in very low delay coding. Then we explain the feature and the merit of our proposal.

# Very low delay coding

## Vertical intra MB line refresh scheme

Very low delay coding realizes the coding delay of less than 1 frame period.

Real-time bi-directional video transmission applications such as TV conference require the codec delay of less than 100 ms for stress-free interaction. Codec venders have tried to reduce the coding delay as much as possible in order to improve user experience. As the result, several codec apparatus with 10 ms delay become available in the market.

In order to achieve very low delay coding, it is required to minimize the size of a bitstream buffer because it is proportional to the codec delay. It is also required to minimize the loss of coding efficiency and to achieve random-access functionality.

The vertical intra MB line refresh scheme (see Figure.1) is designed to satisfy the requirement for very low delay coding.

The vertical intra MB line refresh scheme inserts an intra MB line vertically in each picture and circulates it horizontally from picture to picture so that all pixels are refreshed (i.e. intra coded) at the end of a refresh cycle.

The vertical intra MB line refresh scheme minimizes the fluctuation of (i.e. averages) the number of bits per MB line because intra MBs are distributed uniformly in each MB line. Thus the size of the bitstream buffer can be reduced to the average number of bits per MB line.

The vertical intra MB line refresh scheme also minimize the loss of coding efficiency and achieve random-access functionality because each MB in a picture is coded intra (i.e. refreshed) only once in a refresh cycle.

Figure 1. Vertical intra MB line refresh scheme

## Problem in AVC

### Loss of coding efficiency

The vertical intra MB line refresh scheme can achieve random-access functionality. However, unlike the refresh scheme using an intra picture, proper encoder control is needed in order to guarantee the correct recovery of whole pixels at the end of each refresh cycle when the decoding process starts from a recovery point.

This is because that the decoder process such as the motion compensation process needs to refer a previously decoded pixel. Generally, a pixel which is not refreshed yet by an intra MB line in the current refresh cycle may not be decoded correctly. For a pixel which is once refreshed by the intra MB line, it is required not to refer to a non-refreshed pixel.

(Hereafter, we define a refreshed pixel/area as a pixel/area which is refreshed by an intra MB line in the current refresh cycle. And we define a non-refreshed pixel/area as a pixel/area which is not yet  refreshed by an intra MB line in the current refresh cycle.)

In order to restrict the reference from a refreshed pixel to a non-refreshed pixel in the AVC standard, procedures listed below are needed.

- When the motion estimation process is invoked, the search range of a MV for a refreshed MB shall be restricted so that the MV does not point to an area in a reference picture which contains a non-refreshed pixel.

- When the intra prediction process is invoked, the direction of intra prediction shall be limited so that a non-refreshed pixel in a same picture is not used for intra prediction.

- When the deblocking filter process is invoked, deblocking filtering for the MB edge of a refreshed MB shall be turned off if the refreshed MB is adjacent to a non-refreshed MB.

In [1] and [2], we described the scheme which realize the procedures listed above. The scheme inserts multiple slices in a picture in order to disable deblocking filter at the region boundary.

There is another possible scheme which can achieve the same procedure without inserting multiple slices in a picture (See Figure.2). This scheme sets disable_deblocking_filter_idc parameter to be 1 for a slice which covers whole picture in order to disable deblocking filtering at the region boundary.

Figure 2. The scheme for very low delay coding based on AVC feature. Deblocking filter at MB boundary for all MBs is disabled.

However, there still occurs the loss of coding gain in this scheme.

Firstly, there is a loss of coding efficiency in disabling the deblocking filter process at a MB boundary. The adaptive in-loop filter process specified in the WD of HEVC shall be also off for the similar reason if it is used.

Secondly, there still remains the limitation of a MV for a refreshed MB. This limitation gets worse when the MV is a decimal because the area pointed by the MV extends according to the tap size of the interpolation filter.

### Interoperable low delay coding of less than 1 frame period

The vertical intra MB line refresh scheme theoretically reduces the size of the bitstream buffer to the number of bits per MB line (e.g. 0.2 ms for 1080p60 sequence).

But the HRD specification in AVC only defines the decoding time of a picture. This means that a decoder shall have a bitstream buffer whose size is larger than the number of bits per picture (e.g. 16ms for 1080p60 sequence) because a decoder has to store whole bits of a picture in the bitstream buffer until the decoding time of the picture. Thus the codec delay becomes larger than the interval of a picture if a decoder follows the AVC specification.

In reality, there are several codec apparatus of 10ms delay in the market. But those apparatus need off-band, proprietary protocol for 10ms delay operation.

Interoperable very low delay coding of less than 1 frame period among apparatus of different venders is practically impossible.

# Proposed scheme

## New high-level syntax and semantics

We propose the alternative scheme for achieving the restriction of reference from a refreshed pixel/CU to a non-refreshed pixel/CU while minimizes the loss of coding efficiency.

The proposed scheme consists of the new high-level syntax and semantics.

The new high-level syntax enables to signal the position of the region boundary between a refreshed area and a non-refreshed area in a picture. The position of the region boundary is described by simple parameters. We restrict the position of the region boundary be a discrete value, in unit of smallest CU width/height.

The new high-level semantics consists of the modified definition of CU availability, and the modified definition of input pixels for the motion compensation process and the adaptive in-loop filter process.

The new high-level semantic has unique features. Firstly, the reference from a refreshed pixel/CU to a non-refreshed pixel/CU is only prohibited. Unlike the conventional slice-based approach, the reference from a non-refreshed pixel/CU to a refreshed pixel/CU is allowed. This asymmetric restriction of reference improves coding gain slightly.

Secondly, the restriction of a MV is relaxed by the new definition of input pixels for the motion compensation process. When the motion compensation process is invoked for a refreshed MB, the reference picture(s) for the refreshed MB is modified. Specifically, all non-refreshed pixel in the reference picture(s) are extrapolated using refreshed pixels at the region boundary. That is, the region boundary is treated as a picture boundary. The new definition of the input pixels for the motion compensation process now allows a refreshed MB to have a MV which points partially inside the non-refreshed area, and achieves coding gain of 1-2% in BD-Bitrate.

The new high-level semantics removes the loss in coding efficiency in the conventional schemes because the following issues are removed or relaxed.

- Restriction on motion vector

- Restriction on deblocking filter (and adaptive in-loop filter)

- Redundant insertion of a slice header

Please refer [1][2][3] for detail.

## New buffer model

We also propose the new buffer model which defines the decoding time of a group of MBs (generally consists of one or more MB lines) for the CPB operation.

The new buffer model enables a decoder to start the decoding process of the group of MBs before the decoding time of the picture which contains the group of MBs, and to reduce the delay for display of the picture.

The new buffer model realizes interoperable very low delay coding.

Please refer [2][3] for detail.

# Merit of our proposal

## Negligible increase of decoder complexity

Increase of decoder complexity introduced by the proposed new high-level syntax and semantics is negligible. The reasons are listed below.

- Simple description of the position of the region boundary

- The new high-level semantics only consists of the modified definition of CU availability, and the modified definition of input pixels for the motion compensation process and the adaptive in-loop filter process.

## Coding gain

The coding gain of the proposed scheme from the scheme in subclause 2.2.1 is the following.

- The gain of the proposed scheme from the “disabling deblocking filter” scheme is 6.0% (64x64 LCU), 6.3% (32x32 LCU)

See [4] for detail.

## Interoperability

New buffer model realizes interoperability of very low delay coding less than 1 frame period among apparatus of different venders.

# Conclusion and recommendation

Low delay coding of less than 1 frame period is highly required in real-time bi-directional video transmission application such as TV conference.

We proposed the scheme which minimizes the loss of coding efficiency in very low delay coding of less than one frame period. The proposed scheme consists of the new high-level syntax and semantics. We also proposed the new buffer model that realizes interoperable very low delay coding.

The merits of our proposal are the following.

- Increase of decoder complexity is negligible.

- The coding gain over the conventional scheme (disabling post filter) is 6.0% - 6.3%.

- New buffer model realizes interoperability of very low delay coding less than 1 frame period among apparatus of different venders.

We request to start the discussion of the support of very low delay coding in HEVC.

References

[1] 2nd JCT-VC Meeting, “Proposal of requirement on very low delay coding”, JCTVC-B031, July 2010, Geneva CH

[2] 3rd JCT-VC Meeting, “Evaluation result of JCTVC-B031”, JCTVC-C021, October 2010, Guangzhou, CN

[3] 4th JCT-VC Meeting, “Draft description of proposed syntax and semantics for very low delay coding”, JCTVC-D054, January 2011, Daegu, KR

[3] 4th JCT-VC Meeting, “Updated evaluation result of proposed syntax and semantics for very low delay coding”, JCTVC-D052, January 2011, Daegu, KR

# Patent rights declaration(s)

FUJITSU LIMITED may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).