| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D055 WG11 Number: m18802 |
| --- | --- |

| Title: | Scalable motion vector competition and simplified MVP calculation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

The motion vector competition with five candidates significantly increases the complexity for motion estimation, it is recommended to add the option of motion vector competition with two candidates to allow the high efficiency encoder design at lower complexity. The calculation of the left and upper spatial MVP candidates in the current HEVC design, which uses to up 16 neighboring PUs, is unnecessarily complicated, it is recommended to simplify the design and use only up to 2 neighboring PUs in the calculation.

# Introduction

This document reports the evaluation results of the motion vector competition with five and two motion vector predictor (MVP) candidates, and briefly analyzes its complexity implication especially on the encoder side. Also reported is the quality gain of the complex upper/left spatial MVP candidate calculation by comparing it to the AVC method. The simplified upper/left spatial MVP candidate calculation method is proposed at the end.

# Advanced MVP algorithm in HM1

As shown in Figure 1, in the HEVC the computation of the motion vector predictors (forward and backward directions, or so-called list 0/ list 1) follows the following steps:

  - Form a list of motion vector predictor (MVP) candidates:

- Median vector of the spatial candidates

- Three spatial candidates (left, above, corner)

- One temporal candidate from collocated PU (optional)

  - Conduct special list reordering for PU with 2NxN or Nx2N modes

  - Remove duplicated / redundant candidates from the list

If the number of MVP candidates is more than 1, the index is signaled in bitstream to tell the decoder which MVP is used. In the worst case, there are five MVP candidates for encoder to choose.

Left neighbors

Upper neighbors

Corner neighbors

(Collocated PU)

List of MVP

Candidates

Median

Re-odering

Remove

Redundant

Candidates

A list of MVP

(mv competition is enabeld)

MVP

(mv competition is disabled)

Figure 1. Diagram of computation of the motion vector predictor on the HEVC

As shown in Table 1, the increased number of MVP candidates significantly increases the motion estimation part of encoder complexity, for the motion vector cost calculation, and for search of skip and direct modes.

| Operation | Complexity with 5 MV candidates | Remark |
| --- | --- | --- |
| Remove duplicated MVP candidates | 10 comparison operations | x 2 for B-slice |
| Choose the best MVP candidate | 5 exp-golomb bits calculation 4 comparison operations to search minimum | Worst case x 2 for B-slice |
| Remove redundant MVP candidates | 4 MV reconstructions 4 x 4 MVD calculation 4 x 4 exp-golomb bits calculation 4 x 4 comparison operations | Worst case x 2 for B-slice |
| Search for direct and skip MVs | 5 + 5 + 25 RD cost calculations | Worst case |

Table 1. Number of operations for motion estimation MV cost calculation and skip/direct mode search when five MVP candidates are used for motion vector competition

In the HM1 the MVP candidate calculation is also much more complex than that of AVC.  As shown in Figure 2, the spatial MVP candidates (left, upper, corner) of the current prediction unit (PU)  are calculated from the motion vectors of its neighbouring PUs, and has the following steps:

Current PU

A0

A1

Am

E

B0

B1

Bn

C

D

Figure 2. Illustration of neighboring prediction units (PU) that are involved in the MVP candidate calculation of the current PU in the HEVC

- MVP candidate from left neighbors (mvLXA; X = 0 or 1):

  - Scanning the left neighbors (PU) from top to bottom (A0 … Am; maximum 16 PUs), find the first PU that meets the following conditions:

    - It is available

    - It is inter-coded

    - It has a MV for the same direction (L0 or L1)

    - It uses the same reference index

  - Otherwise, mvLXA is set to (0,0) and marked unavailable.

- MVP candidate from upper neighbors (mvLXB)

  - Do the same procedure (as that of mvLXA) for the upper neighbors, scanning from left to right (B0 … Bn, maximum 16 PUs)

- MVP candidate from corner neighbors (mvLXC)

  - Do the same procedure (as that of mvLXA)  in the order or C, D then E.

- Median of spatial candidates (mvLXMed)

  - If all of mvLXA, mvLXB and mvLXC is unavailable, mvLXMed is set to (0,0).

  - If only one of mvLXA, mvLXB or mvLXC is available, mvLXMed is set to the MV.

  - Otherwise, mvLXMed = clipMv(median(mvLXA, mvLXB, mvLXC))

Here the calculation of left and upper MVP candidate mvLXA and mvLXB is overcomplicated, because up to 16 neighboring candidates need to be checked in the worst case, and each check requires four comparison operations. This is not the case in the AVC. The table below shows the comparion of the neighboring PUs involved in the spatial MVP candidate calculation between HM1 and AVC. Compared to AVC there is roughly 9x increase in the number of memory access and comparison operations.

| MVP | HM1 | AVC |
| --- | --- | --- |
| mvLXA | A0, A1, … Am, up to 16 PUs | A0, 1 PU |
| mvLXB | B0, B1, … Bm, up to 16 PUs | B0, 1 PU |
| mvLXC | C, D, E, 3 PUs | C, D, 2 PUs |

# Scalable motion vector competition

In the current HM1 only two modes are supported, i.e. the motion vector competition with five MVP candidates can be enabled OR disabled. Disabling the motion vector competition will lose about 2.5% - 3.8% quality gain (see Table 2, refer to TMuC0.9-anchor_vsNoAMVP.xls for detail).

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 3.3 | 2.7 | 3.0 | 4.5 | 3.4 | 3.5 |
| Class B | 2.5 | 1.8 | 1.8 | 3.0 | 1.4 | 1.4 |
| Class C | 2.3 | 2.1 | 2.3 | 3.3 | 2.8 | 3.0 |
| Class D | 2.5 | 2.0 | 2.1 | 3.3 | 2.3 | 2.8 |
| Class E |  |  |  |  |  |  |
| All | 2.5 | 2.1 | 2.2 | 3.3 | 2.3 | 2.5 |
| Enc Time[%] | 84% |  |  | 97% |  |  |
| Dec Time[%] | 103% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 2.1 | 1.4 | 0.9 | 3.8 | 2.7 | 2.6 |
| Class C | 2.7 | 2.5 | 2.4 | 4.3 | 3.3 | 3.4 |
| Class D | 2.5 | 1.4 | 2.2 | 3.5 | 2.3 | 1.9 |
| Class E | 2.8 | 1.1 | 1.2 | 3.4 | 2.9 | 3.2 |
| All | 2.5 | 1.6 | 1.6 | 3.8 | 2.8 | 2.7 |
| Enc Time[%] | 90% |  |  | 101% |  |  |
| Dec Time[%] | 104% |  |  | 109% |  |  |

Table 2 Average BD-rate increase by disabling motion vector competition in TMuC0.9.

One trade-off would be using the motion vector competition with two MVP candidates. Idea here is that after forming the list of MVP candidates (i.e. median, left, upper and corner spatial MVP candidate, and temporal MVP candidate) and conducting special re-ordering for PU with 2NxN or Nx2N modes, only the first spatial MVP candidate in the list and the temporal MVP candidate are retained for motion vector competition.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.4 | 0.5 | 1.0 | 1.0 | 1.0 |
| Class B | 0.3 | 0.3 | 0.3 | 0.8 | 0.5 | 0.5 |
| Class C | 0.5 | 0.6 | 0.6 | 1.2 | 1.3 | 1.2 |
| Class D | 0.2 | 0.3 | 0.2 | 1.0 | 0.8 | 1.1 |
| Class E |  |  |  |  |  |  |
| All | 0.4 | 0.4 | 0.4 | 1.0 | 0.9 | 0.9 |
| Enc Time[%] | 89% |  |  | 95% |  |  |
| Dec Time[%] | 99% |  |  | 102% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.3 | 0.2 | -0.1 | 0.4 | 0.2 | 0.1 |
| Class C | 0.4 | 0.4 | 0.3 | 0.6 | 0.3 | 0.5 |
| Class D | 0.3 | -0.3 | 0.3 | 0.3 | 0.4 | 0.1 |
| Class E | -0.7 | -2.4 | -1.9 | -1.2 | -0.2 | -0.4 |
| All | 0.1 | -0.4 | -0.2 | 0.1 | 0.2 | 0.1 |
| Enc Time[%] | 91% |  |  | 97% |  |  |
| Dec Time[%] | 98% |  |  | 103% |  |  |

Table 3 Average BD-rate increase by using 2-candidate based motion vector competition (compared to TMuC-0.9 anchor 5 candidates based competition)

Simulation is carried out to verify the quality loss w.r.t TMuC-0.9 anchor by allowing the motion vector competition with only two candidates. The results are summarized in Table 3 (refer to TMuC0.9-anchor_vs.2CandAMVP.xls for detail). Compared to the 5 MVP candidates based motion  vector competition, the average loss of using only two MVP candidates is  around 0.1% – 0.4% except for Random access LoCo configuration in which 1% loss is measured. That is, around 90% quality gain of motion vector competition can be captured by using two MV candidates.

It is worthy to point out that five candidate based motion vector competition does not always perform better than two candidates based one. As shown in Table 3, in class E sequences the two candidates based motion vector competition outperforms the five candidates based one by 0.7% - 1.2%.

Using the two candidates based motion vector competition significantly reduces the motion estimation complexity for the MV cost calculation and skip/direct mode search.   The complexity comparison between five and two MVP candidates based motion vector competition is listed in Table 4 below.

| Operation | Complexity with 5 MV candidates | Complexity with 2 MV candidates | Complexity factor (5 vs. 2 candidates) |
| --- | --- | --- | --- |
| Remove duplicated MVP candidates | 10 comparison operations | 1 comparison operation | 10 |
| Choose the best MVP candidate | 5 exp-golomb bits calculation 4 comparison operations to search minimum | 2 exp-golomb bits calculation 1 compares to search minimum | 3 |
| Remove redundant MVP candidates | 4 MV reconstructions 4 x 4 MVD calculation 4 x 4 exp-golomb bits calculation 4 x 4 comparison operations | 1 MV reconstruction 1 MVD calculation 1 exp-golomb bits calculation 1 comparison operations | 16 |
| Search for direct and skip MVs | 5 + 5 + 25 RD cost calculations | 2 + 2 + 4 RD cost calculation | 4 |

Table 4. Number of operations for motion estimation MV cost calculation and skip/direct mode search (five MVP candidates vs. two candidates)

It is hence recommended to add the motion vector competition with two MVP candidates to the HEVC test model to facilitate the high efficiency HEVC encoder design.  This high-level flag can be defined as

| 1 | Motion vector competition with two MVP candidates |
| --- | --- |
| 01 | Motion vector competition with five MVP candidates |
| 00 | Motion vector competition disabled |

# Simplified left/upper MVP candidate calculation

Simulation is carried out to quantify the quality gain of the sophisticated HM1 left/upper MVP calculation method that uses up to 16 PUs.  As shown in Table 5 (refer to TMuC0.9-anchor_vs.AMVP_A0B0.xls for detail), compared to the AVC method which only uses A0 and B0 neighbouring PU for the left and upper MVP candidate mvLXA and mvLXB calculation, checking up to 16 neighbouring PUs only provides about 0.1% quality gain. Apparently such a small gain does not justify excessive amount of memory access and comparison operations.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.2 | 0.3 | 0.1 | 0.2 | 0.2 |
| Class B | 0.1 | 0.1 | -0.1 | 0.1 | 0.2 | 0.1 |
| Class C | 0.1 | 0.1 | 0.0 | 0.1 | 0.2 | 0.0 |
| Class D | 0.1 | 0.1 | 0.0 | 0.1 | 0.0 | 0.3 |
| Class E |  |  |  |  |  |  |
| All | 0.1 | 0.1 | 0.0 | 0.1 | 0.1 | 0.2 |
| Enc Time[%] | 98% |  |  | 96% |  |  |
| Dec Time[%] | 96% |  |  | 97% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | 0.1 | 0.0 | 0.1 | -0.1 | 0.1 |
| Class C | 0.1 | 0.1 | 0.1 | 0.1 | 0.0 | 0.2 |
| Class D | 0.1 | -0.5 | 0.2 | 0.1 | 0.0 | -0.2 |
| Class E | -0.1 | 0.1 | 0.0 | 0.0 | 0.2 | 0.2 |
| All | 0.0 | -0.1 | 0.1 | 0.1 | 0.0 | 0.1 |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

Table 5 Average BD-rate increase resulted from using A0 and B0 only for mvLXA and mvLXB calculation.

To simplify the left and upper MVP candidate calculation, we propose to use up to only 2 neighbouring PUs to compute the left and upper MVP candidate mvLXA and mvLXB, respectively.

Current PU

A0

Am

E

B0

Bn

C

D

Figure 3. Illustration of neighboring prediction units (PU) that are involved in the motion vector predictor calculation of the current PU in the proposed method

As shown in Figure 3, for the mvLXA calculation, instead of scanning all the left neighbouring PUs from top to bottom, only up to two (sometimes A0 is same as Am) neighbouring PUs, i.e. A0 and Am, are used in the simplified algorithm. Similarly, to calculate mvLXB only up to neighbouring PUs, i.e. B0 and Bn ,are used.  Note that m and n can be 0 if the current PU has the minimum vector block size allowed by the HEVC standard.

The modified calculation of left/upper MVP candidates is described as follows (highlighted in yellow):

- MVP candidate from left neighbors (mvLXA; X = 0 or 1):

  - Scanning the left two neighbors (PU) A0 and Am from top to bottom (A0, Am; maximum 2 PUs), find the first PU (i.e. A0 or Am) that meets the following conditions:

    - It is available

    - It is inter-coded

    - It has a MV for the same direction (L0 or L1)

    - It uses the same reference index

  - If found, mvLXA is set to clipMv(MV of the PU)

  - Otherwise, mvLXA is set to (0,0) and marked unavailable.

- MVP candidate from upper neighbors (mvLXB)

  - Do the same procedure (as that of mvLXA) for the upper neighbors, scanning upper two neighbors (PU) B0 and Bm from left to right (B0, Bn, maximum 2 PUs)

The rest of steps are unchanged.

Simulation is carried out to verify the effectiveness of the simplified algorithm. As shown in Table 6 (refer to TMuC0.9-anchor_vs.AMVP_A0AmB0Bn.xls for detail), this simplification does not lead to quality degradation. It is recommended to adopt this simplification into the HEVC test model.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.2 | 0.0 | 0.0 | 0.1 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 |
| Class C | 0.0 | 0.0 | 0.1 | 0.0 | 0.1 | 0.0 |
| Class D | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 96% |  |  | 96% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.1 | -0.1 | 0.0 | -0.1 | -0.1 |
| Class C | 0.0 | 0.2 | 0.1 | 0.0 | 0.0 | 0.1 |
| Class D | 0.1 | -0.4 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | -0.2 | 0.0 | 0.6 | 0.0 |
| All | 0.0 | -0.1 | 0.0 | 0.0 | 0.1 | 0.0 |
| Enc Time[%] | 97% |  |  | 100% |  |  |
| Dec Time[%] | 96% |  |  | 99% |  |  |

Table 6 Average BD-rate increase resulted from using A0, Am and B0, Bn for mvLXA and mvLXB calculation (Compared TMuC0.9 anchor).

# Test Conditions

The simulations of this document have used TMuC0.9 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed.

# Conclusions

The motion vector competition with five candidates significantly increases the complexity for motion estimation, it is recommended to add the option of motion vector competition with two candidates to allow the high efficiency encoder design at lower complexity. The calculation of the left and upper spatial MVP candidates in the current HEVC design is overcomplicated and unnecessarily complex, it is recommended to adopt the proposed simplification to simplify the design.

# References

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-C500, 3rd. JCT-VC Meeting, Guangzhou, CN, October 2010.

# Patent rights declaration(s)

Texas Instruments may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).