| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KOR, 20-28 January, 2011 | Document: JCTVC-D056 WG11 Number: m18803 |
| --- | --- |

| Title: | CE3: Luma interpolation using MOMS |  |
| --- | --- | --- |
| Status: | Input Document to JCT-VC |  |
| Purpose: | Report |  |
| Author(s) or Contact(s): | Haricharan Lakshman, Heiko Schwarz, Detlev Marpe, Thomas Wiegand | Tel: +49-30-31002 232 Email: haricharan.lakshman@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |

_____________________________

# Abstract

This contribution is a description of MOMS based interpolation filter for luma motion compensation. It has been tested in the HE profile under CE3 conditions. In this interpolation scheme, the reconstructed picture is 2-tap IIR filtered and the result is used for motion compensation sub-pixel filtering using 4- or 6-tap FIR filters, denoted as 4MOMS and 6MOMS, respectively.

For 6MOMS, average BD-rate savings of 1.9% for the low delay configuration and 0.7% for the random access configuration have been measured. An average decoder run time increase of 18% and an average encoder run time increase of 2% compared to the HE anchor has been observed. A theoretical analysis of complexity and memory accesses is provided.

# Introduction

In a hybrid video codec, the reconstructed picture is generated after completing the in-loop processing. It can then be used as a reference picture for motion compensation of successive pictures in coding order. In order to achieve high quality interpolation, a 12-tap FIR filter is used in the HM 1.0 in the HE profile.

This contribution aims at improving the coding efficiency by using a two stage filtering. The first stage, which involves IIR filtering, operates on the reconstructed picture and is independent of the sub-pixel position pointed by the MVs. Therefore, it can be performed on the reconstructed samples without waiting for the future frame data and stored in the reference picture buffer. The second stage consists of short FIR filters according to the sub-pixel pointed by the MVs.

This document is intended as an overview & performance report of the interpolation schemes and uses the terms 4MOMS and 6MOMS to refer to two variants as described below. An accompanying excel sheet, which is prepared according to the CE3 template, provides a detailed analysis of the complexity of the different stages involved.

# Description of the proposal

The figure below depicts the scheme of the two stage filtering.

The basic architecture of 4MOMS and 6MOMS are similar. The IIR part consists of bi-directional 2-tap filters in horizontal and vertical directions for both variants. The detailed description of the IIR process for 4MOMS is provided below.

IIR filter operations

Let [x, y] denote a sample location inside a picture and S[x, y] denote the reconstructed sample at the sample location [x, y].

| Type | Input | Output | Process |
| --- | --- | --- | --- |
| Left-to-right | S[x] | H1[x] | H1[x]=(S[x]<<2)-(H1[x-1]>>1) |
| Right-to-left | S[x] | H2[x] | H2[x]=(S[x]<<2)-(H2[x+1]>>1) |
| Horizontal result | S[x],H1[x],H2[x] | H[x] | H[x]=H1[x]+H2[x]-(S[x]<<2) |
| Top-to-bottom | H[y] | V1[y] | V1[y]=H[y]-(V1[y-1]>>1) |
| Bottom-to-top | H[y] | V2[y] | V2[y]=H[y]-(V2[y+1]>>1) |
| Vertical result | H[y],V1[y],V2[y] | R[y] | R[y]=V1[y]+V2[y]-H[y] |

In the current implementation, the right-to-left filter does not need the output of the left-to-right filter because both operate on S[x] directly. The dependencies in the vertical filtering case are also similar.

In HE profile, the 12-bit reconstructed samples are IIR filtered and the result is stored in the reference picture buffer using 16-bit per sample.

FIR filter operations

We define the function fir_filt() to illustrate the computation of fractional positions:

Psuedo-code:

result = fir_filt(A,B,C,D,offset)

{

// load filter coefficients

f0=table[offset][0]; .. f3=table[offset][3];

// 4-tap filter

result = A*f0 + B*f1 + C*f2 + D*f3;

// result of filtering

return (result);

}

Notation of different positions in a quarter-pixel grid

| A0 |  |  |  | B0 |  | aa |  | C0 |  |  |  | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A1 |  |  |  | B1 |  | bb |  | C1 |  |  |  | D1 |
|  |  |  |  |  |  | pp |  |  |  |  |  |  |
| A2 |  |  |  | B2 |  | cc |  | C2 |  |  |  | D2 |
| A3 |  |  |  | B3 |  | dd |  | C3 |  |  |  | D3 |

For each position required, the x and y offsets are used to control the 2D separable filtering. Consider the position 'pp' shown in the figure above. Let the sample location of pp be (xoffset, yoffset) with respect to position B1, where xoffset and yoffset are specified in units of quarter luma.

The following horizontal filtering is performed to generate intermediate samples: aa, bb, cc, dd.

aa = fir_filt ( A0, B0, C0, D0, xoffset)

bb = fir_filt ( A1, B1, C1, D1, xoffset)

cc = fir_filt ( A2, B2, C2, D2, xoffset)

dd = fir_filt ( A3, B3, C3, D3, xoffset)

The final interpolated value is obtained through a vertical filtering of the intermediate samples:

pp = fir_filt ( aa, bb, cc, dd, yoffset )

The function fir_filt() is called even when either xoffset or yoffset is equal to 0, in which case the fir_filt() is a symmetric 3-tap filter because of a zero entry in the corresponding table.

The operations of 6-tap MOMS are similar to the 4-tap case. The case of offset=0 is still a symmetric 3-tap filter and values at other offsets are generated using 6-tap filters. The IIR part for using 6-tap MOMS is similar to the 4-tap case except that the pole value is different and a rounding factor is added before right shift. The details can be found in the source code and the differences are accounted in the corresponding complexity analysis excel sheets.

# Experimental results

All experiments for all sequences based on the common conditions, JCTVC-B302r2 [1] have been conducted. The binaries were compiled on a 64 bit Ubuntu 10.10 Linux using the included make files and gcc 4.4.5. The test summary is given below:

|  | Random access HE 6MOMS |  |  | Random access HE 4MOMS |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.1 | 0.1 | 0.0 | 0.2 |
| Class B | -0.2 | 0.0 | -0.1 | 0.0 | 0.0 | -0.1 |
| Class C | -0.9 | -0.3 | -0.3 | -0.1 | -0.1 | -0.1 |
| Class D | -1.7 | -0.3 | -0.2 | 0.0 | 0.1 | 0.4 |
| Class E |  |  |  |  |  |  |
| All | -0.7 | -0.1 | -0.1 | 0.0 | 0.0 | 0.1 |
| Enc Time[%] | 103% |  |  | 98% |  |  |
| Dec Time[%] | 111% |  |  | 110% |  |  |
|  | Low delay HE 6MOMS |  |  | Low delay HE 4MOMS |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.9 | -0.6 | -0.9 | -0.3 | -0.5 | -0.5 |
| Class C | -2.5 | -1.1 | -1.1 | -1.2 | -0.3 | -0.4 |
| Class D | -5.1 | -2.9 | -2.4 | -2.3 | -1.1 | -0.4 |
| Class E | 1.4 | -1.5 | -1.0 | 0.8 | -0.8 | -0.9 |
| All | -1.9 | -1.5 | -1.4 | -0.8 | -0.6 | -0.5 |
| Enc Time[%] | 104% |  |  | 98% |  |  |
| Dec Time[%] | 126% |  |  | 124% |  |  |

## Complexity analysis

The procedure for the complexity analysis of FIR filters is well understood in the CE3 community. Therefore, the complexity of IIR part is described here. The detailed analysis of the operations for the FIR part can be found in the excel sheet associated to this contribution.

The IIR filter routine from source code is reproduced in the table below and the counting of the number of arithmetic operations and memory accesses is illustrated for the case 4MOMS.

| Direction | Process | Shift | Add | Mem read | Mem write |
| --- | --- | --- | --- | --- | --- |
| Left-to-right | cTmp= (pcSrc[iX]<<2)-(cTmp>>1); pcCff[iX]= cTmp; | 2 | 1 | 1 | 1 |
| Right-to-left | cTmp= (pcSrc[iX]<<2)-(cTmp>>1); pcCff[iX]= pcCff[iX]+cTmp-(pcSrc[iX]<<2); | 2 | 3 | 3 | 2 |
| Top-to-bottom | cTmp= *pcSrc-(cTmp>>1); *pcCff= cTmp; | 1 | 1 | 2 | 2 |
| Bottom-to-top | cTmp= *pcSrc-(cTmp>>1); *pcCff= (*pcCff+cTmp-*pcSrc); | 1 | 3 | 4 | 2 |
| Overall | IIR total | 6 | 8 | 10 | 8 |

The sum of all shift and add cycles for each pixel is 6+8 = 14 clock cycles. The total memory accesses for read and write is 10+8=18 bytes. As in the other contributions to CE3, the assumption of 4 bytes/memory access is used to count the clock cycles required for memory accesses.

The theoretical total cycles (operations + memory cycles) of 6MOMS with multiplication enabled implementation is calculated to be around 110% of 12-tap FIR anchor on average. For 4MOMS, the total cycles is calculated to be approx. 80% of 12-tap FIR anchor. One of the reasons for the differences in the complexity analysis and observed encode/decode times is the fact that the complier generated assembly code for the IIR filtering contains memory accesses of 2 bytes/instruction, i.e. 16-bit numbers are loaded/stored each time during the filtering. In an optimized implementation, 32-bit numbers can be loaded and the upper and lower parts can be processed separately to achieve the memory access cycles in accordance with the complexity analysis presented in the excel sheet. It can also explain the negligible run time differences in 4MOMS and 6MOMS, in spite of a clearly lower number of computations in FIR part of 4MOMS compared to 6MOMS.

# Conclusion

A two stage interpolation filtering approach for the HE profile is proposed that provides coding efficiency improvements of 1.3% on average (measured as BD-rate) using the 6MOMS variant. It is found that using the IIR+FIR scheme, particularly large gains can be achieved for those sequences that are sensitive to interpolation filter quality.

# Reference

[1] T. Chujoh, “CE3: Interpolation for MC (Luma),” Joint Collaborative Team on Video Coding, JCTVC-C503r5, Guangzhou, Oct. 2010.

# Patent rights declaration(s)

Fraunhofer HHI may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).