| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KOR, 20-28 January, 2011 | Document: JCTVC-D057 WG11 Number: m18804 |
| --- | --- |

| Title: | CE4: Chroma interpolation using MOMS based FIR filters |  |
| --- | --- | --- |
| Status: | Input Document to JCT-VC |  |
| Purpose: | Report |  |
| Author(s) or Contact(s): | Haricharan Lakshman, Heiko Schwarz, Detlev Marpe, Thomas Wiegand | Tel: +49-30-31002 232 Email: haricharan.lakshman@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |

_____________________________

# Abstract

This contribution reports the performance of MOMS based 4-tap and 6-tap FIR filters for chroma motion compensation. The test results for the case of 6-tap FIR in HE profile and 4-tap FIR in LC profile are provided. In addition to the normally used BDBR metric for each Y,U,V plane, a combined YUV_BDBR metric[1] was adopted for this CE.

According to the conducted experiments, using the 6-tap FIR in HE profile provides an average YUV_BDBR of -2.1% in the random access configuration and -2.2% in the low delay configuration. The 4-tap FIR in LC profile provides an average YUV_BDBR of -2.5% in the random access configuration and -3.2% in the low delay configuration.

# Introduction

HM1.0 uses a bi-linear filter for interpolation during chroma motion compensation. This contribution explores the coding efficiency improvement when a higher number of FIR taps are employed. The interpolation filter for luma is maintained the same as in the anchors.

# Description of the proposal

Instead of the bi-linear filter, the following filters are employed for chroma filtering:

4-tap FIR in LC profile

{ -11,   238,    34,    -5,},	// 1/8

{ -20,   216,    68,    -8,},	// 2/8

{ -18,   181,   107,   -14,},	// 3/8

{ -18,   146,   146,   -18,},	// 4/8

{ -14,   107,   181,   -18,},	// 5/8

{  -8,    68,   216,   -20,},	// 6/8

{  -5,    34,   238,   -11,}		// 7/8

6-tap FIR in HE in profile

{   3,   -16,   243,    35,   -11,     2,},	// 1/8

{   5,   -26,   220,    73,   -19,     3,},	// 2/8

{   6,   -31,   189,   114,   -26,     4,},	// 3/8

{   6,   -32,   154,   154,   -32,     6,},	// 4/8

{   4,   -26,   114,   189,   -31,     6,},	// 5/8

{   3,   -19,    73,   220,   -26,     5,},	// 6/8

{   2,   -11,    35,   243,   -16,     3,}		// 7/8

# Experimental results

All experiments for all sequences based on the common conditions [1] have been conducted. The binaries were compiled on a 64 bit Ubuntu 10.10 Linux using the included make files and gcc 4.4.5. The test summary is given below:

|  | Random access |  |  |  | Random access LoCo |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A | -0.1 | -2.5 | -3.3 | -1.0 | -0.1 | -3.7 | -3.9 | -1.5 |
| Class B | -0.2 | -5.2 | -7.0 | -2.1 | -0.1 | -6.9 | -7.0 | -2.2 |
| Class C | -0.2 | -5.6 | -5.4 | -2.1 | -0.2 | -7.8 | -7.7 | -2.8 |
| Class D | -0.3 | -7.3 | -9.1 | -2.8 | -0.2 | -9.1 | -10.8 | -3.0 |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.2 | -5.5 | -6.6 | -2.1 | -0.2 | -7.3 | -7.8 | -2.5 |
| Enc Time[%] | 101% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 110% |  |  |  | 119% |  |  |  |
|  | Low delay |  |  |  | Low delay LoCo |  |  |  |
|  | Y | U | V | YUV | Y | U | V | YUV |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.5 | -3.8 | -4.9 | -1.7 | -0.9 | -8.4 | -9.8 | -3.3 |
| Class C | -0.6 | -4.3 | -4.7 | -2.2 | -0.9 | -9.9 | -9.2 | -4.5 |
| Class D | -0.8 | -8.5 | -9.1 | -3.1 | -1.3 | -9.9 | -12.0 | -4.1 |
| Class E | -0.2 | -4.4 | -4.6 | -1.5 | -0.3 | 3.8 | -2.2 | -0.2 |
| All | -0.6 | -5.2 | -5.9 | -2.2 | -0.9 | -6.9 | -8.7 | -3.2 |
| Enc Time[%] | 101% |  |  |  | 102% |  |  |  |
| Dec Time[%] | 112% |  |  |  | 118% |  |  |  |

The encoding/decoding times are not very reliable as it was tested using a cluster of computers. When the FIR coefficients are employed with a speed optimized software implementation that exploits symmetries in the half-sample filter coefficients, lower run times can be achieved with no change in RD performance.

# Conclusion

The results reported in this document, showing coding efficiency improvements when using higher number of taps for chroma interpolation filtering, are in accordance to previously reported observation. Average BDBR savings for chroma is found to be considerable with the proposed set of coefficients.

# Reference

[1] E. Alshina, “CE4: Interpolation for MC (Chroma),” Joint Collaborative Team on Video Coding, JCTVC-C504, Guangzhou, Oct. 2010.

# Patent rights declaration(s)

Fraunhofer HHI may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).