| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, Korea, 20-28 January, 2011 | Document: JCTVC-D058 WG11 Number: m18805 |
| --- | --- |

| Title: | CE3: Verification of Nokia adaptive DCT-IF/DIF filters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This document reports the cross-check results of the Nokia proposal of adaptive DCT-IF and DIF filter for both 6-tap and 8-tap. It is verified that the coding efficiency data that Nokia provided is accurate. Additional data is presented in this document to highlight the coding efficiency benefit of adding the DIF filter on the top of DCT-IF filter.

# Introduction

This document reports the cross-check results of the Nokia proposal of adaptive DCT-IF and DIF filter [2], for both 6-tap and 8-tap. The comparison results of the Nokia filter against the TMuC-0.9 results with 6-tap or 8-tap DCT-IF filter in both HE and LOCO configurations as anchor are also reported.

# Algorithm Description

In Nokia adaptive DCT-IF and DIF filter design, a slice can use either DCT-IF or DIF for motion compensation based on the slice activity. In the current encoder implementation, the current frame data is used for computing the slice (frame) activity and make filter switch decision for the current slice (frame). This has implication in memory bandwidth and latency on the encoder side. It is advised that activity collected on the previous frame can be used for filter switch of the current frame to avoid delay and memory bandwidth increase.

For the 6-tap adaptive DCT-IF/DIF filter, the filter coefficients are different for DCT-IF and DIF as listed in table below:

| DCTIF 6-tap 1 | 8, -32, 224, 72, -24, 8 |
| --- | --- |
| DCTIF 6-tap 2 | 8, -40, 160, 160, -40, 8 |
| DCTIF 6-tap 3 | 8, -24, 72, 224, -32, 8 |
| DIF 6-tap 1 | 2, -15, 111 ,37, -10, 2 |
| DIF 6-tap 2 | 3, -17, 78, 78, -17, 3 |
| DIF 6-tap 3 | 2, -10, 37, 11, -15, 3 |
| DIF 12-tap for center point | 5, 5, 5, 22, 22, 5, 5, 22, 22, 5, 5, 5 |

For the 8-tap adaptive DCT-IF/DIF filter, the filter coefficients are same for DCT-IF and DIF, except for the center point of the DIF filter which still uses same 12-tap filter. Having the same filter coefficients in both filters can help simplify the design. The 8-tap filter coefficients are listed in table below:

| DCTIF 8-tap 1 | -4, 16, -40, 228, 76, 28, 12, -4 |
| --- | --- |
| DCTIF 8-tap 2 | -4, 20, -48, 160, 160, -48, 20, -4 |
| DCTIF 8-tap 3 | -4, 12, -28, 76, 228, -40, 16, -4 |
| DIF 8-tap 1 | -4, 16, -40, 228, 76, 28, 12, -4 |
| DIF 8-tap 2 | -4, 20, -48, 160, 160, -48, 20, -4 |
| DIF 8-tap 3 | -4, 12, -28, 76, 228, -40, 16, -4 |
| DIF 12-tap for center point | 5, 5, 5, 22, 22, 5, 5, 22, 22, 5, 5, 5 |

# Test Conditions

The TMuC-0.9 software is used for the evaluation, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed.

# Testing Results

## Comparison results against data provided by Nokia

For 6-tap adaptive DCT-IF/DIF filter, perfect match is achieved. Please refer to CE3_r1_TI_vs.Nokia_difdctif6tap.xls for detail.

For 6-tap adaptive DCT-IF/DIF filter results are slightly off due to the CE3 related encoder fix was not integrated into our 8-tap runs. But Qualcomm has done cross-check for 8-tap with encoder bug fix, and achieved perfect match. Detailed results are provided in CE3_r1_TI_vs.Nokia_difdctif8tap.xls.

## Nokia adaptive filter against TMuC0.9 DCT-IF filter

In order to highlight the benefit of adding the DIF filter on the top of the DCT-IF filter, the Nokia adaptive filter is compared to TMuC-0.9 results with DCT-IF used in both HE and LOCO configurations as anchor. The comparison results for 6-tap and 8-tap are listed in Table 1 and Table 2.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.4 | -0.2 | -0.2 | -1.4 | -0.9 | -0.7 |
| Class B | 0.5 | 0.0 | 0.0 | -0.1 | -0.7 | -0.8 |
| Class C | 0.4 | 0.0 | 0.0 | 0.0 | -0.5 | -0.4 |
| Class D | 0.3 | -0.1 | -0.1 | 0.2 | -0.4 | -0.4 |
| Class E |  |  |  |  |  |  |
| All | 0.4 | -0.1 | 0.0 | -0.2 | -0.6 | -0.6 |
| Enc Time[%] | 104% |  |  | 103% |  |  |
| Dec Time[%] | 104% |  |  | 107% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | 0.2 | 0.2 | -0.5 | 0.3 | 0.4 |
| Class C | -0.2 | 0.1 | 0.1 | -0.6 | 0.0 | -0.1 |
| Class D | -0.1 | -0.2 | 0.0 | -0.1 | 0.0 | -0.1 |
| Class E | -0.1 | 0.1 | -0.5 | -2.3 | -0.1 | 0.5 |
| All | -0.1 | 0.1 | 0.0 | -0.8 | 0.1 | 0.2 |
| Enc Time[%] | 102% |  |  | 106% |  |  |
| Dec Time[%] | 96% |  |  | 108% |  |  |

Table 1 Coding efficiency comparison between Nokia adaptive filter and TMuC-0.9 DCT-IF filter in both HE and LOCO configurations (6-tap, negative numbers mean that Nokia filter has gain)

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | -0.3 | -0.1 | -1.4 | -0.7 | -0.6 |
| Class B | 0.5 | -0.1 | -0.1 | -0.6 | -0.8 | -1.0 |
| Class C | 0.3 | -0.2 | -0.1 | 0.0 | -0.6 | -0.4 |
| Class D | 0.3 | -0.1 | -0.1 | 0.1 | -0.3 | -0.4 |
| Class E |  |  |  |  |  |  |
| All | 0.4 | -0.1 | -0.1 | -0.4 | -0.6 | -0.6 |
| Enc Time[%] | 98% |  |  | 106% |  |  |
| Dec Time[%] | 95% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.2 | 0.1 | 0.1 | -1.1 | -0.1 | -0.2 |
| Class C | -0.2 | 0.1 | 0.0 | -0.5 | -0.3 | -0.4 |
| Class D | -0.1 | -0.1 | -0.2 | -0.3 | -0.3 | -0.2 |
| Class E | 0.4 | 0.7 | 0.2 | -1.3 | -0.2 | 0.3 |
| All | 0.1 | 0.2 | 0.0 | -0.8 | -0.2 | -0.1 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 96% |  |  | 99% |  |  |

Table 2 Coding efficiency comparison between Nokia adaptive filter and TMuC-0.9 DCT-IF filter in both HE and LOCO configurations (8-tap, negative numbers mean that Nokia filter has gain)

For detailed results please refer to

CE3_r1_anchor6tapdctif_vsdifdctif6tap.xls

CE3_r1_anchor8tapdctif_vsdifdctif8tap.xls

# Conclusions

In this document the performance of Nokia adaptive 6-tap and 8-tap DCT-IF/IF filter is verified. Additional data is presented to highlight the coding efficiency benefit of adding the DIF filter on the top of DCT-IF filter.

# References

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-C500, 3rd. JCT-VC Meeting, Guangzhou, CN, October 2010.