| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D059 WG11 Number: m18806 |
| --- | --- |

| Title: | CE3: Verification of Samsung new 6-tap/8-tap DCT-IF filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This document reports the cross-check results of the Samsung new 6-tap/8-tap DCT-IF filter. It is verified that the coding efficiency data that Samsung provided is accurate. Additional data is provided in this document to highlight the coding efficiency benefit of Samsung new 6-tap/8-tap DCT-IF filter compared to the TMuC-0.9 6-tap/8-tap DCT-IF filter.

# Introduction

This document reports the cross-check results of the Samsung proposal of 6-tap/8-tap DCT-IF filter [2]. The comparison results of the Samsung new 6-tap/8-tap DCT-IF filter against the TMuC-0.9 6-tap/8-tap DCT-IF filter are also reported for both HE and LOCO configurations.

# Algorithm Description

In the new design of Samsung 6-tap DCT-IF filter, the filter coefficients are changed to having higher precision. The 6-tap DCT-IF filter coefficients used in TMuC-0.9 and in Samsung new proposal are listed in Table below.

6-tap DCT-IF filter coefficients

| TMuC-0.9 6-tap DCT-IF |  |
| --- | --- |
| DCTIF 6-tap 1 | 8, -32, 224, 72, -24, 8 |
| DCTIF 6-tap 2 | 8, -40, 160, 160, -40, 8 |
| DCTIF 6-tap 3 | 8, -24, 72, 224, -32, 8 |
| Samsung new 6-tap DCT-IF |  |
| DCTIF 6-tap 1 | 8, -35, 227, 73, -23, 6 |
| DCTIF 6-tap 2 | 5, -33, 156, 156, -33, 5 |
| DCTIF 6-tap 3 | 6, -23, 73, 227, -35, 8 |

In Samsung new 6-tap design, filter coefficients for all three filter sets are changed. With the new 6-tap filter coefficients the inter-mediate data precision of the 6-tap DCT-IF filter has grown from 15-bit to 18-bit for 8-bit input video, this has a complexity implication especially for software implementation.

In Samsung new 8-tap design, only symmetric filter coefficients are changed. The inter-mediate data precision remains unchanged (i.e. 16-bit for 8-bit video). The 8-tap DCT-IF filter coefficients used in TMuC-0.9 and in Samsung new proposal are listed in Table below.

8-tap DCT-IF filter coefficients

| TMuC-0.9 8-tap DCT-IF |  |
| --- | --- |
| DCTIF 8-tap 1 | -4, 16, -40, 228, 76, -28, 12, -4 |
| DCTIF 8-tap 2 | -4, 20, -48, 160, 160, -48, 20, -4 |
| DCTIF 8-tap 3 | -4, 12, -28, 76, 228, -40, 16, -4 |
| Samsung new 8-tap DCT-IF |  |
| DCTIF 8-tap 1 | -4, 16, -40, 228, 76, -28, 12, -4 |
| DCTIF 8-tap 2 | -4, 16, -44, 160, 160, -44, 16, -4 |
| DCTIF 8-tap 3 | -4, 12, -28, 76, 228, -40, 16, -4 |

The filter can operate in two modes: i.e. normal mode without encoder restrictions and restricted mode with encoder restrictions to reduce encoder complexity. In this document verification results are reported for 6-tap  normal and restricted modes, and for 8-tap normal mode.

# Test Conditions

The TMuC-0.9 software is used for the evaluation, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed.

# Testing Results

## Comparison results against data provided by Samsung

Perfect match is achieved for 6-tap normal and restricted modes, as well as for  8-tap normal mode. Detailed comparison are provide in the attached spreadsheets.

Results_TIvs.Samsung_CE3_DCTIF6taps_r1.xls

Results_TIvs.Samsung_CE3_DCTIF6tapsRestr_r1.xls

Results_TIvs.Samsung_CE3_DCTIF8taps_r1.xls

## Samsung new 6-tap DCT-IF against TMuC0.9 6-tap DCT-IF filter

In order to investigate the coding efficiency gain, Samsung new 6-tap DCT-IF filter is compared with TMuC-0.9 results with 6-tap DCT-IF used in both HE and LOCO configurations. The comparison results for normal and restricted mode are listed in Table 1 and Table 2.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | 0.0 | 0.1 | -1.1 | 0.1 | 0.0 |
| Class B | -0.1 | 0.0 | 0.0 | -0.8 | 0.1 | 0.1 |
| Class C | 0.0 | 0.2 | 0.1 | -0.2 | 0.2 | 0.2 |
| Class D | 0.0 | 0.3 | 0.3 | 0.0 | 0.3 | 0.3 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.1 | 0.1 | -0.5 | 0.2 | 0.2 |
| Enc Time[%] | 104% |  |  | 109% |  |  |
| Dec Time[%] | 105% |  |  | 116% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.2 | -0.1 | 0.2 | -1.8 | 0.3 | 0.4 |
| Class C | 0.3 | 0.4 | 0.3 | -1.1 | 0.4 | 0.4 |
| Class D | 0.5 | 0.3 | 0.5 | -0.9 | 0.3 | 0.4 |
| Class E | -0.8 | -1.0 | -1.0 | -3.7 | -1.2 | -0.8 |
| All | 0.0 | 0.0 | 0.1 | -1.7 | 0.1 | 0.1 |
| Enc Time[%] | 103% |  |  | 111% |  |  |
| Dec Time[%] | 98% |  |  | 115% |  |  |

Table 1 Coding efficiency comparison between Samsung new and TMuC-0.9 6-tap DCT-IF filter (normal mode, negative numbers mean that Samsung new filter has gain)

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1.8 | 1.3 | 1.1 | 0.8 | 0.9 | 0.7 |
| Class B | 1.5 | 1.1 | 1.1 | 0.8 | 0.3 | 0.4 |
| Class C | 2.2 | 1.3 | 1.3 | 2.0 | 0.9 | 1.0 |
| Class D | 3.3 | 2.1 | 2.2 | 3.2 | 1.8 | 1.9 |
| Class E |  |  |  |  |  |  |
| All | 2.2 | 1.5 | 1.4 | 1.8 | 0.9 | 1.0 |
| Enc Time[%] | 101% |  |  | 97% |  |  |
| Dec Time[%] | 106% |  |  | 108% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 1.7 | 2.2 | 2.3 | -0.5 | 2.3 | 2.8 |
| Class C | 2.5 | 2.6 | 2.6 | 0.9 | 2.5 | 2.4 |
| Class D | 5.5 | 5.1 | 5.6 | 2.8 | 5.0 | 4.8 |
| Class E | 1.2 | 2.5 | 1.8 | -2.5 | 0.9 | 1.9 |
| All | 2.8 | 3.1 | 3.1 | 0.3 | 2.7 | 3.0 |
| Enc Time[%] | 97% |  |  | 99% |  |  |
| Dec Time[%] | 98% |  |  | 112% |  |  |

Table 2 Coding efficiency comparison between Samsung new and TMuC-0.9 6-tap DCT-IF filter (Restricted mode, negative numbers mean that Samsung new filter has gain)

For detailed results please refer to

Results_anchor6tapDCTIF_vs._Samsung6tap.xls

Results_anchor6tapDCTIF_vs._Samsung6tapRestr.xls

## Samsung new 8-tap DCT-IF against TMuC0.9 8-tap DCT-IF filter

Samsung new 8-tap DCT-IF filter is compared with TMuC-0.9 results with 8-tap DCT-IF used in both HE and LOCO configurations. The comparison results for normal mode are given in Table 3.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.2 | -0.6 | 0.1 | 0.1 |
| Class B | -0.1 | 0.1 | 0.1 | -0.9 | -0.1 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | -0.3 | 0.1 | 0.1 |
| Class D | -0.1 | 0.2 | 0.3 | -0.1 | 0.1 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | -0.1 | 0.1 | 0.1 | -0.5 | 0.1 | 0.0 |
| Enc Time[%] | 92% |  |  | 95% |  |  |
| Dec Time[%] | 86% |  |  | 89% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | -0.2 | -1.4 | -0.1 | 0.0 |
| Class C | -0.1 | 0.0 | -0.1 | -0.8 | -0.1 | -0.2 |
| Class D | -0.1 | -0.1 | 0.1 | -1.3 | -0.7 | -0.4 |
| Class E | -0.2 | 0.5 | -0.4 | -1.5 | -0.5 | -0.5 |
| All | -0.1 | 0.1 | -0.1 | -1.2 | -0.3 | -0.2 |
| Enc Time[%] | 95% |  |  | 92% |  |  |
| Dec Time[%] | 93% |  |  | 91% |  |  |

Table 3 Coding efficiency comparison between Samsung new and TMuC-0.9 8-tap DCT-IF filter (normal mode, negative numbers mean that Samsung new filter has gain)

For detailed results please refer to

Results_anchor6tapDCTIF_vs._Samsung8tap.xls

# Conclusions

In this document the performance of Samsung new 6-tap/8-tap DCT-IF filter is verified. Additional data is provided to highlight the coding efficiency benefit of Samsung new 6-tap/8-tap DCT-IF filter compared to the TMuC-0.9 6-tap/8-tap DCT-IF filter.

# References

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-C500, 3rd. JCT-VC Meeting, Guangzhou, CN, October 2010.