| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D062 |
| --- | --- |

| Title: | CE11: Cross-check report from HHI for TI's proposal JCTVC-C227 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | T. Nguyen, D. Marpe, H. Schwarz, T. Wiegand Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49-30-31002-230 tung.nguyen@hhi.fraunhofer.de detlev.marpe@hhi.fraunhofer.de heiko.schwarz@hhi.fraunhofer.de thomas.wiegand@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports a cross-check results of TI’s proposal [1] on parallelization of transform coding in high efficiency configuration. Compared to the anchor, the evaluation of some neighbor in the local template of context model selection for the significance flag is disabled. ADBCH and J denotes as B and C in [1].

# Simulation Environments

## Software Configuration

The code base for the core experiment is TMuC 0.9 software. The evaluation of A, D, B, C, H and J denoted in the code below for context model selection of the significance flag are disabled.

#define TI_REMOVE_AD  1                     // Remove dependency on AD (remove 2)

#define TI_REMOVE_BCHJ  1                   // Remove dependency on BCHJ (remove 4)

UInt uiCtxInc = 0; UInt uiSizeM1 = ( 1 << uiLog2BlkSize ) - 1; if( uiLog2BlkSize <= 3 ) { UInt uiShift = max<UInt>( 0, uiLog2BlkSize - 2 ); uiCtxInc = ( ( uiPosY >> uiShift ) << 2 ) + ( uiPosX >> uiShift ); } else if( uiPosX <= 1 && uiPosY <= 1 ) { uiCtxInc = ( uiPosY << 1 ) + uiPosX; } else if( uiPosY == 0 ) { const int* pData = &pcCoeff[ uiPosX + uiPosY * uiStride ]; int iStride = uiStride; UInt uiCnt = ( pData[ -1 ] ? 1 : 0 ); //I #if (!TI_REMOVE_BCHJ) uiCnt += ( pData[ -2 ] ? 1 : 0 ); //H uiCnt += ( pData[ iStride -2 ] ? 1 : 0 ); //J #endif if( ! bDownLeft ) { uiCnt += ( pData[ iStride -1 ] ? 1 : 0 ); //K } uiCtxInc = 4 + ( ( uiCnt + 1 ) >> 1 ); } else if( uiPosX == 0 ) { const int* pData = &pcCoeff[ uiPosX + uiPosY * uiStride ]; int iStride = uiStride; int iStride2= iStride << 1; UInt uiCnt = ( pData[ -iStride ] ? 1 : 0 ); //F #if (!TI_REMOVE_BCHJ) uiCnt += ( pData[ -iStride2 ] ? 1 : 0 ); //B uiCnt += ( pData[ 1-iStride2 ] ? 1 : 0 ); //C #endif if( bDownLeft ) { uiCnt += ( pData[ 1-iStride ] ? 1 : 0 ); //G } uiCtxInc = 7 + ( ( uiCnt + 1 ) >> 1 ); } else { const int* pData = &pcCoeff[ uiPosX + uiPosY * uiStride ]; int iStride = uiStride; int iStride2= iStride << 1; UInt uiCnt = ( pData[ -iStride ] ? 1 : 0 ); //F uiCnt += ( pData[ -1 ] ? 1 : 0 ); //I uiCnt += ( pData[ -1 -iStride ] ? 1 : 0 ); //E #if (!TI_REMOVE_BCHJ) if( uiPosX > 1 ) { uiCnt += ( pData[ -2 ] ? 1 : 0 ); //H #if (!TI_REMOVE_AD) uiCnt += ( pData[ -2 -iStride ] ? 1 : 0 ); //D #endif if( uiPosY < uiSizeM1 ) { uiCnt += ( pData[ -2 +iStride ] ? 1 : 0 ); //J } } if( uiPosY > 1 ) { uiCnt += ( pData[ -iStride2 ] ? 1 : 0 ); //B #if (!TI_REMOVE_AD) uiCnt += ( pData[ -1 -iStride2 ] ? 1 : 0 ); //A #endif if( uiPosX < uiSizeM1 ) { uiCnt += ( pData[ 1 -iStride2 ] ? 1 : 0 ); //C } } #endif if( bDownLeft ) { if( uiPosX < uiSizeM1 ) { uiCnt += ( pData[ 1 -iStride ] ? 1 : 0 ); //G } } else { if( uiPosY < uiSizeM1 ) { uiCnt += ( pData[ -1 +iStride ] ? 1 : 0 ); //K } } uiCtxInc = 10 + min<UInt>( 4, ( uiCnt + 1 ) >> 1 ); } return uiCtxInc;

## Hardware Configuration

# Results

The results match exactly those reported by TI. Positive numbers indicate a bit rate rising while negative numbers indicate a bit rate saving.

# Remarks

The proposed wavefront scan described in [1] was not included in the cross-check.

# References

- V. Sze, M. Budagavi, “Parallelization of HHI_TRANSFORM_CODING”, JCTVC-C227, Guangzhou, October, 2010.

# Patent rights declaration(s)

Fraunhofer HHI may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).