| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D063 WG11 Number: m18811 |
| --- | --- |

| Title: | CE10: Cross-check report from INRIA on number of intra prediction directions |  |  |
| --- | --- | --- | --- |
| Status: | input Document |  |  |
| Purpose: | Core Experiment report |  |  |
| Author(s) or Contact(s): | Laurent Guillo, Ronan Boitard | Tel: Email: | +33-2-99-84-71-00 laurent.guillo@inria.fr ronan.boitard@inria.fr |
| Source: | INRIA, FRANCE |  |  |

_____________________________

# Abstract

This contribution is a report of core experiment 10 (CE10) on number of intra prediction directions and run by INRIA. There are 12 test cases in CE10 as described in JCTVC-C310. Those test cases are evaluated based on the common conditions in JCTVC-C500.

# Introduction

An increased number of directions for intra prediction was introduced in the TMuC and will be integrated into the HEVC Test Model [1]. The purpose of this CE is to characterize the performance and complexity of using different numbers of intra prediction directions. The characterization was performed by investigating the coding efficiency improvement in terms of BD-BR [2] performance, and by determining the complexity increase for both encoding and decoding using variety of combinations of intra prediction directions for each block size.

# Test points

The test points are identified in the table below with respect to the number of intra prediction directions up to the one defined for the HEVC Test Model default configurations [3]. The test has been conducted using TMuC0.9 with modifications to realize the limitation of number of intra prediction directions.

Table 1. Performance Tests for different combinations of number of intra prediction directions

| Block size | Number of Intra Prediction Directions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Reference | Test |  |  |  |  |  |  |  |  |  |  |
|  |  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 |
| 4x4 | 17 | 17 | 9 | 9 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 9 |
| 8x8 | 34 | 17 | 9 | 34 | 17 | 9 | 34 | 34 | 34 | 34 | 34 | 17 |
| 16x16 | 34 | 17 | 9 | 34 | 34 | 34 | 17 | 9 | 34 | 34 | 34 | 17 |
| 32x32 | 34 | 17 | 9 | 34 | 34 | 34 | 34 | 34 | 17 | 9 | 34 | 17 |
| 64x64 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 3 | 5 |

Test #12: To evaluate the upper limit of the performance of the intra prediction method of the test model,

the reference without fast mode decisions, which means that all intra prediction directions are evaluated using RD cost, is also tested.

Table2. Default configurations in TComRom.cpp file

| Default configurations | #if UNIFIED_DIRECTIONAL_INTRA const UChar g_aucIntraModeNumAng[7] = { 3, // 2x2 17, // 4x4 34, // 8x8 34, // 16x16 34, // 32x32 5, // 64x64 5 // 128x128 }; const UChar g_aucIntraModeBitsAng[7] = { 2, // 2x2 5, // 4x4 6, // 8x8 6, // 16x16 6, // 32x32 3, // 64x64 3 // 128x128 }; |
| --- | --- |

Table3. value for g_aucIntraModeBitsAng

| Number of Directions | 3 | 5 | 9 | 17 | 34 |
| --- | --- | --- | --- | --- | --- |
| Number of bits | 2 | 3 | 4 | 5 | 6 |

INRIA took part to the 12 test cases.

# Experimental Conditions

## Software

TMuC0.9 has been used with modification for the CE tests. The modifications for test cases #1 to #9 and #11 are only modifications of the tables shown in Table 3 and 4 based on Table 1.

For the test case #10, as maximum index for 64x64 intra prediction is 3, mode mapping table for most probable mode calculation has been modified to limit the mapped mode less than 3.

For the test case #12, the best intra prediction mode is selected by evaluating R-D cost for all modes.

The patch for these modifications is attached to this contribution.

INRIA received the source code from Mitsubishi and successfully compiled it using Linux (Fedora core 11 64 bits) with gcc 4.4.1 .

Resources and CPU used for each class are listed in table below:

Table 4. Resources and CPU sorted by class

| Class | CPU | Frequency | RAM |
| --- | --- | --- | --- |
| A | BiXeon, 4 core | 2.33 | 4 |
| B | BiXeon, 4 core | 2.33 | 2 |
| C | BiXeon, 4 core | 1.60 | 1 |
| D | Bipro Intel Xeon | 2.33 | 1 |
| E | BiXeon, 4 core | 2.33 | 1 |

## Test Sequences, Bit Rates and Coding Conditions

Test conditions are the intra-only configuration on both High Efficiency and Low Complexity conditions and test sequences as defined in C500[3], using the reference configuration files also provided in [3].

# Test Results

The following table summarizes the obtained results for the High Efficiency and Low Complexity profiles:

Table 5. Test results on the common test sequences

|  | Block size and number of directions |  |  |  |  | HE |  | LC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4x4 | 8x8 | 16x16 | 32x32 | 64x64 | BDrate | Enc Time | BDrate | Enc Time |
| Reference | 17 | 34 | 34 | 34 | 5 |  |  |  |  |
| Test#1 | 17 | 17 | 17 | 17 | 5 | 1,803 | 94,81% | 1.903 | 88,84% |
| Test#2 | 9 | 9 | 9 | 9 | 5 | 5,123 | 87,70% | 6.320 | 77,43% |
| Test#3 | 9 | 34 | 34 | 34 | 5 | 0,626 | 95,00% | 1.135 | 95,24% |
| Test#4 | 17 | 17 | 34 | 34 | 5 | 0,553 | 96,10% | 1.174 | 94,91% |
| Test#5 | 17 | 9 | 34 | 34 | 5 | 1,157 | 94,65% | 2.217 | 90,67% |
| Test#6 | 17 | 34 | 17 | 34 | 5 | 0,435 | 98,06% | 0.341 | 94,32% |
| Test#7 | 17 | 34 | 9 | 34 | 5 | 0,673 | 95,92% | 0.489 | 93,31% |
| Test#8 | 17 | 34 | 34 | 17 | 5 | 0,110 | 98,37% | 0.005 | 95,41% |
| Test#9 | 17 | 34 | 34 | 9 | 5 | 0,129 | 98,85% | -0.027 | 94,41% |
| Test#10 | 17 | 34 | 34 | 34 | 3 | -0,004 | 95,75% | 0.000 | 95,11% |
| Test#11 | 9 | 17 | 17 | 17 | 5 | 2,581 | 89,79% | 3.433 | 86,04% |
| Test#12 | 17 | 34 | 34 | 34 | 5 | -0,206 | 203,78% | -0.708 | 214,36% |

The results obtained by INRIA perfectly match the results provided by Mitsubishi.

# Analysis on the results

A first general analysis is that reducing the number of directions almost never improves the BDrate while the encoding complexity is almost still the same.

However, a more detailed analysis shows that:

- Reducing the number of directions (from 5 to 3) for 64x64 blocks induces a small BDrate gain and a lower complexity,

- Reducing the number of directions (from 34 to 17 or 9) for 32x32 blocks has two different consequences depending on the profile used. Performances are quite the same for Low Complexity however it saves 5% of time. Regarding the High Efficiency, it saves only 2% of time meanwhile it causes 0.1% bit increase.

- Regarding all the other sizes, reducing the number of directions significantly decreases performances while saving only 4% encoding time in average.

The first two points deserve to be further investigated.

The last test (test 12) which disables the fast mode decision, leads to a small BDrate gain but with a very significant encoding time increase.

# Conclusion

In CE10, INRIA has evaluated 12 test cases based on the common test conditions. The results are summarized according to coding efficiency and complexity. It is suggested to consider these results to decide the number of intra prediction directions when creating the next version of HM.

# References

- T. Wiegand, “HEVC text specification Draft 1”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C403, 3rd Meeting: Guangzhou, CN, Oct., 2010.

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

- F. Bossen, “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C500, 3rd Meeting: Guangzhou, CN, Oct., 2010.

- K. Iguchi, “Performance report of TMuC for Super Hi-Vision”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C055, 3rd Meeting: Guangzhou, CN, Oct., 2010.

# Annex A: Detailed test results on the common test sequences

Table 6. Test Results

| TEST#1 | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 2,4 | 2,6 | 2,7 | 2,5 | 3,3 | 3,3 |
| Class B | 1,8 | 1,8 | 2,1 | 1,9 | 2,7 | 2,9 |
| Class C | 1,6 | 1,5 | 1,5 | 1,7 | 2,2 | 2,3 |
| Class D | 1,1 | 1,1 | 1,2 | 1,0 | 1,4 | 1,4 |
| Class E | 2,7 | 3,0 | 2,6 | 3,1 | 4,1 | 3,8 |
| All | 1,8 | 1,9 | 1,9 | 1,9 | 2,6 | 2,6 |
| Enc Time[%] | 95% |  |  | 89% |  |  |
| Dec Time[%] | 97% |  |  | 99% |  |  |
| TEST#2 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 4,9 | 4,9 | 5,2 | 5,7 | 5,9 | 5,8 |
| Class B | 4,7 | 4,5 | 5,0 | 5,6 | 6,4 | 6,7 |
| Class C | 5,3 | 5,2 | 5,0 | 6,6 | 6,7 | 6,8 |
| Class D | 4,1 | 4,0 | 4,1 | 4,8 | 4,8 | 4,9 |
| Class E | 7,1 | 7,6 | 7,1 | 9,6 | 9,8 | 9,9 |
| All | 5,1 | 5,1 | 5,2 | 6,3 | 6,6 | 6,8 |
| Enc Time[%] | 88% |  |  | 77% |  |  |
| Dec Time[%] | 97% |  |  | 98% |  |  |
| TEST#3 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0,4 | 0,2 | 0,2 | 0,8 | 0,1 | 0,1 |
| Class B | 0,3 | 0,2 | 0,1 | 0,7 | 0,1 | 0,1 |
| Class C | 0,8 | 0,4 | 0,4 | 1,4 | 0,4 | 0,4 |
| Class D | 1,1 | 0,6 | 0,6 | 1,7 | 0,6 | 0,6 |
| Class E | 0,5 | 0,2 | 0,2 | 1,0 | 0,2 | 0,3 |
| All | 0,6 | 0,3 | 0,3 | 1,1 | 0,3 | 0,3 |
| Enc Time[%] | 95% |  |  | 95% |  |  |
| Dec Time[%] | 96% |  |  | 95% |  |  |
| TEST#4 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0,7 | 0,7 | 0,6 | 1,2 | 0,6 | 0,4 |
| Class B | 0,4 | 0,3 | 0,4 | 1,1 | 0,8 | 0,8 |
| Class C | 0,6 | 0,7 | 0,7 | 1,2 | 1,3 | 1,2 |
| Class D | 0,5 | 0,5 | 0,6 | 0,8 | 0,9 | 0,9 |
| Class E | 0,6 | 0,6 | 0,5 | 1,7 | 0,6 | 0,7 |
| All | 0,6 | 0,5 | 0,5 | 1,2 | 0,9 | 0,9 |
| Enc Time[%] | 96% |  |  | 95% |  |  |
| Dec Time[%] | 95% |  |  | 97% |  |  |
| TEST#5 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1,3 | 1,3 | 1,3 | 2,2 | 1,2 | 0,8 |
| Class B | 0,9 | 0,8 | 0,8 | 2,1 | 1,5 | 1,6 |
| Class C | 1,3 | 1,4 | 1,5 | 2,2 | 2,2 | 2,3 |
| Class D | 1,1 | 1,2 | 1,3 | 1,6 | 1,7 | 1,9 |
| Class E | 1,4 | 1,2 | 1,1 | 3,3 | 1,0 | 1,5 |
| All | 1,2 | 1,1 | 1,2 | 2,2 | 1,6 | 1,7 |
| Enc Time[%] | 95% |  |  | 91% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |
| TEST#6 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0,7 | 0,8 | 1,0 | 0,7 | 1,6 | 1,8 |
| Class B | 0,4 | 0,4 | 0,5 | 0,3 | 0,7 | 0,8 |
| Class C | 0,4 | 0,3 | 0,3 | 0,2 | 0,4 | 0,5 |
| Class D | 0,2 | 0,2 | 0,2 | 0,1 | 0,3 | 0,2 |
| Class E | 0,7 | 1,0 | 0,7 | 0,7 | 1,8 | 1,6 |
| All | 0,4 | 0,5 | 0,5 | 0,3 | 0,8 | 0,8 |
| Enc Time[%] | 98% |  |  | 94% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |
| TEST#7 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0,9 | 1,0 | 1,3 | 0,7 | 2,1 | 2,3 |
| Class B | 0,6 | 0,6 | 0,8 | 0,4 | 1,1 | 1,2 |
| Class C | 0,6 | 0,5 | 0,5 | 0,3 | 0,8 | 0,8 |
| Class D | 0,3 | 0,4 | 0,3 | 0,1 | 0,4 | 0,4 |
| Class E | 1,2 | 1,6 | 1,2 | 1,2 | 3,4 | 2,9 |
| All | 0,7 | 0,8 | 0,7 | 0,5 | 1,4 | 1,3 |
| Enc Time[%] | 96% |  |  | 93% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |
| TEST#8 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0,1 | 0,2 | 0,2 | 0,0 | 0,1 | 0,1 |
| Class B | 0,1 | 0,3 | 0,2 | 0,0 | 0,1 | 0,2 |
| Class C | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 |
| Class D | 0,0 | 0,1 | 0,0 | 0,0 | 0,0 | 0,0 |
| Class E | 0,3 | 0,4 | 0,4 | 0,1 | 0,4 | 0,4 |
| All | 0,1 | 0,2 | 0,2 | 0,0 | 0,1 | 0,1 |
| Enc Time[%] | 98% |  |  | 95% |  |  |
| Dec Time[%] | 96% |  |  | 97% |  |  |
| TEST#9 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0,1 | 0,2 | 0,2 | 0,0 | 0,1 | 0,1 |
| Class B | 0,1 | 0,3 | 0,3 | -0,1 | 0,2 | 0,2 |
| Class C | 0,0 | 0,1 | 0,0 | 0,0 | 0,0 | 0,0 |
| Class D | 0,0 | 0,1 | 0,0 | 0,0 | 0,0 | 0,0 |
| Class E | 0,4 | 0,5 | 0,5 | 0,0 | 0,5 | 0,4 |
| All | 0,1 | 0,2 | 0,2 | 0,0 | 0,1 | 0,2 |
| Enc Time[%] | 99% |  |  | 94% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |
| TEST#10 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0,0 | 0,0 | -0,1 | 0,0 | 0,0 | 0,0 |
| Class B | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 |
| Class C | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 |
| Class D | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 |
| Class E | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 |
| All | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 | 0,0 |
| Enc Time[%] | 96% |  |  | 95% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |
| TEST#11 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 2,9 | 2,8 | 3,0 | 3,5 | 3,5 | 3,5 |
| Class B | 2,2 | 2,1 | 2,3 | 2,8 | 3,0 | 3,1 |
| Class C | 2,6 | 2,1 | 2,1 | 3,6 | 3,0 | 3,0 |
| Class D | 2,3 | 1,9 | 1,9 | 3,0 | 2,3 | 2,3 |
| Class E | 3,4 | 3,4 | 3,0 | 4,7 | 4,5 | 4,4 |
| All | 2,6 | 2,3 | 2,4 | 3,4 | 3,1 | 3,2 |
| Enc Time[%] | 90% |  |  | 86% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  |
| TEST#12 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0,2 | 0,2 | 0,2 | -1,0 | -0,1 | -0,2 |
| Class B | -0,3 | 0,0 | 0,0 | -1,0 | -0,1 | -0,2 |
| Class C | -0,1 | 0,3 | 0,2 | -0,4 | 0,0 | 0,0 |
| Class D | -0,1 | 0,3 | 0,3 | -0,4 | 0,0 | 0,0 |
| Class E | -0,4 | 0,1 | 0,0 | -0,9 | -0,5 | -0,4 |
| All | -0,2 | 0,2 | 0,1 | -0,7 | -0,1 | -0,1 |
| Enc Time[%] | 204% |  |  | 214% |  |  |