| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D070 WG11 Number: m18820 |
| --- | --- |

| Title: | Lightweight slicing for entropy coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Jie Zhao and Andrew Segall 5750 Pacific Rim Blvd Camas WA 98607 | Tel: Email: | +1-360-817-8487 misrak@sharplabs.com, jzhao@sharplabs.com, asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

We propose a method for lightweight slicing of entropy coded data, a system that is often referred to as “entropy slices”.  The goal of entropy slices is to provide a simple and direct extension of the HM design to evolving CPU/GPU architectures.  In this scenario, the entropy decoding and reconstruction operations are typically assigned to the CPU and GPU, respectively, and we advocate flexibility in the slicing to address this anticipated use case.  Specifically, we advocate dividing a traditional slice into additional entropy slices that are can be decoded without reference to other entropy slices.  This enables the use of multi-core CPUs to handle the entropy decoding process in an efficient manner.  While the entropy slice system is initially motivated by this use case, we observe that it is useful for many scenarios – including homogeneous decoder architectures.  Here, we describe the design of the entropy slice system and report current results.

- Introduction

Entropy slices extends the slicing concept of the HM to support slicing at both the entropy decoding and reconstruction levels.  To be clear, the current HM slices divide a picture into regiosn that can be completely parsed and reconstructed without reference to other slice data.  The proposed entropy slice concept further divides a slice into units that can be completely parsed without reference to other slices – but reconstruction may require reference to other slice data.

An initial motivator for the entropy slice concept is the evolving use of CPU/GPU architectures for programmable systems. We assert that these architectures must be well supported in the new standard.  Of special interest here is the fact that the entropy parsing and reconstruction processes will be done on the CPU and GPU, respectively.  Moreover, we observe that CPU designs continue to use multi-core architectures, and that the number of cores continues to increase.  This is true for all segments of the market, but it is especially true for high resolution applications that use high resolution displays – a  key target of HEVC.

To address the use case of decoding a high resolution (and high bit-rate) bit-stream with a CPU/GPU system, it is critical that the entropy decoding process be parallelizable within a multi-core CPU.  To achieve this, we advocate slicing the bit-stream to create a large number of entropy slices, with each entropy slice having two important properties.  First, the entropy slice should have bounded complexity.  This allows for meaningful load balancing between parsing engines.  Second, the entropy slice should be completely independent of other entropy slices.  This allows for efficient use of the multi-core system, as dependence between slices (and thus cores) significantly decrease performance.

To achieve these goals, we propose a slicing system that achieves these two, key requirements.  Entropy slices are independent and bounded in the number of bins.  To address the coding efficiency loss introduced by these slices, we advocate the use of a smaller slice header (to reduce overhead).  Additionally, this improved coding efficiency simplifies encoder design, as it is much less critical for an encoder to maximize the size of the payload within an entropy slice.

- Extensions to other scenarios

We conceived the entropy slice concept from the point of view of a CPU/GPU implementation.  As we have discussed, we anticipate the use of the CPU for entropy decoding and the GPU for reconstruction.  By way of example, we show this scenario graphically.  In Figure 1, we illustrate the parsing of a current HM bit-stream.  Here, you can see that the bit-stream is parsed by a single core in the system.  In Figure 2, we illustrate the parsing of the proposed bit-stream, where the entropy slices are assigned to different cores.  The use of the multi-core system improves the throughput of the entropy decoding engine, which we have verified through simulations.

Figure 1. Mapping of a current HM slice to a CPU/GPU architecture.  Note that only one core of the system can be used for entropy coding.

Figure 2. Mapping of an HM slice with entropy slices to a CPU/GPU architecture.  Note that all cores of the system are now used for entropy coding.

While we originally designed the entropy slice system for a CPU/GPU scenario, we observe that the entropy slice approach is well suited for other, homogeneous architectures.  Here though, it is necessary to consider the data that is generated by the parallel, entropy decoding stage.  Due to the parallel processing, the data is not decoded in raster-scan order and should be buffered prior to reconstruction.   In a naïve approach, the decoded symbols could be buffered explicitly.  This obviously results in significant memory bandwidth.  In more practical systems though, we treat the entropy decoding stage as a re-writing process.  Here, we take the bit-stream as input, parse the data, and generate an internal bit-stream as output.  This internal bit-stream still employs entropy coding, but with lower complexity (e.g., CAVLC, ECLC, or another proprietary solutions).  Additionally, the internal format can contain additional meta-data to support architecture specific needs.  For example, it can be targeted to a specific form of parallelization as well as the specific degree of parallelization within the homogeneous platform.  In our testing, we observe that it is possible to re-write the HM bit-stream to a low-complexity, internal format with approximately 10-20% bit-rate increase.  We assert that this is relatively small.  For example, an 80Mbit/s, 4k-by-2k bit-stream would result in a ~100Mbit/s.  By comparison, the transfer of the frame data requires approximately 4Gbit/s (assuming that it is 8-bit)

A visual example of the homogenous architecture appears in Figure 3.

Figure 3. Use of entropy slices in a homogeneous architecture.

In addition to heterogeneous and homogeneous decoding, we also observe that entropy slices are well suited for trans-coding and trans-rating within multi-core architectures.  Here, it is not necessary to fully reconstruct the decoded output, and so the complexity of the process is largely defined by the entropy decoding stage.

- Additional benefits

In previous sections, we asserted that the entropy slice system has benefits for both heterogeneous and homogeneous decoder architectures, as well as common trancoding/transrating applications.  Here, we summarize additional benefits of the entropy slice system.

As a first additional benefit, we observe that the entropy slice concept results in relatively small changes in the current HM slice system.  Specifically, entropy slices use a smaller header and also a second definition for neighborhood definition in the decoding phase.  We assert that this similarity to the current design is a strong benefit, as different implementations/architectures have different needs for parallelization.  In systems that are capable of parsing bit-streams with a serial (or nearly serial) process, then entropy slices can be treated as a traditional slice within the parsing engine.

A second additional benefit of the entropy slice concept is that it does not require targeting a specific degree of decoder parallelization within the bit-stream.  As will be shown in the results section, it is straightforward to generate a bit-stream for 1080p resolution that contains 32 entropy slices and with negligible impact on coding efficiency.  Each of these entropy slices is restricted to include no more than a fixed number of bins, and thus, we assert they have a bounded decoding complexity.  In this case, the 32 entropy slices may be processed by 32 entropy decoders in parallel, with each entropy decoder capable of processing a known number of bins, N, per frame.  Similarly though, the bit-stream may be processed with 16 entropy decoders with each entropy decoder able to process 2*N number of bins per frame.  In this case, each entropy decoder would process two entropy slices serially.  As yet another example, the bit-stream may be processed with 4 entropy decoders, each able to process 8*N bins per frame.  We suspect that it is clear to the reader that the system can be scaled to any target configuration, but we mention that the number of entropy slices does not need to be an integer factor of the number of entropy decoders.  For example, the bit-stream may be processed with 6 entropy decoder, each able to process 6*N bins per frame.  In this case, not all decoders will process the same number of entropy slices.

A third benefit of the entropy slice concept is that it enables straightforward specification of conformance points.  Specifically, by defining the maximum number of bins within an entropy slice in addition to the maximum number of bins within a frame, the JCT-VC can specify the degree of parallelism clearly for both encoder and decoder conformance.

# Syntax

Here, we propose syntax and semantics for the entropy slice system

### Slice header syntax

| slice_header( ) { | C | Descriptor |
| --- | --- | --- |
| first_lctb_in_slice | 2 | ue(v) |
| entropy_slice_flag | 2 | u(1) |
| if(!entropy_slice_flag) { |  |  |
| slice_type | 2 | ue(v) |
| pic_parameter_set_id | 2 | ue(v) |
| Frame_num | 2 | u(v) |
| if( IdrPicFlag ) |  |  |
| idr_pic_id | 2 | ue(v) |
| pic_order_cnt_lsb | 2 | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |  |
| Num_ref_idx_active_override_flag | 2 | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |  |
| num_ref_idx_l0_active_minus1 | 2 | ue(v) |
| If( slice_type = = B ) |  |  |
| num_ref_idx_l1_active_minus1 | 2 | ue(v) |
| } |  |  |
| } |  |  |
| if( nal_ref_idc != 0 ) |  |  |
| Dec_ref_pic_marking( ) | 2 |  |
| if( entropy_coding_mode_flag && slice_type != I ) |  |  |
| cabac_init_idc | 2 | ue(v) |
| slice_qp_delta | 2 | se(v) |
| alf_param() |  |  |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |  |
| mc_interpolation_idc | 2 | ue(v) |
| mv_competition_flag | 2 | u(1) |
| if ( mv_competition_flag ) { |  |  |
| mv_competition_temporal_flag | 2 | u(1) |
| } |  |  |
| } |  |  |
| if ( slice_type = = B && mv_competition_flag) |  |  |
| collocated_from_l0_flag | 2 | u(1) |
| } |  |  |
| else { |  |  |
| if( entropy_coding_mode_flag && slice_type != I ) |  |  |
| cabac_init_idc | 2 | ue(v) |
| } |  |  |

entropy_slice_flag specifies if the slice is an entropy slice.  entropy_slice_flag equal to 1 denotes that the entropy decoding process shall use an alternative definition of neighbor availability than the reconstruction process.  entropy_slice_flag equal to 1 denotes that the entropy decoding and reconstruction processes shall use the same definition of neighbor availability.

# Results

We implemented the proposed entropy slice concept in high efficiency video coding test model (HM) version 0.9. See attached excel file. To configure entropy slices we used the parameters described in our previous proposal JCTVC-C256[2]. We then coded all sequences using the configuration files described in JCTVC-C500 [3]. Full results are available in the attached XLS sheets. However, the summary is as follows:

|  |  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
|  | Class A | 0.1 | 0.0 | 0.0 | 0.4 | 0.2 | 0.1 |
|  | Class B | 0.1 | 0.1 | 0.0 | 0.4 | -0.1 | -0.1 |
|  | Class C | 0.1 | 0.0 | 0.0 | 0.2 | 0.1 | 0.0 |
|  | Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | Class E | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
|  | All | 0.1 | 0.0 | 0.0 | 0.2 | 0.0 | 0.0 |
|  | Enc Time[%] |  |  |  |  |  |  |
|  | Dec Time[%] |  |  |  |  |  |  |
|  |  |  | Random access |  |  | Random access LoCo |  |
|  |  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
|  | Class A | 0.2 | 0.8 | 0.8 | 0.3 | 0.3 | 0.2 |
|  | Class B | 0.2 | 0.7 | 0.3 | 0.1 | 0.0 | 0.0 |
|  | Class C | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 |
|  | Class D | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 |
|  | Class E |  |  |  |  |  |  |
|  | All | 0.1 | 0.4 | 0.3 | 0.1 | 0.1 | 0.1 |
|  | Enc Time[%] |  |  |  |  |  |  |
|  | Dec Time[%] |  |  |  |  |  |  |
|  |  | Low delay |  |  | Low delay LoCo |  |  |
|  |  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
|  | Class A |  |  |  |  |  |  |
|  | Class B | 0.1 | 0.5 | -0.2 | 0.2 | 0.0 | 0.0 |
|  | Class C | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | Class D | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
|  | Class E | 0.0 | -0.2 | -0.2 | 0.1 | 0.2 | 0.0 |
|  | All | 0.1 | 0.2 | -0.1 | 0.1 | 0.0 | 0.0 |
|  | Enc Time[%] |  |  |  |  |  |  |
|  | Dec Time[%] |  |  |  |  |  |  |

# Software

Software is provided with the proposal.

# Conclusions

We propose an entropy slice concept that enables separate definitions for slices for the entropy decoding and reconstruction processes.  We assert that the system benefits CPU/GPU decoders, homogeneous decoders, and transcoders/transraters.  Moreover, we assert that the system does not provide undue burden for decoders that are capable of decoding target bit-streams without additional parallelization.  Entropy slices are designed to be efficient, and current results show a coding efficiency impact of between 0.1% and 0.2%, on average with 32x parallelism for 1080p data.

We propose inclusion of the entropy slice concept in the HM design.

# Patent rights declaration(s)

SHARP may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

[2] Kiran Misra, Jie Zhao, Andrew Segall, “Entropy slices for parallel entropy coding,” JCTVC-C256 / m18297, 3rd Meeting: Guangzhou, CN, 7-15 October, 2010.

[3] Frank Bossen, "Common test conditions and software reference configurations," JCTVC-C500 / m18607, 3rd Meeting: Guangzhou, CN, 7-15 October, 2010.

Note that the definition of the loop filter may use data from neighboring slices in the current design.