| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D071 WG11 Number: m18821 |
| --- | --- |

| Title: | On transform dynamic range |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Louis Kerofsky, Andrew Segall 5750 Pacific Rim Blvd Camas WA 98607 | Tel: Email: | +1-360-817-8487 misrak@sharplabs.com, lkerofsky@sharplabs.com, asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

The dynamic range of an inverse transform is a critical parameter influencing the minimum width of the bus required between the processing unit which carries out the transform and the memory used to store results. Additionally it determines the total size of the memory used for storage and number of coefficients which can be pre-fetched with a single instruction. In this proposal we outline a technique which reduces the required transform dynamic range. More specifically we demonstrate the efficacy of the technique outlined to achieve a reduced dynamic range of 16-bits. This goal is achieved with an increase in the average Y Bjøntegaard Delta (BD) bitrate [1] between 0.0%. and -0.2%. The performance was measured with all the tools in their default test settings, for e.g. there is an internal bit depth increment of 4 for the high-performance case, the transform precision extension is turned ON for the low-complexity case etc. The transform dynamic range is achieved by restricting the bit-depth of coefficients being stored in memory (a) prior to being input to the first inverse 1-D transform and (b) at the output of the first 1-D inverse transform. A combination of QP period based scaling, clipping and transform size dependent bit shifts are used to achieve the 16-bit dynamic range.

- Introduction

A bit-depth limit may be placed on the transform dynamic range for hardware with constrained bus-width and/or memory storage-fetch limitations. In such setups it is highly desirable to have the ability to reduce the required transform dynamic range without giving up too much performance. The variability in transform dynamic range for different transforms has been illustrated earlier in JCTVC-C056 [2], JCT-K031 [3], In JCTVC-C056 [2] it was shown that the transform dynamic range for the inverse transforms at QP=0 is greater than 16-bits. In this proposal we outline a technique which reduces the required transform dynamic range to 16-bits with negligible increase in BD bitrate.

Figure 1 is a generalized block diagram representation of the dequantization and the inverse transform in the decoder. The stages labeled “Memory Storage” identifies the location of where the current proposal restricts the bit-depth of coefficients. We now detail the parameters used in Figure 1 and through the rest of the document:

QP			: Quantization Parameter

Remainder	: (QP + 6*BitIncrement) % 6

Period 		: (QP + 6*BitIncrement) / 6

Figure 1: Generalized block diagram for the dequantization and inverse transform stages along with labeling of locations where coefficient bit-depth has to be restricted for memory storage.

BitIncrement: Bit depth Increment

A			: is a function of transform size and Period

B			: is a function of transform size

Descale		: is a function of Remainder and transform size

- Proposed Approach

In this proposal we consider a 16-bit dynamic range bit-limit on the transform. The simplest approach to achieve this desired dynamic range would be to clip the coefficient data to 16-bits at the highlighted locations. However such a simplistic approach results in significant rate-distortion performance loss. We therefore propose the following approach which minimizes the overall rate-distortion loss while achieving the 16-bit dynamic range.

We first consider restricting the bit-depth of the coefficients being input to the inverse transform. As seen in Figure 1, the dequantizer up-scales values by a power of 2 using bit shifts. We exploit this power of 2(Period+B) operation and modify the memory storage to be executed prior to the up-scaling as shown in Figure 2. Any coefficient data which is not within the 16-bit limit prior to up-scaling is simply clipped to fit within the desired bit-depth. Moving the up-scaling to be executed after the memory storage requires that we store the associated “Period” in memory along with the transform coefficients. As can be seen the actual implementation is slightly different for an 8x8 inverse dequantizer as compared to the others.

Before executing the inverse transform the coefficient data is first up-sampled using the original up-sampling information which was to be used within the dequantizer..ficient data is first up-sampled based on the original up-sampling which was to be used within the de-quanitzer.ignificant 0 Next we consider restricting the bit-depth of the coefficient data which is output by the first 1-D inverse transform. To ensure bit-depth conformance we right bit shift the output of the first 1-D inverse transform and clip it to 16-bits prior to memory storage. This right bit shift is dependent on the transform size.

Figure 3: Performing transform size dependent right-shift and clip prior on the output of the first 1-D transform prior to memory storage; and left-shifting by the same amount before executing the second 1-D inverse transform

Before executing the second 1-D inverse transform the coefficient data fetched from the memory is left bit shifted (with zero padding) by the same amount it was right bit shifted by prior to storage. As a result of these bit shifts some of the least significant bits are lost. However, the overall impact on the rate-distortion performance is marginal. For random-access high-performance configuration with IBDI=4 the shifts for each inverse transform is as follows:

Figure 2: Performing memory storage prior to up-scaling by a power of 2 and clipping to achieve the 16-bit limit.

Right/Left bit shift 4x4 		: 3 bits

Right/Left bit shift 8x8 		: 2 bits

Right/Left bit shift 16x16 	: 8 bits

Right/Left bit shift 32x32 	: 9 bits

The shifts are determined based on the operating dynamic range of each transform and the target 16-bit dynamic range. Figure 3 illustrates the above described approach which is used to ensure that the bit-depth of the first 1-D inverse transform output conforms to the 16-bit limit.

Note, for the proposed approach the required bit-depth can exceed 16-bits within the 1-D inverse transform but this is acceptable in many architectures where the operations of the 1-D inverse transform are carried out within an arithmetic logical unit (ALU) which can support higher bit-depths internally.

# Results

We implemented the proposed transform dynamic range restriction algorithm in high efficiency model (HM) version 0.9. See attached excel file. To configure the encoder we use the parameters described in section 2 above. We then coded all sequences using the configuration files described in JCTVC-C500 [4]. Full results are available in the attached XLS sheets. However, the summary is as follows:

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 102% |  |  | 107% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.1 | 0.0 | 0.1 | 0.1 |
| Class B | 0.0 | 0.0 | -0.1 | 0.0 | 0.1 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 |
| Class D | 0.0 | 0.0 | 0.2 | 0.0 | -0.1 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 98% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 97% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.2 | -0.1 | 0.0 | 0.0 | -0.1 |
| Class C | 0.0 | -0.1 | 0.1 | 0.0 | -0.1 | 0.0 |
| Class D | 0.0 | -0.6 | 0.1 | 0.0 | 0.2 | 0.1 |
| Class E | -0.2 | 0.0 | -0.1 | -0.2 | -0.5 | -0.2 |
| All | 0.0 | -0.2 | 0.0 | 0.0 | -0.1 | 0.0 |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |

Note that for all sequence types the average impact on Y BD Bitrate is between 0.0%. and -0.2%. We assert that this is a negligible impact given that the resulting bitstream requires lower memory bus width and size. In some sequences there is a small improvement in coding performance.

# Conclusions

We proposed a method to reduce the transform dynamic range to 16-bits with negligible change in coding efficiency. As a result the memory bus width and size requirements of the codec are reduced.

# Patent rights declaration(s)

SHARP may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

[2] M. Zhou, V. Sze, “TE12: Evaluation of transform unit (TU) size,” JCTVC-C056 / m18079, 3rd Meeting: Guangzhou, CN, 7-15 October, 2010.

[3] Tao Chen, Jiuhuai Lu, Yoshiichiro Kashiwagi, Shinya Kadono, "Dynamic Range of Outcome of Simplified 8x8 Transform and Quantization/De-quantization," JVT-K031.

[4] Frank Bossen, "Common test conditions and software reference configurations," JCTVC-C500 / m18607, 3rd Meeting: Guangzhou, CN, 7-15 October, 2010.