| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D072 WG11 Number: m18822 |
| --- | --- |

| Title: | CE9: Reduced resolution storage of motion vector data |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yeping Su and Andrew Segall 5750 Pacific Rim Blvd Camas WA 98607 | Tel: Email: | +1-360-817-8487 asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

In the last meeting, we proposed to store the motion vector data from previous frames at lower resolution.  This data is used in the motion vector competition tool in the HM design, and we seek to reduce the resolution by a factor of four in order to reduce the storage requirements.  The technology was tested in the context of CE9, and we observe that memory reduction results in negligible coding efficiency loss.  Specifically, 16x memory size results in 0.4% bit-rate reduction for random access and 0.6-0.7% bit-rate reduction for low delay test configurations.

- Introduction

We report the performance of our previous proposal on storing motion vector data from previous frames at lower resolution.  Results appear in Section 2.  The original proposal is reproduced in Section 3.

# Results

We implemented the proposed method into the HM software.  We then coded all sequences using the high-performance configuration files described in CE9. Full results are available in the attached XLS sheets. However, the summary is as follows:

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.5 | 0.5 | 0.5 | 0.5 | 0.4 | 0.4 |
| Class B | 0.2 | 0.1 | 0.2 | 0.2 | 0.1 | 0.1 |
| Class C | 0.3 | 0.2 | 0.2 | 0.3 | 0.3 | 0.2 |
| Class D | 0.6 | 0.5 | 0.6 | 0.6 | 0.4 | 0.5 |
| Class E |  |  |  |  |  |  |
| All | 0.4 | 0.3 | 0.3 | 0.4 | 0.3 | 0.3 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.4 | 0.2 | 0.2 | 0.4 | 0.1 | 0.2 |
| Class C | 0.8 | 0.8 | 0.7 | 0.9 | 0.7 | 0.8 |
| Class D | 0.9 | 0.4 | 0.8 | 0.9 | 0.6 | 0.6 |
| Class E | 0.5 | 0.3 | 0.3 | 0.6 | 0.6 | 0.7 |
| All | 0.6 | 0.4 | 0.5 | 0.7 | 0.5 | 0.5 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

- Proposal

In this document, we propose a method to reduce the size of the motion vector buffer for storing temporal collocated vectors.  The size of the motion vector buffer is significant, considering the granularity of motion representation and that there are two vectors per motion block for B_SLICE.  For example, we currently estimate the buffer size to be approximately 26Mbits/frame for 4kx2x application, though this size will ultimately depend on the precision and maximum motion vectors supported.  (Here, we assume ¼ pel and limits used in levels 3.x, 4.x and 5.x of H.264/AVC.)

To reduce the size of the buffer, we propose to use decimation to store motion vectors on a larger granularity than the decoding granularity.  The motion vector buffer compression process replaces the decoded motion vector buffer with a lower resolution buffer, with each compressed vector calculated as the component-wise decimation of a block of MVBufferCompRatio-by-MVBufferCompRatio decoded vectors.  We propose to signal the ratio parameter MVBufferCompRatio in the sequence parameter set, though we anticipate the value to ultimately be defined by a profile/level.  The motion vector buffer compression process is performed after the adaptive loop filter process, and before the decoded picture is put into DPB.

# Syntax and Semantics

### Sequence parameter set syntax

| seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| ..... |  |  |
| motion_vector_buffer_comp_flag | 0 | u(1) |
| if ( motion_vector_buffer_comp_flag ) |  |  |
| motion_vector_buffer_comp_ratio_log2 | 0 | u(8) |
| rbsp_trailing_bits( ) | 0 |  |
| } |  |  |

Semantics

motion_vector_buffer_comp_flag equals to 1 specifies the motion vector buffer compression process is applied.

motion_vector_buffer_comp_ratio specifies the compression ratio in the motion vector buffer compression process. When motion_vector_buffer_comp_ratio is not present, it shall be inferred to be equal to 0. The motion vector buffer compression ratio is specified as follows:

MVBufferCompRatio = 1<< motion_vector_buffer_comp_ratio_log2

# Conclusions

We propose a method for storing motion vectors at lower resolution for use in the motion vector competition process.  The method provides 16x memory buffer savings with small impact on coding efficiency.  We propose inclusion of the method in the HM design.

# Patent rights declaration(s)

SHARP may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

[2] Kiran Misra, Jie Zhao, Andrew Segall, “Entropy slices for parallel entropy coding,” JCTVC-C256 / m18297, 3rd Meeting: Guangzhou, CN, 7-15 October, 2010.

[3] Frank Bossen, "Common test conditions and software reference configurations," JCTVC-C500 / m18607, 3rd Meeting: Guangzhou, CN, 7-15 October, 2010.