| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, Korean, 20-28 January, 2011 | Document: JCTVC-D074 WG11 Number: m18824 |
| --- | --- |

| Title: | CE6: Parallel Prediction Unit for Parallel Intra Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Zhao and Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email: | Tel: +1-360-817-8487 asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

In our previous contribution (JCTVC-B112), [1], we proposed the concept of a parallel prediction unit (PPU) within the HEVC test model (HM) design. The goal of the parallel prediction unit is to define a group of pixels that are intra-coded, and where the intra-coding may be done in a parallel fashion.  Parallelization is achieved by partitioning the intra-coded blocks into two sets.  Blocks in the first set are predicted in parallel using available pixels outside PPU; blocks in the second set are also predicted in parallel using available pixels outside the PPU as well as pixels from the first set of blocks.  We implemented parallel intra coding into HM 0.9 and report that the parallel intra coding results in negligible impact on coding efficiency.  For all intra coding, we observe approximately a .3% loss in coding efficiency when the PPU is 16x16.  For random access and low delay configurations, we observe between 0.0% and 0.1% loss in coding efficiency.  For all cases, a PPU of 16x16 means that 4x4 blocks are processed in parallel.

# Introduction

Intra prediction is currently achieved by partitioning a largest coding unit (LCU) into one or more blocks through a recursive splitting process. Each block is predicted spatially and subsequently refined, and the prediction is performed sequentially using neighboring reconstructed blocks. The prediction process requires that the causal neighbors of the current block must be completely reconstructed before processing the current block.  This results in a set of serial dependencies, and these dependencies impact the complexity for both the encoder and decoder processes.

# Parallel Prediction Unit

In our contribution (JCTVC-B112), we introduced the concept of parallel intra-prediction unit (PPU).  The PPU defines the size of a block that can be coded using parallelization.  The motivation for the PPU is straightforward – the serial bottleneck in intra-prediction is not determined by simply the number of blocks within an LCU, but also by the size of the blocks that are being intra-coded.  For blocks larger than a defined PPU size, normal sequential prediction is performed. For blocks fall within a PPU, if they are further partitioned, parallel intra prediction may be employed. We refer such a block as Parallel Prediction Block (PPB). The size of a PPB is equal or less than the size of a PPU.

Let’s use a 16x16 PPU as example.  In our proposal, there are two cases where parallel intra may be applied. In the first case, the PPU is split into 4-8x8 blocks, and at least one of these blocks is further divided into 4x4 blocks.  Blocks that are further divided into 4x4 blocks are each a PPB in our solution, as is shown in Fig. 1 (a). In the second case, a 16x16 PPU is split entirely into 4x4 subblocks and is an entire PPB, as shown in Fig. 1(b).

(a)			     (b)

Fig. 1 - Illustration of Parallel Prediction Block

# Parallel Intra Prediction

## Partitioning the PPB to two sets

The parallel intra prediction approach is designed to reduce the serial dependencies within the intra-prediction process, and it begins by modifying the partitioning strategy for sub-blocks within a parallel prediction block (PPB).  For the PPBs as shown in Fig. 1, we use a checker-board arrangement to partition the blocks to two sets. In Figure 1, blocks in the first set are shown with shading, while blocks in the second set are shown in white. For the PPB in Fig. 1(a), there are two sub-blocks in each set. For the PPB shown in Fig. 1(b), there are 8 sub-blocks in each set. In the next step, the blocks within the first set will be parallel in parallel. Then the blocks within the second set will be processed in parallel.

With the partitioning in Fig. 1, we define that all blocks in a single partition be processed in parallel.  This means that the blocks in the first partition are predicted and refined without reference to each other (or blocks in the second partition).  Blocks in the second partition are also predicted and refined without reference to other blocks in the second partition.  In all cases, a block uses the pixel values from pixels outside of the current PPB for intra-prediction.

By employing the partitioning strategy, we achieved a direct increase in parallelism. This parallelism is shown graphically in Fig. 2.  It is for the case Fig. 1(b) in which a PPB consisting of 16 subblocks. As can be seen in the Fig. 2, "normal" prediction requires 16 sequential steps while our partitioning approach results in only two sequential steps.  This is an increase in parallelism by a factor of 8x. In general, the increase in parallelism is N/2, where N is the number of blocks within the PPB.  (We note that for the case of Figure 1(b), it is possible to use a wavefront reconstruction approach and construct the block in 10 steps while reconstructing two blocks at a time.  We are not aware of an existing parallel method to reconstruct the blocks in Figure 1(a).)

(a)                    (b)

Fig. 2 - Processing order for the 4x4 blocks within a 16x16 intra-predicted PPB using (a) traditional intra-prediction structure and (b) the proposed intra-prediction structure.  Notice that with the proposed intra-prediction structure 8 blocks are predicted in parallel and result in only two sequential steps.  By contrast, the dependencies in the traditional structure require 16 sequential steps.

## Predicting the first set blocks

To predict the first set of blocks, we use pixel values from the upper and left PPB boundaries.  For example, to predict block '13' in Fig. 1(b), we use reconstructed pixel values from the block above block '5' and the block to the left of block '8'. This is equivalent to predicting a PPB using the prediction mode of a sub-block, and then only filling in the first set sub-blocks.

In addition, we modify the method for DC calculation.  This is due to the fact that we use pixel values that are further away from the predicted blocks.  For the DC mode of a first set block, we apply weighted combination to it upper PPB neighbors and left PPB neighbors. For example, for the block ‘5’ in Fig. 1 (b), it is closer to its upper PPB neighbors than to its left PPB neighbors, therefore its upper PPB neighbors will be given more weight in the DC mode prediction.

Following the prediction of first set of blocks, we then added any transmitted residual to the predicted value to generate a reconstructed first set of blocks.

## Predicting the second set blocks

After reconstructing the first set of blocks, we proceed to predict the second set of blocks.  As mentioned before, these blocks are predicted in parallel and use the reconstructed pixel values from blocks in the first partition and blocks outside of the current PPB. For example, consider the block labeled '3' in Fig. 1(b).  This block has top, bottom, left and right neighbors labeled '1', '2', '6' and '9', respectively.  These neighboring pixels are shown in Fig. 3, where p(y,x) denotes the pixel at row y and column x.  As can be seen from the Figure, the current block has both spatially causal and spatially non-causal pixel neighbors.

Fig. 3. - Block with four surrounding neighbor blocks.  Note the pixels AA-DD, II-LL, X2 and X3 are spatially non-causal from the block pixels P(y,x).

For the second set blocks, with the additional right and bottom neighbors, we perform prediction that employs combinations of the right and bottom neighbors, as well as the neighbors to the left and above.  As one step, we use the existing prediction modes from the HM and predict using the above and left neighbors. As a second step, we predict using the available right and bottom neighbors.  These are simply rotated versions of the existing prediction modes. Of course, this may be implemented directly without the rotation.  This second prediction may be used explicitly, or it is combined with the first prediction using a weighting function. For the weighted combination, we use weights that are proportional to the distance between each predicted pixel and the top/bottom and left/right neighbor.  This is expressed as

p(y,x)=w∗p1(y,x)+(1−w)p2(y,x),

where w denotes the weight, and p1(y,x) and p2(y,x) denote the predicted values using the top/left neighbors and bottom/right neighbors, respectively.

# Results

As specified in CE6, we integrated the parallel intra-prediction technology with the PPU concept into the HM0.9 software and tested with the recommended configuration and a PPU of 16x16.  Results are shown in the Tables below.  Here, we report two sets of results with different encoder strategies.  The first is Parallel Intra Prediction with full search; the second is Parallel Intra Prediction with fast search.  The full search method tests for the 8x parallelism case for each PPU.  The fast search method tests for the 8x8 parallelism case only for PPUs where the existing, recursive splitting process selected 4x4 blocks in all or part of the PPU.  Finally, we select the PPU size based on the resolution of the sequence.  In our testing, we currently set the PPU to be 16x16 for HD and above, and 8x8 for low resolution data.  This provides more parallelization for the higher resolution sequences, as we assert that parallelization for the lower resolution data is a much less critical issue.

## Regular Parallel Intra Prediction

Table 1 below shows the summary results of parallel intra vs. anchor. Table 2 listed detailed results of each sequence.

Table 1. Summary RD results of Parallel Intra Prediction vs. HM0.9 Anchor

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.5 | 0.4 | 0.4 | 0.7 | 0.5 | 0.5 |
| Class B | 0.3 | 0.3 | 0.2 | 0.5 | 0.4 | 0.4 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.6 | 0.5 | 0.4 | 0.8 | 0.6 | 0.6 |
| All | 0.3 | 0.2 | 0.2 | 0.3 | 0.3 | 0.3 |
| Enc Time[%] | 114% |  |  | 122% |  |  |
| Dec Time[%] | 102% |  |  | 105% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.2 | 0.3 | 0.4 | 0.1 | 0.2 |
| Class B | 0.1 | 0.2 | -0.1 | 0.2 | 0.1 | 0.2 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.1 | 0.1 | 0.0 | 0.1 | 0.0 | 0.1 |
| Enc Time[%] | 103% |  |  | 106% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | -0.2 | 0.1 | 0.1 | -0.1 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.1 | 0.5 | 0.4 | 0.1 | -0.2 | -0.9 |
| All | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | -0.2 |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 2. BD rates of Parallel Intra Prediction vs. HMC0.9 Anchor

| Sequences / Configurations |  | Intra | Intra LoCo | Random access | Random Access LoCo | Low Delay | Low Delay LoCo |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class A (4K crop) | S01-Traffic | 0.4 | 0.5 | 0.3 | 0.4 |  |  |
|  | S02-PeopleOnStreet | 0.6 | 0.9 | 0.2 | 0.3 |  |  |
| HD (1080p) | S03-Kimono | 0.1 | 0.2 | -0.1 | 0.0 | 0.0 | 0.1 |
|  | S04-ParkScene | 0.1 | 0.2 | 0.1 | 0.2 | 0.0 | 0.0 |
|  | S05-Cactus | 0.7 | 0.9 | 0.3 | 0.3 | 0.1 | 0.1 |
|  | S06-BasketballDrive | 0.6 | 0.7 | 0.1 | 0.1 | 0.1 | 0.0 |
|  | S07-BQTerrace | 0.2 | 0.3 | 0.3 | 0.4 | -0.2 | 0.1 |
| 720p (1280x720) | S16-vidyo1 | 0.6 | 0.8 |  |  | 0.1 | 0.2 |
|  | S17-vidyo3 | 0.9 | 0.9 |  |  | 0.2 | 0.1 |
|  | S18-vidyo4 | 0.4 | 0.7 |  |  | -0.1 | 0.0 |
| AVERAGE |  | 0.46 | 0.61 | 0.17 | 0.24 | 0.03 | 0.08 |

As can be seen from the tables, the impact on coding efficiency is less than 1% for all intra configurations, and less than 0.3 and 0.1% for random access and low delay cases.

## 2. Fast Parallel Intra Prediction

This set of fast parallel intra prediction code reduces the encoding time by speeding up the RD decision for parallel intra. The impact to RD performance is little. It achieves parallel intra prediction with negligible rate increase and very low coding complexity increase. Tables below show the performance of our fast parallel intra prediction vs. HM0.9 anchor.

Table 3. Summary RD results of Fast Parallel Intra Prediction vs. HM0.9 Anchor

|  | Intra |  |  |  | Intra LoCo |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate | V BD-rate | Y BD-rate |  | U BD-rate | V BD-rate |
| Class A | 0.5 |  | 0.4 | 0.4 | 0.7 |  | 0.5 | 0.4 |
| Class B | 0.3 |  | 0.3 | 0.3 | 0.5 |  | 0.3 | 0.3 |
| Class C | 0.0 |  | 0.0 | 0.0 | 0.0 |  | 0.0 | 0.0 |
| Class D | 0.0 |  | 0.0 | 0.0 | 0.0 |  | 0.0 | 0.0 |
| Class E | 0.6 |  | 0.5 | 0.5 | 0.8 |  | 0.6 | 0.6 |
| All | 0.3 |  | 0.2 | 0.2 | 0.3 |  | 0.2 | 0.2 |
| Enc Time[%] | 103% |  |  |  | 105% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 102% |  |  |  |
|  |  | Random access |  |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate |  | V BD-rate | Y BD-rate | U BD-rate |  | V BD-rate |
| Class A | 0.3 | 0.1 |  | 0.5 | 0.3 | 0.3 |  | 0.1 |
| Class B | 0.2 | 0.1 |  | 0.1 | 0.2 | 0.2 |  | 0.1 |
| Class C | 0.0 | 0.0 |  | 0.0 | 0.0 | 0.0 |  | 0.0 |
| Class D | 0.0 | 0.0 |  | 0.0 | 0.0 | 0.0 |  | 0.0 |
| Class E |  |  |  |  |  |  |  |  |
| All | 0.1 | 0.0 |  | 0.1 | 0.1 | 0.1 |  | 0.0 |
| Enc Time[%] | 101% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 102% |  |  |  | 100% |  |  |  |
|  | Low delay |  |  |  | Low delay LoCo |  |  |  |
|  | Y BD-rate | U BD-rate |  | V BD-rate | Y BD-rate | U BD-rate |  | V BD-rate |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 |  | 0.0 | 0.1 | 0.0 |  | -0.1 |
| Class C | 0.0 | 0.0 |  | 0.0 | 0.0 | 0.0 |  | 0.0 |
| Class D | 0.0 | 0.0 |  | 0.0 | 0.0 | 0.0 |  | 0.0 |
| Class E | 0.1 | -0.5 |  | -0.5 | 0.2 | -0.1 |  | -0.2 |
| All | 0.0 | -0.1 |  | -0.1 | 0.1 | 0.0 |  | -0.1 |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 102% |  |  |  |

Table 4. BD rates of Fast Parallel Intra Prediction vs. HM0.9 Anchor

| Sequences / Configurations |  | Intra | Intra LoCo | Random access | Random Access LoCo | Low Delay | Low Delay LoCo |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class A (4K crop) | S01-Traffic | 0.3 | 0.5 | 0.2 | 0.4 |  |  |
|  | S02-PeopleOnStreet | 0.7 | 0.9 | 0.3 | 0.3 |  |  |
| HD (1080p) | S03-Kimono | 0.1 | 0.1 | 0.0 | 0.1 | 0.0 | 0.0 |
|  | S04-ParkScene | 0.1 | 0.2 | 0.1 | 0.2 | 0.0 | 0.0 |
|  | S05-Cactus | 0.7 | 0.9 | 0.4 | 0.4 | 0.2 | 0.2 |
|  | S06-BasketballDrive | 0.5 | 0.7 | 0.2 | 0.2 | 0.0 | 0.1 |
|  | S07-BQTerrace | 0.2 | 0.4 | 0.1 | 0.3 | 0.0 | 0.0 |
| 720p (1280x720) | S16-vidyo1 | 0.6 | 0.8 |  |  | -0.1 | 0.1 |
|  | S17-vidyo3 | 0.8 | 1.0 |  |  | 0.5 | 0.2 |
|  | S18-vidyo4 | 0.4 | 0.7 |  |  | 0.0 | 0.4 |
| AVERAGE |  | 0.44 | 0.62 | 0.19 | 0.27 | 0.08 | 0.13 |

# Syntax

We propose to support the parallel prediction unit with the following syntax and semantics

### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | C |  | Descriptor |  |
| --- | --- | --- | --- | --- |
| profile_idc | 0 |  | u(8) |  |
| reserved_zero_8bits /* equal to 0 */ | 0 |  | u(8) |  |
| level_idc | 0 |  | u(8) |  |
| seq_parameter_set_id | 0 |  | ue(v) |  |
| bit_depth_luma_minus8 | 0 |  | ue(v) |  |
| bit_depth_chroma_minus8 | 0 |  | ue(v) |  |
| increased_bit_depth_luma | 0 |  | ue(v) |  |
| increased_bit_depth_chroma | 0 |  | ue(v) |  |
| log2_max_frame_num_minus4 | 0 |  | ue(v) |  |
| log2_max_pic_order_cnt_lsb_minus4 | 0 |  | ue(v) |  |
| max_num_ref_frames | 0 |  | ue(v) |  |
| gaps_in_frame_num_value_allowed_flag | 0 |  | u(1) |  |
| log2_min_coding_unit_size_minus3 |  | 0 |  | ue(v) |
| max_coding_unit_hierarchy_depth |  | 0 |  | ue(v) |
| log2_min_transform_unit_size_minus2 |  | 0 |  | ue(v) |
| max_transform_unit_hierarchy_depth |  | 0 |  | ue(v) |
| log2_parallel_unit_size |  | 0 |  | ue(v) |
| pic_width_in_luma_samples | 0 |  | u(16) |  |
| pic_height_in_luma_samples | 0 |  | u(16) |  |
| rbsp_trailing_bits( ) | 0 |  |  |  |
| } |  |  |  |  |

### Prediction unit syntax

| prediction_unit( x0, y0, currPredUnitSize ) { | C | Descriptor |
| --- | --- | --- |
| if( slice_type != I ) |  |  |
| skip_flag |  |  |
| if( skip_flag ) { |  |  |
| if( mv_competition_flag ) { |  |  |
| if( inter_pred_idc != Pred_L1 && NumMVPCand( L0 ) > 1 ) |  |  |
| mvp_idx_l0 | 2 | ue(v) ¦ ae(v) |
| if( inter_pred_idc != Pred_L0 && NumMVPCand( L1 ) > 1 ) |  |  |
| mvp_idx_l1 | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| } |  |  |
| else { |  |  |
| if(!entropy_coding_mode_flag) |  |  |
| mode_table_idx |  | vlc(n,v) |
| else { |  |  |
| if( slice_type != I ) |  |  |
| pred_mode | 2 | u(1) ¦ ae(v) |
| } |  |  |
| if( PredMode == MODE_INTRA ) { |  |  |
| planar_flag | 2 | u(1) ¦ ae(v) |
| if(planar_flag) { |  |  |
| planar_delta_y = getPlanarDelta() |  |  |
| planar_delta_uv_present_flag | 2 | u(1) ¦ ae(v) |
| if(planar_delta_uv_present_flag) { |  |  |
| planar_delta_u = getPlanarDelta() |  |  |
| planar_delta_v = getPlanarDelta() |  |  |
| } |  |  |
| } else { |  |  |
| if(entropy_coding_mode_flag) |  |  |
| intra_split_flag | 2 | ae(v) |
| combined_intra_pred_flag | 2 | u(1) ¦ ae(v) |
| for( i = 0; i < ( intra_split_flag ? 4 : 1 ); i++ ) { |  |  |
| prev_intra_luma_pred_ flag | 2 | u(1) ¦ ae(v) |
| if( !prev_intra_pred_luma_flag ) |  |  |
| rem_intra_luma_pred_mode | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| } |  |  |
| if( (currPredUnitSize < ParallelPredictionUnitSize && isFirstIntraInParallelPredUnit()) ¦¦ (currPredUnitSize == ParallelPredictionUnitSize && intra_split_flag ) { |  |  |
| weighted_intra_block_flag |  |  |
| } |  |  |
| if( chroma_format_idc != 0 ) |  |  |
| intra_chroma_pred_mode | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| } |  |  |
| else if( PredMode == MODE_INTER ) { |  |  |
| if(entropy_coding_mode_flag) |  |  |
| inter_partitioning_idc | 2 | ue(v) ¦ ae(v) |
| for( i= 0; i < NumPuParts( inter_partitioning_idc ); i++ ) { |  |  |
| if( NumMergeCandidates > 0 ) { |  |  |
| merge_flag[ i ] | 2 | ue(1) ¦ ae(v) |
| if( merge_flag[ i ] && NumMergeCandidates > 1 ) |  |  |
| merge_left_flag[ i ] | 2 | ue(1) ¦ ae(v) |
| } |  |  |
| if( !merge_flag[i] ) { |  |  |
| if( slice_type == B ) |  |  |
| inter_pred_idc[ i ] | 2 | ue(v) ¦ ae(v) |
| if( inter_pred_idc[ i ] != Pred_L1 ) { |  |  |
| if( num_ref_idx_l0_active_minus1 > 0 ) |  |  |
| ref_idx_l0[ i ] | 2 | ue(v) ¦ ae(v) |
| mvd_l0[ i ][ 0 ] | 2 | se(v) ¦ ae(v) |
| mvd_l0[ i ][ 1 ] | 2 | se(v) ¦ ae(v) |
| if( mv_competition_flag && NumMVPCand( L0, i ) > 1 ) |  |  |
| mvp_idx_l0[ i ] | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| if( inter_pred_idc[ i ] != Pred_L0 ) { |  |  |
| if( num_ref_idx_l1_active_minus1 > 0 ) |  |  |
| ref_idx_l1[ i ] | 2 | ue(v) ¦ ae(v) |
| mvd_l1[ i ][ 0 ] | 2 | se(v) ¦ ae(v) |
| mvd_l1[ i ][ 1 ] | 2 | se(v) ¦ ae(v) |
| if( mv_competition_flag && NumMVPCand( L1, i ) > 1 ) |  |  |
| mvp_idx_l1[ i ] | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| else if( PredMode == MODE_DIRECT ) { |  |  |
| if( slice_type == B ) |  |  |
| inter_pred_idc | 2 | ue(v) ¦ ae(v) |
| if( mv_competition_flag ) { |  |  |
| if( inter_pred_idc != Pred_L1 && NumMVPCand( L0 ) > 1 ) |  |  |
| mvp_idx_l0 | 2 | ue(v) ¦ ae(v) |
| if( inter_pred_idc != Pred_L0 && NumMVPCand( L1 ) > 1 ) |  |  |
| mvp_idx_l1 | 2 | ue(v) ¦ ae(v) |
| } |  |  |
| } |  |  |
| } |  |  |

SEMANTICS

log2_parallel_unit_size specifies the size of the parallel prediction unit.

The variable ParallelPredictionUnitSize is derived as

ParallelPredictionUnitSize = MaxTransformUnitSize >> log2_parallel_unit_size

weighted_intra_block_flag equal 1 defines that weighted prediction shall be used for second pass units within a parallel prediction unit.  weighted_intra_block_flag equal 0 defines that weighted prediction shall not be used for second pass blocks within a parallel prediction unit.

# Conclusion

We propose a parallel prediction unit, or PPU, for the HEVC HM design.  The parallel prediction unit supports the parallelization of intra-coded blocks within the PPU in a two step, parallel process.  Results show that the increased parallelization comes with little loss in coding efficiency.  Specifically, we observe an average loss of 0.0%-0.1% for inter coding scenarios.  We assert that this loss is negligible and well justified by the parallelization capability.

Given this level of performance and the significance of parallelization for future standards, we request the JCT-VC adopt the parallel intra prediction method into the HM.

# Patent rights declaration(s)

SHARP Labs of America may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

- JCTVC B112, “Parallel Prediction Unit for Parallel Intra Coding,” 2nd JCT-VC Meeting, Geneva, July. 2010.

- JCTVC-C506, “Description of Core Experiment 6: Intra Prediction Improvement”, 3rd JCT-VC Meeting, Guangzhou, Oct. 2010.

- JCTVC-C500, “Common test conditions and software reference configurations,”, 3rd JCT-VC Meeting, Guangzhou, Oct. 2010.