| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D076 |
| --- | --- |

| Title: | CE3: Verification results of Nokia’s Proposal (JCTVC-D323) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Liwei Guo and In Suk Chong and Marta Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: 1-858-658-5673 Email: liweig@qualcomm.com |  |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution shows verification results of JCTVC-D323 for CE3.  Coding efficiency results are confirmed.  No mismatch between encoder and decoder is observed.  Relative encoding and decoding time have some degree of deviation.  Considering the difference of operating the environments, results are basically confirmed.

# Introduction

This contribution is a report of verification results of JCTVC-D323, which is a contribution to CE3. Our cross check covers only part of the proposal which is 8tap cases.  And within that category there are three variations which are Verification is performed under the CE3 test conditions that refer to JCTVC-C500.  In addition, source code is reviewed.

# Experimental Results

Our encoding environment is using 64bit linux cluster and decoding was done on sinlge 64 bit windows PC. There was no mismatch between encoder and decoder. There is no mismatch of overall BD-rate between the verification results and the proponent’s results.

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | -0.2 | 0.1 | 0.2 | 0.2 | 0.1 |
| Class B | 0.4 | -0.2 | -0.2 | -0.4 | 0.2 | 0.2 |
| Class C | 1.2 | 0.1 | 0.2 | -4.7 | -1.1 | -1.4 |
| Class D | 1.6 | 0.7 | 0.8 | -10.6 | -4.7 | -4.6 |
| Class E |  |  |  |  |  |  |
| All | 0.9 | 0.2 | 0.2 | -4.2 | -1.4 | -1.5 |
| Enc Time[%] | 97% |  |  | 117% |  |  |
| Dec Time[%] | 120% |  |  | 136% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.2 | 0.0 | -1.2 | -1.5 | -1.8 |
| Class C | 0.8 | 0.9 | 0.9 | -4.9 | -4.7 | -4.9 |
| Class D | 0.8 | 1.3 | 1.6 | -10.1 | -10.0 | -10.4 |
| Class E | 0.6 | 0.3 | -0.1 | -0.2 | -0.8 | -0.5 |
| All | 0.5 | 0.7 | 0.6 | -4.2 | -4.3 | -4.5 |
| Enc Time[%] | 100% |  |  | 120% |  |  |
| Dec Time[%] | 123% |  |  | 136% |  |  |

We also cross checked their fast version of decoder. There was no mismatch between encoder and decoder. Decoding time was different from above table in 2% on average.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).