| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D078 WG11 Number: m18828 |
| --- | --- |

| Title: | CE7: Cross-check of Qualcomm’s proposal by Institute for Infocomm Research |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Chuohao Yeo Yih Han Tan Zhengguo Li 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 chyeo@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

This contribution provides a summary of the cross-checks performed by the Institute for Infocomm Research on Qualcomm’s proposal on MDDT improvements. The simulation results match the results provided by Qualcomm.

- Introduction

Qualcomm has proposed two closely related methods for improving the performance of MDDT [1] [2]. In the first, “Adaptive switch scan”, the transform to be used depends on the intra prediction mode as in MDDT. However, the coefficient scan (out of zig-zag, horizontal and vertical) to be used is explicitly signaled to the decoder; therefore, the encoder has to test each scan to determine which results in the best RD cost.

In the second method, “Adaptive switch KLT+scan”, the transform to be used also depends on the coefficient scan. If the zig-zag scan is used, then the transform to be used depends on the intra prediction mode, as above. However, if the horizontal or vertical scan is signaled, then the transform used is fixed and depends on whether it is the horizontal scan or the vertical scan.

# Simulation Results

The test conditions as specified in CE7 [3] are followed where appropriate; in particular, since the proposed improvements only apply for High Efficiency coding, tests were conducted on the high efficiency variants of intra coding, random access and low delay coding configurations [4]. The implementation is on TMuC 0.9.

## RD results

Table 2 shows the BD-Rate [5] results for both “Adaptive switch scan” and “Adaptive switch KLT+scan”.

Table 1. BD-Rate results for “Adaptive switch scan” and “Adaptive switch KLT+scan” (High-Efficiency only)

|  | Adaptive switch scan |  |  | Adaptive switch KLT+scan |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Intra |  |  | Intra |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -3.2 | -3.8 | -3.8 | -3.5 | -2.8 | -3.0 |
| Class B | -2.0 | -2.9 | -2.9 | -2.5 | -1.7 | -1.8 |
| Class C | -2.5 | -2.5 | -2.5 | -3.1 | -1.4 | -1.3 |
| Class D | -2.4 | -2.2 | -2.3 | -2.8 | -1.2 | -1.2 |
| Class E | -3.5 | -4.3 | -4.0 | -3.9 | -2.9 | -2.8 |
| All | -2.6 | -3.0 | -2.9 | -3.1 | -1.9 | -1.9 |
| Enc Time[%] | 135% |  |  | 296% |  |  |
| Dec Time[%] | 111% |  |  | 112% |  |  |
|  | Random access |  |  | Random access |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.3 | -0.9 | -0.9 | -1.5 | -0.5 | -0.4 |
| Class B | -1.0 | -1.1 | -1.2 | -1.3 | -0.6 | -0.5 |
| Class C | -1.2 | -1.1 | -1.1 | -1.5 | -0.5 | -0.5 |
| Class D | -0.8 | -0.8 | -0.6 | -1.1 | -0.2 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | -1.1 | -1.0 | -1.0 | -1.3 | -0.4 | -0.4 |
| Enc Time[%] | 118% |  |  | 133% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.6 | -0.7 | -0.8 | -0.7 | -0.5 | -0.9 |
| Class C | -0.5 | -0.6 | -0.7 | -0.6 | -0.5 | -0.4 |
| Class D | -0.4 | -1.0 | -0.3 | -0.4 | -1.1 | -0.6 |
| Class E | 0.0 | 0.0 | -0.3 | -0.5 | 0.3 | -0.7 |
| All | -0.4 | -0.6 | -0.6 | -0.5 | -0.5 | -0.7 |
| Enc Time[%] | 118% |  |  | 137% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

More detailed coding results can be found in the Excel attachment.

## Qualitative results

We have not noticed any visible visual artifacts in the decoded frames.

## Complexity

Due to the need to test out each scan explicitly, there is an increase in encoding time for both methods. Furthermore, because the transform to be used in “Adaptive switch KLT+scan” depends on what scan is signaled, the testing of the scans has to be accompanied by performing the appropriate transform; in other words, when computing the RD cost of coding a block, the block transform has to be repeated for each scan being tested, instead of being done just one. Therefore, the encoding time for “Adaptive switch KLT+scan” is higher than that of “Adaptive switch scan”.

## Training

Training appears to be needed to obtain one of the KLT matrix.

# Conclusion

We have cross-checked Qualcomm’s proposal, studied the algorithm and verified their coding simulation results.

# References

[1] Y. Ye and M. Karczewicz, “Improved Intra Coding,” ITU-T Q.6/SG11 VCEG, VCEG-AG11, Shenzhen, China, Oct 2007.

[2] P. Chen, R. Joshi, Y. Zheng, M. Coban and M. Karczewicz, “CE7: Mode dependent intra residual coding analysis,” JCT-VC D399, Daegu, Korea, Jan 2011.

[3] R. Cohen et al., “CE7: Alternative Transforms,” JCTVC-B507, Guangzhou, China, Oct 2010.

[4] Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-C500, Guangzhou, China, Oct 2010.

[5] Gisle Bjontegaard, “Calculation of Average PSNR Differences between RD curves,” ITU-T SC16/Q6, VCEG-M33, Austin, USA, Apr 2001.

For avoidance of doubt, positive BD-Rate implies that the proposed method uses more rate, while a negative BD-Rate implies that the proposed method improves coding performance.