| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D079 WG11 Number: m18829 |
| --- | --- |

| Title: | Verification of Qualcomm’s proposal on alternative large transform architecture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Chuohao Yeo Yih Han Tan Zhengguo Li 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 chyeo@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

This contribution provides a summary of verifications performed by the Institute for Infocomm Research on Qualcomm’s proposal on large transforms factorizations for 16-point and 32-point transforms. The simulation results match the results provided by Qualcomm.

- Introduction

Qualcomm has previously proposed an alternative transform factorization for DCT-II [1]. In the proposal being tested [2], this is used for 16-point and 32-point transforms. The forward transform has a structure as illustrated in the Figure 1 below:

Figure 1. Transform architecture for DCT-II

The DCT-IV kernel has a forward transform structure as shown in Figure 2.

Figure 2. Transform architecture for DCT-IV

With such a factorization, a 16-point DCT-II transform could use 3 identical 4-point DCT-II blocks and 1 4-point DCT-IV block [1]. A 32-point transform could re-use the 16-point DCT-II transform and use an additional 16-point DCT-IV transform, which can in turn re-use the 8-point DCT-II block (and the DCT-II and DCT-IV blocks within). These factorizations are outlined in Figure 3 below.

Figure 3. Dependencies among forward transforms

The Givens rotations are implemented with integer arithmetic. Scale factors are incorporated into the quantization matrices.

# Simulation Results

The test conditions as specified in the coming test conditions are followed and tests were conducted on all coding configurations [3]. The implementation is on TMuC 0.9.

## RD results

Table 2 shows the BD-Rate [4] results for the proposed scheme vs the TMuC 0.9 anchors.

Table 1. BD-Rate results for proposed large transforms architecture [2]

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | 0.0 | -0.1 | 0.0 | 0.1 | 0.1 |
| Class B | 0.0 | -0.1 | -0.1 | 0.1 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | -0.1 | -0.1 | -0.2 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | 0.1 | 0.1 | 0.0 | 0.1 | 0.2 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.1 | 0.0 | -0.1 | -0.1 |
| Class D | 0.0 | 0.0 | 0.1 | 0.1 | 0.0 | 0.2 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.1 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | -0.2 |
| Class C | 0.0 | 0.0 | -0.1 | 0.1 | -0.1 | 0.0 |
| Class D | 0.0 | -0.8 | 0.0 | -0.1 | 0.1 | -0.1 |
| Class E | -0.1 | -0.2 | -0.4 | -0.1 | -0.2 | 0.2 |
| All | 0.0 | -0.3 | -0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

More detailed coding results can be found in the Excel attachment.

## Qualitative results

We have not noticed any visible visual artifacts in the decoded frames.

## Complexity

There is no significant change in encoding or decoding time.

# Conclusion

We have cross-checked Qualcomm’s proposal, studied the algorithm and verified their coding simulation results.

# References

[1] R. Joshi, Y. A. Reznik, and M. Karczewicz, “Efficient Large Size Transforms for High-Performance Video Coding,” in Proc. SPIE 7798, 2010.

[2] R. Joshi, Y. Reznik, J. Sole, and M. Karczewicz, “Efficient 16 and 32-point transforms,” JCTVC-D256, Daegu, Korea, Jan 2011.

[3] Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-C500, Guangzhou, China, Oct 2010.

[4] Gisle Bjontegaard, “Calculation of Average PSNR Differences between RD curves,” ITU-T SC16/Q6, VCEG-M33, Austin, USA, Apr 2001.

For avoidance of doubt, positive BD-Rate implies that the proposed method uses more rate, while a negative BD-Rate implies that the proposed method improves coding performance.