| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D083_r1 |
| --- | --- |

| Title: | Non-directional intra prediction for coding efficiency improvement |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjoon Jeon Seungwook Park Byeongmoon Jeon #221 Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Email: | yongjoon.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

HM has up to 33 directional modes and 1 non-directional mode, which is the intra DC mode. At the previous meeting, it is mentioned in [1] that having only 1 non-directional mode may be insufficient for accuracy on prediction units (PU) with complex texture. In this context, this contribution proposes a new non-directional intra prediction method which utilizes Planar prediction of TMuC but modifies it to be more efficient in terms of coding efficiency. In the proposed method, the current intra DC mode is replaced by the proposed non-directional intra mode. The proposed scheme achieves 0.5% and 0.7% for Intra High Efficiency and Intra Low Complexity configurations respectively and the maximum gain of 1.5% is found at Vidyo1 sequence under Intra Low Complexity configuration.

# Introduction

HM has 34 intra prediction modes, which are 33 directional modes and 1 non-directional mode, which is DC mode. It is mentioned in [1] that, even though 33 directional modes produce higher accuracy of prediction on blocks with angular texture, using only DC mode for non-directional intra prediction is a weakness and there’s some room for improvement. DC prediction is efficient for static background image regions but not efficient for gradient image regions as much as other non-directional prediction schemes such as Plane prediction of H.264/AVC or Planar prediction of TMuC. Gradient image regions can be predicted more efficiently by employing those plane-like predictors. Besides the 34 predictors of HM, those predictors have been proposed to compensate the weakness of DC prediction.

Plane Prediction

Integration of Plane prediction into Unified Intra Prediction (UIP) was proposed in [1]. Two kinds of schemes are proposed in [1], which are the integration of Plane prediction and grouping of intra prediction modes. In [1], intra prediction mode number is predicted based on the group number and group index. This contribution shows improvement in coding efficiency but the gain comes from the combination of the two schemes and the gain of Plane prediction itself has not been shown.

Planar Prediction

Planar prediction is a coding tool to improve the visual quality of coded video at smooth areas, especially at mid and low bitrates [2]. It is first proposed in Dresden meeting and adopted in TMuC. It shows visual quality improvement at smooth regions especially at mid and low bitrates. Despite of the advantage it provides, it is not adopted in HM due to the inefficiency of coding performance. When this tool is enabled, there was no gain in BD-Rate in all the high-efficiency configurations and it even causes a loss of 0.9% and 0.2% BD-Rate in Intra Low Complexity and Random Access Low Complexity configurations respectively [2][3]. One of the main reasons for the loss is the signaling overhead. It signals 1 bit flag (planar_flag) in every intra CU even though it is not used often due to the RD decision process at encoder, which results in coding loss. It is inefficient in terms of RD perspective despite of the advantage for subjective quality. To compensate the loss for the low-complexity configurations caused by planar prediction, joint coding of planar_flag and intra_split_flag was proposed in [2]. It shows 0.6% BD-Rate gain in Intra Low Complexity configuration. However, the scheme has not shown any gain in High Efficiency configuration.

Both schemes show the need for an additional non-directional predictor and this topic has been studying under CE6 4.1.5 : Optimizing the combination of current intra predictors [4]. The goal of this CE is to evaluate and investigate the predictors as well as the existing intra predictors in HM and find an optimal combination of these predictors along with the predictors in Unified Intra Prediction (UIP).

In this context, this contribution proposes a way to optimize current intra predictors especially focusing on non-directional predictor.

# Proposed method

The proposed scheme focuses on the way to compensate the weakness of DC prediction as well as optimizing the combination of current intra predictors.

The proposed scheme utilizes Planar prediction of TMuC [5] but modifies it in an efficient way in the RD optimization sense.

- Planar prediction can predict gradient image regions well but it is not efficient in terms of RD performance due to the overhead it needs. For planar prediction, the quantized difference value of right-bottom sample of the block and DC predicted value should be sent to decoder together with the signaling bit for the usage of it. In the proposed method, the quantized difference value is not sent to decoder. Instead, the predicted block is refined to compensate the weakness of not sending the difference value. This process is shown in step4 of Fig.1. In this step, each predicted sample is refined using the weighted average of each sample and its neighboring samples which are left and top samples. The whole process is shown in Fig.1. In step1, the right-bottom sample is set to the DC predicted value which is the output of the DC prediction of HM. Step2 and 3 are same as Planar prediction. Step4 describes the refinement process.

Step1. Set right-bottom sample to DC predicted value. Step2. Interpolate rightmost and bottom samples. Step3. Bi-linear interpolation of inner samples. Step4. Refinement of predicted samples - X’ = { (X<<1) + L + T +1} >> 1

Fig. 1 – Prediction process of the proposed method

The proposed method replaces the current DC mode of HM to optimize the combination of intra predictors.

- DC prediction is efficient in some image regions such as static background but those regions can also be predicted well by the proposed method, which means the proposed method covers the current DC mode. The proposed method can predict static background regions as well as gradient image regions. In this sense, it is wasteful to have two kinds of non-directional intra predictors, DC mode and the proposed method.

The proposed method can predict more efficiently for gradient image regions as well as static image regions. Therefore coding efficiency can be improved by replacing the current DC mode with the proposed method without any additional overhead.

# Experimental results

TMuC0.9 software is used for the HM anchor and the proposed scheme is implemented in the same software and tested under the test conditions of [6]. Table 1 shows the BD-rate of the proposed scheme relative to the HM anchor. The proposed method shows 0.5% and 0.7% BD-Rate improvement in Intra High Complexity and Intra Low Complexity configurations respectively. BD-Rate-Y for Intra High Complexity and Intra Low Complexity configurations are shown in Fig.2 and 3. Maximum gain is 1.5%, which can be found in Vidyo1 sequence of Intra Low Complexity configuration. Table1 shows BD-Rate of the proposed scheme for all the configurations of [6].

Fig. 2 – BD rate-Y of the proposed method relative to HM anchor for Intra HE configuration

Fig. 3 – BD rate-Y of the proposed method relative to HM anchor for Intra LC configuration

Table 1 – Coding Gain of the proposed method

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.6 | 0.7 | 0.9 | -0.9 | -0.2 | 0.3 |
| Class B | -0.6 | 0.2 | 0.4 | -0.6 | -0.2 | 0.3 |
| Class C | -0.3 | 0.7 | 0.9 | -0.5 | 0.2 | 0.3 |
| Class D | -0.4 | 0.7 | 0.9 | -0.5 | 0.1 | 0.2 |
| Class E | -0.7 | -0.2 | 0.2 | -0.9 | -0.7 | -0.8 |
| All | -0.5 | 0.4 | 0.7 | -0.7 | -0.1 | 0.1 |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | 0.9 | 1.1 | -0.3 | 0.7 | 1.4 |
| Class B | -0.4 | 0.2 | 0.1 | -0.2 | 0.7 | 1.2 |
| Class C | -0.3 | 0.3 | 0.4 | -0.3 | 0.7 | 0.8 |
| Class D | -0.2 | 0.4 | 0.7 | -0.2 | 0.8 | 1.1 |
| Class E |  |  |  |  |  |  |
| All | -0.3 | 0.4 | 0.5 | -0.2 | 0.7 | 1.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | -0.1 | 0.2 | -0.1 | 0.1 | 0.3 |
| Class C | -0.2 | -0.1 | 0.0 | 0.0 | 0.1 | 0.1 |
| Class D | 0.0 | -0.4 | 0.4 | -0.1 | 0.4 | 0.1 |
| Class E | 0.0 | 0.7 | 0.1 | 0.2 | 0.3 | -0.2 |
| All | -0.1 | 0.0 | 0.2 | 0.0 | 0.2 | 0.1 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Fig.4 and Fig.5 show the subjective quality comparison between HM anchor and the proposed method. The left/top image is from HM anchor and the right/bottom image is from the proposed method.

Fig. 4 – Subjective quality comparison – Kimono, Intra LC, QP37, frame#147

Fig. 5 – Subjective quality comparison – Vidyo3, Intra LC, QP37, frame#51

# Conclusion

Recommend the proposed scheme to be studied within CE6 to find the optimal combination of current intra predictors.

# Patent rights declaration(s)

LG Electronics may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- G. Li, L. Liu, N. Ling, J. Zheng, P. Zhang, “Integration of Plane Mode in Unified Intra Prediction”, JCTVC-C111, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, GuangZhou, China, Oct. 2010

- J.Lainema, K.Ugur, O.Bici, “TE12.3: Results for planar prediction tests by Nokia”, JCTVC-C188_r1, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, GuangZhou, China, Oct. 2010

- D.Flynn, “TE12.3: Report on planar mode”, JCTVC-C244, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, GuangZhou, China, Oct. 2010

- A.Tabatabai, A.Tanizawa, A.Segall, H.Yu, S.Lei, J.Xu, J.Chen, Y.Jeon, G.Li, M.Mrak, M.Coban, “Core Experiment 6: Intra Prediction Improvement”, JCTVC-C506, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, GuangZhou, China, Oct. 2010

- “Test Model under Consideration”, JCTVC-B205, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, Jul., 2010

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-C500, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, GuangZhou, China, Oct. 2010