| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D084 |
| --- | --- |

| Title: | Improvement on signaling method for prediction modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jaehyun Lim Byoengmoon Jeon Convergence Lab., LG Electronics Inc. 221, Yangjae-dong, Seocho-gu, Seoul, Korea | Tel: Email: | jaehyun.lim@lge.com bm.jeon@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This document presents an enhanced scheme for signaling of prediction modes in inter slices. Different from the specification of HM, prediction mode and partitioning type for inter slices are jointly coded with the unary-type codeword in the HM software. In this contribution, the distribution of prediction modes has been investigated and the codeword representing prediction mode and partitioning type is rearranged according to the portion of each prediction mode. The simulation result shows that the proposed method achieves 0.0%, 0.3% bit rate reduction for HE and LC configurations respectively in CU MRG case and 0.0%, 0.5% in PU MRG case without any additional signaling overhead and complexity increase.

# Introduction

At October JCT-VC meeting, the HM was released. In the Specification of HM, prediction mode and partitioning type are signaled by the syntax element pred_mode and inter_partitioning_idc respectively as shown in Table.1. Different from the specification, the current HM Software (TMuC 0.9) exploits a set of unary code for signaling both prediction mode and partition size jointly for coding efficiency. These codewords are shown as Table.2.

Table.1 Syntax table of Coding Unit

| coding_unit( x0, y0, log2CUSize ) { | Descriptor |
| --- | --- |
| ︙ |  |
| if( slice_type != I ) |  |
| pred_mode | u(1) ¦ ae(v) |
| ︙ |  |
| if( PredMode = = MODE_INTER ) |  |
| inter_partitioning_idc | ue(v) ¦ ae(v) |
| ︙ |  |
| } |  |

Table.2 Codewords for representing each prediction mode

| prediction mode | partition | codewords |  |
| --- | --- | --- | --- |
| direct |  | 0 |  |
| intra | 2Nx2N | 1 | 00000 |
|  | NxN | 1 | 00001 |
| inter | 2Nx2N | 1 | 1 |
|  | 2NxN | 1 | 01 |
|  | Nx2N | 1 | 001 |
|  | NxN | 1 | 0001 |

Each prediction mode is signaling as shown in Figure.1. After the decision of MODE_SKIP and MODE_MERGE, other prediction modes (MODE_DIRECT, MODE_INTER and MODE_INTRA) are signaled by corresponding codewords as shown in Table.1

Figure.1 Prediction mode signaling process with CU merge mode

As shown in Table.2, the codeword for each prediction mode has different length. By the source coding theorem, the length of each codeword is approximately proportional to the negative logarithm of the probability. Therefore, the best coding efficiency is achieved when the most common symbols use the shortest codes.

In the proposed method, each codeword corresponding with the prediction mode is reordered through the analysis of the distribution on the prediction modes. By the rearrangement of the codewords, the proposed method can increase the coding efficiency without any additional encoding/decoding complexity increase.

# Proposed Methods

Before rearranging the codewords, an analysis for the distribution of prediction modes on the HM anchor bitstream was performed for maximizing the coding efficiency of HM. Table.3 and Figure.2 show the average distribution of prediction modes on each test configuration.

Table.3 Distribution of prediction modes in inter slices

| class | skip | mrg | direct | intra |  | Inter |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 2Nx2N | NxN | 2Nx2N | 2NxN | Nx2N | NxN |
| RAHE | 40.6 | 26.5 | 8.5 | 2.9 | 1.3 | 11.4 | 3.9 | 4.4 | 0.6 |
| LDHE | 37.3 | 25.6 | 12.2 | 1.9 | 0.6 | 11.9 | 4.5 | 5.4 | 0.6 |
| RALC | 54.9 | 13.6 | 6.5 | 1.8 | 1.9 | 10.7 | 5.0 | 4.5 | 1.2 |
| LDLC | 48.0 | 19.6 | 7.2 | 1.1 | 0.9 | 11.6 | 5.3 | 5.0 | 1.1 |
| avg. | 45.2 | 21.3 | 8.6 | 1.9 | 1.2 | 11.4 | 4.7 | 4.8 | 0.9 |

Figure.2 Distribution of prediction modes in inter slices

This table and figure show following characteristics in common.

portion of 2Nx2N inter mode > portion of direct mode

portion of Nx2N inter mode > portion of 2NxN inter mode

As shown in Table.1, in the current HM, shorter codeword than the codeword corresponding to the 2Nx2N inter mode was assigned for the direct mode. Similar codewords assignment is observed in the relation between the 2NxN inter mode and the Nx2N inter mode.

Therefore, in the proposed method, following 4 prediction modes are rearranged for the codeword.

2Nx2N inter mode is switched with direct mode

2NxN inter mode is switched with Nx2N inter mode

Table.4 shows the codewords for representing each prediction mode after rearrangement by the proposed method.

Table.4 Rearranged codewords for representing each prediction mode

| prediction mode | partition | codewords |  |
| --- | --- | --- | --- |
| direct |  | 1 | 1 |
| intra | 2Nx2N | 1 | 00000 |
|  | NxN | 1 | 00001 |
| inter | 2Nx2N | 0 |  |
|  | 2NxN | 1 | 001 |
|  | Nx2N | 1 | 01 |
|  | NxN | 1 | 0001 |

The proposed method can be applied also when CU base merge is substituted with PU based merge which is being studied in CE9. If PU merge is exploited, each prediction mode is signaled as shown in Figure.4. After the decision of MODE_SKIP, other prediction modes are signaled with the same corresponding codewords as shown in Table.2. In case of MODE_INTER, a merge_flag is signaled for each prediction partition in order to indicate whether MODE_MERGE is applied.

Figure.3 Prediction mode signaling process with PU merge mode

Table.5 and Figure.4 show that the distribution of prediction modes when PU based merge is exploited. In this table and figure, similar pattern of distribution is observed for the 4 prediction modes to be arranged.

Table.5 Distribution of prediction modes in inter slices (PU MRG use case)

| class | skip | direct | intra |  | Inter |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2Nx2N | NxN | 2Nx2N | 2NxN | Nx2N | NxN |
| RAHE | 42.8 | 7.8 | 2.9 | 1.2 | 32.2 | 5.4 | 6.5 | 1.2 |
| LDHE | 36.6 | 13.3 | 1.8 | 0.6 | 33.0 | 6.0 | 7.3 | 1.4 |
| RALC | 53.2 | 12.2 | 1.9 | 1.9 | 13.7 | 7.4 | 6.6 | 3.0 |
| LDLC | 46.8 | 14.8 | 1.2 | 1.0 | 18.7 | 7.8 | 7.0 | 2.8 |
| avg. | 44.9 | 12.0 | 1.9 | 1.2 | 24.4 | 6.6 | 6.9 | 2.1 |

Figure.4 Distribution of prediction modes in inter slices (PU MRG case)

# Experimental Results

The proposed method was implemented on the HM (TMuC 0.9) software and the performance was measured under both Random Access and Low Delay configurations. The current HM allows the merging scheme in the CU level only. Therefore, the performance of the proposed method using PU merge mode was measured with the CE9 software. Detail test condition for the proposed method is as shown in Table.6.

Table.7 shows the experimental results in terms of BD rate reduction [2]. The average gains for HE and LC configurations are 0.0%, -0.3% respectively in CU MRG case and 0.0%, -0.5% in PU MRG case.

Table.6 Test condition for the proposed method

| software | CU MRG case : HM software (TMuC 0.9) PU MRG case : CE9 software (Modified TMuC 0.9 for PU MRG) |
| --- | --- |
| test sequences | All seq. (class A/B/C/D for RA, class B/C/D/E for LD) |
| coding structure | Random Access HE/LC Low Delay HE/LC |
| etc | the same as the HM(TMuC 0.9) software configuration |

Table.7 BD rate (%) reduction of the proposed method

| class | CU MRG case (anchor : HM) |  |  |  | PU MRG case (anchor : HM with PU MRG) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RAHE | LDHE | RALC | LDLC | RAHE | LDHE | RALC | LDLC |
| A | 0.0 |  | -0.2 |  | 0.0 |  | -0.3 |  |
| B | 0.0 | 0.0 | -0.2 | -0.2 | -0.1 | 0.0 | -0.4 | -0.6 |
| C | 0.0 | 0.0 | -0.3 | -0.3 | 0.0 | 0.0 | -0.5 | -0.7 |
| D | 0.0 | 0.0 | -0.3 | -0.4 | -0.1 | 0.0 | -0.5 | -0.7 |
| E |  | -0.2 |  | -0.4 |  | -0.1 |  | -0.7 |
| avg. | 0.0 | 0.0 | -0.2 | -0.3 | 0.0 | 0.0 | -0.4 | -0.7 |
| HE/LC avg. | 0.0 |  | -0.3 |  | 0.0 |  | -0.5 |  |
| Enc Time | 100% | 100% | 100% | 102% | 99% | 99% | 101% | 101% |
| Dec Time | 100% | 100% | 100% | 100% | 100% | 100% | 100% | 100% |

There are no noticeable encoder and decoder complexity differences because the proposed method deals with only the codeword change.

# Conclusion

In this document, enhanced prediction mode signaling method for inter slice was proposed. The proposed scheme shows bit-rate reduction without any additional signal overhead. Moreover, it does not cause any encoder and decoder complexity increase. It is recommended the HM to include the codeword table for joint coding of the prediction mode and the partitioning type in the decoding process for coding unit. And it is also recommended to use the proposed codeword rearrangement in order to achieve better coding efficiency.

# Patent rights declaration(s)

LG Electronics may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] “WD1: Working Draft 1 of High-Efficiency Video Coding”, JCT-VC, 3rd Meeting, Guangzhou, China, October 2010, Doc. JCTVC-C403.

[2] G. Bjontegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SC16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.