| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D085_r2 WG11 Number: m18835 |
| --- | --- |

| Title: | Summary of Core Experiment 8 on In-Loop filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage1 Keiichi Chono2 Yu-Wen Huang3 Matthias Narroschke4 In Suk Chong5 | Email: | tomoo.yamakage@toshiba.co.jp chono@ct.jp.nec.com yuwen.huang@mediatek.com Matthias.Narroschke@panasonic.de ichong@qualcomm.com |
| Source: | CE coordinators (TOSHIBA1, NEC2, MediaTek3, Panasonic4 and Qualcomm5) |  |  |

_____________________________

# Abstract

This contribution is a summary of core experiment 8 (CE8) on In-Loop filtering. There are three Subsets in CE8, Deblocking/debanding filters (Subset 1), Wiener-based in-loop filters (Subset 2) and Image clipping and offset (Subset 3) as described in JCTVC-C508. The number of proposals for each Subset is four, three and two, respectively, and those are evaluated based on the common conditions in JCTVC-C500. Detailed results will be reported by proponents.

# Introduction

In this tool experiment, in-loop filtering has been tested for the following functionalities:

- Deblocking/debanding filters (four proposals)

- Wiener-based in-loop filters (three proposals, including one unified proposal of three parties)

- Image clipping and offset (two proposals)

As for the details of each proposal, please refer the documents listed in section 2.

# Document list

- Summary document (this document)

  - JCTVC-D085 [CE coordinators] Summary of Core Experiment 8 on In-Loop filtering

- Proponent's and verification/study documents

Subset 1: Deblocking/debanding filters

  - JCTVC-D163 [MediaTek] CE 8 subtest1: Improved deblocking filter

    - Cross-check: JCTVC-D379 [Ericsson], JCTVC-D203[Toshiba]

  - JCTVC-D137 [Microsoft] CE8, Subset 1: Report of Content-Adaptive Deblocking

    - Cross-check: JCTVC- D332 [SKT/SKKU]

  - JCTVC-D043 [NEC] CE8: Conditional joint deblocking-debanding filter

    - Cross-check: JCTVC-D144 [Microsoft]

  - JCTVC-D334 [SK Telecom and Sungkyunkwan University] CE8 Subset 1: CE8 subset 1: Results of intra deblocking filter testing by SKKU/SKT (JCTVC-C130)

    - Cross-check: JCTVC-D206 [NEC]

Subset 2: Wiener-based in-loop filters

  - JCTVC-D119 [MediaTek/Qualcomm/Toshiba] CE8 subset 2: CE8 Subtest2: A Joint Proposal on Improving the Adaptive Loop Filter in TMuC0.9 by MediaTek, Qualcomm, and Toshiba

    - Cross-check: JCTVC-D170 [Intel], JCTVC-D216 [Panasonic], JCTVC-D114 [Sharp]

  - JCTVC-D217 [Panasonic] CE 8: Results for adaptive loop filter using prediction and residual (3-Input-ALF)

    - Cross-check: JCTVC-D219 [MediaTek/Qualcomm/Toshiba]

  - JCTVC-D115 [Sharp] CE8: DF-combined adaptive loop filter

    - Cross-check: JCTVC-D215 [Panasonic], JCTVC-D211 [Toshiba]

Subset 3: Image clipping and offset

  - JCTVC-D123 [MediaTek] CE8 Subtest3: Controlled Clipping

    - Cross-check: JCTVC-D077 [Qualcomm]

  - JCTVC-D122 [MediaTek] CE8 Subtest3: Picture Quadtree Adaptive Offset

    - Cross-check: JCTVC-D183 [Samsung]

# Cross-verification assignment

Proponents conducted source code exchange for each other. Cross-checkers run the software independently and compared their results with proponent’s results.

Table 1  Cross checkers

| Subset | Proponent | Cross-checkers |
| --- | --- | --- |
| 1 | MediaTek | Ericsson, Toshiba |
|  | Microsoft | SKT/SKKU |
|  | NEC | Microsoft |
|  | SKT/SKKU | NEC |
| 2 | MediaTek/Qualcomm/Toshiba | Intel, Panasonic, Sharp |
|  | Panasonic | MediaTek/Qualcomm/Toshiba |
|  | Sharp | Panasonic, Toshiba |
| 3 | MediaTeK | Qualcomm |
|  | MediaTek | Samsung |

# Experimental results

Based on the common conditions [1], CE8 defined subsets of test conditions and sequences as mandatory tests [2] as follows:

Subset 1: All six test cases and additional results using ALF off anchor for High Efficiency cases shall be provided.

Subsets 2 and 3: All six test cases.

The following evaluation criteria are used:

- Coding efficiency (BD-PSNR and BD-Rate defined in JCTVC-C500)

- Complexity measurement based on the factor of encoder/decoder execution time against the anchor

- Estimated complexity (encoding passes, number of operations in one encoding pass) for Subset 2

- Subjective quality for Subset 1 (CE8 plans to perform informal subjective viewing)

Informal subjective viewing was conducted on Jan 20 and 21.  Dr. Vittorio Baroncini checked results on Jan 23.

In the followings,

IO means High Efficiency I Only case,

LD means High Efficiency Low Delay case, and

RA means High Efficiency Random Access case.

## Subset 1: Deblocking/debanding filters

### Coding efficiency

Table 2  BD-Rate (Y) for High Efficiency (positive value means loss)

| Proponent | MediaTek |  |  | Microsoft |  |  | NEC |  |  | SKKU/SKT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | IO | LD | RA | IO | LD | RA | IO | LD | RA | IO | LD | RA |
| Low Complexity | -1.5 | -1.6 | -0.8 | 0.2 | 0.0 | 0.0 | 0.5 | 0.1 | 0.1 | -0.4 | -0.1 | -0.3 |
| High Efficiency | -1.3 | -1.6 | -1.0 | 0.2 | 0.0 | 0.1 | 0.1 | 0.0 | 0.1 | -0.6 | -0.1 | -0.3 |
| HE w/o QC_ALF | -1.4 | -0.7 | -0.8 | 0.2 | 0.0 | 0.1 | 0.5 | 0.1 | 0.1 | -0.4 | -0.1 | -0.2 |

### Complexity – relative encoding/decoding time against the anchor -

Table 3  Summary of Relative Encoding Time (%) for Subset 1 (against the anchor)

| Proponent | MediaTek |  |  | Microsoft |  |  | NEC |  |  | SKKU/SKT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | IO | LD | RA | IO | LD | RA | IO | LD | RA | IO | LD | RA |
| Low Complexity | 99 | 102 | 103 | 100 | 100 | 100 | 101 | 100 | 101 | 104 | 100 | 100 |
| High Efficiency | 99 | 100 | 104 | 101 | 101 | 102 | 101 | 100 | 100 | 102 | 101 | 99 |
| HE w/o QC_ALF | 102 | 103 | 103 | 98 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |

Table 4  Summary of Relative Decoding Time (%) for Subset 1 (against the anchor)

| Proponent | MediaTek |  |  | Microsoft |  |  | NEC |  |  | SKKU/SKT |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | IO | LD | RA | IO | LD | RA | IO | LD | RA | IO | LD | RA |
| Low Complexity | 104 | 101 | 102 | 106 | 103 | 106 | 103 | 101 | 102 | 102 | 100 | 102 |
| High Efficiency | 100 | 100 | 100 | 101 | 101 | 102 | 105 | 100 | 101 | 101 | 100 | 100 |
| HE w/o QC_ALF | 103 | 98 | 96 | 99 | 95 | 97 | 102 | 101 | 101 | 101 | 100 | 100 |

### Comments/Remarks from Cross Checkers

- MediaTek’s proposal

Ericsson has partially cross verified the Mediatek proposal C142. The software was compiled successfully on linux using gcc 4.3.3 after minor modification of one include file. The decoding time and the coding efficiency numbers match those of Mediatek except for PSNR for some sequences and QPs but that is likely due to different platforms. The source-code has been studied and found to have been re-designed compared to C142 but corresponds to D163. The pure deblocking time is increased with 25% compared to TMuC 0.9 measured on the same platform which is larger than Mediatek reports. Visual inspections at low bitrates for random access and low-delay configurations show more blocking artifacts than TMuC 0.9. Visual inspections for all intra coding configurations show similar performance as TMuC 0.9. The monitor used for the subjective inspection was HP LP2475w.

Toshiba checked MediaTek’s proposal, and confirmed that the coding efficiency against the HM anchor and relative encoding/decoding time are almost same as the proponent’s results.

- Microsoft’s proposal

SKT/SKKU checked Microsoft’s proposal, and confirmed the results obtained by SKT/SKKU match with those provided by the proponent. The source code provided by the proponent was studied thoroughly, and the code is compiled and executed by the cross-checker to obtain the test result. Simple internal viewing was conducted using a monitor (Samsung SyncMaster 305T PLUS) of diagonal size (TFT - 30"), and the cross-checker found improved cases although it was not consistently the case.

- NEC’s proposal

Microsoft checked NEC’s proposal, and got the same RD results and similar encoding/decoding times as what reported by the proponent. Visual comparison was also performed and verified that for the sky area of Kimono sequences, the proposed scheme can improve the visual quality by reducing the banding artifacts. Even at high bit-rates(Qp=22), the visual improvement is perceivable. No obvious degradation was observed in the playback video.

- SKT/SKKU’s proposal

NEC checked SKT/SKKU’s proposal, and verified that its cross-check results are the same as what reported by the proponent. Viewing test with a 56-inch professional monitor (SONY SRM L560) was conducted for all results excluding Class A and D, and significant improvement and degradation by the proposal were not observed in the playback video. Accurate edge reconstruction is observed in some frames of BQMall (IO LC QP=37) by comparing the individual frames of Anchor and Proposal results.

Note: As for subjective picture quality, the preliminary results of the informal viewing held in the afternoon on Jan. 20 were inconsistent.

## Subset 2: Wiener-based in-loop filters

### Note: For Low Complexity cases, the anchor is QC_ALF off.  For High Efficiency, the anchor is QC_ALF on.

### Coding efficiency

Table 5  Summary of BD-Rate (Y) for Subset 2 (positive value means loss)

| Proponent | MediaTek/Qualcomm/Toshiba |  |  | Panasonic |  |  | Sharp |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | IO | LD | RA | IO | LD | RA | IO | LD | RA |
| Low Complexity | -3.5 | -6.0 | -5.3 | -4.6 | -5.5 | -5.9 | -3.9 | -5.4 | -5.4 |
| High Efficiency | 0.1 | -1.9 | -0.4 | -0.9 | -0.4 | -0.8 | -0.2 | -0.7 | -0.2 |

### Complexity – relative encoding/decoding time against the anchor -

Table 6  Summary of Relative Encoding Time (%) for Subset 2 (against the anchor)

| Proponent | MediaTek/Qualcomm/Toshiba |  |  | Panasonic |  |  | Sharp |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | IO | LD | RA | IO | LD | RA | IO | LD | RA |
| Low Complexity | 154 | 108 | 109 | 195 | 119 | 118 | 135 | 105 | 106 |
| High Efficiency | 100 | 98 | 98 | 110 | 104 | 104 | 91 | 98 | 98 |

Table 7  Summary of Relative Decoding Time (%) for Subset 2 (against the anchor)

| Proponent | MediaTek/Qualcomm/Toshiba |  |  | Panasonic |  |  | Sharp |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | IO | LD | RA | IO | LD | RA | IO | LD | RA |
| Low Complexity | 176 | 175 | 169 | 216 | 204 | 196 | 198 | 196 | 194 |
| High Efficiency | 85 | 83 | 85 | 103 | 107 | 106 | 85 | 93 | 93 |

### Complexity – number of encoding passes -

Table 8  Summary of Worst Case Number of Encoding Passes for Subset 2

| Proponent | MediaTek/Qualcomm/Toshiba |  |  | Panasonic |  |  | Sharp |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case | IO | LD | RA | IO | LD | RA | IO | LD | RA |
| # of passses | 1 | 1 | 1 | 15 | 15 | 15 | 13 | 13 | 13 |

Note: Detailed explanation is described in section 4.2.4.

### Complexity – Description of operations in each of the encoding passes -

If a proposal expects frame memory (e.g. deblocked pixels, pre-deblocked pixels, predicted pixels, etc.) to design filter coefficients and/or control filtering, please describe which frame memory is accessed.

MediaTek/Qualcomm/Toshiba:

During encoding and deblocking LCUs, autocorrelation and crosscorrelation can be calculated and stored from original pixels and deblocked pixels without any additional picture buffer access. Next, the number of filter sets, the parameters of each FS including adaptive filter coefficients or adaptive offsets, the size of FUs, and the selection of FS in each FU are derived by RDO with the fast distortion estimation by reusing autocorrelation and crosscorrelation. To this step, there is no filtering, and only on/off flags for ALF control blocks are not yet decided.

Then, as for the 1st additional pass, each pixel is filtered according to the RDO decision, which requires reading the deblocked picture for one time. If the deblocked picture is stored in an on-chip buffer, this external memory access can be saved. Meanwhile, the on/off control is performed, which requires reading the original picture for one time. If the original picture is stored in an on-chip buffer, this external memory access can be saved. Finally, filtered pixels and deblocked pixels are outputted for regions where filtering is enabled and for regions where filtering is disabled, respectively, which requires writing the external picture buffer one time.

To sum up, at most two reading operations and one writing operation per pixel are required, and they can be done in parallel to achieve single-pass encoding.

Panasonic:

Details of operations and frame memory accesses are provided in the table below.

In order to achieve the goal of the CE8 to clearly understand the advantages and disadvantages of individual loop filter tools, in this particular case the additional use of the prediction and quantized prediction error, only this tool has been added to the reference and been evaluated in this experiment. Intentionally, the encoder control of the reference, which is significantly responsible for long run times and a large number of encoding passes to control other loop filter tool, is not changed in this experiment. In addition, the information in the table only reflects the operations and frame memory accesses of the software implementation. They do not reflect the ones of an optimized implementation in which e.g. the filtered signal could be written only once in the final pass.

| Pass | Operations | Access |  |  |  |  |  | note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Resi. (1x1) | Pred. (1x1) | Rec. (5x5,..,9x9) | Org. | Filt. | Corr. Mat. |  |
| 0 | 1. Correlation matrix calculation | read | read | read | read |  | write | During encoding: 5x5 diamond +3 |
| 1 | 1. Filtering 2. Control map decision 3. Correlation matrix calculation | read | read | read | read | Write | read & write | Depth = 0, RD = 0 |
| 2 | 1. Filtering 2. Control map decision 3. Correlation matrix calculation | read | read | read | read | Write | read & Write | Depth = 0, RD = 1 |
| 3 | 1. Filtering 2. Control map decision 3. Correlation matrix calculation | read | read | read | read | Write | read & Write | Depth = 0, RD = 2 |
| 4-12 | … | read | read | read | read | write | read & Write |  |
| 13 | 1. Filtering 2. Control map decision 3. Correlation matrix calculation | read | read | read | read | write | read & Write | redesign of 5x5 diamond + 3 |
| 14 | 1. Filtering 2. Control map decision 3. Correlation matrix calculation | read | read | read | read | write | read & Write | redesign of 7x7 diamond + 3 |
| 15 | 1. Filtering 2. Control map decision | read | read | read | read | write | read | redesign of 9x9 diamond + 3 |

Remark:

Although not explicitly requested by the CE8 description, it is worth to mention that the complexity of the decoder increases only negligibly by using only one additional sample of the prediction signal and quantized prediction error signal in addition to the 5x5,..,9x9 samples of the deblocked signal.

Sharp:

During encoding: design filter coefficients (Design) using deblocked pixels(post-DF), pre-deblocked pixels(pre-DF) and store correlation matrices in temporal buffer.

1-9 additional pass: repetition of Filtering and control filtering control (Control) and Design to optimize on/off control and filter coefficients. Control uses filtered image pixels and. Design only uses stored correlation matrices.

10- 12 additional pass: repetition of Filtering and Control and Design to optimize filter tap.

13 additional pass: reconstruct best filtered image.

Details are shown in the following Table.

| pass | operations | access |  |  |  |  | note |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | pre-DF | post-DF | orig | filter | corr. matrices |  |
| 0 | Design | yes | yes | yes | no | yes | duaring encoding: 7x7 diamond +2 |
| 1 | Filtering | yes | yes | no | yes | no |  |
|  | Control | no | no | yes | yes | no | Depth=0, RD=0 |
|  | Design | no | no | no | no | yes | Depth=0, RD=1 |
| 2 | Filtering |  |  |  |  |  | Depth=0, RD=1 |
|  | Control |  |  |  |  |  | Depth=0, RD=1 |
|  | Design |  |  |  |  |  | Depth=0, RD=2 |
| 3 | Filtering |  |  |  |  |  | Depth=0, RD=2 |
|  | Control |  |  |  |  |  | Depth=0, RD=2 |
|  | Design |  |  |  |  |  | Depth=1, RD=1 |
| 4-8 | … |  |  |  |  |  |  |
| 9 | Filtering |  |  |  |  |  | Depth=3, RD=2 |
|  | Control |  |  |  |  |  | Depth=3, RD=2 |
|  | Design |  |  |  |  |  | redesign of 7x7 diamond + 2 |
| 10 | Filtering |  |  |  |  |  |  |
|  | Control |  |  |  |  |  |  |
|  | Design |  |  |  |  |  | 9x9 diamond + 2 |
| 11 | Filtering |  |  |  |  |  |  |
|  | Control |  |  |  |  |  |  |
|  | Design |  |  |  |  |  | 11x11 diamond + 2 |
| 12 | Filtering |  |  |  |  |  |  |
|  | Control |  |  |  |  |  |  |
| 13 | Reconstruct | no | no | no | yes | no |  |

### Comments/Remarks from Cross Checkers

It is observed that relative encoding/decoding time is not consistent depending on the operating environment such as operating system, compiler, turbo mode on multicore machines, output of decoded image, etc.  There are a couple of test points whose results mismatch slightly between proponent and cross-checkers’.  This mismatch is also related to the operating environment.

MediaTek/Qualcomm/Toshiba’s proposal

Intel checked MediaTek/Qualcomm/Toshiba’s proposal, and confirmed the coding gain.

Panasonic checked MediaTek/Qualcomm/Toshiba’s proposal, and confirmed the coding gain.

Sharp checked MediaTek/Qualcomm/Toshiba’s proposal, and confirmed the coding gain.

- Panasonic’s proposal

MediaTek Qualcomm and Toshiba checked Panasonic’s proposal, and confirmed the coding gain.

- Sharp’s proposal

Panasonic checked Sharp’s proposal, and confirmed the coding gain.

Toshiba checked Sharp’s proposal, and confirmed the coding gain.

## Subset 3: Image clipping and offset

### Coding efficiency

Table 9  Summary of BD-Rate (Y) for Subset 3 (positive value means loss)

| Proponent |  | MediaTek (CC) |  |  |  |  | MediaTek (PQAO) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case |  | IO |  | LD |  | RA | IO | LD |  | RA |  |  |  |
| Low Complexity |  | -0.2 |  | -0.3 |  | -0.2 | -1.1 | -2.7 |  | -1.3 |  |  |  |
| High Efficiency |  | -0.3 |  | -0.4 |  | -0.6 | -0.8 | -2.2 |  | -1.5 |  |  |  |

### Complexity – relative encoding/decoding time against the anchor -

Table 10  Summary of Relative Encoding Time (%) for Subset 3 (against the anchor)

| Proponent |  | MediaTek (CC) |  |  |  |  | MediaTek (PQAO) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case |  | IO |  | LD |  | RA | IO | LD |  | RA |  |  |  |
| Low Complexity |  | 100 |  | 100 |  | 100 | 107 | 101 |  | 102 |  |  |  |
| High Efficiency |  | 101 |  | 100 |  | 100 | 103 | 101 |  | 101 |  |  |  |

Table 11  Summary of Relative Decoding Time (%) for Subset 3 (against the anchor)

| Proponent |  | MediaTek (CC) |  |  |  |  | MediaTek (PQAO) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test case |  | IO |  | LD |  | RA | IO | LD |  | RA |  |  |  |
| Low Complexity |  | 101 |  | 100 |  | 101 | 107 | 108 |  | 109 |  |  |  |
| High Efficiency |  | 98 |  | 98 |  | 99 | 102 | 103 |  | 102 |  |  |  |

### Comments/Remarks from Cross Checkers

- MediaTek’s proposal, controlled clipping (CC)

Qualcomm checked MediaTek/Sharp’s proposal, and the result is match.

- MediaTek’s proposal, picture quadtree adaptive offset (PQAO)

Samsung checked MediaTek’s proposal, and the result is match.

# Conclusion

In CE8, nine proposals are evaluated based on the test conditions. The results are summarized according to coding efficiency and complexity. Informal subjective viewing will be conducted during the Daegu meeting for Subset 1, and results will be reported during the meeting.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCTVC-C500, Guangzhou, October 2010.

[2]	T. Yamakage, T. Chujoh, Y.W. Huang, K. Chono, I.S. Chong, T. Yamamoto, J. Lim, B. Jeon, J. Xu and M. Narroschke, “Description of CE8: In-loop filtering,” JCT-VC Document, JCTVC-D508_r1, Guangzhou, October 2010.