| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D086 WG11 Number: m18836 |
| --- | --- |

| Title: | Constrained intra prediction for reducing visual artifacts caused by lossy decoder-side memory compression |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Keiichi Chono (NEC) Hirofumi Aoki (NEC) 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN Xuan Jing (Panasonic) Viktor Wahadaniah (Panasonic) ChongSoon Lim (Panasonic) Sue Mon Thet Naing (Panasonic) Block 1022 Tai Seng Avenue #06-3530 Singapore 534415 | Tel: Email: | (81) 504317595 chono@ct.jp.nec.com hao@bu.jp.nec.com (65) 65505415 viktor.wahadaniah@sg.panasonic.com |
| Source: | NEC Corporation, Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution provides an investigation report on the effects of constrained intra prediction in HEVC codec, in particular, on its performance in reducing visual artifacts caused by encoder-decoder mismatch associated with lossy decoder-side memory compression. Using constrained intra prediction, experimental results show average BD-rate losses of 2.1% (Y), 3.6% (U), 3.5% (V) for random access high efficiency setting, 1.8% (Y), 2.6% (U), 2.8% (V) for random access low complexity setting, 1.7% (Y), 3.7% (U), 3.6% (V) for low delay high efficiency setting, and 1.4% (Y), 2.9% (U), 3.1% (V) for low delay low complexity setting. This contribution also shows the benefit of constrained intra prediction in reducing visual artifacts when decoder-side memory compression is applied.

It is proposed that:

- Integrate the proposed constrained intra prediction into HM software and improve its implementations;

- Study the benefit of constrained intra prediction in reducing visual artifacts with different decoder-side memory compression schemes.

# Introduction

Constrained intra prediction is an essential tool to avoid spatial noise propagations caused by spatial intra prediction with encoder-decoder mismatched reference pixels. The encoder-decoder mismatched reference pixels can appear when packet loss happens in transmitting inter-coded slices. They can also appear when lossy decoder-side memory compression is used. Such spatial noise propagations are fixed-pattern noise and unacceptably noticeable. Therefore constrained intra prediction would be useful not only for enabling robust video transmission over unreliable networks but also for allowing decoders to use their own lossy decoder-side memory compression technique.

This contribution describes our constrained intra prediction implementation in TMuC0.9 and demonstrates its benefits in reducing visual artifacts when decoder-side memory compression is applied

# Constrained Intra Prediction Technique

The proposed constrained intra prediction scheme is similar to that of H.264/MPEG-4 AVC. When constrained intra prediction is enabled, inter-predicted samples are marked as not available for intra prediction. Though this concept is simple, we should additionally consider that availability of neighboring samples may be different pixel by pixel, since the size of coding units is variable in TMuC while that of macroblocks is fixed in AVC. For simplification, availability of neighboring samples is decided every PuPartSize samples in this proposal.

Figure 1 shows an example for availability decision of neighboring samples in the proposed constrained intra prediction. If any above PUs are inter-predicted PUs, all above neighboring samples are marked as “not available for intra prediction.” Otherwise, all above neighboring samples are marked as “available for intra prediction.” Availability of above right, above left, left and below left samples is derived in a similar way. The complete definition of neighboring samples availability is shown in Appendix.

Figure 1 – Neighboring samples availability for constrained intra prediction

# Experimental Setting

The proposed constrained intra prediction scheme was implemented into TMuC 0.9 software. NEC and Panasonic implemented the proposed scheme separately and subsequently conducted cross-checks including code exchanges.

Simulations were conducted using four out of six settings specified in JCTVC-C500 [2], namely:

•	Random Access, High Efficiency

•	Random Access, Low Complexity

•	Low Delay, High Efficiency

•	Low Delay, Low Complexity

Full test sequences and quantization parameter values as specified in JCTVC-C500 were used.

# Results

## Objective Quality

Coding efficiency performance of the proposed constrained intra prediction scheme is summarized in Table 1.

Table 1 – Summary of R-D and complexity performance against TMuC 0.9 anchors

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 2.0 | 2.5 | 2.5 | 1.6 | 1.7 | 2.1 |
| Class B | 1.7 | 3.4 | 3.1 | 1.5 | 2.6 | 2.7 |
| Class C | 2.7 | 4.5 | 4.4 | 2.3 | 3.4 | 3.4 |
| Class D | 2.0 | 3.5 | 3.7 | 1.7 | 2.2 | 2.5 |
| Class E |  |  |  |  |  |  |
| All | 2.1 | 3.6 | 3.5 | 1.8 | 2.6 | 2.8 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 1.6 | 3.8 | 3.8 | 1.4 | 2.9 | 3.4 |
| Class C | 2.8 | 5.4 | 5.4 | 2.3 | 3.8 | 4.1 |
| Class D | 1.6 | 3.4 | 3.6 | 1.2 | 2.6 | 2.7 |
| Class E | 0.6 | 1.5 | 1.2 | 0.6 | 2.2 | 1.8 |
| All | 1.7 | 3.7 | 3.6 | 1.4 | 2.9 | 3.1 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

The exact same BD-rates are observed in both NEC's and Panasonic's results, while encoding and decoding times shown above are based on NEC’s results.

## Subjective Quality

Viewing evaluations were conducted separately by NEC and Panasonic. No visual artifacts, which are caused by the proposed constrained intra prediction, we observed in playback video. (Note that by freezing video playback and comparing individual frames of Anchor and Proposal results, visual differences are observed for Cactus at QP=37. However, the visual differences are invisible as long as video is playbacked.) Therefore, it can be concluded that the proposed constrained intra prediction does not affect the subjective quality performance.

## Error Resilience

This section reports the investigation findings on the usefulness of constrained intra prediction in improving error resilience. In this investigation, lossy decoder-side reference memory compression was used to simulate the occurrence of encoder-decoder mismatch. Coded bitstreams were input to a non-conformant decoder which performs reference memory compression at compression ratio of 50%. Simple spatial compression/decompression methods of checkerbox downsampling and quincunx upscaling as shown in Figure 2 were used in this experiment.

Figure 2 – Memory compression using checkerbox downsampling and quincunx upscaling

Figure 3 illustrates the benefits of constrained intra prediction. In the images on the left, lossy decoder-side reference memory compression causes dark patches to appear on the decoder reconstructed image. Such artifacts are not present for coded bitstreams using constrained intra prediction, as shown in the images on the right.

(a) BQMall frame 552, random access, high efficiency, QP=32

(b) BQSquare frame 312, random access, high efficiency, QP=32

Figure 3 – Effect of constrained intra prediction in reducing visual artifacts caused by decoder-side reference memory compression

It should be noted here that NEC and Panasonic evaluated the effect of reference memory compression schemes presented in JCTVC-C095 [2] and JCTVC-C073 [3]. Both schemes do not show any visual degradation even when they are used as decoder-side memory compression schemes, at least with the current test set ([1]). However, lossy decoder-side reference memory compression can cause unacceptable visual degradation as shown in Figure 3 unless the same lossy reference memory compression is used by encoder. Therefore it would be beneficial to have constrained intra prediction for allowing decoders to use their lossy decoder side memory compression schemes, if reference memory compression is not standardized.

# Conclusions

This contribution has provided the description and simulation results of constrained intra prediction in TMuC0.9 and demonstrated its benefits in reducing visual artifacts when decoder-side memory compression is applied

It is recommended that:

- Integrate the proposed constrained intra prediction into HM software and improve its implementations;

- Study the benefit of constrained intra prediction in reducing visual artifacts with different decoder-side memory compression schemes.

# Patent rights declaration(s)

NEC Corporation and Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- F. Bossen, "Common test conditions and software reference configurations", JCTVC-C500, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd meeting, Guangzhou, 7-15 Oct 2010.

- H. Aoki, K. Chono, K. Senzaki, J. Tajime, Y. Senda, “Performance improvement of DPCM-based memory compression by adaptive quantization,” JCT-VC document JCTVC-C095 (WG11 m18118), Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd meeting, Guangzhou, October 2010.

- V. Wahadaniah, C.S. Lim, S.M.T. Naing, H.W. Sun, "TE2: Reference frame compression using image coder", JCTVC-C073, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd meeting, Guangzhou, 7-15 Oct 2010.

# Appendix: Proposed change of the TMuC text for constrained intra prediction

The proposed change of the TMuC text is shown as follows.

In clause 6.2.1.2 “Intra sample prediction,”

...

- Each sample p[ x, y ] with x = -1, y = -1..PuPartSize*2 – 1 and x = 0..PuPartSize*2 – 1, y = -1 is derived as follows.

- If any of the following conditions is true, the sample p[ x, y ] is marked as “not available for intra prediction”

  - lcuAddrN is not available,

  - partIdxN is not available

- Otherwise, the sample p[ x, y ] is marked as “available for intra prediction” and the luma sample at luma location ( xW, yW ) inside the largest coding unit lcuAddrN is assigned to p[ x, y ].

When samples p[ x, -1 ], with x = PuPartSize..PuPartSize*2 – 1 are marked as “not available for intra prediction,” and the sample p[ PuPartSize – 1, -1 ] is marked as “available for intra prediction,” the sample value of p[ PuPartSize – 1, -1 ] is substituted for samples values p[ x, -1 ], with x = PuPartSize..PuPartSize*2 – 1 and samples p[ x, -1 ], with x = PuPartSize..PuPartSize*2 – 1 are marked as “available for intra prediction.”

When samples p[ -1, y ], with y = PuPartSize..PuPartSize*2 – 1 are marked as “not available for intra prediction,” and the sample p[ -1, PuPartSize – 1 ] is marked as “available for intra prediction,” the sample value of p[ -1, PuPartSize – 1 ] is substituted for samples values p[ -1, y ], with y = PuPartSize..PuPartSize*2 – 1 and samples p[ -1, y ], with y = PuPartSize..PuPartSize*2 – 1 are marked as “available for intra prediction.”

is replaced with

...

- Each sample p[ x, y ] with x = -1, y = -1..PuPartSize*2 – 1 and x = 0..PuPartSize*2 – 1, y = -1 is derived as follows.

- If any of the following conditions is true, the sample p[ x, y ] is marked as “not available for intra prediction”

  - lcuAddrN is not available,

  - partIdxN is not available

  - puIdxN is available, PredMode of puIdxN is not MODE_INTRA, and constrained intra prediction is enabled

- Otherwise, the sample p[ x, y ] is marked as “available for intra prediction” and the luma sample at luma location ( xW, yW ) inside the largest coding unit lcuAddrN is assigned to p[ x, y ].

When any samples p[ x, -1 ], with x = 0..PuPartSize-1 are marked as “not available for intra prediction,” samples p[ x, -1 ], with x = 0..PuPartSize-1 are marked as “not available for intra prediction.”

When any samples p[ -1, y ], with y = 0..PuPartSize-1 are marked as “not available for intra prediction,” samples p[ -1, y ], with y = 0..PuPartSize-1 are marked as “not available for intra prediction.”

When any samples p[ x, -1 ], with x = PuPartSize..PuPartSize*2-1 are marked as “not available for intra prediction,” the following applies.

- If the sample p[ PuPartSize-1, -1 ] is marked as “available for intra prediction,” the sample value of p[ PuPartSize-1, -1 ] is substituted for samples values p[ x, -1 ], with x = PuPartSize..PuPartSize*2-1 and samples p[ x, -1 ], with x = PuPartSize..PuPartSize*2-1 are marked as “available for intra prediction.”

- Otherwise, samples p[ x, -1 ], with x = PuPartSize..PuPartSize*2-1 are marked as “not available for intra prediction.”

When any samples p[ -1, y ], with y = PuPartSize..PuPartSize*2-1 are marked as “not available for intra prediction,” the following applies.

- If sample p[ -1, PuPartSize-1 ] is marked as “available for intra prediction,” the sample value of p[ -1, PuPartSize-1 ] is substituted for samples values p[ -1, y ], with y = PuPartSize..PuPartSize*2-1 and samples p[ -1, y ], with y = PuPartSize..PuPartSize*2-1 are marked as “available for intra prediction.”

- Otherwise, samples p[ -1, y ], with y = PuPartSize..PuPartSize*2-1 are marked as “not available for intra prediction.”