| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D087 |
| --- | --- |

| Title: | Encoding complexity reduction by removal of NxN partition type |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungsun Kim Byeongmoon Jeon #221 Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Tel: Email: | 82-2-6272-9179 Jungsun.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes removal of NxN partition type for both inter and intra predicted CU to reduce encoding complexity. In the current HM, a CU may split into several partitions. Two types of splitting and four types of splitting are used for intra and inter CU respectively. In the proposed methods, NxN partitioning is prevented except when the CU has the maximum depth. The proposed scheme shows approximately 29% encoding time reduction with 0.2% loss in BD-rate [4] under the condition of intra high efficiency and 18% encoding time reduction with 0.3% loss in BD-rate under the condition of random access high efficiency.

- Introduction

In the working draft of HEVC [1], a Coding Unit (CU) is specified for the quadtree segmentation of the given square region. The minimum size and maximum size of CUs are specified in Sequence Parameter Set by log2_min_coding_block_Size_minus3 and log2_diff_max_min_coding_block_size. If a CU is larger than minimum CU size, it could split into four smaller CUs recursively. When a CU does not split further, i.e., a CU has the minimum size, the CU is defined as a Prediction Unit (PU). A PU is basic unit for specifying the prediction process. A PU can be split into multiple partitions which have rectangular or square shape as described in Fig. 1. Intra CUs have only two partition types of 2Nx2N partition and NxN partition but inter CUs have all of four types of partitions.

Fig. 1 Types of partitions in PUs

# Proposed Methods

We proposed to prevent a PU splitting into NxN partitions when the size is larger than minimum CU size in both intra and inter intra prediction. It is extended work of JCTVC-C218 which proposed to disable NxN partition type in intra prediction when the size is larger than minimum CU size[2].

In the current HM, one prediction size is checked twice in two different hierarchical depth levels. For example, the RD cost of 2mx2m intra prediction block is calculated as one of partitions of NxN PU when the CU size is 2m+1x2m+1 and the partition type is NxN. And it is calculated once more as a partition of 2Nx2N PU when the CU size is 2mx2m and the partition type is 2Nx2N. Fig. 2 shows repetition of RD cost calculation for an intra prediction block in two different hierarchical CU levels. The RD costs of a shaded region are calculated as one of NxN partition in Level D and as a 2Nx2N partition in Level D+1.

Fig. 2 Calculations of RD cost for a region in different hierarchical CU levels for intra prediction

To reduce encoding time and computational complexity, we eliminate the process of calculating RD cost of NxN partition for each PU since prediction blocks with the corresponding size can be covered in smaller CU with partition type 2Nx2N. However we keep NxN partition for minimum sized PUs since the PUs do not have smaller CU that could cover the block size. Therefore, in the proposed methods, available partition types are as below:

Table 1 Available partition types for each prediction and CU size

| CU size | Inter Prediction | Intra Prediction |
| --- | --- | --- |
| Minimum | 2Nx2N, 2NxN, Nx2N, NxN | 2Nx2N, NxN |
| Otherwise | 2Nx2N, 2NxN, Nx2N | 2Nx2N |

# Experimental results

Table 2 shows the brief experimental results using the configuration files [3] provided with HM0.9. The anchor is HM0.9(rev. 340) distributed on 2nd November 2010. The experiments for both the anchor and the test are conducted at WindowXP 64bit.

It shows the encoding complexity reduces to 71% and 66% for intra high efficiency case and low complexity case respectively. For random access case it reduces to 82% and 77% for high efficiency and low complexity respectively. For low delay case, it reduces to 79% and 75% for high efficiency and low complexity respectively. The coding losses in bdrate are less than 0.3% for high efficiency case and less than 0.9% for low complexity case. For low complexity case it shows small gains for chroma components with slightly increased PSNRs. More detailed results are described in the accompanied spreadsheet.

Table 2 Experimental Results

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.5 | 0.7 | 1.1 | -0.3 | -0.4 |
| Class B | 0.2 | 0.3 | 0.4 | 1.1 | -0.3 | -0.3 |
| Class C | 0.2 | 0.2 | 0.3 | 0.6 | -0.1 | -0.1 |
| Class D | 0.1 | 0.1 | 0.1 | 0.4 | -0.2 | -0.1 |
| Class E | 0.3 | 0.7 | 0.7 | 1.8 | -1.1 | -0.6 |
| All | 0.2 | 0.3 | 0.4 | 0.9 | -0.4 | -0.3 |
| Enc Time[%] | 71% |  |  | 66% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.3 | 0.4 | 0.9 | -0.8 | -1.0 |
| Class B | 0.2 | 0.2 | 0.1 | 0.8 | -0.4 | -0.2 |
| Class C | 0.3 | 0.3 | 0.3 | 0.8 | 0.2 | 0.2 |
| Class D | 0.3 | 0.4 | 0.5 | 0.6 | -0.1 | 0.4 |
| Class E |  |  |  |  |  |  |
| All | 0.3 | 0.3 | 0.3 | 0.8 | -0.2 | -0.1 |
| Enc Time[%] | 82% |  |  | 77% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | 0.2 | 0.3 | 0.3 | -0.1 | 0.0 |
| Class C | 0.3 | 0.2 | 0.2 | 0.5 | 0.0 | 0.2 |
| Class D | 0.3 | -0.2 | 0.7 | 0.4 | 0.4 | 0.2 |
| Class E | 0.0 | 0.6 | 0.7 | 0.3 | -0.8 | -1.1 |
| All | 0.2 | 0.2 | 0.4 | 0.4 | -0.1 | -0.1 |
| Enc Time[%] | 79% |  |  | 75% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

# Conclusions

In this contribution, we propose to disable NxN partition when a PU size is larger than minimum CU size. It reduces encoding time 29% for intra high efficiency case with 0.2% loss and 34% for intra low complexity case with 0.9% loss. And it reduces encoding time around 20% for random access case with 0.55% loss in average. For low delay case, it reduces encoding time 22.5% with 0.3% loss in average. Since the encoding time can be critical obstacle to adoption of sophisticated tools we suggest reducing encoding time by restricted usage of NxN partition which causes redundant computation.

# Patent rights declarations

LG electronics may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “WD1: Working Draft 1 of High-Efficiency Video Coding”, JCTVC-C403, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Guangzhou, China, Oct. 2010

- Jungsun Kim, Byeongmoon Jeon, “Encoding complexity reduction for Intra prediction by Disabling NxN Partition”, JCTVC-C218, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, China, Oct 2010

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-C500, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, China, Oct. 2010

- Gisle Bjontegaard, “Improvements of the BD-PSNR model”, ITU-T Q.6/SG16 VCEG, VCEG-AI11, Berlin, Germany, July 2008