| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28, January, 2011 | Document: JCTVC-D088 |
| --- | --- |

| Title: | CE7: Cross-check for Samsung's Proposal on Mode-dependent DCT/DST for intra prediction in video coding by NHK |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yasuko Sugito Atsuro Ichigaya NHK Science & Technology Research Laboratories 1-10-11, Kinuta, Setagaya-ku, Tokyo, JAPAN | Tel: Email: | +81-3-5494-3353 sugitou.y-gy@nhk.or.jp ichigaya.a-go@nhk.or.jp |
| Source: | NHK (Japan Broadcasting Corporation) |  |  |

_____________________________

- Abstract

The purpose of this document is to report the result of cross-check on Mode-dependent DCT/DST for intra prediction on/off conducted by Samsung according to the conditions specified in JCTVC-D033 [1]. The verification task has been done successfully.

- Verification

Samsung integrated two types of Mode-dependent DCT/DST into TMuC 0.9 code. One is that DCT/DST is used at all block sizes and the other is that DCT/DST is used at block sizes 4*4 and 8*8 only. NHK cross-checked the former.

The code was compiled using Microsoft® Visual Studio® 2005, and executed on the PCs with the following configurations: Microsoft® Windows® XP 64bits, Intel® Xeon® CPU W3530 @2.80GHz, 16GB of RAM. And -o encoder option (YUV writing) was used to confirm that a reconstructed sequence matches its decoded sequence.

The simulation includes:

- Full Class A, B, C, D and E sequences, under Intra, high efficiency test condition.

- Full Class A, B, C, D and E sequences, under Intra, low complexity test condition.

- Full Class A, B, C, D and E sequences, under Random access, high efficiency test condition.

- Full Class A, B, C, D and E sequences, under Random access, low complexity test condition.

- Results

Results in simulation a and b and results in simulation c and d are shown Table 1 and 2, respectively.

Results in simulation b and d (low complexity test condition) match exactly provided by Samsung, but results in simulation a and c (high efficiency test condition) show minor mismatches in bit rate and PSNR. The difference of bit rate is about -0.2% to +0.09%.

It is informed that Samsung uses Linux to perform the simulation. The mismatches are likely caused by different platforms.

Rates of encoder and decoder times denote the same tendency of Samsung’s.

Table 1: Summary of BD-rate and Encoding/Decoding Time (simulation a and b)

Table 2: Summary of BD-rate and Encoding/Decoding Time (simulation c and d)

- Conclusion

We have cross-checked Mode-dependent DCT/DST for intra prediction conducted by Samsung. The verification task has been done successfully.

- References

| [1] | Ankur Saxena et al, “Mode-Dependent DCT/DST for intra prediction in video coding,” Doc JCTVC-D033, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Daegu, KR, January 2011 |
| --- | --- |