| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D089r1 |
| --- | --- |

| Title: | Redundancy of Bi-directional Inter Prediction in Generalized P and B picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Joonyoung Park Younghee Choi Byeongmoon Jeon 221, Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Tel: Email: {jy.park, younghee.choi, bm.jeon}@lge.com |  |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This document describes a redundancy of bi-directional inter prediction in Generalized P and B picture (GPB) and reports a marginal gain when it is removed. Compared to TMuC 0.9 anchors, there are 0.1% BD-rate reduction for random-access high-efficiency configuration and 0.1% BD-rate reduction for low-delay high-efficiency configuration, on average.

# Introduction

According to the previous version of TMuC (TMuC 0.7), a reference picture list 0 (List_0) and a reference picture list 1 (List_1) are same in a GPB picture. Since List_0 is identical to List_1, there is redundancy in uni-directional inter prediction. In the current TMuC (TMuC 0.9), it has been resolved by disabling uni-directional prediction from list 1, as previously proposed [1], [2]. But there still remains redundancy of bi-directional inter prediction.

# Redundancy of Bi-directional Inter Prediction

In the current test condition [3], the number of reference pictures for each list in a GPB picture is 2. Then, the combination of reference indices for bi-directional prediction is as below (Table 1).

Table 1: Combination of the reference index for bi-directional prediction

| List_0 List_1 | ref_idx 0 | ref_idx 1 |
| --- | --- | --- |
| ref_idx 0 | A | B |
| ref_idx 1 | C | D |

Since List_0 is identical to List_1 for a GPB picture, there is redundancy between the case B and C which have same reference pictures. The redundancy of bi-directional prediction can be reduced by removing either the case B or C.

For instance, assume the case C is eliminated among 4 cases. For the case A of bi-prediction, a reference index for List_0 is transmitted to be 0; whereas a reference index for List_1 is inferred to be 0 (not transmitted). And for the case B and D, the parsing process is same as that of TMuC 0.9. The method to reduce redundancy can be expanded for the case of N reference picture.

# Simulation Results

This simple change is applied to the GPB picture in both low-delay and random access case and tested in both low-delay and random access configurations [3]. An issue in the motion estimation for B0 pictures in RA configurations (ticket #115) is also considered in this test.

Table 2: Test conditions

| Anchor | TMuC 0.9 + ticket #115 fixed |
| --- | --- |
| Test | TMuC 0.9 + ticket #115 fixed + redundancy reduction of bi-prediction |

Table 3: Experimental results of random access configurations

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class B | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 | 0.2 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 4: Experimental results of low delay configurations

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | -0.2 | -0.3 | -0.1 | -0.2 | -0.2 |
| Class C | -0.1 | -0.2 | -0.1 | 0.0 | -0.2 | 0.0 |
| Class D | -0.2 | -0.5 | 0.1 | -0.1 | -0.1 | -0.1 |
| Class E | 0.0 | 0.2 | 0.5 | 0.1 | 0.0 | -0.2 |
| All | -0.1 | -0.2 | 0.0 | 0.0 | -0.1 | -0.1 |
| Enc Time[%] | 100% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |

In this experiment, the average BD-rate reductions are 0.1% for each random-access high-efficiency configuration and 0.1% for low-delay high-efficiency configuration. And there is no significant difference in the complexity compared to the anchor.

# Conclusion

It can be concluded that redundancy of bi-directional prediction can be reduced to improve the coding efficiency. Simulation results show that the redundancy reduction achieves 0.1% BD-rate reduction for random-access high-efficiency configuration and 0.1% BD-rate reduction for low-delay high-efficiency configuration, on average.

# References

- Bin Li, Jizheng Xu, Gary Sullivan, Feng Wu, Houqiang Li, “Redundancy reduction in B-frame coding at temporal level zero”, JCTVC-C278, Guangzhou, October 2010.

- Wei-Jung Chien, Peisong Chen, Xianglin Wang, Marta Karczewicz, “Modified Uni-directional Inter Prediction in Generalized P and B Picture”, JCTVC-C285, Guangzhou, October 2010.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-C500, Guangzhou, October 2010.

- Thomas Wiegand, Woo-Jin Han, Benjamin Bross, Jens-Rainer Ohm, Gary J. Sullivan, “WD1: Working Draft 1 of High-Efficiency Video Coding”, JCTVC-C403, Guangzhou, October 2010.

# Patent rights declaration(s)

LG Electronics may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).