| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D090-v2 MPEG number : m18841 |
| --- | --- |

| Title: | CE3 : Results on Bi/Single MC interpolation filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo and Teruhiko Suzuki Gate City Osaki West 12F 1-11-1 Osaki, Shinagawa-ku Tokyo, 141-0032, JAPAN | Tel: Email: | +81-3-5435-3308 kenji.kondo@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution reports results on enhanced MC filter, Bi/Single switching interpolation filter [1], in Core Experiment3 (CE3). In the test, the coding performance and complexity was measured under common test conditions in CE3, which is defined in JCTVC-C503r5 [2]. Several tap length ( 4, 6, 8, and 12 tap) were tested. In this test, for BD% of Bi/Single filter 8 tap, +0.1% (loss) and +0.1% (loss) for RA HE and LD HE, -4.6% (gain) and -3.8% (gain) for RA LC and LD LC can be achieved. The detailed results are summarized in the attached Excel sheet.

# Introduction

The goal of CE3 is to further investigate interpolation filtering for motion compensation (Luma) technology. This contribution focuses on Bi/Single filter switching that is one of investigation items.

The technique of Bi/Single filter was introduced in [1]. The contribution focused on the 6 tap Bi/Single filter. And, the excepted 6 tap such as 12 tap was not premature. So, this contribution further studies 12, 8 and 4 tap.

And this contribution also studies theoretical complexity analysis.

# Proposed method

This Bi/Single interpolation filter improves quality of prediction in MC as shown in [1], and two filter coefficients sets are switched either inter prediction mode is bi-predictive or not. For example, if current MC block mode is bi-predictive, filter coefficients for bi-prediction is used. In Table 1, the filter coefficients for single and bi-prediction are shown. This single prediction means L0 or L1 prediction.

Table 1 Filter coefficients

| # tap | Sub pel | Single prediction | Bi-prediction |
| --- | --- | --- | --- |
| 4 tap | 1/2 pel | -17, 145, 145, -17 | -32, 160, 160, -32 |
|  | 1/4 pel | -9, 200, 78, -13 | -27, 239, 53, -9 |
| 6 tap | 1/2 pel | 2, -20, 146, 146, -20, 2 | 11, -44, 161, 161, -44, 11 |
|  | 1/4 pel | 2, -16, 208, 81, -25, 6 | 11, -41, 239, 63, -23, 7 |
| 8 tap | 1/2 pel | -2, 15, -41, 156, 156, -41, 15, -2 | -6, 19, -44, 159, 159, -44, 19, -6 |
|  | 1/4 pel | -1, 9, -32, 223, 79, -35, 17, -4 | -7, 19, -43, 241, 62, -23, 12, -5 |
| 12 tap | 1/2 pel | -2, 6, -10, 21, -45, 158, 158, -45, 21, -10, 6, -2 | -3, 9, -17, 28, -52, 163, 163, -52, 28, -17, 9, -3 |
|  | 1/4 pel | 1, 0, -1, 5, -27, 216, 86, -38, 20, -9, 3, 0 | -4, 9, -15, 26, -48, 236, 72, -29, 16, -10, 5, -2 |

# Experiment

## Test condition and machine platform

Experiment was carried out based on HM software version 0.9 that was modified to bug fix #110 and #111. This bug fix is to follow CE3 mandate. And encoding time and decoding time were measured on same platform machines that spec is Core i7 CPU 2.8GHz, and 8 GB memory. And the Linux 64 bit (called Cent OS) is installed in the platform machine.

## Test result

We tested a Bi/Single filter 12 tap for high efficiency case. Anchor is DCTIF 12 tap in HM 0.9. The summary results of BD performance are shown in Table 2.

Table 2 Result of Bi/Single filter 12 tap for high efficiency case (compared with DCTIF 12 tap)

|  | Random access |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.1 |  |  |  |
| Class B | -0.2 | 0.0 | 0.0 | -0.2 | -0.2 | -0.3 |
| Class C | -0.5 | -0.2 | -0.2 | -0.9 | -0.5 | -0.7 |
| Class D | -1.0 | -0.2 | 0.0 | -2.0 | -1.9 | -1.5 |
| Class E |  |  |  | 0.7 | -0.6 | -0.9 |
| All | -0.4 | -0.1 | 0.0 | -0.6 | -0.8 | -0.8 |
| Enc Time[%] | 107% |  |  | 107% |  |  |
| Dec Time[%] | 112% |  |  | 108% |  |  |

For low complexity case, Bi/Single filter 8 tap was compared with directional interpolation filter (DIF) as anchor. The result is shown in Table 3.

Table 3 Result of Bi/Single filter 8 tap for low complexity case (compared with DIF 6 tap)

|  | Random access LoCo |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1.3 | 1.0 | 0.8 |  |  |  |
| Class B | -0.2 | 1.0 | 1.0 | -0.3 | -1.3 | -1.7 |
| Class C | -5.6 | -0.4 | -0.8 | -5.2 | -4.6 | -4.9 |
| Class D | -11.9 | -4.8 | -4.4 | -10.7 | -11.0 | -11.0 |
| Class E |  |  |  | 1.5 | -1.4 | -0.2 |
| All | -4.6 | -0.9 | -0.9 | -3.8 | -4.6 | -4.6 |
| Enc Time[%] | 114% |  |  | 116% |  |  |
| Dec Time[%] | 133% |  |  | 125% |  |  |

Forr low complexity case, Bi/Single filter 6 tap was compared with directional interpolation filter (DIF) as anchor. The result is shown in Table 4.

Table 4 Result of Bi/Single filter 6 tap for low complexity case

|  | Random access LoCo |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.6 | 0.6 | 0.3 |  |  |  |
| Class B | -0.3 | 0.5 | 0.5 | -0.5 | -1.3 | -1.5 |
| Class C | -4.0 | -0.4 | -0.7 | -3.1 | -3.5 | -3.7 |
| Class D | -8.9 | -3.8 | -3.4 | -6.8 | -7.6 | -7.6 |
| Class E |  |  |  | 0.7 | -0.5 | -0.4 |
| All | -3.4 | -0.9 | -0.9 | -2.5 | -3.3 | -3.4 |
| Enc Time[%] | 110% |  |  | 110% |  |  |
| Dec Time[%] | 123% |  |  | 118% |  |  |

For high complexity case, we tested the Bi/Single filter 8 tap compared with DCTIF 8 tap. The result is shown in Table 5

Table 5 Result of Bi/Single filter 8 tap for high efficiency (compared with DCTIF 12 tap)

|  | Random access HE |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.2 |  |  |  |
| Class B | -0.2 | 0.0 | -0.1 | -0.3 | -0.1 | -0.4 |
| Class C | 0.3 | 0.2 | 0.1 | 0.4 | 0.4 | 0.3 |
| Class D | 0.3 | 0.5 | 0.7 | -0.1 | 0.1 | 0.9 |
| Class E |  |  |  | 0.4 | 0.2 | 0.0 |
| All | 0.1 | 0.2 | 0.2 | 0.1 | 0.1 | 0.2 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 106% |  |  | 105% |  |  |

For high complexity case, we tested the Bi/Single filter 8 tap compared with DCTIF 8 tap. The result is shown in Table 6.

Table 6 Result of Bi/Single filter 8 tap for high efficiency case (compared with DCTIF 8 tap)

|  | Random access HE |  |  | Low delay |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | 0.1 |  |  |  |
| Class B | -0.1 | 0.1 | 0.0 | -0.1 | -0.1 | -0.4 |
| Class C | -0.5 | -0.2 | -0.3 | -0.5 | -0.6 | -0.7 |
| Class D | -1.0 | -0.4 | -0.3 | -1.0 | -1.2 | -0.8 |
| Class E |  |  |  | 0.2 | 0.7 | -0.1 |
| All | -0.5 | -0.1 | -0.1 | -0.4 | -0.3 | -0.5 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Table 7 Result of Bi/Single filter 6 tap for high efficiency case (compared with DCTIF 6tap)

|  | Random access LoCo |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.9 | -0.3 | -0.4 |  |  |  |
| Class B | -0.3 | -0.3 | -0.4 | -0.9 | -0.1 | 0.0 |
| Class C | -1.6 | -0.1 | -0.3 | -1.6 | -0.5 | -0.7 |
| Class D | -2.8 | -1.1 | -0.6 | -1.6 | -1.3 | -1.3 |
| Class E |  |  |  | -1.6 | -0.6 | 0.1 |
| All | -1.4 | -0.5 | -0.4 | -1.4 | -0.6 | -0.5 |
| Enc Time[%] | 108% |  |  | 109% |  |  |
| Dec Time[%] | 118% |  |  | 113% |  |  |

Table 8 Results of Bi/Single filter 4 tap for low complexity case (compared with DCTIF 4 tap)

|  | Random access LoCo |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.0 | -1.0 | -0.9 |  |  |  |
| Class B | -1.3 | -0.6 | -0.9 | -2.9 | -1.3 | -1.6 |
| Class C | -1.4 | -0.4 | -0.5 | -2.3 | -0.9 | -1.1 |
| Class D | -1.8 | -0.9 | -0.5 | -2.2 | -1.3 | -0.9 |
| Class E |  |  |  | -5.7 | -2.9 | -3.9 |
| All | -1.6 | -0.7 | -0.7 | -3.1 | -1.5 | -1.7 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 108% |  |  | 107% |  |  |

# Complexity analysis

In software code, Bi/Single switching is very simple for implementation. It is just to change pointer address to point to filter coefficients array by either bi-predictive is used or not. When one block has two references (namely bi-predictive prediction), the pointer of filter coefficients is switched to coefficients for Bi. When one block has only L0 or L1 reference, the pointer is set to coefficients for single-prediction. We expect that the time to pointer switching can be neglected to compare with FIR process.

As shown in Table 6, 8 tap was used to both Bi/Single filter and DCTIF, we can see that encoding time and decoding time is almost same.

On the other hand, at Table 2, 12 tap was used to both Bi/Single filter and DCTIF, the encoding and decoding time of Bi/Single filter increase +7~12%. This increase mainly comes from implementation issue. DCTIF 12 tap is implemented using adder and shift, and DCTIF 8 tap is implemented using multiplication and adder. Also Bi/Single filter is implemented using multiplication and adder style. So, if Bi/Single filter will be refined as adder and shift style, we believe that encoding and decoding time of DCTIF 12 tap will be almost same as DCTIF 12 tap.

# References

[1] K. Kondo and T. Suzuki, “TE3 subset4: Results on Bi/Single filter switching in FIF,” JCTVC-C164, 3rd. JCT-VC Meeting, Guangzhou, CN, 7-15 Oct, 2010.

[2] C. Chujo, “CE3: Interpolation for MC (Luma)”, JCTVC-C504r4, 3rd. JCT-VC Meeting, Guangzhou, CN, 7-15 Oct, 2010.

[2] F. Bossen, et al. "AHG report: Software development and TMuC software technical evaluation” JCT-VC document JCTVC-B003, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 2 nd Meeting: Geneva, CH, 21-28 July, 2010

Sony Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).