| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D091-v2 MPEG number : m18842 |
| --- | --- |

| Title: | CE3: Verification of Samsung new 8-tap DCT-IF filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kenji Kondo and Teruhiko Suzuki Gate City Osaki West 12F 1-11-1 Osaki, Shinagawa-ku Tokyo, 141-0032, JAPAN | Tel: Email: | +81-3-5435-3308 kenji.kondo@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution reports results of cross check for Samsung proposal that is new DCT based interpolation filter (DCTIF) 8 tap and the restricted DCTIF that is discussed in Core Experiment 3 (CE3). The two proposals were verified that exactly match to provided Samsung’s result for BD performance. The detailed result is shown in attached excel sheet.

# Introduction

The goal of CE3 is to further investigate interpolation filtering for motion compensation (Luma) technology. In the CE3, The Samsung proposed new DCTIF [1]. The source code and their test results were provided from Samsung. We verified either matches to Samsung’s result for BD performance. Also, we checked the encoding time and decoding time.

The Samsung had two proposals that first one is new DCTIF and second one is the restricted test as encoder issue. So, we show the results for both proposals in this contribution.

# Experiment

## Test condition and machine platform

Experiment was carried out based on HM software version 0.9 that was modified to bug fix #110 and #111. This bug fix is to follow CE3 mandate. The configuration and common condition [2] are followed.

And encoding time and decoding time were measured on same platform machines that spec is Core i7 CPU 2.8GHz, and 8 GB memory. And the Linux 64 bit (called Cent OS) is installed in the platform machine.

## Test result

### Verification of new DCTIF 8 tap

We show the result for new DCTIF 8 tap compared with conventional DCTIF 8 tap in HM0.9 in Table 1. This result is exact match to Samsung’s result for BD performance. The encoding time and decoding time was not verified because it’s not shown in Samsung’s excel sheet.

Table 1 Result of DCTIF 8 tap compared with HM0.9 DCTIF 8 tap

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.1 | 0.7 | 1.1 | 0.8 |
| Class B | -0.1 | -0.1 | -0.2 | -1.0 | 0.9 | 1.2 |
| Class C | 0.9 | 0.4 | 0.4 | -4.6 | -0.5 | -0.7 |
| Class D | 1.8 | 1.4 | 1.3 | -10.1 | -4.0 | -3.9 |
| Class E |  |  |  |  |  |  |
| All | 0.7 | 0.4 | 0.4 | -4.2 | -0.8 | -0.8 |
| Enc Time[%] | 99% |  |  | 114% |  |  |
| Dec Time[%] | 101% |  |  | 130% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.4 | -0.1 | -0.2 | -2.2 | -1.4 | -1.7 |
| Class C | 0.7 | 1.0 | 1.1 | -5.3 | -4.2 | -4.4 |
| Class D | 0.9 | 1.0 | 2.2 | -10.7 | -9.4 | -9.6 |
| Class E | -0.4 | -0.5 | -0.7 | -1.3 | -1.7 | -1.5 |
| All | 0.2 | 0.4 | 0.6 | -4.9 | -4.2 | -4.3 |
| Enc Time[%] | 99% |  |  | 114% |  |  |
| Dec Time[%] | 100% |  |  | 124% |  |  |

### Verification of restricted new DCTIF 8 tap

We show the result of restricted new DCTIF 8 tap compared with HM0.9 DCTIF 8 tap. As same as above, the encoding time and decoding time was not verified because it’s not shown in Samsung’s excel sheet.

Table 2 Result of restricted new DCTIF 8 tap

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1.9 | 1.2 | 1.2 | 2.6 | 1.8 | 1.5 |
| Class B | 1.5 | 1.0 | 0.9 | 0.7 | 1.3 | 1.3 |
| Class C | 3.2 | 1.6 | 1.7 | -2.4 | 0.1 | 0.0 |
| Class D | 5.0 | 3.2 | 3.4 | -7.2 | -2.4 | -2.4 |
| Class E |  |  |  |  |  |  |
| All | 3.0 | 1.8 | 1.8 | -2.0 | 0.1 | 0.0 |
| Enc Time[%] | 92% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 123% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 1.4 | 2.1 | 2.1 | -0.5 | 0.5 | 0.6 |
| Class C | 3.1 | 3.3 | 3.4 | -3.1 | -2.2 | -2.3 |
| Class D | 6.3 | 7.1 | 7.9 | -6.8 | -5.5 | -5.4 |
| Class E | 1.5 | 2.3 | 2.1 | 0.3 | 1.0 | 1.3 |
| All | 3.1 | 3.7 | 3.9 | -2.6 | -1.6 | -1.5 |
| Enc Time[%] | 91% |  |  | 100% |  |  |
| Dec Time[%] | 97% |  |  | 115% |  |  |

# Conclusion

In this contribution, BD performance and encoding and decoding time of new DCTIF 8 tap and the restricted DCTIF 8 tap were shown. For the BD performance, provided Samsung’s result was checked that it is match to our result.

# References

- JCT-VC, “CE3: Experimental results of DCTIF by Samsung”, JCTVC-D344, 4th. JCT-VC Meeting, Daegu, October 2010.

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-C500, 3rd. JCT-VC Meeting, Guangzhou, CN, October 2010.

Sony Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).