| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D092 |
| --- | --- |

| Title: | Study on coding performance of HM1 with new test sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki, Yoshitaka Morigami and Ali Tabatabai 1-11-1 Osaki, Shinagawa-ku, Tokyo 141-0032, JAPAN 1730 N 1st Street, San Jose, CA 95112, USA | Tel: Email: | +81-3-5435-3308 teruhikos@jp.sony.com Yoshitaka.Morigami@jp.sony.com +1-408-352-4710 Ali.Tabatabai@am.sony.com |
| Source: | Sony Corp. and Sony Electronics Inc. |  |  |

_____________________________

# Abstract

As JCTVC-C168 pointed out, it is important to test more variety of test materials during the development of HEVC. The current test sequences are similar. For example, a lot of panning sequences is included. New test materials to cover more variety of scenes should be collected. In this contribution, we report the coding performance of existing test materials of VCEG/MPEG, as a starting point of the discussion. The coding performance is compared with high profile by JM 17.2.

# Experiments

HM 0.9 is compared with high profile results using JM 17.2. HM conditions are the same as anchor conditions (both random access and low delay) defined by JCTVC-C500.

The HM coding performance is compared with JM for high efficiency and low complexity, and of intra-only, random-access, and low-delay cases.

The parameter settings are as Table 1. QP is set to 22, 27, 32, and 37.

| Sequence name | size | Frame count | Frame rate | Random access | Low-delay |
| --- | --- | --- | --- | --- | --- |
| CrowdRun | 1920x1080 | 100 | 50fps | X | X |
| ParkJoy | 1920x1080 | 100 | 50fps | X | X |
| DucksTakeOff | 1920x1080 | 100 | 50fps | X | X |
| InToTree | 1920x1080 | 100 | 50fps | X | X |
| OldTownCross | 1920x1080 | 100 | 50fps | X | X |
| blue_sky | 1920x1080 | 100 | 50fps | X | X |
| pedestrian | 1920x1080 | 100 | 25fps | X | X |
| riverbed | 1920x1080 | 100 | 25fps | X | X |
| rush_hour | 1920x1080 | 100 | 25fps | X | X |
| station2 | 1920x1080 | 100 | 25fps | X | X |
| sunflower | 1920x1080 | 100 | 25fps | X | X |
| toys_and_calendar | 1920x1080 | 100 | 25fps | X | X |
| tractor | 1920x1080 | 100 | 25fps | X | X |
| shuttlestart | 1280x720 | 100 | 60fps | X | X |
| shields | 1280x720 | 100 | 60fps | X | X |
| sheriff | 1280x720 | 100 | 60fps | X | X |
| raven | 1280x720 | 100 | 60fps | X | X |
| night | 1280x720 | 100 | 60fps | X | X |
| jets | 1280x720 | 100 | 60fps | X | X |
| harbour | 1280x720 | 100 | 60fps | X | X |
| crew | 1280x720 | 100 | 60fps | X | X |
| city | 1280x720 | 100 | 60fps | X | X |
| bigships | 1280x720 | 100 | 60fps | X | X |

Table 1: Test sequences

## Performance

As objective quality performance measure, BD-Rates are computed the basis of fitting 4 test points with the three order polynomial. The results are summarized in Table 2, Table 3, Table 4 and Table 5. Figure 1, Figure 2, Figure 3 and Figure 4 show BD-Rate for each test sequence.

|  | Y BD SNR (db) | Y BD Bit Rate (%) | U BD SNR (db) | U BD Bit Rate (%) | V BD SNR (db) | V BD Bit Rate (%) |
| --- | --- | --- | --- | --- | --- | --- |
| CrowdRun | 0.0 | 1.2 | -0.3 | 7.8 | -0.3 | 10.4 |
| ParkJoy | -0.1 | 1.6 | -0.2 | 5.1 | -0.4 | 15.6 |
| DucksTakeOff | 0.4 | -11.3 | 0.9 | -25.1 | 0.2 | -13.6 |
| InToTree | 0.3 | -14.2 | 0.1 | -8.9 | 0.1 | -6.2 |
| OldTownCross | 0.4 | -14.0 | 0.1 | -8.2 | 0.1 | -6.1 |
| blue_sky | 1.0 | -24.1 | 0.4 | -10.2 | 0.3 | -8.6 |
| pedestrian_area | 0.8 | -20.9 | 0.3 | -10.1 | 0.4 | -10.9 |
| riverbed | 0.2 | -4.4 | -0.3 | 9.4 | 0.0 | -0.5 |
| rush_hour | 1.1 | -33.7 | 0.0 | 1.2 | 0.4 | -11.8 |
| station2 | 1.4 | -36.7 | 0.3 | -16.8 | 0.4 | -19.1 |
| sunflower | 2.1 | -44.9 | 0.5 | -11.7 | 0.2 | -6.3 |
| toys_and_calendar | 0.9 | -34.3 | 0.4 | -23.8 | 0.6 | -23.4 |
| tractor | 1.0 | -23.3 | 0.2 | -9.1 | 0.1 | -2.7 |
| shuttlestart | 0.8 | -27.6 | 1.1 | -42.7 | 0.9 | -48.8 |
| shields | 0.7 | -23.9 | 0.5 | -21.1 | 0.4 | -21.4 |
| sheriff | 0.4 | -12.1 | -0.3 | 15.7 | -0.1 | 4.0 |
| raven | 1.3 | -31.6 | 0.0 | -0.7 | 0.1 | -5.8 |
| night | 0.4 | -11.4 | 0.2 | -5.0 | 0.0 | 0.3 |
| jets | 1.2 | -42.7 | 1.0 | -33.8 | 1.2 | -42.1 |
| harbour | 0.4 | -9.9 | -0.4 | 21.7 | -0.5 | 25.9 |
| crew | 0.5 | -20.2 | 0.5 | -27.3 | 0.4 | -17.0 |
| city | 0.5 | -14.2 | 0.0 | -2.2 | -0.1 | 4.8 |
| bigships | 0.4 | -13.5 | 0.1 | -4.5 | -0.1 | 8.4 |

Table 2: Low delay (Low complexity)

|  | Y BD SNR (db) | Y BD Bit Rate (%) | U BD SNR (db) | U BD Bit Rate (%) | V BD SNR (db) | V BD Bit Rate (%) |
| --- | --- | --- | --- | --- | --- | --- |
| CrowdRun | 0.7 | -14.7 | -0.2 | 3.0 | -0.2 | 4.7 |
| ParkJoy | 0.8 | -13.7 | -0.1 | 2.3 | -0.4 | 14.1 |
| DucksTakeOff | 1.1 | -28.6 | 1.3 | -35.1 | 0.3 | -18.5 |
| InToTree | 0.6 | -24.4 | 0.1 | -12.6 | 0.1 | -21.1 |
| OldTownCross | 0.9 | -40.6 | 0.3 | -34.7 | 0.4 | -35.7 |
| blue_sky | 2.1 | -42.4 | 1.5 | -32.8 | 1.1 | -29.3 |
| pedestrian_area | 1.5 | -35.1 | 0.4 | -14.3 | 0.5 | -14.8 |
| riverbed | 1.2 | -24.5 | -0.1 | -0.6 | 0.0 | -3.2 |
| rush_hour | 1.5 | -43.7 | -0.2 | 7.6 | 0.2 | -6.0 |
| station2 | 2.5 | -55.1 | 0.8 | -34.5 | 1.1 | -38.3 |
| sunflower | 3.2 | -61.0 | 1.8 | -37.1 | 1.3 | -31.4 |
| toys_and_calendar | 1.4 | -48.7 | 0.7 | -40.5 | 1.1 | -39.9 |
| tractor | 1.8 | -38.6 | 0.5 | -18.2 | 0.3 | -11.2 |
| shuttlestart | 1.6 | -46.8 | 1.2 | -47.4 | 0.8 | -45.5 |
| shields | 1.7 | -52.3 | 1.3 | -52.9 | 1.5 | -56.2 |
| sheriff | 0.9 | -27.6 | 0.0 | -1.0 | 0.4 | -20.9 |
| raven | 2.1 | -47.4 | 0.2 | -8.1 | 0.4 | -17.1 |
| night | 1.0 | -22.8 | 0.2 | -6.9 | 0.0 | 0.3 |
| jets | 2.0 | -60.6 | 2.0 | -54.0 | 1.9 | -59.0 |
| harbour | 1.1 | -24.3 | -0.7 | 33.7 | -0.7 | 36.7 |
| crew | 1.1 | -38.5 | 0.5 | -29.4 | 0.4 | -16.8 |
| city | 1.5 | -41.5 | 0.4 | -27.1 | 0.3 | -19.0 |
| bigships | 1.1 | -34.7 | 0.4 | -26.4 | 0.4 | -26.8 |

Table 3: Low delay (high complexity)

|  | Y BD SNR (db) | Y BD Bit Rate (%) | U BD SNR (db) | U BD Bit Rate (%) | V BD SNR (db) | V BD Bit Rate (%) |
| --- | --- | --- | --- | --- | --- | --- |
| CrowdRun | 0.5 | -10.9 | 0.2 | -7.1 | 0.2 | -6.2 |
| ParkJoy | 0.4 | -9.0 | 0.2 | -6.4 | 0.1 | -4.3 |
| DucksTakeOff | 0.7 | -22.3 | 0.6 | -21.9 | 0.4 | -21.5 |
| InToTree | 0.6 | -35.7 | 0.3 | -30.4 | 0.3 | -32.8 |
| OldTownCross | 0.6 | -37.5 | 0.3 | -30.5 | 0.4 | -29.5 |
| blue_sky | 1.8 | -34.3 | 1.0 | -23.9 | 1.1 | -26.8 |
| pedestrian_area | 1.2 | -30.0 | 0.6 | -23.0 | 0.7 | -20.1 |
| riverbed | 0.6 | -12.2 | -0.6 | 31.4 | -0.1 | 9.2 |
| rush_hour | 1.3 | -37.2 | 0.5 | -23.1 | 0.8 | -26.9 |
| station2 | -0.6 | 11.5 | 1.8 | -46.2 | 0.6 | -19.6 |
| sunflower | 5.0 | -74.1 | 0.0 | -4.9 | 1.1 | -32.0 |
| toys_and_calendar | 1.5 | -48.2 | 0.5 | -38.3 | 1.2 | -40.5 |
| tractor | 1.7 | -38.5 | 1.0 | -32.9 | 1.0 | -31.9 |
| shuttlestart | 1.1 | -34.5 | 1.3 | -42.9 | 0.9 | -41.0 |
| shields | 0.8 | -25.7 | 0.5 | -20.2 | 0.6 | -22.4 |
| sheriff | 0.8 | -26.2 | 0.5 | -21.3 | 0.7 | -28.4 |
| raven | 1.9 | -41.8 | 0.6 | -24.7 | 0.8 | -28.2 |
| night | 0.9 | -22.0 | 0.5 | -16.4 | 0.4 | -12.7 |
| jets | 1.0 | -34.5 | 1.0 | -29.7 | 1.2 | -34.4 |
| harbour | 0.9 | -22.1 | 0.3 | -11.3 | 0.2 | -9.5 |
| crew | 1.0 | -36.2 | 0.7 | -37.6 | 1.0 | -34.4 |
| city | 1.0 | -27.1 | 0.5 | -20.5 | 0.3 | -15.0 |
| bigships | 0.8 | -25.6 | 0.7 | -29.0 | 0.4 | -22.0 |

Table 4: Random access (low complexity)

|  | Y BD SNR (db) | Y BD Bit Rate (%) | U BD SNR (db) | U BD Bit Rate (%) | V BD SNR (db) | V BD Bit Rate (%) |
| --- | --- | --- | --- | --- | --- | --- |
| CrowdRun | 1.3 | -26.0 | 0.5 | -16.2 | 0.5 | -16.5 |
| ParkJoy | 1.3 | -25.3 | 0.5 | -15.4 | 0.3 | -11.7 |
| DucksTakeOff | 1.5 | -42.8 | 1.2 | -39.1 | 0.7 | -32.5 |
| InToTree | 1.0 | -50.7 | 0.5 | -44.8 | 0.5 | -45.7 |
| OldTownCross | 0.9 | -52.5 | 0.5 | -48.3 | 0.8 | -49.4 |
| blue_sky | 2.9 | -48.9 | 2.3 | -45.0 | 2.0 | -44.0 |
| pedestrian_area | 1.9 | -43.7 | 0.9 | -30.5 | 1.0 | -28.4 |
| riverbed | 1.6 | -30.9 | -0.4 | 17.9 | 0.0 | 4.3 |
| rush_hour | 1.8 | -48.6 | 0.6 | -28.9 | 0.9 | -32.7 |
| station2 | 0.2 | -15.2 | 2.1 | -55.1 | 1.1 | -34.4 |
| sunflower | 5.9 | -81.0 | 0.6 | -22.9 | 1.6 | -43.6 |
| toys_and_calendar | 1.9 | -58.4 | 0.7 | -50.1 | 1.6 | -52.2 |
| tractor | 2.6 | -51.7 | 1.5 | -44.1 | 1.5 | -42.1 |
| shuttlestart | 1.8 | -50.5 | 1.5 | -48.2 | 1.0 | -42.1 |
| shields | 1.4 | -43.1 | 1.1 | -40.8 | 1.2 | -42.6 |
| sheriff | 1.3 | -40.5 | 0.8 | -34.7 | 1.0 | -40.4 |
| raven | 2.6 | -53.4 | 1.0 | -33.8 | 1.2 | -36.3 |
| night | 1.4 | -33.5 | 0.8 | -24.4 | 0.6 | -20.0 |
| jets | 1.6 | -51.1 | 1.7 | -43.1 | 1.3 | -41.9 |
| harbour | 1.6 | -36.5 | 0.5 | -19.5 | 0.5 | -19.0 |
| crew | 1.5 | -50.7 | 0.8 | -41.8 | 1.1 | -38.4 |
| city | 1.9 | -45.0 | 0.9 | -38.3 | 0.7 | -31.7 |
| bigships | 1.5 | -41.2 | 1.0 | -39.4 | 0.7 | -34.6 |

Table 5: Random access (high complexity)

Figure 1: Low delay (low complexity)

Figure 2: Low delay (high complexity)

Figure 3: Random access (low complexity)

Figure 4: Random access (high complexity)

# Discussions

## Coding performance of HM

These results show that average coding gain is about -20.3 % with Low delay for low complexity, -37.7 % with Lowdelay for high complexity, -30.6 % with Random access for low complexity, and 45.4 with Random access for high complexity.

It is observed that the current HM1 low complexity setting performs better than high profile of AVC.

## Current test sequence in JCTVC-C500

The current test sequences used in the common test conditions defined in JCTVC-C500 is the same as the sequences used for CfP. Those sequences were selected from the point of view to conduct subjective test (except some cases, e.g. class A). Although those sequences were carefully selected, similar types of scenes are included, e.g. panning. The sequences with fast motion or complicated motion are missing. Even within the existing VCEG/MPEG sequences, we observed difficult contents, for example,

Crowd run, park joy (this is panning), night, harbour, etc

Those sequences are good to test during the development of HEVC and we propose to include those sequences. (especially crowd run (for class B) and night, harbour (for class E))

# Conclusion

In this contribution, coding performance of HM compared with JM is reported. Although new good test materials to cover more variety of scene should be continued, we believe some of the existing sequences are good to test during the codec development. There fore we propose

- to include Crowd run (for class B), Night and harbour (for class E)

- to continue to collect new test materials to cover variety of test sequences (especially high resolution video)

Sony Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).