| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 Jan, 2011 | Document: JCTVC-D093 |
| --- | --- |

| Title: | Reference Lists For B Pictures Under Low Delay Constraints |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | ChongSoon Lim Sue Mon Thet Naing Viktor Wahadaniah Xuan Jing Blk 1022 Tai Seng Avenue #06-3530 Singapore 534415 | Tel: Email: | (65) 65505487 ChongSoon.Lim@sg.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution provides an investigation report on the different reference picture lists settings when QP fluctuation is used for low delay constraints.  The best coding performance can be achieved when the reference picture list 0 for B pictures are ordered based on quality while the reference pictures list 1 is ordered based on POC.  The proposed technique provides an average gain of 11.0% for high efficiency and 11.5% for low complexity settings under low delay constraints.

# Introduction

From the Dresden meeting, the document [1] provides a report on the subjective test results of responses to the Joint Call for Proposals (CfP).  In that report, one interesting observation that can be made is that the beta anchor performed subjectively better than gamma anchor for most cases in low delay constraints.  One of the possible reasons for the subjective improvement is the use of QP fluctuation in the beta anchor.

In this proposal, we explore the impact of the use of QP fluctuation under low delay constraints and report on the experiments conducted to further improve the coding efficiency by introducing different settings for reference picture lists of B pictures under low delay constraints.





Figure 1 – Overall average MOS results over all Classes
for Low Delay (constraint set 2) coding conditions

# QP Fluctuation

HM 0.9 supports QP fluctuation for low delay condition by setting the RateGOPSize equal to 4.

The QP Scaling used in HM is shown below.

QP scaling: QP (I picture), QP+1 (first B layer), QP+4 (second B layer), QP+5 (third B layer)

# Reference Lists For B Pictures

## Reference lists Construction in HM

### Case1:  No QP Fluctuation (RateGOPSize equal to 1) (Default)

When QP Fluctuation is not used, currently HM uses the nearest reference pictures for both lists.  For example, as shown in Figure 2, the reference pictures in List0 and List1 are ordered based on POC.

Figure 2 – Reference Lists For HM (RateGOPSize equal to 1)

### Case 2: With QP Fluctuation (Rate GOP Size equal to 4) (Default)

When QP Fluctuation is used, currently HM uses the higher quality reference pictures for both lists.  For example, as shown in Figure 2, the reference pictures in List0 and List1 are ordered based on quality.

Figure 3 – Reference Lists For HM (RateGOPSize equal to 4)

### Case 3: With QP Fluctuation (Rate GOP Size equal to 4) & Nearest Reference Pictures

Instead of using higher quality pictures as reference pictures, we tried to compare the case when the nearest pictures are used as reference pictures when QP fluctuation is used.  For a fair comparison, we restricted the number of reference indexes to be similar to case 2.

Figure 4 – Nearest Reference Pictures Lists For HM (RateGOPSize equal to 4)

### Case 4: With QP Fluctuation (Rate GOP Size equal to 4) & Proposed Reference Pictures Lists

In this setting, reference picture list 0 is ordered based on quality while reference picture list 1 is ordered based on POC.  Similar to case 3, we restricted the number of reference indexes to be similar to case 2 for a fair comparison.

Figure 5 – Proposed Reference Pictures Lists For HM (RateGOPSize equal to 4)

To have a fair comparison with the proposed setting relative to case 2 and case 3, total number of reference frames used for both lists is increased to two reference pictures for all POC.

### Case 5: With QP Fluctuation (Rate GOP Size equal to 4) (Default) & Two Higher Quality Reference Pictures

Based on Case 2, the following setting used QP Fluctuation with two higher quality reference pictures for each list.  An example of default reference lists using 2 reference pictures is shown in Figure 6, two reference pictures in each List0 and List1 are ordered based on quality.

Figure 6 – Higher Quality Reference Pictures Lists For HM using 2 Reference Pictures (RateGOPSize equal to 4)

### Case 6: With QP Fluctuation (Rate GOP Size equal to 4) & Two Nearest Reference Pictures

Similar to Case 3, the reference pictures for both lists are ordered based on nearest reference pictures when QP fluctuation is used.  However, the numbers of reference indexes used were fixed to 2 reference pictures for each list, instead of 1 reference picture.

Figure 7 – Nearest Reference Pictures Lists For HM using 2 Reference Pictures (RateGOPSize equal to 4)

# Simulation Results

## Simulation Conditions

Experiments were conducted using 0.9-hm revision 345 low delay setting:

- Anchor: 0.9-hm  low delay default setting (RateGOPSize = 1)

  - No QP fluctuation

  - Default reference lists

- Low delay default setting with RateGOPSize = 4

  - QP fluctuation

  - Default reference lists (Higher quality pictures as reference pictures for both list 0 and list 1)

- Nearest POC reference picture lists with RateGOPSize = 4

  - QP fluctuation

  - Nearest pictures as reference pictures for both list 0 and list 1

- Proposed reference picture lists construction with RateGOPSize = 4

  - QP fluctuation

  - Higher quality picture as reference picture for list 0 and

  - Nearest picture as reference picture for list 1

- Default reference picture lists with 2 reference pictures and RateGOPSize = 4

  - QP fluctuation

  - 2 Higher quality pictures as reference pictures for each list 0 and list 1

- Nearest POC reference picture lists with 2 reference pictures and RateGOPSize = 4

  - QP fluctuation

  - 2 Nearest pictures as reference pictures for each list 0 and list 1

## Results Comparison For Different Reference Lists Settings

### Rate distortion performance for QP fluctuation (Rate GOP Size equal to 4) with different reference lists settings relative to No QP fluctuation (Rate GOP Size equal to 1)

| High Efficiency Low Delay | Case 2: |  |  | Case 3: |  |  | Case 4: |  |  | Case 5: |  |  | Case 6: |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RateGOPSize=4 (Default) |  |  | RateGOPSize=4 |  |  | RateGOPSize=4 |  |  | RateGOPSize=4 |  |  | RateGOPSize=4 |  |  |
|  | (H-quality Frame) |  |  | (Nearest POC) |  |  | (Proposed) |  |  | (H-quality 2 Ref Frames) |  |  | (Nearest POC 2 Ref Frames) |  |  |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class B | 5.3% | -23.1% | -24.6% | -2.7% | -26.0% | -25.4% | -7.2% | -30.6% | -31.0% | 0.3% | -26.0% | -27.3% | -6.0% | -28.8% | -28.6% |
| Class C | 2.6% | -17.6% | -17.1% | -5.0% | -21.9% | -21.0% | -10.9% | -26.8% | -26.1% | -2.0% | -20.9% | -20.8% | -8.7% | -24.9% | -24.1% |
| Class D | -2.6% | -23.8% | -21.2% | -1.2% | -21.5% | -19.2% | -12.5% | -30.9% | -28.6% | -6.5% | -26.3% | -23.8% | -8.2% | -27.3% | -24.4% |
| Class E | -5.1% | -26.8% | -27.9% | -9.2% | -25.0% | -23.6% | -15.6% | -33.4% | -33.3% | -7.4% | -29.3% | -30.0% | -12.3% | -29.4% | -28.1% |
| All | 0.7% | -22.6% | -22.5% | -4.1% | -23.7% | -22.4% | -11.0% | -30.3% | -29.6% | -3.4% | -25.4% | -25.3% | -8.4% | -27.5% | -26.3% |

Table 4.2.1 –High Efficiency: average BD-Rate gain for QP fluctuation (RateGOPSize=4) with different reference lists settings

| Low Complexity Low Delay | Case 2: |  |  | Case 3: |  |  | Case 4: |  |  | Case 5: |  |  | Case 6: |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RateGOPSize=4 (Default) |  |  | RateGOPSize=4 |  |  | RateGOPSize=4 |  |  | RateGOPSize=4 |  |  | RateGOPSize=4 |  |  |
|  | (H-quality Frame) |  |  | (Nearest POC) |  |  | (Proposed) |  |  | (H-quality 2 Ref Frames) |  |  | (Nearest POC 2 Ref Frames) |  |  |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class B | 4.4% | -20.3% | -21.3% | 2.5% | -16.2% | -14.8% | -8.1% | -27.3% | -27.2% | -0.3% | -23.7% | -24.9% | -4.4% | -22.7% | -22.4% |
| Class C | 3.5% | -18.1% | -16.7% | 0.7% | -15.9% | -14.3% | -10.9% | -27.4% | -25.9% | -2.6% | -22.4% | -21.2% | -6.2% | -22.2% | -20.8% |
| Class D | -0.9% | -24.4% | -21.2% | 6.9% | -13.9% | -11.0% | -12.7% | -31.8% | -28.8% | -6.9% | -28.1% | -25.2% | -5.8% | -24.7% | -21.7% |
| Class E | -5.7% | -18.2% | -16.7% | -6.1% | -16.5% | -14.9% | -16.2% | -25.9% | -24.9% | -9.0% | -20.1% | -19.3% | -11.9% | -22.6% | -21.3% |
| All | 0.9% | -20.4% | -19.3% | 1.5% | -15.6% | -13.8% | -11.5% | -28.2% | -26.8% | -4.2% | -23.8% | -23.0% | -6.6% | -23.1% | -21.6% |

Table 4.2.2 - Low Complexity: average BD-Rate gain for QP fluctuation (RateGOPSize=4) with different reference lists settings

| Sequence Name | BD-Rate Luma (High Efficiency) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Case 2: | Case 3: | Case 4: | Case 5: | Case 6: |
|  | Default | Nearest | Proposed | Default (2 Ref) | Nearest (2 Ref) |
|  | Ref List | Ref List | Ref List | Ref List | Ref List |
| Kimono | 11.5% | -1.5% | -3.2% | 10.9% | -2.5% |
| ParkScene | -6.4% | -9.8% | -14.4% | -8.5% | -11.8% |
| Cactus | 6.5% | -3.3% | -8.2% | 0.0% | -6.3% |
| BasketballDrive | 14.7% | 1.6% | -0.8% | 13.1% | -1.1% |
| BQTerrace | 0.0% | -0.6% | -9.6% | -13.9% | -8.1% |
| BasketballDrill | -0.2% | -10.8% | -15.0% | -7.0% | -13.7% |
| BQMall | 4.6% | -6.1% | -9.6% | 3.7% | -8.9% |
| PartyScene | -17.8% | -11.2% | -24.3% | -23.3% | -17.7% |
| RaceHorses | 23.7% | 8.2% | 5.3% | 18.7% | 5.7% |
| BasketballPass | 13.1% | -2.5% | -3.4% | 12.0% | -3.6% |
| BQSquare | -27.8% | 7.3% | -26.4% | -35.1% | -15.0% |
| BlowingBubbles | -13.6% | -12.6% | -20.4% | -17.9% | -15.4% |
| RaceHorses | 17.9% | 3.0% | 0.2% | 15.2% | 1.1% |
| Vidyo1 | -5.4% | -9.7% | -14.3% | -6.8% | -12.0% |
| Vidyo3 | -3.6% | -7.1% | -15.6% | -5.7% | -10.2% |
| Vidyo4 | -6.4% | -10.8% | -17.0% | -9.7% | -14.7% |
| Average (All Seq) | 0.7% | -4.1% | -11.0% | -3.4% | -8.4% |

Table 4.2.3 - High Efficiency: Per Sequence BD-Rate gain for QP fluctuation (RateGOPSize=4) with different reference lists settings

| Sequence Name | BD-Rate Luma (Low Complexity) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Case 2: | Case 3: | Case 4: | Case 5: | Case 6: |
|  | Default | Nearest | Proposed | Default (2 Ref) | Nearest (2 Ref) |
|  | Ref List | Ref List | Ref List | Ref List | Ref List |
| Kimono | 14.4% | 1.9% | -1.1% | 13.4% | -0.4% |
| ParkScene | -6.6% | -4.7% | -15.3% | -9.3% | -10.3% |
| Cactus | 7.5% | -1.0% | -7.9% | 0.3% | -6.2% |
| BasketballDrive | 15.5% | 5.8% | -0.6% | 13.5% | 0.4% |
| BQTerrace | -8.8% | 10.4% | -15.6% | -19.6% | -5.7% |
| BasketballDrill | 0.3% | -7.8% | -15.6% | -7.1% | -13.0% |
| BQMall | 3.8% | -1.9% | -10.5% | 2.1% | -7.7% |
| PartyScene | -17.3% | -2.1% | -25.0% | -26.8% | -12.9% |
| RaceHorses | 27.4% | 14.5% | 7.4% | 21.3% | 8.7% |
| BasketballPass | 17.2% | 0.9% | -1.4% | 15.4% | -1.5% |
| BQSquare | -29.8% | 25.6% | -31.0% | -42.0% | -12.5% |
| BlowingBubbles | -13.6% | -7.7% | -20.9% | -19.3% | -12.7% |
| RaceHorses | 22.5% | 9.0% | 2.3% | 18.4% | 3.4% |
| Vidyo1 | -6.1% | -6.1% | -15.1% | -8.3% | -11.3% |
| Vidyo3 | -5.1% | -5.8% | -17.0% | -9.0% | -11.1% |
| Vidyo4 | -6.0% | -6.6% | -16.6% | -9.8% | -13.2% |
| Average (All Seq) | 0.9% | 1.5% | -11.5% | -4.2% | -6.6% |

Table 4.2.4 - Low Complexity: Per Sequence BD-Rate gain for QP fluctuation (RateGOPSize=4) with different reference lists settings

# Conclusion

This contribution provides results for different reference lists construction for B pictures under low delay constraints.  Among the different reference picture lists setting, the best setting is the setting with reference list 0 ordered based on quality and the reference list 1 ordered based on POC.  We recommend to enable this functionality in HM as it provides an average gain of 11.0% for high efficiency and 11.5% for low complexity settings as compared to the current low delay configurations.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] V. Baroncini, J. Ohm, G. Sullivan, “Report of Subjective Test Results of Responses to the Joint Call for Proposals (CfP) on Video Coding Technology for High Efficiency Video Coding (HEVC)”, ITU-T/ISO JCT-VC, JCTVC-A204, Dresden, Germany, April 2010