| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D094 |
| --- | --- |

| Title: | Constrained Intra Prediction Scheme for Flexible-Sized Prediction Units in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Viktor Wahadaniah ChongSoon Lim Sue Mon Thet Naing Xuan Jing Block 1022 Tai Seng Avenue #06-3530 Singapore 534415 | Tel: Email: | (65) 65505415 viktor.wahadaniah@sg.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution provides an investigation report on the possible improvements of constrained intra prediction scheme in HEVC. By adapting the constrained intra prediction scheme to the characteristics of HEVC video coding scheme, in particular the flexible-sized prediction units, experimental results show average BD-rate differences over H.264-style constrained intra prediction scheme of -0.2% (Y), -0.5% (U), -0.5% (V) for random access high efficiency setting, -0.2% (Y), -0.3% (U), -0.3% (V) for random access low complexity setting, -0.1% (Y), -0.2% (U), +0.1% (V) for low delay high efficiency setting, and -0.1% (Y), -0.1% (U), -0.2% (V) for low delay low complexity setting. It is concluded that the coding efficiency loss introduced by H.264-style constrained intra prediction scheme can be further reduced by considering HEVC characteristics in the constrained intra prediction design. It is firstly recommended to include the functionality of constrained intra prediction into HEVC. It is also recommended to conduct further investigation on the suitable constrained intra prediction design for HEVC.

# Introduction

Constrained intra prediction has been shown to be beneficial for error resilience especially in reducing error propagation from spatially-predicted intra-coded samples (Error: Reference source not found, Error: Reference source not found). Constrained intra prediction restricts the intra prediction process for a target prediction unit to use only the intra-coded neighboring samples. H.264 features a constrained intra prediction scheme in which specific intra prediction modes are disabled when corresponding reference samples are not intra-predicted. An adaptation of H.264-style constrained intra prediction scheme is presented in JCTVC-D086 (Error: Reference source not found).

# Constrained Intra Prediction for Flexible-Sized Prediction Units

As shown in Error: Reference source not found, constrained intra prediction is useful in reducing visual artifacts caused by spatial error drifts. As a consequence of the reduction in available reference samples and intra prediction modes, it is reported in Error: Reference source not found that H.264-style constrained intra prediction scheme introduces average coding efficiency losses of 2.1% (Y), 3.6% (U), 3.5% (V) for random access high efficiency setting, 1.8% (Y), 2.6% (U), 2.8% (V) for random access low complexity setting, 1.7% (Y), 3.7% (U), 3.6% (V) for low delay high efficiency setting, and 1.4% (Y), 2.9% (U), 3.1% (V) for low delay low complexity setting.

H.264 constrained intra prediction requires every reference sample (except from top-right neighboring locations) to be intra-predicted. Such a requirement is appropriate for H.264, where intra/inter prediction type is specified for each 16x16 macroblock unit. In contrast, intra/inter prediction type in HEVC is specified for flexible-sized prediction units whose sizes range from 4x4 to 64x64. As such, there are instances where some intra prediction reference samples are intra-coded while others are inter-coded, as shown in Figure 1 below.

Figure 1 - Example of H.264-style constrained intra prediction in an inter picture

When H.264-style constrained intra prediction scheme is used for the above example, all intra prediction modes except DC mode is disabled. Even for H.264-style DC intra prediction mode, the partially available intra-predicted reference samples are not used in calculating prediction sample values.

Above example in Figure 1 suggests that it may be beneficial for the coding efficiency to use the partially available intra-predicted reference samples in a constrained intra prediction scheme for HEVC. One such constrained intra prediction scheme is proposed in this document. The proposed scheme replaces reference sample values at inter-predicted sample locations with values calculated from adjacent intra-prediction reference samples on the left or below and on the right or above. After the recalculation and replacement, the inter-predicted reference samples are allowed to be used as intra prediction reference samples. As a result, all intra prediction modes can remain available and the encoder can have the freedom to select a most efficient mode.

Recalculation process for intra prediction reference samples in the proposed scheme is described as follows:

- Each intra prediction reference sample p[x, y] with x = -1, y = -1 .. PuPartSize*2 – 1 and x = 0 .. PuPartSize*2 – 1, y = -1 is marked as “available for intra prediction” or “not available for intra prediction” as specified in section 8.3.2.1 of HEVC specification JCTVC-C403 (Error: Reference source not found) with the additional condition that an inter-predicted reference sample is marked as “not available for intra prediction”. PuPartSize denotes the prediction unit size, e.g. 16 in the example in Figure 1.

- The following steps are performed for below-left reference sample p[-1, PuPartSize*2 – 1]:

- If p[-1, PuPartSize*2 – 1] is marked as “not available for intra prediction”:

  - Search for a nearest sample marked as “available for intra prediction” starting from p[-1, PuPartSize*2 – 1] to p[-1, -1]. If the nearest sample marked as “available for intra prediction” is not found, continue to search starting from p[0, -1] to p[PuPartSize*2 – 1, -1].

  - If the nearest sample marked as “available for intra prediction” is found, assign its value to p[-1, PuPartSize*2 – 1] and mark p[-1, PuPartSize*2 – 1] as “available for intra prediction”.

- The following steps are performed for left and below-left reference samples starting from p[-1, PuPartSize*2 – 2] to p[-1, -1]:

- If a current sample p[-1, y] is marked as “not available for intra prediction”:

  - Search for a first nearest sample marked as “available for intra prediction” starting from p[-1, PuPartSize*2 – 1] to p[-1, y + 1].

  - If the first nearest sample marked as “available for intra prediction” is found, assign its value to the intermediate sample p1 and mark p1 as “available”. Otherwise, mark p1 as “not available”

  - Search for a second nearest sample marked as “available for intra prediction” starting from p[-1, y – 1] to p[-1, -1]. If the second nearest sample marked as “available for intra prediction” is not found, continue to search starting from p[0, -1] to p[PuPartSize*2 – 1, -1].

  - If the second nearest sample marked as “available for intra prediction” is found, assign its value to the intermediate sample p2 and mark p2 as “available”. Otherwise, mark p2 as “not available”

  - If p1 and p2 are available, calculate p[-1, y] as p[-1, y] = ((p1 + p2 + 1) >> 1) and mark p[-1, y] as “available for intra prediction”. Otherwise, if p1 is available, set p[-1, y] = p1 and mark p[-1, y] as “available for intra prediction”. Otherwise, if p2 is available, set p[-1, y] = p2 and mark p[-1, y] as “available for intra prediction”.

  - Otherwise, continue to the next sample.

- The following steps are performed for above-left reference sample p[-1, -1]:

- If p[-1, -1] is marked as “not available for intra prediction”:

  - Search for a first nearest sample marked as “available for intra prediction” starting from p[-1, PuPartSize*2 – 1] to p[-1, 0].

  - If the first nearest sample marked as “available for intra prediction” is found, assign its value to the intermediate sample p1 and mark p1 as “available”. Otherwise, mark p1 as “not available”

  - Search for a second nearest sample marked as “available for intra prediction” starting from p[0, -1] to p[PuPartSize*2 – 1, -1].

  - If the second nearest sample marked as “available for intra prediction” is found, assign its value to the intermediate sample p2 and mark p2 as “available”. Otherwise, mark p2 as “not available”

  - If p1 and p2 are available, calculate p[-1, -1] as p[-1, -1] = ((p1 + p2 + 1) >> 1) and mark p[-1, -1] as “available for intra prediction”. Otherwise, if p1 is available, set p[-1, -1] = p1 and mark p[-1, -1] as “available for intra prediction”. Otherwise, if p2 is available, set p[-1, -1] = p2 and mark p[-1, -1] as “available for intra prediction”.

- The following steps are performed for above and above-right reference samples starting from p[0, -1] to p[PuPartSize*2 – 2, -1]:

- If a current sample p[x, -1] is marked as “not available for intra prediction”:

  - Search for a first nearest sample marked as “available for intra prediction” starting from p[x – 1, -1] to p[-1, -1]. If the first nearest sample marked as “available for intra prediction” is not found, continue to search starting from p[-1, 0] to p[-1, PuPartSize*2 – 1].

  - If the first nearest sample marked as “available for intra prediction” is found, assign its value to the intermediate sample p1 and mark p1 as “available”. Otherwise, mark p1 as “not available”

  - Search for a second nearest sample marked as “available for intra prediction” starting from p[x + 1, -1] to p[PuPartSize*2 – 1, -1].

  - If the second nearest sample marked as “available for intra prediction” is found, assign its value to the intermediate sample p2 and mark p2 as “available”. Otherwise, mark p2 as “not available”

  - If p1 and p2 are available, calculate p[x, -1] as p[x, -1] = ((p1 + p2 + 1) >> 1) and mark p[x, -1] as “available for intra prediction”. Otherwise, if p1 is available, set p[x, -1] = p1 and mark p[x, -1] as “available for intra prediction”. Otherwise, if p2 is available, set p[x, -1] = p2 and mark p[x, -1] as “available for intra prediction”.

  - Otherwise, continue to the next sample.

- The following steps are performed for above-right reference sample p[PuPartSize*2 – 1, -1]:

- If p[PuPartSize*2 – 1, -1] is marked as “not available for intra prediction”:

  - Search for a nearest sample marked as “available for intra prediction” starting from p[PuPartSize*2 – 2, -1] to p[-1, -1]. If the nearest sample marked as “available for intra prediction” is not found, continue to search starting from p[-1, 0] to p[-1, PuPartSize*2 – 1].

  - If the nearest sample marked as “available for intra prediction” is found, assign its value to p[PuPartSize*2 – 1, -1] and mark p[PuPartSize*2 – 1, -1] as “available for intra prediction”.

Applying the proposed scheme to the example in Figure 1 produces the following results:

- p[-1, 23] to p[-1, 0] are calculated as (p[-1, 24] + p[-1, -1] + 1) >> 1).

- p[0, -1] to p[11, -1] are calculated as (p[-1, -1] + p[12, -1] + 1) >> 1).

- p[16, -1] to p[31, -1] are copied from p[15, -1].

# Experimental Setting

The proposed constrained intra prediction scheme was implemented into TMuC software version 0.9 revision 345. In order to keep intra picture coding unchanged from reference TMuC software, recalculation of intra prediction reference samples and enabling of all intra prediction modes are not done when the reference samples are located physically outside picture boundaries. In future, further investigation may be conducted to evaluate whether reference samples recalculation and intra prediction mode enabling beyond picture boundaries are beneficial to the coding efficiency of intra pictures.

Simulations were conducted using four out of six settings specified in JCTVC-C500 Error: Reference source not found, namely:

- Random Access, High Efficiency

- Random Access, Low Complexity

- Low Delay, High Efficiency

- Low Delay, Low Complexity

Full test sequences and quantization parameter values as specified in Error: Reference source not found were used. Two separate sets of simulations were conducted on the full test set:

- H.264-style constrained intra prediction scheme (Error: Reference source not found).

- Proposed constrained intra prediction scheme.

# Experimental Results

Experimental results of the proposed constrained intra prediction scheme are shown in Table 1 and Figure 2. It is observed that coding efficiency of proposed scheme is higher than H.264-style constrained intra prediction. In addition, the proposed scheme does not increase computational complexity, as suggested by the encoding and decoding time. It should be noted additionally that results reported in Error: Reference source not found show that encoding and decoding time of H.264-style constrained intra prediction does not differ significantly from the TMuC anchor encoding and decoding time.

Table 1 - Coding efficiency of proposed constrained intra prediction scheme compared to H.264-style constrained intra prediction scheme

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | 0.0 | 0.0 | -0.1 | -0.2 | -0.1 |
| Class B | -0.2 | -0.5 | -0.6 | -0.2 | -0.3 | -0.4 |
| Class C | -0.3 | -0.5 | -0.4 | -0.3 | -0.4 | -0.4 |
| Class D | -0.2 | -0.6 | -0.7 | -0.2 | -0.4 | -0.3 |
| Class E |  |  |  |  |  |  |
| All | -0.2 | -0.5 | -0.5 | -0.2 | -0.3 | -0.3 |
| Enc Time[%] | 92% |  |  | 93% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | -0.2 | 0.3 | 0.0 | 0.0 | -0.2 |
| Class C | -0.1 | -0.2 | 0.0 | -0.1 | -0.1 | 0.0 |
| Class D | 0.0 | -0.2 | 0.0 | -0.1 | -0.1 | -0.2 |
| Class E | -0.1 | -0.1 | 0.2 | -0.1 | -0.4 | -0.3 |
| All | -0.1 | -0.2 | 0.1 | -0.1 | -0.1 | -0.2 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 103% |  |  | 98% |  |  |

Figure 2 - Per sequence coding efficiency of proposed constrained intra prediction scheme compared to H.264-style constrained intra prediction scheme

# Conclusions

This contribution provides investigation results for an improved constrained intra prediction scheme adapted to the flexible-sized prediction units in HEVC video coding scheme. Experimental results show that the proposed scheme provides BD-rate gain of up to 0.7% (luma component) and 1.2% (chroma component) as compared to H.264-style constrained intra prediction without increasing the encoding and decoding time. It can be concluded therefore that the coding efficiency loss introduced by H.264-style constrained intra prediction scheme can be further reduced by considering characteristics of the HEVC video coding scheme. We would firstly recommend JCT-VC to include the functionality of constrained intra prediction into HEVC. We would also recommend further investigation on the design of HEVC-adapted constrained intra prediction schemes such as the current proposed scheme.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- H. Aoki, K. Chono, X. Jing, V. Wahadaniah, C.S. Lim, S.M.T. Naing, "Constrained intra prediction for reducing visual artifacts caused by lossy decoder-side memory compression," JCT-VC document JCTVC-D086, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 4th meeting, Daegu, January 2011.

- T. Wiegand, G.J. Sullivan, G. Bjontegaard, A. Luthra, "Overview of the H.264/AVC video coding standard," IEEE Transactions on Circuits and Systems for Video Technology, Vol. 13 Issue 7, pp. 560 - 576, July 2003.

- T. Wiegand, W.J. Han, B. Bross, J.R. Ohm, G.J. Sullivan, "WD1: Working Draft 1 of High-Efficiency Video Coding," JCT-VC document JCTVC-C403, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd meeting, Guangzhou, October 2010.

- F. Bossen, "Common test conditions and software reference configurations," JCT-VC document JCTVC-C500, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 3rd meeting, Guangzhou, October 2010.