| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D095 |
| --- | --- |

| Title: | Improvements on median motion vectors of AMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joonyoung Park Seungwook Park Byeongmoon Jeon #221 Yangjae-dong, Seocho-gu, Seoul 137-130, Korea Wei-Jung Chien Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +82-10-6272-9179 jy.park@lge.com wchien@qualcomm.com |
| Source: | LG electronics Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution presents an improved method on median motion vectors (MVs) of Advanced Motion Vector Prediction (AMVP). In AMVP, MVs with only a same reference index and a same reference list are regarded as spatial neighboring MV. But in the proposed method, MVs with a different reference index or a different reference list are also considered. The first available MV is chosen with pre-determined order, and it is scaled and inverted according to temporal distances and prediction directions. And then, it is used to calculate median MV predictors. The simulation results show that the proposed method achieves 0.4% and 0.5% bit rate reduction under random access high efficiency (RAHE) and random access low complexity (RALC) configurations on average for class A, B, C, D sequences. For RAHE and RALC, the encoding times are 100% and 98%, and the decoding times are 100% and 103%, respectively.

# Introduction

For the current HEVC Test Model (HM), AMVP is adopted as a method of motion vector prediction [1]. In AMVP, The motion vector predictor list, mvpListLX, is constructed of elements which are followed as {mvLXMed, mvLXA, mvLXB, mvLXC, mvLXCol}. mvLXMed is calculated from the motion vectors mvLXN and the availability flags availableFlagLXN with N being replaced by A, B, or C.

Figure 1. spatial motion vector neighbours

The motion vector mvLXN and the availability flag availableFlagLXN are derived in the following steps (with N being replaced by A, B, or C).

- Initially, availableFlagLXN and the both components of mvLXN are all set to be 0.

- For Nk from N0 to NnN-1, the following applies repeatedly until availableFlagLXN is equal to 1.

  - If predFlagLX[Nk] is equal to 1 and refIdxLX[Nk] is equal to refIdxLX, which is the reference index of the current prediction unit,

    - availableFlagLXN is set equal to 1 and mvLXN is set equal to mvLX[Nk].

From the above derivation process, availableFlagLXN is derived to be 0 in many cases. And it may give a bad effect on the accuracy of mvLXMed. It usually occurs in conventional video codec. To overcome this, the scaled motion vector prediction was currently proposed from several sources [2, 3, and 4]. And the method was also included as a technique for the further investigation in HM [1].

In this contribution, we propose an improved method for median MVs on AMVP, based on current HM and the scaled motion vector prediction method.

# Proposed algorithm

We propose some additional process for obtaining three spatial motion vectors mvLXN (with N being replaced by A, B, or C), which are used in the calculation of a median vector. In the proposed method, it is scanned and checked if there is a neighboring MV which satisfies each following condition in order.

- same reference picture and same reference list (with those of current prediction unit)

- same reference picture and different reference list

- different reference picture and same reference list

- different reference picture and different reference list

The first available MV is chosen in the above process. And then, the chosen MV is scaled by a factor of (tb / td), and inverted if its prediction direction is different from that of current MV, as figure 2 shows.

Figure 2. The example for MV scaling

The condition 1 represents original AMVP method. And the condition 2, 3, and 4 are added parts in this proposal.

The three spatial motion vectors, obtained by the above proposed method, are used only to calculate a median vector. Other MVP candidates are derived as AMVP does.

# Experimental results

The proposed method is implemented on HM 0.9 software and simulated under the common test conditions. And the performance is compared in terms of Bjøntegaard Delta (BD) Bit Rate [5]. Table 1 summarizes the experimental results.

Table 1. Experimental results on random access configuration

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -0.2 | -0.2 | -0.1 | -0.1 | -0.1 |
| Class B | -0.4 | -0.2 | -0.1 | -0.5 | -0.4 | -0.5 |
| Class C | -0.4 | -0.4 | -0.4 | -0.6 | -0.3 | -0.5 |
| Class D | -0.5 | -0.2 | -0.2 | -0.5 | -0.4 | -0.2 |
| Class E |  |  |  |  |  |  |
| All | -0.4 | -0.3 | -0.2 | -0.5 | -0.4 | -0.4 |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 103% |  |  |

Table 2. Experimental results on low delay configuration

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.2 | 0.1 | -0.1 | 0.0 | 0.0 | 0.2 |
| Class C | 0.0 | 0.3 | -0.1 | -0.1 | 0.1 | 0.2 |
| Class D | 0.0 | -0.4 | 0.1 | -0.2 | 0.2 | 0.2 |
| Class E | 0.1 | 0.2 | -0.3 | 0.2 | 0.1 | 0.3 |
| All | 0.1 | 0.0 | -0.1 | 0.0 | 0.1 | 0.2 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 103% |  |  | 101% |  |  |

The proposed method achieves 0.4% and 0.5% BD bit rate reduction under RAHE and RALC configurations. The encoding times are 100% and 98%, and the decoding times are 100% and 103%, respectively. On the other hand, for the LDHE and LDLC configurations, there were no BD bit rate reductions. The encoding times are 99% and 100%, and the decoding times are 103% and 101%, respectively. The detailed results for each sequence and QP are attached in the excel sheet.

# Conclusion

In this contribution, an improved method for obtaining median motion vectors on AMVP was presented. It produces 0.4% and 0.5% BD bit rate reduction under RAHE and RALC configurations with slight complexity changes. We propose to adopt our method into HM.

# References

[1] TK Tan, Gary J. Sullivan and Jens-Rainer Ohm, “Summary of HEVC working draft 1 and HEVC test model (HM)”, JCTVC-C405, Guangzhou, China, October 2010.

[2] ByeongMoon Jeon, SeungWook Park, JungSun Kim and JoonYoung Park, “Description of video coding technology proposal by LG Electronics”, JCTVC-A110, Dresden, Germany, April 2010

[3] Marta Karczewicz, Peisong Chen, Rajan Joshi, Xianglin Wang, Wei-Jung Chien and Rahul Panchal , “Video coding technology proposal by Qualcomm Inc.”, JCTVC-A121, Dresden, Germany, April 2010

[4] Yu-Wen Huang, Chih-Ming Fu, Yu-Pao Tsai, Jian-Liang Lin, Yulin Chang, Jyh-Huei Guo, Ching-Yeh Chen and Shawmin Lei, “A Technical Description of MediaTek’s Proposal to the JCT-VC CfP”, JCTVC-A109, Dresden, Germany, April 2010

[5] G. Bjøntegaard, “Calculation of average PSNR differences between RD-Curves,” ITU-T Q.6/SG16 Doc., VCEG-M33, Austin, Apr. 2001

# Patent rights declaration(s)

(NOTE – Activities in the JCT-VC and contributions to the JCT-VC are subject to the common patent policy for ITU-T/ITU-R/ISO/IEC. A statement of that policy can be found at
http://www.itu.int/ITU-T/dbase/patent/patent-policy.html, with further information available at http://www.itu.int/ITU-T/ipr/index.html and in the ISO/IEC Directives. The form to be used for the formal reporting of patent rights to ITU-T/ITU-R/ISO/IEC can be found at http://www.itu.int/ITU-T/ipr/index.html. Contributions to the JCT-VC proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The provided informal notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form. This obligation to provide an informal notice is supplemental to, and does not replace, any existing obligations of parties with technology included in a final or draft standard to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC. An example of an informal IPR notification statement for a contribution is provided below.)

LG may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).