| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D097 |
| --- | --- |

| Title: | Report of CE9 on Motion Vector Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Shun-ichi Sekiguchi Kazuo Sugimoto Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Sekiguchi.Shunichi@eb.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution is to report a performance verification result on CE9 approved at the last Guangzhou meeting.

# Introduction

A performance verification on CE9 approved at the last Guangzhou meeting was conducted. The verified method is a combination of skip/direct mode with a modified AMVP and CU merging, which is experiment 3.1.h defined in CE9 document [1]. The evaluation was performed using a modified TMuC 0.9 software that has been distributed to CE9 participants. In average, the results show reduction of encoding time up to 10%, while performance loss up to 1.3% BD-rate increase is observed.

# Simulation results

Table 1 shows test conditions used in our experiment, which is to test configuration 3.1.h defined in the CE9 document [1]. We verified that the CE9 software generates bitstreams consistent with those can be obtained by using original TMuC 0.9 software with HM-1 default configuration. Given this, we adopted original TMuC 0.9 encoding results as our anchor data, especially for comparison of encoding/decoding time. Encoding and decoding time for both anchor and the test case were measured using “time” command in Linux platform (Fedora x86_64).

Table 1  Test Condition

| Software | CE9 software (Modified TMuC0.9 software that has been distributed to CE9 participants) |
| --- | --- |
| Tested configuration | Skip: Modified AMVP 1 (disable left, above, median candidates) Direct: Modified AMVP 1 (disable left, above, median candidates) Merge: CU based (Need to check mrg_cbf flag’s impact) |
| QP setting | QPI=22,37,32,37 |
| Constraint Sets | CS2(HE-LD), CS3(LC-LD), CS4(HE-RA), CS5(LC-RA) |

BD rate reduction and ratio of encoding/decoding time with the evaluated configuration are listed in Table 2 and 3, respectively. From these results, it is observed that the tested configuration generally decreases coding performance of HM-1 (maximum loss is 1.7% in class C of LC-LD condition) and encoding time can be reduced ranging from 5 to 10%. The performance loss is more significant in LC conditions than in HE conditions, and in Low Delay conditions than in Random Access conditions. There is no impact to decoder-side complexity.

Table 2 Performance for Random Access Conditions

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.8 | 1.1 | 0.5 | 1.1 | 1.2 |
| Class B | 0.1 | 0.7 | 0.9 | 0.4 | 0.8 | 0.7 |
| Class C | 0.1 | 0.5 | 0.5 | 1.0 | 1.4 | 1.3 |
| Class D | 0.0 | 0.6 | 0.6 | 1.0 | 1.3 | 1.5 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.6 | 0.7 | 0.7 | 1.1 | 1.2 |
| Enc Time[%] | 90% |  |  | 93% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |

Table 3 Performance for Low Delay Conditions

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.7 | 1.1 | 1.0 | 1.3 | 1.6 | 1.4 |
| Class C | 0.3 | 0.6 | 0.5 | 1.7 | 1.9 | 1.9 |
| Class D | 0.3 | 0.1 | 0.4 | 1.5 | 1.9 | 2.1 |
| Class E | 0.0 | -1.8 | -1.4 | 0.6 | 1.8 | 1.1 |
| All | 0.4 | 0.2 | 0.3 | 1.3 | 1.8 | 1.6 |
| Enc Time[%] | 93% |  |  | 95% |  |  |
| Dec Time[%] | 97% |  |  | 100% |  |  |

# Conclusion

We tested an experimental configuration approved in CE9, which decreases the number of MVP candidates for AMVP process. Reported simulation results suggest that original set of MVP candidates in HM-1 is contributing to coding performance especially for LC conditions, but requires some sort of additional encoding complexity.

References:

- J. Jung and B. Bross, “Core Experiment 9: Motion Vector Coding,” JCTVC-C509, Oct. 2010.

- Frank Bossen, “Common test conditions and software reference configurations,” JCTVC- C500, Oct. 2010.