| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D099 |
| --- | --- |

| Title: | CE1: Report of implicit direct vector derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information and Proposal |  |  |
| Author(s) or Contact(s): | Yusuke Itani Shun-ichi Sekiguchi Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Itani.Yusuke@dx.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution is to report results of performance verification on implicit direct vector derivation technique that was approved as a part of CE1[1] at the last Guangzhou meeting. A performance evaluation using TMuC 0.9 was conducted according to the test condition defined in CE1 and the results show 0.5% coding gain in average and up to 1.3% at maximum gain for random access condition. The increase of decoding time was about 25% relative to TMuC 0.9 with the default configuration and encoding time was almost the same as that of TMuC 0.9.

# Introduction

In this contribution, we report simulation results on our implicit adaptive direct vector derivation scheme [2], which was approved in Guangzhou as an evaluation item in CE1. The evaluated technique provides a new efficient design of B-skip and B-direct modes with locally adaptive direct MV derivation relying on a simple decoder-side decision. It has been observed that the proposed technique can achieve BD rate reduction around 0.5% in average and up to 1.3% for random access condition relative to TMuC 0.9 with the default configuration. The increase of decoding time was about 25% and encoding time was almost the same as that of TMuC 0.9.

# Simulation results

B-skip and B-direct modes using the proposed technique has been implemented into TMuC version0.9 software that was made available to the group. Table 1 shows test conditions used for our experiment in the CE1 document. The anchor streams have been generated with TMuC 0.9 default configurations[3].

Table 1  Test Condition

| Codec software | Implementation of the tool proposed in JCTVC-C124[2] based on TMuC0.9 software |
| --- | --- |
| Test configuration | JCTVC-C500[3] |
| QP setting | QPI=22,37,32,37 |
| Constraint Sets | CS2, CS3, CS4, CS5 |

The BD rate reductions in each condition obtained by the proposed method are listed in Table 2 to 5 respectively. It is observed that the proposed direct mode can achieve consistent coding gain especially for the sequences where spatially detailed texture and stable background scene co-exist, such as PeopleOnStreet or Cactus. In such cases, spatially adjacent motion vector may not work as sufficient estimator for direct vector, but temporal co-located candidate could contribute to improvement of direct vector accuracy.

Table 2 Performance in BD measurements (low delay)

|  | BD-rate[%] |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Kimono | 0.7 | 0.1 | 0.0 |
| ParkScene | 0.1 | 0.4 | -0.4 |
| Cactus | 0.8 | 0.4 | -0.7 |
| BasketballDrive | 1.3 | 1.2 | 1.7 |
| BQTerrace | 0.0 | 0.3 | -2.5 |
| BasketballDrill | 1.1 | 1.1 | 1.9 |
| BQMall | 0.9 | 1.2 | 0.4 |
| PartyScene | 0.4 | 0.4 | 0.2 |
| RaceHorses | 0.2 | 0.4 | 0.6 |
| BasketballPass | 1.1 | 0.7 | 0.9 |
| BQSquare | -0.1 | -1.4 | -0.4 |
| BlowingBubbles | 0.5 | -0.1 | -0.2 |
| RaceHorses | 0.3 | 0.3 | 0.6 |
| Vidyo1 | -0.5 | -3.8 | -5.1 |
| Vidyo3 | 0.5 | -4.8 | -4.0 |
| Vidyo4 | 0.2 | -2.8 | -3.8 |
| Average Class B | 0.6 | 0.5 | -0.4 |
| Average Class C | 0.7 | 0.8 | 0.8 |
| Average Class D | 0.5 | -0.1 | 0.2 |
| Average Class E | 0.1 | -3.8 | -4.3 |
| Average All | 0.5 | -0.4 | -0.7 |

Table 3 Performance in BD measurements (low delay Loco)

|  | BD-rate[%] |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Kimono | 1.2 | 1.2 | 1.4 |
| ParkScene | -0.1 | -0.9 | -0.5 |
| Cactus | 0.6 | 0.6 | -0.2 |
| BasketballDrive | 1.8 | 1.1 | 1.4 |
| BQTerrace | -0.2 | -2.2 | -3.6 |
| BasketballDrill | 0.6 | 1.1 | 1.2 |
| BQMall | 1.4 | -0.3 | 0.5 |
| PartyScene | 1.3 | 0.2 | -0.2 |
| RaceHorses | 0.9 | 0.8 | 0.8 |
| BasketballPass | 1.4 | 0.8 | 0.9 |
| BQSquare | 0.7 | -1.4 | -0.7 |
| BlowingBubbles | 1.2 | -0.3 | -1.0 |
| RaceHorses | 0.9 | 0.4 | 0.7 |
| Vidyo1 | -1.2 | -2.0 | -0.8 |
| Vidyo3 | -0.6 | -0.5 | -0.7 |
| Vidyo4 | -0.5 | 0.2 | 0.5 |
| Average Class B | 0.7 | 0.0 | -0.3 |
| Average Class C | 1.0 | 0.5 | 0.6 |
| Average Class D | 1.0 | -0.1 | 0.0 |
| Average Class E | -0.8 | -0.8 | -0.3 |
| Average All | 0.6 | -0.1 | 0.0 |

Table 4 Performance in BD measurements (random access)

|  | BD-rate[%] |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Traffic | -1.1 | -0.7 | -0.7 |
| PeopleOnStreet | -0.7 | -0.5 | 0.0 |
| Kimono | -0.2 | -0.1 | -0.3 |
| ParkScene | -0.5 | -0.3 | -0.3 |
| Cactus | -1.3 | -0.9 | -1.1 |
| BasketballDrive | 0.2 | 0.3 | 0.6 |
| BQTerrace | -0.3 | -0.3 | -0.3 |
| BasketballDrill | -0.8 | -0.9 | -0.8 |
| BQMall | -0.7 | -0.3 | -0.6 |
| PartyScene | -1.0 | -0.3 | -0.6 |
| RaceHorses | 0.2 | 0.7 | 0.7 |
| BasketballPass | -0.3 | 0.0 | 0.1 |
| BQSquare | -1.0 | -1.0 | -0.5 |
| BlowingBubbles | -1.2 | -0.7 | -0.5 |
| RaceHorses | -0.1 | 0.0 | 0.6 |
| Average Class A | -0.9 | -0.6 | -0.4 |
| Average Class B | -0.4 | -0.2 | -0.3 |
| Average Class C | -0.3 | 0.1 | 0.0 |
| Average Class D | -0.6 | -0.4 | -0.1 |
| Average All | -0.5 | -0.2 | -0.1 |

Table 5 Performance in BD measurements (random access LoCo)

|  | BD-rate[%] |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Traffic | -1.0 | -1.0 | -0.9 |
| PeopleOnStreet | -0.6 | -0.4 | -0.3 |
| Kimono | -0.6 | -0.9 | -0.9 |
| ParkScene | -0.5 | -0.7 | -0.7 |
| Cactus | -1.4 | -0.9 | -1.4 |
| BasketballDrive | 0.6 | 0.3 | 0.4 |
| BQTerrace | -0.3 | -0.7 | -0.7 |
| BasketballDrill | 0.3 | 0.4 | 0.3 |
| BQMall | -0.3 | 0.0 | 0.0 |
| PartyScene | 0.1 | 0.3 | 0.2 |
| RaceHorses | 0.6 | 0.8 | 0.8 |
| BasketballPass | -0.1 | -0.2 | 0.2 |
| BQSquare | 1.5 | 0.9 | 1.7 |
| BlowingBubbles | -0.3 | -0.2 | -0.1 |
| RaceHorses | 0.3 | 0.4 | 0.6 |
| Average Class A | -0.8 | -0.7 | -0.6 |
| Average Class B | -0.4 | -0.6 | -0.7 |
| Average Class C | 0.2 | 0.4 | 0.3 |
| Average Class D | 0.3 | 0.2 | 0.6 |
| Average All | -0.1 | -0.1 | -0.1 |

Table 6 to 9 show encoding and decoding time those are measured using “time” command in Linux platform. In these tables, decoding time was measured with YUV outputs. They were also compared with that of the anchor streams using the same platform (Fedora x86_64). In average, the increase of decoding time was about 25% relative to the anchor, and encoding time was almost the same as that of the anchor.

The derivation process of direct vector with the proposed technique requires doubled memory access to reference pictures to obtain all prediction block candidates and computation of two SAD calculations per PU at decoder-side, relative to HM-1. Our proposed technique requires derivation of the temporal co-located vector and the spatial neighboring vector, which does not have any additional impact to memory size required for motion vector storage if direct vector derivation process relies on temporal motion vector reference.

Table 6 Encoding and Decoding time (low delay)

| sequence | encoder | decoder |
| --- | --- | --- |
| Kimono | 101.91 | 107.71 |
| ParkScene | 102.18 | 107.01 |
| Cactus | 105.78 | 115.34 |
| BasketballDrive | 102.50 | 103.91 |
| BQTerrace | 101.34 | 104.13 |
| BasketballDrill | 100.42 | 104.22 |
| BQMall | 100.44 | 109.25 |
| PartyScene | 101.26 | 105.06 |
| RaceHorses | 101.81 | 101.42 |
| BasketballPass | 100.46 | 108.89 |
| BQSquare | 101.67 | 116.11 |
| BlowingBubbles | 100.38 | 110.99 |
| RaceHorses | 101.46 | 104.44 |
| Vidyo1 | 99.83 | 111.99 |
| Vidyo3 | 100.25 | 107.57 |
| Vidyo4 | 100.55 | 110.29 |
| Average Class B | 102.74 | 107.62 |
| Average Class C | 100.98 | 104.99 |
| Average Class D | 104.14 | 109.63 |
| Average Class E | 101.92 | 104.02 |
| Average All | 101.39 | 108.02 |

Table 7 Encoding and Decoding time (low delay Loco)

| sequence | encoder | decoder |
| --- | --- | --- |
| Kimono | 100.83 | 113.69 |
| ParkScene | 101.08 | 125.53 |
| Cactus | 100.48 | 119.46 |
| BasketballDrive | 101.50 | 112.67 |
| BQTerrace | 101.07 | 122.94 |
| BasketballDrill | 99.78 | 110.58 |
| BQMall | 100.96 | 119.18 |
| PartyScene | 101.66 | 113.57 |
| RaceHorses | 101.23 | 108.15 |
| BasketballPass | 100.67 | 111.14 |
| BQSquare | 101.56 | 117.76 |
| BlowingBubbles | 100.67 | 115.28 |
| RaceHorses | 101.37 | 111.83 |
| Vidyo1 | 99.67 | 130.68 |
| Vidyo3 | 99.01 | 124.76 |
| Vidyo4 | 99.95 | 130.96 |
| Average Class B | 100.99 | 118.86 |
| Average Class C | 100.91 | 112.87 |
| Average Class D | 100.99 | 116.07 |
| Average Class E | 101.29 | 117.81 |
| Average All | 100.72 | 118.01 |

Table8 Encoding and Decoding time (random access)

| sequence | encoder | decoder |
| --- | --- | --- |
| Traffic | 103.98 | 117.94 |
| PeopleOnStreet | 102.45 | 114.29 |
| Kimono | 106.19 | 128.23 |
| ParkScene | 105.49 | 125.47 |
| Cactus | 102.22 | 123.64 |
| BasketballDrive | 104.93 | 121.90 |
| BQTerrace | 105.16 | 131.08 |
| BasketballDrill | 101.42 | 111.72 |
| BQMall | 103.73 | 131.35 |
| PartyScene | 102.47 | 135.10 |
| RaceHorses | 103.33 | 110.45 |
| BasketballPass | 102.76 | 122.04 |
| BQSquare | 103.41 | 149.31 |
| BlowingBubbles | 104.12 | 147.31 |
| RaceHorses | 103.66 | 117.10 |
| Average Class A | 103.22 | 116.11 |
| Average Class B | 104.80 | 126.06 |
| Average Class C | 102.74 | 122.15 |
| Average Class D | 103.49 | 133.94 |
| Average All | 103.69 | 125.79 |

Table9 Encoding and Decoding time (random access Loco)

| sequence | encoder | decoder |
| --- | --- | --- |
| Traffic | 105.28 | 129.26 |
| PeopleOnStreet | 101.25 | 127.41 |
| Kimono | 104.12 | 135.27 |
| ParkScene | 104.26 | 139.41 |
| Cactus | 102.07 | 128.11 |
| BasketballDrive | 103.73 | 133.90 |
| BQTerrace | 104.07 | 141.65 |
| BasketballDrill | 100.85 | 115.34 |
| BQMall | 103.64 | 136.67 |
| PartyScene | 103.42 | 130.37 |
| RaceHorses | 103.17 | 115.99 |
| BasketballPass | 102.73 | 122.44 |
| BQSquare | 104.92 | 133.02 |
| BlowingBubbles | 104.29 | 136.91 |
| RaceHorses | 103.66 | 122.79 |
| Average Class A | 103.27 | 128.33 |
| Average Class B | 103.65 | 135.67 |
| Average Class C | 102.77 | 124.59 |
| Average Class D | 103.90 | 128.79 |
| Average All | 103.43 | 129.90 |

# Conclusion

The implicit adaptive direct vector derivation technique was evaluated under the common condition specified in CE1. Our experimental results show coding gain to various high-resolution sequences under random access configurations. We propose to continue further study on the impact of DMVD features including the proposed approach based on HM.

References:

- Yi-Jen Chiu, et.al. “Core Experiment 1: Decoder-Side Motion Vector Derivation,” JCTVC-C501, Oct. 2010.

- Y. Itani, et.al, “TE1 report: implicit direct vector derivation,” JCTVC-C124, Oct. 2010.

- Frank Bossen, “Common test conditions and software reference configurations,” JCTVC- C500, Oct. 2010.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).