| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D100 |
| --- | --- |

| Title: | CE10: Summary of CE10 on number of intra prediction directions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Core Experiment report |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto (Mitsubishi Electric Corporation) | Tel: Email: | +81-467-41-2463 Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Source: | CE coordinator |  |  |

_____________________________

# Abstract

This contribution is a summary of core experiment 10 (CE10) on number of intra prediction directions. There are 12 test cases in CE10 as described in JCTVC-C310. 9 organizations conducted the cross-check, and those test cases are evaluated based on the common conditions in JCTVC-C500. Detailed results will be reported by proponents.

# Introduction

An increased number of directions for intra prediction was introduced in the TMuC and will be integrated into the HEVC Test Model[1]. The purpose of this CE is to characterize the performance and complexity of using different numbers of intra prediction directions. The characterization was performed by investigating the coding efficiency improvement in terms of BD-BR [2] performance, and by determining the complexity increase for both encoding and decoding using variety of combinations of intra prediction directions for each block size.

# Document list

- Summary document (this document)

  - JCTVC-D100 [CE coordinator] CE10: Summary of CE10 on number of intra prediction directions

- verification/study documents

  - JCTVC-D050 [I2R] CE10: Cross-check report from Institute for Infocomm Research

  - JCTVC-D063 [INRIA] CE10: Cross-check report from INRIA on number of intra prediction directions

  - JCTVC-D101 [Mitsubishi] CE10: Cross-check report on number of intra prediction directions from Mitsubishi

    - An additional test on cropped version of SHV sequences is also reported

    - An additional test on a recommended setting is also reported

  - JCTVC-D102 [Toshiba] CE10: Cross check report for number of intra prediction directions from Toshiba

    - An additional test on a recommended setting is also reported

  - JCTVC-D198 [Nokia] CE10: Cross verification of directional intra prediction configurations

  - JCTVC-D207 [NEC] CE10: Cross-check report on the number of intra prediction directions

    - An additional test on cropped version of SHV sequences is also reported

  - JCTVC-D291 [MediaTek] CE10 : Crosscheck report for number of intra by MediaTek

  - JCTVC-D358 [Samsung] CE10: Samsung’s crosscheck on number of intra prediction directions

# Test points and Cross-checkers

The test points are identified in the table below with respect to the number of intra prediction directions up to the one defined for the HEVC Test Model default configurations[3]. The test has been conducted using TMuC0.9 with modifications to realize the limitation of number of intra prediction directions.

Table 1. Performance Tests for different combinations of number of intra prediction directions

| Block size | Number of Intra Prediction Directions |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Reference | Test |  |  |  |  |  |  |  |  |  |  |
|  |  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 |
| 4x4 | 17 | 17 | 9 | 9 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 9 |
| 8x8 | 34 | 17 | 9 | 34 | 17 | 9 | 34 | 34 | 34 | 34 | 34 | 17 |
| 16x16 | 34 | 17 | 9 | 34 | 34 | 34 | 17 | 9 | 34 | 34 | 34 | 17 |
| 32x32 | 34 | 17 | 9 | 34 | 34 | 34 | 34 | 34 | 17 | 9 | 34 | 17 |
| 64x64 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 3 | 5 |

Test #12: To evaluate the upper limit of the performance of the intra prediction method of the test model,

the reference without fast mode decisions, which means that all intra prediction directions are evaluated using RD cost, is also tested.

Table2. Default configurations in TComRom.cpp file

| Default configurations | #if UNIFIED_DIRECTIONAL_INTRA const UChar g_aucIntraModeNumAng[7] = { 3, // 2x2 17, // 4x4 34, // 8x8 34, // 16x16 34, // 32x32 5, // 64x64 5 // 128x128 }; const UChar g_aucIntraModeBitsAng[7] = { 2, // 2x2 5, // 4x4 6, // 8x8 6, // 16x16 6, // 32x32 3, // 64x64 3 // 128x128 }; |
| --- | --- |

Table3. value for g_aucIntraModeBitsAng

| Number of Directions | 3 | 5 | 9 | 17 | 34 |
| --- | --- | --- | --- | --- | --- |
| Number of bits | 2 | 3 | 4 | 5 | 6 |

Cross-checkers for each test are shown in Table4.

Table4. Cross-checkers

| Cross-checker | Test |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 | #12 |
| INRIA | x | x | x | x | x | x | x | x | x | x | x | x |
| I2R | x | x | x | x | x | x | x | x | x | x | x | x |
| NEC | x | x | x | x | x | x | x | x | x | x | x | x |
| MediaTek | x | x | x | x | x | x | x | x | x | x | x | x |
| Mitsubishi | x | x | x | x | x | x | x | x | x | x | x | x |
| Nokia | x | x | x | x | x | x | x | x | x | x | x | x |
| Samsung | x | x | x | x | x | x | x | x | x | x | x | x |
| Santa Clara University | x | x | x | x | x | x | x | x | x | x | x | x |
| Toshiba | x | x | x | x | x | x | x | x | x | x | x | x |

# Experimental Conditions

## Software

TMuC0.9 has been used with modification for the CE tests. The modifications for test cases #1 to #9 and #11 are only modifications of the tables shown in Table 3 and 4 based on Table 1.

For the test case #10, as maximum index for 64x64 intra prediction is 3, mode mapping table for most probable mode calculation has been modified to limit the mapped mode less than 3.

For the test case #12, the best intra prediction mode is selected by evaluating R-D cost for all modes.

The patch for these modifications is attached to this contribution.

## Test Sequences, Bit Rates and Coding Conditions

Test conditions are the intra-only configuration on both High Efficiency and Low Complexity conditions and test sequences as defined in C500[3], using the reference configuration files also provided in [3].

# Test Results

All test results reported from participants are identical in terms of BD-rate except Windows/Linux mismatches. Encoding and decoding time differs slightly, and the summary of the results including averaged encoding and decoding time are shown in Table 5. Detailed results for each test cases with averaged time are shown in Annex A.

Table 5. Test results on the common test sequences

|  | Block size and number of directions |  |  |  |  | HE |  | LC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4x4 | 8x8 | 16x16 | 32x32 | 64x64 | BDrate | Enc Time | Bdrate | Enc Time |
| Reference | 17 | 34 | 34 | 34 | 5 |  |  |  |  |
| Test#1 | 17 | 17 | 17 | 17 | 5 | 1.803 | 95.11% | 1.903 | 90.12% |
| Test#2 | 9 | 9 | 9 | 9 | 5 | 5.123 | 88.94% | 6.320 | 79.91% |
| Test#3 | 9 | 34 | 34 | 34 | 5 | 0.626 | 96.08% | 1.135 | 96.40% |
| Test#4 | 17 | 17 | 34 | 34 | 5 | 0.553 | 97.38% | 1.174 | 96.24% |
| Test#5 | 17 | 9 | 34 | 34 | 5 | 1.157 | 95.02% | 2.217 | 91.07% |
| Test#6 | 17 | 34 | 17 | 34 | 5 | 0.435 | 97.56% | 0.341 | 96.79% |
| Test#7 | 17 | 34 | 9 | 34 | 5 | 0.673 | 97.60% | 0.489 | 95.31% |
| Test#8 | 17 | 34 | 34 | 17 | 5 | 0.110 | 98.57% | 0.005 | 97.21% |
| Test#9 | 17 | 34 | 34 | 9 | 5 | 0.129 | 98.05% | -0.027 | 96.54% |
| Test#10 | 17 | 34 | 34 | 34 | 3 | -0.004 | 97.53% | 0.000 | 99.07% |
| Test#11 | 9 | 17 | 17 | 17 | 5 | 2.581 | 92.05% | 3.433 | 86.59% |
| Test#12 | 17 | 34 | 34 | 34 | 5 | -0.206 | 207.63% | -0.708 | 212.79% |

In addition to the common test sequences, some participants conducted the test on SHV sequences cropped to Class A size provided by NHK[4]. The results are shown in Table 6.

Table 6. Test results on cropped SHV sequences

|  | Block size and number of directions |  |  |  |  | HE |  |  | LC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4x4 | 8x8 | 16x16 | 32x32 | 64x64 | BDrate | Enc Time |  | Bdrate | Enc Time |
| Reference | 17 | 34 | 34 | 34 | 5 |  |  |  |  |  |
| Test#1 | 17 | 17 | 17 | 17 | 5 | 0.639 | 95.19% | 0.516 |  | 89.75% |
| Test#2 | 9 | 9 | 9 | 9 | 5 | 1.672 | 89.09% | 2.106 |  | 78.98% |
| Test#3 | 9 | 34 | 34 | 34 | 5 | 0.044 | 97.15% | 0.083 |  | 96.28% |
| Test#4 | 17 | 17 | 34 | 34 | 5 | 0.161 | 97.74% | 0.489 |  | 95.66% |
| Test#5 | 17 | 9 | 34 | 34 | 5 | 0.353 | 95.73% | 1.199 |  | 91.35% |
| Test#6 | 17 | 34 | 17 | 34 | 5 | 0.213 | 98.68% | 0.001 |  | 96.85% |
| Test#7 | 17 | 34 | 9 | 34 | 5 | 0.438 | 97.87% | 0.289 |  | 95.29% |
| Test#8 | 17 | 34 | 34 | 17 | 5 | 0.083 | 98.93% | -0.053 |  | 97.47% |
| Test#9 | 17 | 34 | 34 | 9 | 5 | 0.119 | 98.32% | -0.124 |  | 96.11% |
| Test#10 | 17 | 34 | 34 | 34 | 3 | -0.006 | 98.41% | -0.005 |  | 99.58% |
| Test#11 | 9 | 17 | 17 | 17 | 5 | 0.701 | 92.42% | 0.632 |  | 86.26% |
| Test#12 | 17 | 34 | 34 | 34 | 5 | -0.222 | 194.93% | -1.169 |  | 216.31% |

# Analysis on the results

According to the test results, these observations are recognized:

1) Reducing the number of directions almost never resulted in coding performance improvement.

2) Increasing the number of directions is more beneficial for smaller PU sizes.

3) The fast decision algorithm in the reference resulted in only a 50% reduction in encoding time over Test 12 despite performing R-D checks on significantly less than 50% of all possible directions. This indicates that there are other tools contributing significantly to the complexity of the encoder.

4) Comparing the reference with test #12, it appears that the current intra prediction mode decision algorithm achieves only 0.2 bit increase from R-D based mode decision with 50% encoding time savings.

## 4x4 block

Comparing the reference with test #3, it appears that limiting the number of intra prediction directions for 4x4 blocks to 9 from 17 causes 0.6 and 1.1 % bit increase for HE and LC on the common test sequences with around 4% savings of encoding time.

17 directions for 4x4 blocks would be necessary to avoid performance degradation.

## 8x8 block

Comparing the reference with test #4, it appears that limiting the number of intra prediction directions for 8x8 blocks to 17 from 34 causes 0.6 and 1.2 % bit increase for HE and LC on the common test sequences with around 3-4% savings of encoding time.

Comparing the reference with test #5, it appears that limiting the number of intra prediction directions for 8x8 blocks to 9 from 34 causes 1.2 and 2.2 % bit increase for HE and LC on the common test sequences with around 5% and 9% savings of encoding time.

34 directions for 8x8 blocks would be necessary to avoid performance degradation.

## 16x16 block

Comparing the reference with test #6, it appears that limiting the number of intra prediction directions for 16x16 blocks to 17 from 34 causes 0.4 and 0.3 % bit increase for HE and LC on the common test sequences with around 2-3% savings of encoding time.

Comparing the reference with test #7, it appears that limiting the number of intra prediction directions for 16x16 blocks to 9 from 34 causes 0.7 and 0.5 % bit increase for HE and LC on the common test sequences with around 3-4% savings of encoding time.

34 or 17 directions for 16x16 blocks would be necessary to avoid performance degradation.

## 32x32 block

Comparing the reference with test #8, it appears that limiting the number of intra prediction directions for 32x32 blocks to 17 from 34 causes 0.1 and 0.0 % bit increase for HE and LC on the common test sequences with around 2-3% savings of encoding time.

Comparing the reference with test #9, it appears that limiting the number of intra prediction directions for 32x32 blocks to 9 from 34 causes 0.1 and 0.0 % bit increase for HE and LC on the common test sequences with around 2-3% savings of encoding time.

More than 9 directions for 32x32 blocks would not be necessary to improve the performance.

## 64x64 block

Comparing the reference with test #10, limiting the number of intra prediction directions for 64x64 blocks to 3 from 5 causes even small bit savings for HE and LC on the common test sequences with around 1-2% savings of encoding time.

More than 3 directions for 64x64 blocks would not be necessary to improve the performance.

## Consideration on the results on SHV sequences

For SHV sequences, 9 modes for 4x4 blocks, 17 modes for 8x8 blocks, 17 modes for 16x16 blocks, 9 modes for 32x32 blocks and 3 modes for 64x64 blocks still gives the coding performance of the reference. More variations of test sequences would give more clear idea on the optimal configurations of number of intra prediction modes.

# Conclusion

In CE10, 12 test cases are evaluated based on the common test conditions. The results are summarized according to coding efficiency and complexity. It is suggested to consider these results to decide the number of intra prediction directions when creating the next version of HM.

# References

- T. Wiegand, “HEVC text specification Draft 1”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C403, 3rd Meeting: Guangzhou, CN, Oct., 2010.

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

- F. Bossen, “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C500, 3rd Meeting: Guangzhou, CN, Oct., 2010.

- K. Iguchi, “Performance report of TMuC for Super Hi-Vision”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C055, 3rd Meeting: Guangzhou, CN, Oct., 2010.

# Annex A: Detailed test results on the common test sequences

Table 7. Test Results

| TEST#1 | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 2.4 | 2.6 | 2.7 | 2.5 | 3.3 | 3.3 |
| Class B | 1.8 | 1.8 | 2.1 | 1.9 | 2.7 | 2.9 |
| Class C | 1.6 | 1.5 | 1.5 | 1.7 | 2.2 | 2.3 |
| Class D | 1.1 | 1.1 | 1.2 | 1.0 | 1.4 | 1.4 |
| Class E | 2.7 | 3.0 | 2.6 | 3.1 | 4.1 | 3.8 |
| All | 1.8 | 1.9 | 1.9 | 1.9 | 2.6 | 2.6 |
| Enc Time[%] | 95% |  |  | 90% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
| TEST#2 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 4.9 | 4.9 | 5.2 | 5.7 | 5.9 | 5.8 |
| Class B | 4.7 | 4.5 | 5.0 | 5.6 | 6.4 | 6.7 |
| Class C | 5.3 | 5.2 | 5.0 | 6.6 | 6.7 | 6.8 |
| Class D | 4.1 | 4.0 | 4.1 | 4.8 | 4.8 | 4.9 |
| Class E | 7.1 | 7.6 | 7.1 | 9.6 | 9.8 | 9.9 |
| All | 5.1 | 5.1 | 5.2 | 6.3 | 6.6 | 6.8 |
| Enc Time[%] | 89% |  |  | 80% |  |  |
| Dec Time[%] | 101% |  |  | 104% |  |  |
| TEST#3 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.4 | 0.2 | 0.2 | 0.8 | 0.1 | 0.1 |
| Class B | 0.3 | 0.2 | 0.1 | 0.7 | 0.1 | 0.1 |
| Class C | 0.8 | 0.4 | 0.4 | 1.4 | 0.4 | 0.4 |
| Class D | 1.1 | 0.6 | 0.6 | 1.7 | 0.6 | 0.6 |
| Class E | 0.5 | 0.2 | 0.2 | 1.0 | 0.2 | 0.3 |
| All | 0.6 | 0.3 | 0.3 | 1.1 | 0.3 | 0.3 |
| Enc Time[%] | 96% |  |  | 96% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
| TEST#4 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.7 | 0.7 | 0.6 | 1.2 | 0.6 | 0.4 |
| Class B | 0.4 | 0.3 | 0.4 | 1.1 | 0.8 | 0.8 |
| Class C | 0.6 | 0.7 | 0.7 | 1.2 | 1.3 | 1.2 |
| Class D | 0.5 | 0.5 | 0.6 | 0.8 | 0.9 | 0.9 |
| Class E | 0.6 | 0.6 | 0.5 | 1.7 | 0.6 | 0.7 |
| All | 0.6 | 0.5 | 0.5 | 1.2 | 0.9 | 0.9 |
| Enc Time[%] | 97% |  |  | 96% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
| TEST#5 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1.3 | 1.3 | 1.3 | 2.2 | 1.2 | 0.8 |
| Class B | 0.9 | 0.8 | 0.8 | 2.1 | 1.5 | 1.6 |
| Class C | 1.3 | 1.4 | 1.5 | 2.2 | 2.2 | 2.3 |
| Class D | 1.1 | 1.2 | 1.3 | 1.6 | 1.7 | 1.9 |
| Class E | 1.4 | 1.2 | 1.1 | 3.3 | 1.0 | 1.5 |
| All | 1.2 | 1.1 | 1.2 | 2.2 | 1.6 | 1.7 |
| Enc Time[%] | 95% |  |  | 91% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
| TEST#6 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.7 | 0.8 | 1.0 | 0.7 | 1.6 | 1.8 |
| Class B | 0.4 | 0.4 | 0.5 | 0.3 | 0.7 | 0.8 |
| Class C | 0.4 | 0.3 | 0.3 | 0.2 | 0.4 | 0.5 |
| Class D | 0.2 | 0.2 | 0.2 | 0.1 | 0.3 | 0.2 |
| Class E | 0.7 | 1.0 | 0.7 | 0.7 | 1.8 | 1.6 |
| All | 0.4 | 0.5 | 0.5 | 0.3 | 0.8 | 0.8 |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
| TEST#7 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.9 | 1.0 | 1.3 | 0.7 | 2.1 | 2.3 |
| Class B | 0.6 | 0.6 | 0.8 | 0.4 | 1.1 | 1.2 |
| Class C | 0.6 | 0.5 | 0.5 | 0.3 | 0.8 | 0.8 |
| Class D | 0.3 | 0.4 | 0.3 | 0.1 | 0.4 | 0.4 |
| Class E | 1.2 | 1.6 | 1.2 | 1.2 | 3.4 | 2.9 |
| All | 0.7 | 0.8 | 0.7 | 0.5 | 1.4 | 1.3 |
| Enc Time[%] | 98% |  |  | 95% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
| TEST#8 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.2 | 0.2 | 0.0 | 0.1 | 0.1 |
| Class B | 0.1 | 0.3 | 0.2 | 0.0 | 0.1 | 0.2 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.3 | 0.4 | 0.4 | 0.1 | 0.4 | 0.4 |
| All | 0.1 | 0.2 | 0.2 | 0.0 | 0.1 | 0.1 |
| Enc Time[%] | 99% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
| TEST#9 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.2 | 0.2 | 0.0 | 0.1 | 0.1 |
| Class B | 0.1 | 0.3 | 0.3 | -0.1 | 0.2 | 0.2 |
| Class C | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.4 | 0.5 | 0.5 | 0.0 | 0.5 | 0.4 |
| All | 0.1 | 0.2 | 0.2 | 0.0 | 0.1 | 0.2 |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
| TEST#10 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
| TEST#11 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 2.9 | 2.8 | 3.0 | 3.5 | 3.5 | 3.5 |
| Class B | 2.2 | 2.1 | 2.3 | 2.8 | 3.0 | 3.1 |
| Class C | 2.6 | 2.1 | 2.1 | 3.6 | 3.0 | 3.0 |
| Class D | 2.3 | 1.9 | 1.9 | 3.0 | 2.3 | 2.3 |
| Class E | 3.4 | 3.4 | 3.0 | 4.7 | 4.5 | 4.4 |
| All | 2.6 | 2.3 | 2.4 | 3.4 | 3.1 | 3.2 |
| Enc Time[%] | 92% |  |  | 87% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
| TEST#12 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.2 | 0.2 | 0.2 | -1.0 | -0.1 | -0.2 |
| Class B | -0.3 | 0.0 | 0.0 | -1.0 | -0.1 | -0.2 |
| Class C | -0.1 | 0.3 | 0.2 | -0.4 | 0.0 | 0.0 |
| Class D | -0.1 | 0.3 | 0.3 | -0.4 | 0.0 | 0.0 |
| Class E | -0.4 | 0.1 | 0.0 | -0.9 | -0.5 | -0.4 |
| All | -0.2 | 0.2 | 0.1 | -0.7 | -0.1 | -0.1 |
| Enc Time[%] | 208% |  |  | 213% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |