| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D101 |
| --- | --- |

| Title: | CE10: Cross-check report on number of intra prediction directions from Mitsubishi |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto Shun-ichi Sekiguchi 5-1-1 Ofuna, Kamakura-shi Kanagawa, 247-8501 Japan | Tel: Email: | +81-467-41-2463 Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This contribution reports the simulation results and cross verification of Core Experiment 10 as described in JCTVC-C510.

# Experimental Conditions

## Software

We used the software provided from the CE10 coordinator.

## Test Sequences, Bit Rates and Coding Conditions

Test conditions are the intra-only configuration on both High Efficiency and Low Complexity conditions and test sequences as defined in C500[1], using the reference configuration files also provided in [1]. Further more, we conducted the combination of Test#9 and Test#10 as Test#13, and the combination of Test#6 and Test#13 as Test#14. In addition to the common test sequences, we conducted Test#1 to #12 on SHV sequences cropped to Class A size provided by NHK[2]. The Test#13 is cross-checked by Toshiba[3], and tests on SHV sequences are cross-checked by NEC[4].

Our tests are conducted on Linux OS(Fedora x86_64) with the same CPU/Memory equipped PCs.

# Test Results

The test results on the common test sequences are shown in Table 1.

Table 1. Test results on the common test sequences

| TEST#1 | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 2.4 | 2.6 | 2.7 | 2.5 | 3.3 | 3.3 |
| Class B | 1.8 | 1.8 | 2.1 | 1.9 | 2.7 | 2.9 |
| Class C | 1.6 | 1.5 | 1.5 | 1.7 | 2.2 | 2.3 |
| Class D | 1.1 | 1.1 | 1.2 | 1.0 | 1.4 | 1.4 |
| Class E | 2.7 | 3.0 | 2.6 | 3.1 | 4.1 | 3.8 |
| All | 1.8 | 1.9 | 1.9 | 1.9 | 2.6 | 2.6 |
| Enc Time[%] | 96% |  |  | 90% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
| TEST#2 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 4.9 | 4.9 | 5.2 | 5.7 | 5.9 | 5.8 |
| Class B | 4.7 | 4.5 | 5.0 | 5.6 | 6.4 | 6.7 |
| Class C | 5.3 | 5.2 | 5.0 | 6.6 | 6.7 | 6.8 |
| Class D | 4.1 | 4.0 | 4.1 | 4.8 | 4.8 | 4.9 |
| Class E | 7.1 | 7.6 | 7.1 | 9.6 | 9.8 | 9.9 |
| All | 5.1 | 5.1 | 5.2 | 6.3 | 6.6 | 6.8 |
| Enc Time[%] | 90% |  |  | 79% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
| TEST#3 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.4 | 0.2 | 0.2 | 0.8 | 0.1 | 0.1 |
| Class B | 0.3 | 0.2 | 0.1 | 0.7 | 0.1 | 0.1 |
| Class C | 0.8 | 0.4 | 0.4 | 1.4 | 0.4 | 0.4 |
| Class D | 1.1 | 0.6 | 0.6 | 1.7 | 0.6 | 0.6 |
| Class E | 0.5 | 0.2 | 0.2 | 1.0 | 0.2 | 0.3 |
| All | 0.6 | 0.3 | 0.3 | 1.1 | 0.3 | 0.3 |
| Enc Time[%] | 97% |  |  | 96% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
| TEST#4 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.7 | 0.7 | 0.6 | 1.2 | 0.6 | 0.4 |
| Class B | 0.4 | 0.3 | 0.4 | 1.1 | 0.8 | 0.8 |
| Class C | 0.6 | 0.7 | 0.7 | 1.2 | 1.3 | 1.2 |
| Class D | 0.5 | 0.5 | 0.6 | 0.8 | 0.9 | 0.9 |
| Class E | 0.6 | 0.6 | 0.5 | 1.7 | 0.6 | 0.7 |
| All | 0.6 | 0.5 | 0.5 | 1.2 | 0.9 | 0.9 |
| Enc Time[%] | 98% |  |  | 96% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
| TEST#5 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1.3 | 1.3 | 1.3 | 2.2 | 1.2 | 0.8 |
| Class B | 0.9 | 0.8 | 0.8 | 2.1 | 1.5 | 1.6 |
| Class C | 1.3 | 1.4 | 1.5 | 2.2 | 2.2 | 2.3 |
| Class D | 1.1 | 1.2 | 1.3 | 1.6 | 1.7 | 1.9 |
| Class E | 1.4 | 1.2 | 1.1 | 3.3 | 1.0 | 1.5 |
| All | 1.2 | 1.1 | 1.2 | 2.2 | 1.6 | 1.7 |
| Enc Time[%] | 96% |  |  | 91% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
| TEST#6 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.7 | 0.8 | 1.0 | 0.7 | 1.6 | 1.8 |
| Class B | 0.4 | 0.4 | 0.5 | 0.3 | 0.7 | 0.8 |
| Class C | 0.4 | 0.3 | 0.3 | 0.2 | 0.4 | 0.5 |
| Class D | 0.2 | 0.2 | 0.2 | 0.1 | 0.3 | 0.2 |
| Class E | 0.7 | 1.0 | 0.7 | 0.7 | 1.8 | 1.6 |
| All | 0.4 | 0.5 | 0.5 | 0.3 | 0.8 | 0.8 |
| Enc Time[%] | 99% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
| TEST#7 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.9 | 1.0 | 1.3 | 0.7 | 2.1 | 2.3 |
| Class B | 0.6 | 0.6 | 0.8 | 0.4 | 1.1 | 1.2 |
| Class C | 0.6 | 0.5 | 0.5 | 0.3 | 0.8 | 0.8 |
| Class D | 0.3 | 0.4 | 0.3 | 0.1 | 0.4 | 0.4 |
| Class E | 1.2 | 1.6 | 1.2 | 1.2 | 3.4 | 2.9 |
| All | 0.7 | 0.8 | 0.7 | 0.5 | 1.4 | 1.3 |
| Enc Time[%] | 98% |  |  | 95% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
| TEST#8 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.2 | 0.2 | 0.0 | 0.1 | 0.1 |
| Class B | 0.1 | 0.3 | 0.2 | 0.0 | 0.1 | 0.2 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.3 | 0.4 | 0.4 | 0.1 | 0.4 | 0.4 |
| All | 0.1 | 0.2 | 0.2 | 0.0 | 0.1 | 0.1 |
| Enc Time[%] | 99% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
| TEST#9 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.2 | 0.2 | 0.0 | 0.1 | 0.1 |
| Class B | 0.1 | 0.3 | 0.3 | -0.1 | 0.2 | 0.2 |
| Class C | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.4 | 0.5 | 0.5 | 0.0 | 0.5 | 0.4 |
| All | 0.1 | 0.2 | 0.2 | 0.0 | 0.1 | 0.2 |
| Enc Time[%] | 98% |  |  | 96% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
| TEST#10 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
| TEST#11 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 2.9 | 2.8 | 3.0 | 3.5 | 3.5 | 3.5 |
| Class B | 2.2 | 2.1 | 2.3 | 2.8 | 3.0 | 3.1 |
| Class C | 2.6 | 2.1 | 2.1 | 3.6 | 3.0 | 3.0 |
| Class D | 2.3 | 1.9 | 1.9 | 3.0 | 2.3 | 2.3 |
| Class E | 3.4 | 3.4 | 3.0 | 4.7 | 4.5 | 4.4 |
| All | 2.6 | 2.3 | 2.4 | 3.4 | 3.1 | 3.2 |
| Enc Time[%] | 93% |  |  | 86% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
| TEST#12 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.2 | 0.2 | 0.2 | -1.0 | -0.1 | -0.2 |
| Class B | -0.3 | 0.0 | 0.0 | -1.0 | -0.1 | -0.2 |
| Class C | -0.1 | 0.3 | 0.2 | -0.4 | 0.0 | 0.0 |
| Class D | -0.1 | 0.3 | 0.3 | -0.4 | 0.0 | 0.0 |
| Class E | -0.4 | 0.1 | 0.0 | -0.9 | -0.5 | -0.4 |
| All | -0.2 | 0.2 | 0.1 | -0.7 | -0.1 | -0.1 |
| Enc Time[%] | 203% |  |  | 215% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
| TEST#13 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.2 | 0.2 | 0.0 | 0.1 | 0.1 |
| Class B | 0.1 | 0.2 | 0.3 | -0.1 | 0.1 | 0.2 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.4 | 0.6 | 0.4 | 0.0 | 0.4 | 0.4 |
| All | 0.1 | 0.2 | 0.2 | 0.0 | 0.1 | 0.2 |
| Enc Time[%] | 99% |  |  | 97% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
| TEST#14 | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1.2 | 1.4 | 1.7 | 0.8 | 2.0 | 2.2 |
| Class B | 0.9 | 1.1 | 1.3 | 0.4 | 1.3 | 1.4 |
| Class C | 0.5 | 0.4 | 0.4 | 0.2 | 0.5 | 0.6 |
| Class D | 0.4 | 0.4 | 0.4 | 0.1 | 0.4 | 0.3 |
| Class E | 1.5 | 1.9 | 1.6 | 0.9 | 2.7 | 2.4 |
| All | 0.8 | 1.0 | 1.0 | 0.4 | 1.2 | 1.2 |
| Enc Time[%] | 96% |  |  | 93% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

The summary of test results is shown in Table 2.

Table 2. Test results on the common test sequences

|  | Block size and number of directions |  |  |  |  | HE |  | LC |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4x4 | 8x8 | 16x16 | 32x32 | 64x64 | BDrate | Enc Time | Bdrate | Enc Time |
| Reference | 17 | 34 | 34 | 34 | 5 | 0.000 | 100.00% | 0.000 | 100.00% |
| Test#1 | 17 | 17 | 17 | 17 | 5 | 1.803 | 95.86% | 1.903 | 90.28% |
| Test#2 | 9 | 9 | 9 | 9 | 5 | 5.123 | 90.22% | 6.320 | 79.30% |
| Test#3 | 9 | 34 | 34 | 34 | 5 | 0.626 | 97.44% | 1.135 | 96.23% |
| Test#4 | 17 | 17 | 34 | 34 | 5 | 0.553 | 98.02% | 1.174 | 95.76% |
| Test#5 | 17 | 9 | 34 | 34 | 5 | 1.157 | 96.30% | 2.217 | 91.39% |
| Test#6 | 17 | 34 | 17 | 34 | 5 | 0.435 | 98.76% | 0.341 | 96.80% |
| Test#7 | 17 | 34 | 9 | 34 | 5 | 0.673 | 98.10% | 0.489 | 95.32% |
| Test#8 | 17 | 34 | 34 | 17 | 5 | 0.110 | 99.00% | 0.005 | 97.48% |
| Test#9 | 17 | 34 | 34 | 9 | 5 | 0.129 | 98.46% | -0.027 | 96.42% |
| Test#10 | 17 | 34 | 34 | 34 | 3 | -0.004 | 98.44% | 0.000 | 99.30% |
| Test#11 | 9 | 17 | 17 | 17 | 5 | 2.581 | 93.19% | 3.433 | 86.30% |
| Test#12 | 17 | 34 | 34 | 34 | 5 | -0.206 | 203.33% | -0.708 | 215.34% |
| Test#13 | 17 | 34 | 34 | 9 | 3 | 0.125 | 98.67% | -0.028 | 97.32% |
| Test#14 | 17 | 34 | 17 | 9 | 3 | 0.830 | 95.71% | 0.425 | 92.62% |

The summary of test results on SHV test sequences is shown in Table 3.

Table 3. Test results on cropped SHV sequences

|  | 4x4 | 8x8 | 16x16 | 32x32 | 64x64 | HE | Enc | LC | Enc |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference | 17 | 34 | 34 | 34 | 5 | 0.000 | 100.00% | 0.000 | 100.00% |
| Test#1 | 17 | 17 | 17 | 17 | 5 | 0.639 | 95.19% | 0.516 | 89.75% |
| Test#2 | 9 | 9 | 9 | 9 | 5 | 1.672 | 89.09% | 2.106 | 78.98% |
| Test#3 | 9 | 34 | 34 | 34 | 5 | 0.044 | 97.15% | 0.083 | 96.28% |
| Test#4 | 17 | 17 | 34 | 34 | 5 | 0.161 | 97.74% | 0.489 | 95.66% |
| Test#5 | 17 | 9 | 34 | 34 | 5 | 0.353 | 95.73% | 1.199 | 91.35% |
| Test#6 | 17 | 34 | 17 | 34 | 5 | 0.213 | 98.68% | 0.001 | 96.85% |
| Test#7 | 17 | 34 | 9 | 34 | 5 | 0.438 | 97.87% | 0.289 | 95.29% |
| Test#8 | 17 | 34 | 34 | 17 | 5 | 0.083 | 98.93% | -0.053 | 97.47% |
| Test#9 | 17 | 34 | 34 | 9 | 5 | 0.119 | 98.32% | -0.124 | 96.11% |
| Test#10 | 17 | 34 | 34 | 34 | 3 | -0.006 | 98.41% | -0.005 | 99.58% |
| Test#11 | 9 | 17 | 17 | 17 | 5 | 0.701 | 92.42% | 0.632 | 86.26% |
| Test#12 | 17 | 34 | 34 | 34 | 5 | -0.222 | 194.93% | -1.169 | 216.31% |

# Analysis

By comparing each result of each test case, the numbers of intra prediction directions for each block size which gives almost comparable performance with the reference are as follows:

[for the common test sequences]

4x4: 17 modes

8x8: 34 modes

16x16: 34 modes

32x32: 9 modes

64x64: 3 modes

[for SHV sequences]

4x4: 9 modes

8x8: 17 modes

16x16: 17 modes

32x32: 9 modes

64x64: 3 modes

# Conclusion

For 32x32 blocks, 9 or less directions would be preferable, and for 64x64 blocks, 3 or less directions would be preferable. For smaller blocks, more variations of test sequences would be necessary to get more clear idea on the optimal number of intra prediction modes.

# References

- F. Bossen, “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C500, 3rd Meeting: Guangzhou, CN, Oct., 2010.

- K. Iguchi, “Performance report of TMuC for Super Hi-Vision”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C055, 3rd Meeting: Guangzhou, CN, Oct., 2010.

- A. Tanizawa, “CE10: Cross check report for number of intra prediction directions from Toshiba”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D102, 4th Meeting: Daegu, KR, Jan., 2011.

- K. Chono, “CE10: xxxxxxxxxxxxxxxxxx”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-Dxxx, 4th Meeting: Daegu, KR, Jan., 2011.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).