| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D102 |
| --- | --- |

| Title: | CE10: Cross check report for number of intra prediction directions from Toshiba |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Akiyuki Tanizawa Taichiro Shiodera 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 akiyuki.tanizawa@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This contribution contains the cross-checking results for Core Experiment 10 (Number of intra prediction directions).  An increased number of directions for intra prediction was introduced in the TMuC software and was integrated into the HEVC Test Model 1.  The purpose of this CE is to characterize the performance and complexity of using different numbers of intra prediction directions.  The verification task has been done successfully and the results are attached in this contribution.

# Introduction

At JCT-VC Guangzhou meeting, Core Experiment 10 on Number of intra prediction directions [1] was established and the following tasks have been decided to be performed in this CE;

- Determine the performance and complexity of using different numbers of intra prediction directions as compared to the HEVC Test Model.

- Identify the best combination of the number of intra prediction directions for each partition size, from the viewpoint of performance/complexity balance.

# Test points and experimental conditions

The test points (12 cases) are identified in the Table 1 below with respect to the number of intra prediction directions up to the one defined for the HEVC Test Model 1 default configurations.  TEST #13, which is not official test case described in [1] is an additional test case to evaluate combination of official test cases.  All test points for I slice only coding structure on both high efficiency condition and low complexity condition are tested based on the common coding conditions [2].  BD-rate [3] is measured by Excel sheet (CE10_results_template.xls) provided on the JCT-VC CE10 coordinator.  The software based on TMuC version 0.9 was provided on November 4th by CE10 coordinator.

Table 1. Performance Tests for different combinations of number of intra prediction directions

| Block size | Number of Intra Prediction Directions |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Reference | Test |  |  |  |  |  |  |  |  |  |  |  |
|  |  | #1 | #2 | #3 | #4 | #5 | #6 | #7 | #8 | #9 | #10 | #11 | #13 |
| 4x4 | 17 | 17 | 9 | 9 | 17 | 17 | 17 | 17 | 17 | 17 | 17 | 9 | 17 |
| 8x8 | 34 | 17 | 9 | 34 | 17 | 9 | 34 | 34 | 34 | 34 | 34 | 17 | 34 |
| 16x16 | 34 | 17 | 9 | 34 | 34 | 34 | 17 | 9 | 34 | 34 | 34 | 17 | 34 |
| 32x32 | 34 | 17 | 9 | 34 | 34 | 34 | 34 | 34 | 17 | 9 | 34 | 17 | 9 |
| 64x64 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 3 | 5 | 3 |

Test #12: To evaluate the upper limit of the performance of the intra prediction method of the test model, the reference without fast mode decisions, which means that all intra prediction directions are evaluated using RD cost, is also tested.

# Experimental results

Table 2 shows the experimental results of overall cases. This table shows that loss of BD-rate gain of TEST #8, TEST #9, TEST #10 and TEST #13 is negligible and the encoding time of them is reduced slightly. Although TEST #12, which utilizes the mode decision based on full R-D optimization, can achieve 0.21% gain on high efficiency condition and 0.71% gain on low complexity condition, the encoding time is highly increased (about twice). The accompanied spreadsheet contains the detailed data of the cross-checking results.

Table 1:  BD-rate, the encoding time and the decoding time for all test cases

| TEST # | Intra, High efficiency |  |  |  |  | Intra, low complexity |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD- rate | U BD- rate | V BD- rate | Enc Time | Dec time | Y BD- rate | U BD- rate | V BD- rate | Enc Time | Dec time |
| #1 | 2.36 | 2.57 | 2.70 | 96% | 102% | 1.90 | 2.60 | 2.62 | 89% | 102% |
| #2 | 5.12 | 5.09 | 5.17 | 94% | 116% | 6.32 | 6.62 | 6.78 | 90% | 131% |
| #3 | 0.63 | 0.34 | 0.31 | 97% | 101% | 1.13 | 0.29 | 0.29 | 96% | 102% |
| #4 | 0.55 | 0.54 | 0.52 | 98% | 101% | 1.17 | 0.87 | 0.88 | 96% | 106% |
| #5 | 1.16 | 1.14 | 1.17 | 96% | 101% | 2.22 | 1.58 | 1.71 | 91% | 105% |
| #6 | 0.43 | 0.48 | 0.47 | 99% | 102% | 0.34 | 0.85 | 0.84 | 96% | 101% |
| #7 | 0.67 | 0.75 | 0.74 | 98% | 100% | 0.49 | 1.38 | 1.34 | 95% | 98% |
| #8 | 0.11 | 0.19 | 0.17 | 99% | 100% | 0.01 | 0.12 | 0.14 | 97% | 100% |
| #9 | 0.13 | 0.21 | 0.18 | 98% | 101% | -0.03 | 0.14 | 0.15 | 96% | 99% |
| #10 | 0.00 | 0.00 | -0.01 | 98% | 102% | 0.00 | 0.00 | 0.00 | 100% | 101% |
| #11 | 2.58 | 2.35 | 2.36 | 93% | 100% | 3.43 | 3.12 | 3.15 | 85% | 98% |
| #12 | -0.21 | 0.18 | 0.14 | 221% | 99% | -0.71 | -0.11 | -0.13 | 208% | 97% |
| #13 | 0.12 | 0.21 | 0.17 | 96% | 99% | -0.03 | 0.14 | 0.15 | 95% | 98% |

# References

[1] 	"Core Experiment 10: Number of Intra Prediction Directions ", ITU-T and ISO/IEC JCT-VC 3rd Meeting Guangzhou, JCTVC-C510, Oct. 2010.

[2] 	F. Bossen, “Common test conditions and software reference configurations “, ITU-T and ISO/IEC JCT-VC 3rd Meeting Guangzhou, JCTVC-C500, Oct. 2010.

[3] 	Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.