| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D104 |
| --- | --- |

| Title: | CE7: Cross check report of I2R's proposal (JCTVC-D046) from Toshiba |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Akiyuki Tanizawa Jun Yamaguchi 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 akiyuki.tanizawa@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This contribution contains the cross-checking the software containing the tool of JCTVC-C039 proposed by Institute for Infocomm Research. The verification task has been done successfully and the results match those proposed in JCTVC-D046.

# Cross-checking

The software based on TMuC version 0.9 and the experimental results were provided on Dec. 27 2010 by Institute for Infocomm Research. The software was complied fine and the cross-checking was performed on 64bit environments. The simulation has been conducted under CE common test conditions [1] for both high efficiency and low complexity conditions in the case of I-slice only coding structure and the random access coding structure based on CE7 test conditions [2]. The encoded images match those of the decoded ones, and the PSNR and bitrate values are identical to the results provided by Institute for Infocomm Research. Table 1 and Table 2 show the cross-checking results of overall cases from Toshiba.

We checked the source code of the provided software and confirmed that the technique integrated in the software matches JCTVC-D046 contributed by Institute for Infocomm Research completely.

Table 1: BD-Bitrate, the encoding time and the decoding time for I slice only coding structure

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.2 | -2.5 | -2.6 | -3.6 | -1.5 | -1.0 |
| Class B | -0.9 | -1.1 | -1.1 | -1.7 | -1.1 | -0.9 |
| Class C | -1.2 | -1.0 | -1.0 | -1.9 | -1.1 | -1.1 |
| Class D | -1.3 | -1.1 | -1.0 | -1.9 | -1.0 | -1.0 |
| Class E | -1.5 | -1.7 | -1.5 | -2.7 | -0.4 | -0.9 |
| All | -1.3 | -1.4 | -1.3 | -2.2 | -1.0 | -1.0 |
| Enc Time[%] | 100% |  |  | 103% |  |  |
| Dec Time1[%] | 102% |  |  | 104% |  |  |
| Dec Time2[%] | 100% |  |  | 100% |  |  |

Table 2:  BD-Bitrate, the encoding time and the decoding time for random access coding structure

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.9 | -0.5 | -0.2 | -1.2 | -0.2 | 0.1 |
| Class B | -0.5 | -0.3 | -0.4 | -0.6 | -0.3 | -0.3 |
| Class C | -0.6 | -0.3 | -0.4 | -0.6 | -0.3 | -0.3 |
| Class D | -0.6 | -0.3 | -0.1 | -0.5 | -0.2 | -0.2 |
| Class E |  |  |  |  |  |  |
| All | -0.6 | -0.3 | -0.3 | -0.7 | -0.2 | -0.2 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time1[%] | 101% |  |  | 102% |  |  |
| Dec Time2[%] | 98% |  |  | 93% |  |  |

In the above tables, it is noted that the Dec Time1 means the decoding time ratio without outputting the yuv file and the Dec time2 means the decoding time ratio with outputting the yuv file.

The accompanied spreadsheet contains the detailed data of the cross-checking results

# References

[1] 	F. Bossen ," Common test conditions and software reference configurations ", ITU-T and ISO/IEC JCT-VC 3rd Meeting Guangzhou, JCTVC-C500, Oct. 2010.

[2] 	"Core Experiment 7: Alternative transform", ITU-T and ISO/IEC JCT-VC 3rd Meeting Guangzhou, JCTVC-C507, Oct. 2010.