| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D106 |
| --- | --- |

| Title: | High Efficient 1 byte fixed length coding for Low Complexity Entropy Coding – PIPE/V2F |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto Ryoji Hattori Shun-ichi Sekiguchi Yoshiaki Kato Kohtaro Asai Tokumichi Murakami Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Kazuo.Sugimoto@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

In this contribution, a new entropy coding scheme, PIPE/V2F, for low complexity condition is proposed. The proposed scheme realizes 1 byte fixed length coding with higher coding efficiency compared to LCEC which is currently used for low complexity condition in HM-1. In the proposed scheme, V2F(Variable Length to Fixed Length) coders are used instead of V2V(Variable Length to Variable Length) coder for PIPE, and all V2F coders are designed to generate 4 bit fixed length code. Context adaptation is disabled to accelerate the entropy coding/decoding in the proposed scheme. We implemented the proposed scheme on TMuC0.9, and simulations are conducted using common test configurations of low complexity settings. The proposed scheme achieves BD-rate reduction 3.2% on average for random access case, and 4.1% on average for low delay case. The increase of decoding complexity is around 8% when compared to LCEC.

# Introduction

LCEC is adopted in HM-1 as a low complexity entropy coder, however, the functionality is completely different from those of CABAC. On the other hand, most of the functionalities PIPE uses are also used in common with CABAC. We modified PIPE to realize low complexity entropy coding which gives an advantage for designing hardware decoder to support both CABAC and low complexity entropy coding schemes.

# Proposed scheme

In our proposed scheme, 2 modifications are applied to PIPE/V2V coding as follows:

- use V2F tables instead of V2V tables

- disable context adaptation

The proposed scheme, PIPE/V2F, uses 12 different V2F coder shown in table 1.

Table 1.

1111

1110

1101

1100

1011

1010

1001

1000

0111

0110

0101

0100

0011

0010

0001

0000

1111

1110

1101

1100

1011

1010

1001

1000

0111

0110

0101

0100

0011

0010

0001

0000

V2F #0

1111

1110

1101

1100

1011

1010

1001

1000

0111

0110

0101

0100

0011

0010

0001

0000

As shown in table 1, all bins are converted to 4 bit fixed length codes. In the decoder, bitstream reading is conducted in 1 byte unit by reading a couple of 4 bit fixed length codes together.

Context adaptation is reported as a bottleneck of computation for PIPE in [1], therefore in the proposed scheme all context adaptations are disabled to realize low computational complexity entropy coding.

# Implementation affinity with CABAC

As the proposed scheme is based on PIPE coding, most of the algorithms are in common with CABAC. To evaluate the difference with CABAC, we counted the program lines and the table sizes necessary to implement the proposed scheme and LCEC on top of CABAC as shown in Table 2 and 3 respectively.

Table 2. Necessary additional program lines and table size on top of CABAC for encoder

| coder | PIPE/V2F | LCEC |
| --- | --- | --- |
| program lines for CABAC | 2141 |  |
| table size for CABAC | 10312 |  |
| additional program lines | 286 | 1487 |
| additional table size | 9664 | 27608 |

Table 3. Necessary additional program lines and table size on top of CABAC for decoder

| coder | PIPE/V2F | LCEC |
| --- | --- | --- |
| program lines for CABAC | 2161 |  |
| table size for CABAC | 10312 |  |
| additional program lines | 155 | 1585 |
| additional table size | 1852 | 29464 |

According to Table 2 and 3, it is observed that:

- additional program size for PIPE/V2F encoder is around 20% that of LCEC

- additional program size for PIPE/V2F decoder is around 10% that of LCEC

- additional table size for PIPE/V2F encoder is around 35% that of LCEC

- additional table size for PIPE/V2F decoder is around 6% that of LCEC

# Simulations & Results

The proposed scheme is implemented on top of TMuC v0.9, and the simulations are conducted using the common test configurations[2] for low complexity settings. Table 4 shows the comparison results between the default configuration of TMuC v0.9 and the one with the proposed scheme. Each Class has been evaluated on the same CPU equipped PC with the same memory size. To measure the decoding time as accurate as possible, decoding has been conducted sequentially using single CPU. The decoding has been done 3 times, and the average decoding time are used as the result in Table 4.

Table 4. BD Bitrate saving of the proposed scheme compared to LCEC

|  | Intra LoCo |  |  |
| --- | --- | --- | --- |
|  | Y psnr | U psnr | V psnr |
| Class A | -0.3 | 4.5 | 4.5 |
| Class B | -0.2 | 4.2 | 3.3 |
| Class C | -0.8 | 1.2 | 0.5 |
| Class D | -1.1 | 0.3 | -0.3 |
| Class E | -0.9 | 3.9 | 5.4 |
| All | -0.7 | 2.6 | 2.3 |
| Enc Time[%] | 181% |  |  |
| Dec Time[%] | 127% |  |  |
|  | Random access LoCo |  |  |
|  | Y psnr | U psnr | V psnr |
| Class A | -3.5 | 3.6 | 4.5 |
| Class B | -4.7 | 6.3 | 5.6 |
| Class C | -4.0 | 3.4 | 3.0 |
| Class D | -3.9 | 3.4 | 3.0 |
| Class E |  |  |  |
| All | -4.1 | 4.4 | 4.1 |
| Enc Time[%] | 128% |  |  |
| Dec Time[%] | 109% |  |  |
|  | Low delay LoCo |  |  |
|  | Y psnr | U psnr | V psnr |
| Class A |  |  |  |
| Class B | -3.6 | 17.2 | 22.0 |
| Class C | -2.5 | 9.9 | 10.8 |
| Class D | -3.0 | 16.0 | 15.6 |
| Class E | -3.4 | 21.6 | 18.5 |
| All | -3.2 | 15.9 | 17.0 |
| Enc Time[%] | 128% |  |  |
| Dec Time[%] | 108% |  |  |

As the complexity of entropy coder highly depends on the bitrate, and the bit amount generated by LCEC, CABAC and PIPE/V2F differs, we also measured decoding time divided by bitrate to normalize the decoding speed for each coder. The increase of the time of PIPE/V2F compared to LCEC and CABAC in this measurement is shown in Table 5.

Table 5. comparison of decoding time per bit

|  | LCEC | CABAC |
| --- | --- | --- |
| Intra Only | 22.16% | -10.66% |
| Random Access | 8.00% | -9.11% |
| Low Delay | 6.10% | -9.38% |

The balance of the decoding complexity per bit and coding efficiency of each scheme on random access condition and low delay condition are shown in Figures 1 and 2 respectively.

Figure 1. Complexity vs Performance on Random Access condition

Figure 2. Complexity vs Performance on Low Delay condition

# Conclusion

In this contribution, a new entropy coding scheme, PIPE/V2F, for low complexity condition is proposed. The proposed scheme realizes 1 byte fixed length coding with higher coding efficiency compared to LCEC which is currently used for low complexity condition in HM-1.

PIPE/V2F achieves BD-rate reduction 3.2% on average for random access case, and 4.1% on average for low delay case. The increases of decoding complexity per bit are around 8% and 6% for random access and low delay cases respectively.

Another advantage of the proposed scheme is smaller amount of additional program and table sizes on top of CABAC. The necessary additional program size are only 20 and 10% compared to LCEC for encoder and decoder, and the necessary additional table size are only 35% and 6% compared to LCEC for decoder.

We propose the proposed scheme to be verified in CE by the next meeting as a candidate for the entropy coder of low complexity condition.

# Reference

- Vivienne Sze, et al., “TE12: Evaluation of entropy coders: PIPE tested against CABAC” , Doc. JCTVC-C058, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, 3rd Meeting: Guangzhou, CN, Oct., 2010.

- JCT-VC, “Common test conditions and software reference configurations” , Doc. JCTVC-C500, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, 3rd Meeting: Guangzhou, CN, Oct., 2010.

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).