| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D107 |
| --- | --- |

| Title: | CE7: Experimental results for one-dimensional directional unified transform |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Akiyuki Tanizawa Jun Yamaguchi Taichiro Shiodera Takeshi Chujoh Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 akiyuki.tanizawa@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This contribution presents the experimental results of 1 Dimensional Directional Unified Transform (1DDUT) for Core Experiment 7 (Alternative transforms in HEVC).  1DDUT is a spatial transform scheme for intra coding. 1DDUT has only two 1-D transform matrices, which are Discrete Cosine Transform (DCT) and the directional transform based on Karhunen-Loeve Transform (KLT). It’s applied for 4x4 block and 8x8 block only. 1DDUT was proposed in the contribution of JCTVC-B042 at the JCT-VC Geneva meeting and JCTVC-C080 at the JCT-VC Guangzhou meeting.

In this document, the experimental results in TMuC software version 0.9 on both high efficiency conditions and low complexity conditions based on the common test conditions for both I slice only coding structure and the random access coding structure defined by CE7 are reported.  For I slice only coding structure, the BD-Bitrate gain compared with the anchor is 2.2% on average, and for the random access coding structure, the BD-Bitrate gain is 0.8% on average.  Experimental results indicate that 1DDUT can improve the coding efficiency for intra coding with minimum complexity increase.

# Introduction

At JCT-VC Guangzhou meeting, Core Experiment 7 on Alternative transform [1] was established and the following tasks have been decided to be performed in this CE;

- Specify the software platform, common software configurations and common test conditions for these experiments;

- Evaluate compression efficiency, complexity, and subjective performance of the alternative transforms studied in this CE;

- Investigate interactions between alternative transforms and coefficient scanning schemes;

- Report the results and conclusions of these experiments.

# 1 Dimensional Directional Unified Transform (1DDUT)

## Algorithm description

1DDUT was firstly included in the CfP submission of JCTVC-A117 [2] and the improved method was proposed in the contribution of JCTVC-B042 [3] at JCT-VC Geneva meeting and JCTVC-C080 [4] at JCT-VC Guangzhou meeting.  1DDUT is a spatial transform scheme for intra coding. 1DDUT has only two 1-D transform matrices, which are Discrete Cosine Transform (DCT) and the directional transform designed based on Karhunen-Loeve Transform (KLT). The directional transform was trained offline and pre-determined.

Unified Intra Prediction (UIP) [5] adopted in HEVC Test Model 1 (TM1) has up to 34 prediction modes (up to 33 directional prediction modes and DC prediction mode).  Since the intra prediction and the transform have a strong relation, it is desirable to define a specific transform for each intra prediction direction.  However, defining a number of transforms according to the number of intra prediction directions of UIP, it may have some impact on a certain implementation.  Therefore, 1DDUT utilizes only two 1-D transform matrices, which can specify four combinations of 2-D transform.  A combination is selected for each intra prediction mode.  It means that the intra prediction modes are assigned into four categories, which are three directions and no direction (DC prediction).  Error: Reference source not found shows the relationship between the transform index and each 1D transform matrix, and Table 2 shows the relationship between intra prediction modes of UIP and transform index. T1 shows the directional transform designed based on KLT and T2 shows DCT.

Table 1:  Relationship between the TransformIdx and each 1 D transform matrix

| TransformIdx | Vertical 1D Transform | Horizontal 1D Transform |
| --- | --- | --- |
| 0 | T1 | T2 |
| 1 | T2 | T1 |
| 2 | T2* | T2* |
| 3 | T1 | T1 |

Table 2:  Relationship between the IntraPredMode and the TransformIdx

| IntraPredMode [ puPartIdx ] | IntraPredType [ puPartIdx ] | IntraPredAngleID [ puPartIdx ] | TransformIdx [ puPartIdx ] |
| --- | --- | --- | --- |
| 0 | Intra_Vertical | 0 | 0 |
| 1 | Intra_Horizontal | 0 | 1 |
| 2 | Intra_DC | - | 2 |
| 3 | Intra_Vertical | -8 | 3 |
| 4 | Intra_Vertical | -4 | 3 |
| 5 | Intra_Vertical | +4 | 0 |
| 6 | Intra_Vertical | +8 | 0 |
| 7 | Intra_Horizontal | -4 | 3 |
| 8 | Intra_Horizontal | +4 | 1 |
| 9 | Intra_Horizontal | +8 | 1 |
| 10 | Intra_Vertical | -6 | 3 |
| 11 | Intra_Vertical | -2 | 0 |
| 12 | Intra_Vertical | +2 | 0 |
| 13 | Intra_Vertical | +6 | 0 |
| 14 | Intra_Horizontal | -6 | 3 |
| 15 | Intra_Horizontal | -2 | 1 |
| 16 | Intra_Horizontal | +2 | 1 |
| 17 | Intra_Horizontal | +6 | 1 |
| 18 | Intra_Vertical | -7 | 3 |
| 19 | Intra_Vertical | -5 | 3 |
| 20 | Intra_Vertical | -3 | 3 |
| 21 | Intra_Vertical | -1 | 0 |
| 22 | Intra_Vertical | +1 | 0 |
| 23 | Intra_Vertical | +3 | 0 |
| 24 | Intra_Vertical | +5 | 0 |
| 25 | Intra_Vertical | +7 | 0 |
| 26 | Intra_Horizontal | -7 | 3 |
| 27 | Intra_Horizontal | -5 | 3 |
| 28 | Intra_Horizontal | -3 | 3 |
| 29 | Intra_Horizontal | -1 | 1 |
| 30 | Intra_Horizontal | +1 | 1 |
| 31 | Intra_Horizontal | +3 | 1 |
| 32 | Intra_Horizontal | +5 | 1 |
| 33 | Intra_Horizontal | +7 | 1 |

After intra prediction, the residual signal is transformed using the sets of 1-D transform matrices derived according to the above categories. Transform coefficients are converted into 1D signals from 2D signals using nine fixed scanning orders each prediction group.  One of them is same as that of the common condition, which is used for the scanning order for DCT (TransformIdx = 2).

In this contribution, the same matrices of the directional transform as the previous proposal [3] are used and 1DDUT is introduced for 4x4 and 8x8 block only.

## Complexity analysis

For the understanding of complexity, the following metrics for 1DDUT are explained;

- Operations counts (e.g., number of multiplies and additions) and operation precision for individual transform: 16 multiples and 12 adds for 4-point transform, 64 multiplies and 56 adds for 8-point transform)

- Coefficient scanning process: nine fixed scan for 4x4 and 8x8 block

- Bit-precision: 32-bit (for inverse transform and inverse quantization)

- Memory sizes: (compared with conventional DCT and zigzag scan)

  - Transform matrix: additional 16byte for 4x4 block and 64byte for 8x8 block

  - Scanning order: additional 16byte (4x16) for 4x4 block and 384byte (6x64) for 8x8 block

- Encoding time and Decoding time compared with anchor are shown in Section 3

# Experimental results

1DDUT is integrated into TMuC software version 0.9.  I slice only coding structure, the random access coding structure and the low delay coding structure are tested based on CE7 coding conditions [1], and the other conditions are completely same as the common coding conditions [6].  BD-Bitrate (ΔBitrate [%]) [7] is measured by Excel sheet (HEVC_results_template.xls) provided on the JCTVC reflector.

Table 4 shows the experimental results of 1DDUT compared with common anchor on I slice only coding structure.  BD-Bitrate gain of 1DDUT compared to the anchor is 2.12% on average on high efficiency condition and 2.31% on average on low complexity condition, respectively. Table 5 and Table 6 show the experimental results on random access and low delay coding structures.  BD-Bitrate gain of 1DDUT compared to the anchor is 0.93% and 0.30% on average on high efficiency condition and 0.76% and 0.24% on average on low complexity condition, respectively.  1DDUT does not affect the encoding time and the decoding time significantly compared with the common condition anchor.

Table 4:  BD-Bitrate, the encoding time and the decoding time for I slice only coding structure

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.70 | -2.97 | -2.86 | -2.97 | -1.35 | -0.94 |
| Class B | -1.36 | -1.76 | -1.69 | -1.66 | -1.58 | -1.51 |
| Class C | -2.19 | -1.96 | -1.94 | -2.37 | -1.87 | -1.91 |
| Class D | -2.18 | -1.93 | -1.92 | -2.15 | -1.86 | -1.84 |
| Class E | -2.84 | -2.74 | -2.45 | -3.12 | -0.91 | -1.50 |
| All | -2.12 | -2.14 | -2.05 | -2.31 | -1.57 | -1.60 |
| Enc Time[%] | 106% |  |  | 105% |  |  |
| Dec Time1[%] | 104% |  |  | 107% |  |  |
| Dec Time2[%] | 103% |  |  | 104% |  |  |

Table 5:  BD-Bitrate, the encoding time and the decoding time for random access coding structure

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.16 | -0.51 | -0.20 | -1.03 | -0.18 | 0.04 |
| Class B | -0.77 | -0.53 | -0.55 | -0.63 | -0.48 | -0.51 |
| Class C | -0.99 | -0.62 | -0.61 | -0.88 | -0.68 | -0.59 |
| Class D | -0.95 | -0.72 | -0.33 | -0.66 | -0.37 | -0.19 |
| Class E |  |  |  |  |  |  |
| All | -0.93 | -0.60 | -0.46 | -0.76 | -0.47 | -0.37 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time1[%] | 105% |  |  | 105% |  |  |
| Dec Time2[%] | 104% |  |  | 92% |  |  |

Table 6:  BD-Bitrate, the encoding time and the decoding time for low delay coding structure

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.24 | -0.35 | -0.10 | -0.20 | -0.18 | -0.25 |
| Class C | -0.42 | -0.41 | -0.47 | -0.35 | -0.34 | -0.41 |
| Class D | -0.23 | -0.84 | -0.31 | -0.23 | -0.03 | -0.25 |
| Class E | -0.36 | 0.08 | 0.24 | -0.15 | 0.40 | -0.22 |
| All | -0.30 | -0.41 | -0.18 | -0.24 | -0.07 | -0.29 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time1[%] | 101% |  |  | 102% |  |  |
| Dec Time2[%] | 99% |  |  | 101% |  |  |

In the above tables, it is noted that the Dec Time1[%] means the decoding time ratio without outputting the yuv file and the Dec time2[%] means the decoding time ratio with outputting the yuv file.

# Conclusions

This contribution proposes 1 Dimensional Directional Unified Transform (1DDUT) and presents the experimental results for Core Experiment 7 (Alternative transforms in HEVC).  From the experimental results, the average BD-Bitrate gain of 2.2% for I slice only coding structure, 0.8% for the random access coding structure and 0.3% for low delay coding structure compared with TMuC common condition anchor are reported without significant increase of the encoding time and the decoding time.

It’s suggested that 1 Dimensional Directional Unified Transform (1DDUT) is introduced in the HEVC test model as the tool of coding efficiency improvement for intra coding.

# References

[1] 	"Core Experiment 7: Alternative transform", ITU-T and ISO/IEC JCT-VC 3rd Meeting Guangzhou, JCTVC-C507, Oct. 2010.

[2] 	T. Chujoh, A. Tanizawa and T. Yamakage, “Description of video coding technology proposal by TOSHIBA,” Joint Collaborative Team on Video Coding, JCTVC-A117, Dresden, April 2010.

[3] 	A. Tanizawa, T. Shiodera, T. Chujoh and T. Yamakage, “Improvement of Bidirectional Intra Prediction (BIP) and 1 Dimensional Directional Unified Transform,” ITU-T and ISO/IEC JCT-VC 2nd Meeting Geneva, JCTVC-B042, July 2010.

[4] 	A. Tanizawa, T. Shiodera, T. Chujoh and T. Yamakage, “TE7: 1 Dimensional Directional Unified Transform,” ITU-T and ISO/IEC JCT-VC 3rd Meeting Guangzhou, JCTVC-C080, Oct. 2010.

[5] 	J.-H. Min et. al. , "Unification of the Directional Intra Prediction Methods in TMuC ", ITU-T and ISO/IEC JCT-VC 2nd Meeting Geneva, JCTVC-B100, July 2010.

[6] 	F. Bossen ," Common test conditions and software reference configurations ", ITU-T and ISO/IEC JCT-VC 3rd Meeting Guangzhou, JCTVC-C500, Oct. 2010.

[7] 	Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

When TransformIdx = 2, DCT, is applied in the current TMuC software, which is same as AVC.