| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D108 |
| --- | --- |

| Title: | CE6 Subset A: Bidirectional intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Taichiro Shiodera Akiyuki Tanizawa Takeshi Chujoh Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 taichiro.shiodera@toshiba.co.jp akiyuki.tanizawa@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

____________________________

# Abstract

This contribution presents the experimental results of Bidirectional Intra Prediction (BIP) for Core Experiment 6 (CE6) on intra prediction improvement.  BIP was included in the CfP submission of JCTVC-A117 and was proposed in the contribution of JCTVC-C079 at the JCT-VC Guangzhou meeting.

In this document, the experimental results in TMuC software version 0.9 under the common test conditions defined by CE6 are reported.  For I slice only coding structure, the average BD-rate gain is 2.0% on low complexity condition and 1.4% on high efficiency condition. Experimental results indicate that BIP can improve the coding efficiency for intra coding with minimum complexity increase.

# Bidirectional Intra Prediction

Bidirectional intra prediction (BIP) was firstly proposed at the VCEG Marrakech meeting [1], and the improvement of BIP was reported in [2].  At the last JCT-VC meeting, experimental results based on TE6 were reported in JCTVC-C079 [3].

## Algorithm description

Bidirectional Intra Prediction (BIP)[1][2][3] is a spatial prediction scheme for intra coding. BIP combines two unidirectional intra predictions results by a weighted sum according to the distance between the predicted pixel and the reference pixel(s) used for prediction. These unidirectional intra predictions are based on Unified Intra Prediction (UIP) integrated in current TMuC 0.9 software. BIP consists of the unidirectional intra prediction modes (UIP modes) and bidirectional intra prediction modes (BIP modes), and one of the modes is selected for each PU. Figure 1 shows examples of UIP and BIP modes.

Figure 1  UIP and BIP modes.

The sample in the bidirectional prediction at pixel position (n), predBi[n], is derived from the following equation:

predBi[n] = ( w[n] * predL0[n] + ((1<<w_shift) - w[n]) * predL1[n] + (1<<(w_shift-1))) >> (w_shift)

where predL0[n] and predL1[n] are the samples of the two UIP modes, and w[n] is the weighting list for predL0[n] according to the sample position (n). w_shift shows the precision of the weighting list. The weighting list w[n] is pre-determined according to the difference of distance from corresponding reference pixels of two prediction directions. An example of weighted sum of two unidirectional intra prediction is shown in Figure 2, which w_shift is set to 10. By combining two UIP mode predictions with weighted averaging, it is expected to predict multi-directional texture and reduce the coding noise of the reference pixels.

Details of derivation process of the bidirectional prediction samples are shown in [3].

( w[n] *              +(1024-w[n]) *                     +512 ) >> 10

Mode 0

Mode 3

Figure 2  An example of weighted sum of two unidirectional intra prediction

### Derivation of bidirectional intra prediction modes

As explained above, two kinds of UIP modes, intraPredModeL0 and intraPredModeL1, are selected from several UIP mode candidates for current PU. In the case of HM1 which has maximum of 34 UIP modes, there are all 561 (= C(34,2)) possible combinations of UIP modes. This number is not acceptable since an overhead incurred by mode signaling increases significantly. In the previous contribution [3], a limited set of 16 combinations are used in order to reduce the overhead.

In this proposal, an implicit bidirectional mode derivation method (IBMD) is introduced. IBMD exploits the correlation between UIP mode of available neighboring PUs and the BIP mode of current PU. Figure 3 shows five IBMD modes. Left PU A and above PU B is a left and above neighboring PUs, and UIP mode "a" and "b" corresponds to prediction modes of Block A and B respectively. In IBMD, IntraPredModeL0 and IntraPredModeL1 of BIP modes are set as follows:

(1) combination of "a" and "b":

IntraPredModeL0 = a, IntraPredModeL1 = b,

(2) combination of "a" and neighboring direction of "a":

IntraPredModeL0 = a, IntraPredModeL1 = Nb[a],

(3) combination of "b" and neighboring direction of "b":

IntraPredModeL0 = b, IntraPredModeL1 = Nb[b],

(4) combination of "a" and counter direction of "a":

IntraPredModeL0 = a, IntraPredModeL1 = Ct[a],

(5) combination of "b" and counter direction of "b":

IntraPredModeL0 = b, IntraPredModeL1 = Ct[b],

where, Nb[x] is a table which map UIP mode "x" to the neighbor prediction direction of "x", and Ct[x] is a table which map UIP mode "x" to the counter prediction direction of "x" in the range of available directions. Table A-5 specifies the details of tables Nb[] and Ct[].

Figure 3  Implicit bi-directional mode derivarion

### Specification of intra prediction mode

Table 1 shows the number of UIP modes, BIP modes and total modes for each PU size. BIP modes are added when PU size is from 4x4 to 32x32. A number in brackets of BIP modes means the number of IBMD modes in BIP modes. Tables A-1 to A-3 specify the details of intra prediction mode set. When the PU size is 64x64, the prediction mode set is same as set specified in original UIP (unidirectional only).

Table 1  Number of intra prediction mode for each PU size

| PuSize | Number of UIP modes (IntraUniModeNum) | Number of BIP modes | Total Number of pred. modes |
| --- | --- | --- | --- |
| PU_4x4 | 17 | 8 (5) | 25 |
| PU_8x8 | 34 | 8 (5) | 42 |
| PU_16x16 | 34 | 8 (5) | 42 |
| PU_32x32 | 34 | 4 (3) | 38 |
| PU_64x64 | 5 | 0 | 5 |

### Coding of intra prediction mode

The coding of intra prediction mode is same as [3] except the number of suffix bins S and T for each PU size.

For PU_4x4 to PU_32x32, the selected intra prediction mode from the UIP and BIP modes are coded based on luma_pred_mode_code_type, which specifies the coding type for intra prediction mode as shown in Table 2. When luma_pred_mode_code_type is equal to 0, the prediction mode is the most probable mode (MPM). When luma_pred_mode_code_type is equal to 1, the prediction mode is one of the UIP modes except MPM.  When luma_pred_mode_code_type is equal to 2, the prediction mode is one of BIP modes.

MPM is derived in a similar fashion as [4] except the case that the neighbor PU uses one of the BIP modes. In this case, IntraPredModeL0 of the BIP mode is used.

In this table, S means the number of Fixed Length Bins (FLB) for the rest of UIP modes, and T means the number of FLB for the BIP modes as shown in Table 3.

Table 2  Associated names of luma_pred_mode_code_type

| luma_pred_mode_code_type | Name of luma_pred_mode_code_type | Prefix bins | Suffix bins |
| --- | --- | --- | --- |
| 0 | UIP MostProbable Mode | 00 | -- |
| 1 | Rest of UIP Modes | 01 | S bins |
|  |  | 10 | S bins + Esc. code |
| 2 | BIP Modes | 11 | T bins |

Table 3  Number of suffix bins for each PU size

| PU Size | S | T |
| --- | --- | --- |
| 4x4 | 3 | 3 |
| 8x8 | 4 | 3 |
| 16x16 | 4 | 3 |
| 32x32 | 4 | 2 |

The coding of intra prediction mode for PU_64x64 is same as TMuC 0.9 software.

### Mode decision (encoder only)

A fast mode decision method [5] for intra prediction integrated into TMuC 0.9 software is used in this proposal.  The number of intra mode candidates is completely same as TMuC 0.9 software and the best mode is selected based on Hadamard Transformed coefficients of residual signal (HSAD) with mode bits for each PU size.

# Experimental results

TMuC software version 0.9 with the default configuration is used in this experiment.  I slice only coding structure is tested based on CE6 [6] coding conditions [7].  BD-rate (ΔBitrate [%]) [8] is measured by Excel sheet (HEVC_results_template.xls) provided on the JCTVC reflector.

Table 4 shows the experimental results of BIP for I slice only coding structure. The negative value indicates gain. The BD-rate gain of BIP compared to the TMuC 0.9 anchor is 1.4% (High efficiency) and 2.0% (Low complexity) on average.

Table 4  BD-Rate[%] and relative encoding/decoding time[%] for Intra-only case

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.6 | -1.0 | -1.1 | -2.3 | -1.7 | -1.7 |
| Class B | -1.6 | -1.0 | -1.1 | -2.0 | -1.5 | -1.5 |
| Class C | -1.4 | -0.7 | -0.7 | -1.7 | -1.1 | -1.1 |
| Class D | -1.1 | -0.5 | -0.5 | -1.6 | -0.9 | -0.9 |
| Class E | -1.7 | -1.3 | -1.1 | -2.6 | -2.2 | -1.8 |
| All | -1.4 | -0.9 | -0.9 | -2.0 | -1.4 | -1.4 |
| Enc Time[%] | 107% |  |  | 114% |  |  |
| Dec Time1[%] | 106% |  |  | 110% |  |  |
| Dec Time2[%] | 104% |  |  | 108% |  |  |

In the above tables, it is noted that the Dec Time1 means the decoding time ratio without outputting the yuv file and the Dec time2 means the decoding time ratio with outputting the yuv file.

Figure 4 (1) ~ (4) show additional results for each PU size compared with TMuC 0.9 anchor in order to compare the gain of additional UIP modes and BIP modes.

Here the X-axis is the total number of intra modes which consists (i) UIP modes only and (ii) UIP and BIP modes for one particular PU size. The Y-axis is the average Y BD-Rate value on both intra high efficiency and low complexity conditions.  The number of prediction modes for PU sizes other than the target PU size is same as TMuC 0.9 anchor, which means that BIP modes are not used.  The blue solid line indicates the results of UIP modes based on CE10 results.  The blue dashed line indicates the upper limit of coding gain of UIP modes in case that the number of UIP modes is over 34.  This dashed line is extrapolated from slope of Y BD-Rate of the blue solid line.  For example, in the case of PU 8x8, the average Y BD-Rate is +0.86 % for 17 UIP modes, and 0.00% for 34 UIP modes (same as TMuC 0.9 anchor).  Therefore, the average Y BD-Rate is projected to -0.86 % for 51 UIP modes.  This dashed line is connected points of (34, 0.00%) and (51, -0.86%).  Since the slope is generically lower as the number of UIP modes is increased, this dashed line can be treated as the upper limit of coding gain of UIP modes.  The red line indicates the result of BIP modes on top of UIP modes.

As shown in these figures, it is observed that the gain of additional BIP modes is higher than the one of additional UIP modes for every PU size.

(1) PU_4x4

(2) PU_8x8

(3) PU_16x16

(4) PU_32x32

Figure 4  Total number of intra prediction modes - Y BD-Rate plots for each PU size.

# Conclusions

In this contribution, BIP is proposed and the experimental results of Bidirectional Intra Prediction for CE6 were reported.  The average BD-rate gain of 1.4% for high efficiency condition and 2.0% for low complexity condition compared with TMuC common condition anchor are achieved without significant increase of the encoding time and the decoding time.

It’s suggested that the Bidirectional Intra Prediction (BIP) is introduced in the HEVC test model (HM) as a tool of the coding efficiency improvement.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1]	T. Shiodera, A. Tanizawa, T. Chujoh, "Bidirectional Intra Prediction", ITU-T SG16/Q.6 VCEG-AE14, Marrakech, Jan. 2007.

[2]	T. Shiodera, A. Tanizawa, T. Chujoh and T. Yamakage, “Improvement of Bidirectional Intra Prediction,” ITU-T SG16 Q.6 Document, VCEG-AG08, Oct 2007.

[3]	T. Shiodera, A. Tanizawa, T. Chujoh and T. Yamakage, “ TE 6a: Bidirectional Intra Prediction,” Joint Collaborative Team on Video Coding, JCTVC-C079, Guangzhou, July 2010.

[4]	J.-H. Min et. al. , "Unification of the Directional Intra Prediction Methods in TMuC ", ITU-T and ISO/IEC JCT-VC 2nd Meeting Geneva, JCTVC-B100, July 2010.

[5]	Y. Piao et. al., “Encoder improvement of unified intra prediction," Joint Collaborative Team on Video Coding, JCTVC-C207, Guangzhou, October, 2010.

[6]	“Core Experiment 6: Intra Prediction Improvement," Joint Collaborative Team on Video Coding, JCTVC-B306, Geneva, July 2010.

[7]	“Common test conditions and software reference configurations," Joint Collaborative Team on Video Coding, JCTVC-C500, Guangzhou, October 2010.

[8] 	Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

# Annex

- Table A-1  Spacification for IntraPredMode for PU_4x4

| IntraPred- Mode [ puPartIdx ] | IntraBipred- Flag [ puPartIdx ] | IntraBipred- TypeIdx [ puPartIdx ] | IntraPred- TypeL0 [ puPartIdx ] | IntraPred- AngleIdL0 [ puPartIdx ] | IntraPred- TypeL1 [ puPartIdx ] | IntraPred- AngleIdL1 [ puPartIdx ] |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | - | Intra_Vertical | 0 | - | - |
| 1 | 0 | - | Intra_Horizontal | 0 | - | - |
| 2 | 0 | - | Intra_DC | - | - | - |
| 3 | 0 | - | Intra_Vertical | -8 | - | - |
| 4 | 0 | - | Intra_Vertical | -4 | - | - |
| 5 | 0 | - | Intra_Vertical | +4 | - | - |
| 6 | 0 | - | Intra_Vertical | +8 | - | - |
| 7 | 0 | - | Intra_Horizontal | -4 | - | - |
| 8 | 0 | - | Intra_Horizontal | +4 | - | - |
| 9 | 0 | - | Intra_Horizontal | +8 | - | - |
| 10 | 0 | - | Intra_Vertical | -6 | - | - |
| 11 | 0 | - | Intra_Vertical | -2 | - | - |
| 12 | 0 | - | Intra_Vertical | +2 | - | - |
| 13 | 0 | - | Intra_Vertical | +6 | - | - |
| 14 | 0 | - | Intra_Horizontal | -6 | - | - |
| 15 | 0 | - | Intra_Horizontal | -2 | - | - |
| 16 | 0 | - | Intra_Horizontal | +2 | - | - |
| 17 | 1 | 0 | Intra_Horizontal | 0 | Intra_Vertical | 0 |
| 18 | 1 | 0 | Intra_DC | - | Intra_Vertical | 0 |
| 19 | 1 | 0 | Intra_DC | - | Intra_Horizontal | 0 |
| 20 | 1 | 1 | IntraPredModeAL0 | - | IntraPredModeBL0 | - |
| 21 | 1 | 2 | IntraPredModeAL0 | - | Nb[IntraPredModeAL0] | - |
| 22 | 1 | 3 | IntraPredModeBL0 | - | Nb[IntraPredModeBL0] | - |
| 23 | 1 | 4 | IntraPredModeAL0 | - | Ct[IntraPredModeAL0] | - |
| 24 | 1 | 5 | IntraPredModeBL0 | - | Ct[IntraPredModeBL0] | - |

- Table A-2  Spacification for IntraPredMode for for PU_8x8, PU16x16

| IntraPred- Mode [ puPartIdx ] | IntraBipred- Flag [ puPartIdx ] | IntraBipred- TypeIdx [ puPartIdx ] | IntraPred- TypeL0 [ puPartIdx ] | IntraPred- AngleIdL0 [ puPartIdx ] | IntraPred- TypeL1 [ puPartIdx ] | IntraPred- AngleIdL1 [ puPartIdx ] |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | - | Intra_Vertical | 0 | - | - |
| 1 | 0 | - | Intra_Horizontal | 0 | - | - |
| 2 | 0 | - | Intra_DC | - | - | - |
| 3 | 0 | - | Intra_Vertical | -8 | - | - |
| 4 | 0 | - | Intra_Vertical | -4 | - | - |
| 5 | 0 | - | Intra_Vertical | +4 | - | - |
| 6 | 0 | - | Intra_Vertical | +8 | - | - |
| 7 | 0 | - | Intra_Horizontal | -4 | - | - |
| 8 | 0 | - | Intra_Horizontal | +4 | - | - |
| 9 | 0 | - | Intra_Horizontal | +8 | - | - |
| 10 | 0 | - | Intra_Vertical | -6 | - | - |
| 11 | 0 | - | Intra_Vertical | -2 | - | - |
| 12 | 0 | - | Intra_Vertical | +2 | - | - |
| 13 | 0 | - | Intra_Vertical | +6 | - | - |
| 14 | 0 | - | Intra_Horizontal | -6 | - | - |
| 15 | 0 | - | Intra_Horizontal | -2 | - | - |
| 16 | 0 | - | Intra_Horizontal | +2 | - | - |
| 17 | 0 | - | Intra_Horizontal | +6 | - | - |
| 18 | 0 | - | Intra_Vertical | -7 | - | - |
| 19 | 0 | - | Intra_Vertical | -5 | - | - |
| 20 | 0 | - | Intra_Vertical | -3 | - | - |
| 21 | 0 | - | Intra_Vertical | -1 | - | - |
| 22 | 0 | - | Intra_Vertical | +1 | - | - |
| 23 | 0 | - | Intra_Vertical | +3 | - | - |
| 24 | 0 | - | Intra_Vertical | +5 | - | - |
| 25 | 0 | - | Intra_Vertical | +7 | - | - |
| 26 | 0 | - | Intra_Horizontal | -7 | - | - |
| 27 | 0 | - | Intra_Horizontal | -5 | - | - |
| 28 | 0 | - | Intra_Horizontal | -3 | - | - |
| 29 | 0 | - | Intra_Horizontal | -1 | - | - |
| 30 | 0 | - | Intra_Horizontal | +1 | - | - |
| 31 | 0 | - | Intra_Horizontal | +3 | - | - |
| 32 | 0 | - | Intra_Horizontal | +5 | - | - |
| 33 | 0 | - | Intra_Horizontal | +7 | - | - |
| 34 | 1 | 0 | Intra_Horizontal | 0 | Intra_Vertical | 0 |
| 35 | 1 | 0 | Intra_DC | - | Intra_Vertical | 0 |
| 36 | 1 | 0 | Intra_DC | - | Intra_Horizontal | 0 |
| 37 | 1 | 1 | IntraPredModeAL0 | - | IntraPredModeBL0 | - |
| 38 | 1 | 2 | IntraPredModeAL0 | - | Nb[IntraPredModeAL0] | - |
| 39 | 1 | 3 | IntraPredModeBL0 | - | Nb[IntraPredModeBL0] | - |
| 40 | 1 | 4 | IntraPredModeAL0 | - | Ct[IntraPredModeAL0] | - |
| 41 | 1 | 5 | IntraPredModeBL0 | - | Ct[IntraPredModeBL0] | - |

- Table A-3  Spacification for IntraPredMode for for PU_32x32

| IntraPred- Mode [ puPartIdx ] | IntraBipred- Flag [ puPartIdx ] | IntraBipred- TypeIdx [ puPartIdx ] | IntraPred- TypeL0 [ puPartIdx ] | IntraPred- AngleIdL0 [ puPartIdx ] | IntraPred- TypeL1 [ puPartIdx ] | IntraPred- AngleIdL1 [ puPartIdx ] |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | - | Intra_Vertical | 0 | - | - |
| 1 | 0 | - | Intra_Horizontal | 0 | - | - |
| 2 | 0 | - | Intra_DC | - | - | - |
| 3 | 0 | - | Intra_Vertical | -8 | - | - |
| 4 | 0 | - | Intra_Vertical | -4 | - | - |
| 5 | 0 | - | Intra_Vertical | +4 | - | - |
| 6 | 0 | - | Intra_Vertical | +8 | - | - |
| 7 | 0 | - | Intra_Horizontal | -4 | - | - |
| 8 | 0 | - | Intra_Horizontal | +4 | - | - |
| 9 | 0 | - | Intra_Horizontal | +8 | - | - |
| 10 | 0 | - | Intra_Vertical | -6 | - | - |
| 11 | 0 | - | Intra_Vertical | -2 | - | - |
| 12 | 0 | - | Intra_Vertical | +2 | - | - |
| 13 | 0 | - | Intra_Vertical | +6 | - | - |
| 14 | 0 | - | Intra_Horizontal | -6 | - | - |
| 15 | 0 | - | Intra_Horizontal | -2 | - | - |
| 16 | 0 | - | Intra_Horizontal | +2 | - | - |
| 17 | 0 | - | Intra_Horizontal | +6 | - | - |
| 18 | 0 | - | Intra_Vertical | -7 | - | - |
| 19 | 0 | - | Intra_Vertical | -5 | - | - |
| 20 | 0 | - | Intra_Vertical | -3 | - | - |
| 21 | 0 | - | Intra_Vertical | -1 | - | - |
| 22 | 0 | - | Intra_Vertical | +1 | - | - |
| 23 | 0 | - | Intra_Vertical | +3 | - | - |
| 24 | 0 | - | Intra_Vertical | +5 | - | - |
| 25 | 0 | - | Intra_Vertical | +7 | - | - |
| 26 | 0 | - | Intra_Horizontal | -7 | - | - |
| 27 | 0 | - | Intra_Horizontal | -5 | - | - |
| 28 | 0 | - | Intra_Horizontal | -3 | - | - |
| 29 | 0 | - | Intra_Horizontal | -1 | - | - |
| 30 | 0 | - | Intra_Horizontal | +1 | - | - |
| 31 | 0 | - | Intra_Horizontal | +3 | - | - |
| 32 | 0 | - | Intra_Horizontal | +5 | - | - |
| 33 | 0 | - | Intra_Horizontal | +7 | - | - |
| 34 | 1 | 0 | Intra_Horizontal | 0 | Intra_Vertical | 0 |
| 35 | 1 | 1 | IntraPredModeAL0 | - | IntraPredModeBL0 | - |
| 36 | 1 | 2 | IntraPredModeAL0 | - | Nb[IntraPredModeAL0] | - |
| 37 | 1 | 3 | IntraPredModeBL0 | - | Nb[IntraPredModeBL0] | - |

- Table A-4  Spacification for IntraPredMode for for PU_64x64 (Same as UIP)

| IntraPredMode [ puPartIdx ] | IntraBipredFlag [ puPartIdx ] | IntraPredTypeL0 [ puPartIdx ] | IntraPredAngleIdL0 [ puPartIdx ] | IntraPredTypeL1 [ puPartIdx ] | IntraPredAngleIdL1 [ puPartIdx ] |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | Intra_Vertical | 0 | - | - |
| 1 | 0 | Intra_Horizontal | 0 | - | - |
| 2 | 0 | Intra_DC | - | - | - |
| 3 | 0 | Intra_Vertical | -8 | - | - |
| 4 | 0 | Intra_Vertical | -4 | - | - |

- Table A-5  Spacification for NeighborDirTable[] and CounterDirTable[].

| IntraPredModeL0 | Nb[IntraPredModeL0] | Ct[IntraPredModeL0] |
| --- | --- | --- |
| 0 | 21 | 9 |
| 1 | 30 | 6 |
| 2 | 9 | 7 |
| 3 | 26 | 6 |
| 4 | 27 | 3 |
| 5 | 23 | 9 |
| 6 | 13 | 9 |
| 7 | 28 | 5 |
| 8 | 32 | 6 |
| 9 | 17 | 6 |
| 10 | 18 | 6 |
| 11 | 20 | 9 |
| 12 | 22 | 9 |
| 13 | 24 | 9 |
| 14 | 27 | 6 |
| 15 | 29 | 6 |
| 16 | 31 | 6 |
| 17 | 33 | 6 |
| 18 | 10 | 6 |
| 19 | 10 | 6 |
| 20 | 11 | 6 |
| 21 | 11 | 6 |
| 22 | 12 | 9 |
| 23 | 12 | 9 |
| 24 | 13 | 9 |
| 25 | 13 | 9 |
| 26 | 14 | 6 |
| 27 | 14 | 6 |
| 28 | 15 | 6 |
| 29 | 15 | 6 |
| 30 | 16 | 6 |
| 31 | 16 | 6 |
| 32 | 17 | 6 |
| 33 | 17 | 6 |