| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D109 |
| --- | --- |

| Title: | LUT-based adaptive filtering on intra prediction samples |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto Shun-ichi Sekiguchi Akira Minezawa Information Technology R&D Center Mitsubishi Electric Corporation 5-1-1, Ofuna, Kamakura, JAPAN Kazuhisa Iguchi Yoshiaki Shishikui NHK Science & Technology Research Laboratories 1-10-11, Kinuta, Setagaya-ku, Tokyo, JAPAN | Tel: Email: | +81-467-41-2463 Kazuo.Sugimoto@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation and NHK(Japan Broadcasting Corporation) |  |  |

_____________________________

# Abstract

In this contribution, an adaption of filtering on intra prediction samples is proposed to improve the coding performance. In the proposed scheme, the prediction samples generated by unified intra prediction are filtered just before generating residual samples according to the prediction unit size and the intra prediction mode. We implemented the proposed scheme on TMuC0.9, and simulations are conducted using common test configurations with Class A-E sequences and SHV sequences cropped to Class A size[1]. The proposed scheme achieves up to 1.3% (0.7% on average) BD-rate reduction for high efficiency, and up to 4.6% (1.6% on average) BD-rate reduction for low complexity intra configuration respectively. Simplified version of the scheme is also proposed in this contribution.

# Introduction

In [2], we have shown the possibility of the advantage in terms of coding efficiency using filtering on intra prediction samples compared to intra reference sample smoothing.

According to our further investigations, using various filters depending on the prediction unit size and the intra prediction mode of the coded block provide better coding efficiency when compared to only one filter for intra prediction samples. In this contribution, the filtering decision is based on a lookup table.

# Proposed scheme

In HM-1, intra prediction unit size can be 4x4, 8x8, 16x16, 32x32, and 64x64 with up to 33 directional intra prediction modes. The proposed filtering scheme on intra prediction samples is applied to up to 16x16 PU size with three different filters. The filter coefficients and the LUT are shown in figure 1 and Table 1 respectively.  Mode index in the table 1 indicates intra prediction mode index shown in Figure 8-1 of the HEVC Working Draft[3].

Figure 1.  filter coefficients

Table 1.  LUT for each PU size and intra prediction mode

| PU Size | Filter mode |  |  |  |
| --- | --- | --- | --- | --- |
|  | Filter #0 | Filter #1 | Filter #2 | Filter #3 |
| 4x4 | - | mode 3, 6, 9 | mode 2 | - |
| 8x8 | - | mode 2 | - | mode 3-9 |
| 16x16 | mode 2 | - | - | mode 3-33 |
| 32x32 | - | - | - | - |
| 64x64 | - | - | - | - |

As shown in figure 1, filters #0 to #2 uses the current, upper and left samples for filtering, while filter #3 uses the current and four neighboring samples.

We also developed simplified version of the proposed method by enabling filtering only on DC prediction modes, where only left and upper most pixel lines of the prediction samples are filtered using 2 tap filters shown in Figure 2, and the rest of the prediction samples are not filtered. The LUT for this simplified version is also shown in table 2. In our proposed simplified version, simplified adaptive intra smoothing(SAIS) proposed in [4] is also applied so that prediction samples except DC prediction mode can be smoothed with almost no complexity increase.

Figure 2. filter coefficients for simplified version

Table 2.  LUT for each PU size and intra prediction mode with simplified version

| PU Size | Filter mode |  |  |
| --- | --- | --- | --- |
|  | Filter #0 | Filter #1 | Filter #2 |
| 4x4 | - | - | mode 2 |
| 8x8 | - | mode 2 | - |
| 16x16 | mode 2 | - | - |
| 32x32 | - | - | - |
| 64x64 | - | - | - |

# Simulations & Results

The proposed scheme is implemented on top of TMuC v0.9, and the simulations are conducted using the common test configurations[5] for intra only conditions with Class A-E sequences and SHV sequences cropped to Class A size. Table 3 shows the comparison results in high efficiency and low complexity intra coding respectively.

Table 3. BD Bitrate saving of the proposed scheme

We also conducted a simulation on the simplified version of the proposal, and the results are shown in Table 4. In this case, we implemented simplified adaptive intra smoothing proposed in [4] on top of TMuC v0.9, and the simplified version of our proposal is also implemented on top of it. The improvement without SAIS is also shown in Table 5.

Table 4. BD Bitrate saving of the simplified version of the proposed scheme

Table 5. BD Bitrate saving of the simplified version of the proposed scheme without SAIS

# Conclusion

In this contribution, an intra prediction filtering scheme is proposed. The proposed scheme is based on a simple but efficient partitioning rule, which avoid encoder complexity and additional signaling overhead. The proposed scheme achieves up to 1.3% ( 0.7% on average) BD-rate reduction for high efficiency, and up to 3.6% ( 1.6% on average) BD-rate reduction for low complexity intra configuration respectively. The encoding and decoding complexity increase is very small compared to TMuC 0.9 default setting.

We also demonstrated a simplified version of intra prediction filtering scheme which achieves up to 0.9% ( 0.5% on average) BD-rate reduction for high efficiency, and up to 2.4% ( 1.1% on average) BD-rate reduction for low complexity intra configuration respectively. The encoding and decoding complexity of the simplified version is negligible. Considering the R-D performance trade-off, the simplified version might be preferable.

We propose intra prediction filtering scheme to be verified in CE by the next meeting.

# Reference

- Kazuhisa Iguchi, et al., “Performance report of TMuC for Super Hi-Vision”, Doc. JCTVC-C055, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, CN, October 2010

- Shun-ichi Sekiguchi, et Al., “DCT-based noise filtering for intra prediction samples”, Doc. JCTVC-B067, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, July 2010

- “WD1: Working Draft 1 of High-Efficiency Video Coding”, Doc. JCTVC-C403, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, CN, October 2010

- Yunfei Zheng, et al., “Simplified Intra Smoothing”, Doc. JCTVC-C234, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, CN, October 2010

- JCT-VC, “Common test conditions and software reference configurations” , Doc. JCTVC-C500, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, July 2010

# Patent rights declaration(s)

Mitsubishi Electric Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NHK may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).