| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D110 |
| --- | --- |

| Title: | CE6 Subset A: Cross check report of Samsung's proposal (JCTVC-C206) from Toshiba |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Akiyuki Tanizawa Taichiro Shiodera 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, JAPAN | Tel: Email: | +81-44-549-2152 akiyuki.tanizawa@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

This contribution contains the cross-checking the software containing the tool of JCTVC-C206 proposed by Samsung Electronics Co., Ltd.. The verification task has been done successfully and the results match those provided by proponents.

# Cross-checking

The software based on TMuC version 0.9 and the experimental results were provided on Dec. 28 2010 by Samsung Electronics Co., Ltd.. Proponent offered four test cases and we performed T2 case.

- T1: JCTVC-C206 (Chroma intra prediction by reconstructed luma samples) with code-word re-assignment

- T2: JCTVC-C206 simplified version, three modes (LM, DM and DC) are used for chroma intra coding

- T3: Chroma intra mode coding improvement by code-word re-assignment (assign shortest code-word to DM mode since it is most probable mode.

- T4: Chroma intra mode coding improvement and encoding complexity reduction by using only DM and DC mode

The software was complied fine and the cross-checking was performed on 64bit environments. The simulation has been conducted under CE common test conditions [1][2] for both high efficiency and low complexity conditions in the case of I-slice only, random access and low delay coding structures. The encoded images match those of the decoded ones, and the PSNR and bitrate values are identical to the results provided by Samsung Electronics Co., Ltd.. Table 1, Table 2 and Table 3 show the cross-checking results of overall cases from Toshiba.

Table 1: BD-Bitrate, the encoding time and the decoding time for I slice only coding structure

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.4 | -7.3 | -3.6 | -1.1 | -7.5 | -2.8 |
| Class B | -1.5 | -7.6 | -3.9 | -1.2 | -7.7 | -3.5 |
| Class C | -1.8 | -7.5 | -8.0 | -1.7 | -6.8 | -7.3 |
| Class D | -1.2 | -5.0 | -5.0 | -1.1 | -4.7 | -4.4 |
| Class E | -1.1 | -4.3 | -5.1 | -0.5 | -3.6 | -4.0 |
| All | -1.4 | -6.4 | -5.2 | -1.2 | -6.1 | -4.5 |
| Enc Time[%] | 97% |  |  | 96% |  |  |
| Dec Time1[%] | 103% |  |  | 101% |  |  |
| Dec Time2[%] | 102% |  |  | 103% |  |  |

Table 2:  BD-Bitrate, the encoding time and the decoding time for random access coding structure

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.6 | -7.1 | -3.0 | -0.4 | -6.6 | -1.6 |
| Class B | -0.7 | -8.0 | -3.5 | -0.5 | -5.5 | -1.3 |
| Class C | -0.8 | -5.9 | -6.0 | -0.7 | -4.7 | -4.7 |
| Class D | -0.5 | -3.7 | -3.1 | -0.5 | -2.7 | -2.6 |
| Class E |  |  |  |  |  |  |
| All | -0.7 | -6.2 | -4.0 | -0.6 | -4.7 | -2.6 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time1[%] | 102% |  |  | 102% |  |  |
| Dec Time2[%] | 99% |  |  | 93% |  |  |

Table 3:  BD-Bitrate, the encoding time and the decoding time for low delay coding structure

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.2 | -2.3 | -1.3 | -0.1 | -2.1 | -1.2 |
| Class C | -0.3 | -2.4 | -2.6 | -0.2 | -2.1 | -1.9 |
| Class D | -0.1 | -1.9 | -1.3 | -0.2 | -1.1 | -1.0 |
| Class E | -0.2 | -1.5 | -3.8 | 0.0 | -2.9 | -3.1 |
| All | -0.2 | -2.1 | -2.1 | -0.1 | -2.0 | -1.7 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time1[%] | 102% |  |  | 102% |  |  |
| Dec Time2[%] | 100% |  |  | 96% |  |  |

In the above tables, it is noted that the Dec Time1 means the decoding time ratio without outputting the yuv file and the Dec time2 means the decoding time ratio with outputting the yuv file.

The accompanied spreadsheet contains the detailed data of the cross-checking results.

# References

[1] 	F. Bossen ," Common test conditions and software reference configurations ", ITU-T and ISO/IEC JCT-VC 3rd Meeting Guangzhou, JCTVC-C500, Oct. 2010.

[2] 	"Core Experiment 6: Intra Prediction Improvement ", ITU-T and ISO/IEC JCT-VC 3rd Meeting Guangzhou, JCTVC-C506, Oct. 2010.