| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D112 WG11 Number: m18863 |
| --- | --- |

| Title: | Evaluation of Most Probable Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomoyuki Yamamoto 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 yamamoto.tomoyuki@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

In this contribution, the effectiveness of Most Probable Mode (MPM) on TMuC 0.9 is reported. In addition, the effectiveness of MPM on TMuC 0.9 with the coding tool called Differential Coding on Intra Modes (DCIM) is reported. The results indicate that MPM introduces some gain when it is used with TMuC 0.9 coding tools. Meanwhile, it is also indicated that MPM causes some loss when it is used with DCIM.

# Introduction

At the last meeting, HEVC test model was created which includes intra prediction tool so called UIP (Unified Intra Prediction). UIP is a simple extension of AVC intra prediction tools in the sense that it has DC and directional modes which use neighboring pixels. UIP also inherits MPM from AVC. Although some aspects of UIP such as the number of predictors are being investigated in CE10 [1], the effectiveness of MPM for UIP has not been reported yet. Therefore, we investigated on the topic and it is reported in the following sections. In addition to UIP, there are some intra prediction tools studied in CE6 [2]. Since it is assumed that the variety of intra predictors has relation to the effectiveness of MPM, the relation was also investigated. Specifically, we evaluated the effectiveness of MPM for DCIM used with UIP.

# Effectiveness of Most Probable Mode

The following softwares were used for evaluating MPM.

a) UIP/MPM: TMuC 0.9 software (anchor)

b) UIP/MPM0: Modify a) not to use MPM

c) DCIM/MPM: DCIM software used in CE6

d) DCIM/MPM0: Modify b) not to use MPM

In order to modify the software “not to use MPM”, we changed the MPM derivation process so that DC mode is always selected as “MPM”. In other words, the shortest bit is always assigned to DC mode, while the longer bits are assigned to other directional modes. The encoding was done based on the Intra only configuration attached to TMuC 0.9 with both High Efficiency (HE) and Low Complexity (LC) settings. The results are shown in Table. 1 through 3 and Fig. 1.

Table. 1: Performance of UIP/MPM0

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.1 | 0.1 | 0.5 | 0.5 | 0.4 |
| Class B | 0.3 | 0.2 | 0.2 | 1.2 | 0.8 | 0.9 |
| Class C | 0.4 | 0.5 | 0.4 | 1.1 | 0.9 | 0.7 |
| Class D | 0.2 | 0.2 | 0.2 | 0.7 | 0.5 | 0.5 |
| Class E | 0.7 | 0.6 | 0.8 | 1.9 | 1.2 | 1.0 |
| All | 0.3 | 0.3 | 0.3 | 1.1 | 0.8 | 0.7 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

Table. 2: Performance of DCIM/MPM1

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.7 | -0.6 | -0.4 | -1.5 | -0.7 | -0.4 |
| Class B | -2.4 | -1.4 | -1.4 | -2.3 | -1.7 | -1.8 |
| Class C | -2.4 | -1.1 | -1.3 | -2.9 | -1.8 | -2.2 |
| Class D | -1.7 | -0.8 | -0.3 | -2.3 | -1.4 | -1.2 |
| Class E | -3.2 | -2.2 | -1.5 | -3.2 | -1.6 | -2.0 |
| All | -2.3 | -1.3 | -1.0 | -2.5 | -1.5 | -1.6 |
| Enc Time[%] | 110% |  |  | 117% |  |  |
| Dec Time[%] | 108% |  |  | 124% |  |  |

Table. 3: Performance of DCIM/MPM0

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.9 | -1.0 | -0.8 | -1.9 | -1.0 | -0.5 |
| Class B | -2.5 | -1.8 | -1.7 | -2.6 | -1.9 | -1.9 |
| Class C | -2.6 | -1.3 | -1.4 | -3.0 | -2.0 | -2.4 |
| Class D | -1.9 | -1.1 | -0.8 | -2.4 | -1.2 | -1.4 |
| Class E | -3.3 | -2.3 | -2.0 | -3.2 | -1.7 | -2.0 |
| All | -2.5 | -1.5 | -1.4 | -2.7 | -1.6 | -1.7 |
| Enc Time[%] | 108% |  |  | 115% |  |  |
| Dec Time[%] | 108% |  |  | 124% |  |  |

[%]

(a) Intra Only, High Efficiency

[%]

(b) Intra Only, Low Complexity

Fig. 1: bd-rate reduction compared to the reference (UIP/MPM)

The result for UIP/MPM0 shows that disabling MPM in TMuC 0.9 leads to coding efficiency degradation by 0.3% and 1.0% for HE and LC settings respectively, while the difference of DCIM/MPM and DCIM/MPM0 shows that disabling MPM in DCIM software leads to coding efficiency gain by about 0.2% for both HE and LC settings.  In addition to about 2% gain achieved by introducing DCIM, 0.2% gain can be added by just disabling MPM.

It could be said that MPM is effective to UIP (or TMuC 0.9), but not to the combination of UIP and DCIM. One fact that causes this difference might be that DCIM includes the additional mode based on the estimated edge direction to which shorter bits are assigned. Since a probable predictor is derived using the information of neighboring region in both MPM and the additional DCIM mode, the functionalities are overwrapped. Although it seems that the DCIM mode is superior to MPM in terms of deriving more probable directional predictor, it is not good at evaluating whether DC mode is probable or not. Therefore, it would be reasonable to assign shorter bits to both DC mode and the additional DCIM mode (which is the configuration used in the experiment).

As for the complexity, no significant impact on encoding and decoding time can be observed in the result. But it is clear that not doing MPM derivation process can reduce some complexity in terms of number of operations and the memory for intra modes.

# Conclusion

In this contribution, the effectiveness of Most Probable Mode (MPM) for TMuC 0.9 as well as for TMuC 0.9 with DCIM is reported. The results indicate that MPM is effective to TMuC 0.9, but not when DCIM is added. We suggest whether or not to use MPM should be studied carefully in the context of available intra coding tools.

# Reference

- JCT-VC, “Description of Core Experiment 10: Number of Intra Prediction Directions”, JCTVC-C510, 3rd JCT-VC Meeting, Guangzhou, October 2010.

- Ali Tabatabai et al. “Description of Core Experiment 6: Intra Prediction Improvement”, JCTVC-C506, 3rd JCT-VC Meeting, Guangzhou, October 2010.

# Patent rights declaration(s)

SHARP Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).