| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D113 WG11 Number: m18864 |
| --- | --- |

| Title: | CE11: Cross-check report for Fraunhofer HHI’s proposal |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yukinobu Yasugi Tomoyuki Yamamoto 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | Tel: +81-43-299-8526 yasugi.yukinobu@sharp.co.jp, yamamoto.tomoyuki@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution provides a cross-check result of the context selection of coeff_level_greater_one and coeff_abs_level_minus1 in HHI_TRANSFORM_CODING by Fraunhofer HHI [1] in the context of CE11 (CE11: Coefficient Scanning and Coding) [2]. The experiments defined in CE11 were carried out on our Linux-based grid system using the software provided by Fraunhofer HHI.

# Experimental condition

The evaluation of the context selection method defined in CE11 was carried out with TMuC_0.9-CE11 software by using following conditions:

Reference:	CE11_DISABLE_HHI_ABS_LEVEL_M1_CTX_MOD = 0 (the default settings)

Compared:	CE11_DISABLE_HHI_ABS_LEVEL_M1_CTX_MOD = 1

The computing platform is summarized in Table 1. In the experiments, encoding was done using the grid system composed of variety of PCs, but it was configured so that the same-performance PC is used to encode both anchor and the proposed method for each test point. Decoding was done using the same machine for both anchor and the proposed method.

Table 1: Computing platforms

|  | Decoding | Encoding |
| --- | --- | --- |
| OS | Linux (64bit) | Linux (64bit) |
| CPU | Intel Core2 Quad Q9550 2.83GHz Intel Xeon X5667 3.07GHz Intel Core2 Quad 2.66GHz | Intel Core2 Quad Q9550 2.83GHz Intel Xeon X5667 3.07GHz Intel Core2 Quad 2.66GHz |
| Compiler | gcc 4.4.3 | gcc 4.4.3 |

# Performance experiments

The experimental results are summarized in Table 2. Detailed results are included in the attachment (JCTVC-D113.xls).

Table 2: Experimental results

|  | Intra |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig_flag count |
| Class A | 0.7 | 0.5 | 0.5 | -2.02% |
| Class B | 0.9 | 0.9 | 0.9 | -2.45% |
| Class C | 0.3 | 0.6 | 0.5 | -2.30% |
| Class D | 0.3 | 0.5 | 0.5 | -1.47% |
| Class E | 0.7 | 0.4 | 0.5 | -2.07% |
| All | 0.6 | 0.6 | 0.6 | -2.06% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 103% |  |  |  |
|  | Random access |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig_flag count |
| Class A | 0.1 | 0.9 | 0.7 | -2.22% |
| Class B | 0.3 | 1.5 | 1.1 | -2.97% |
| Class C | 0.1 | 1.3 | 1.1 | -4.26% |
| Class D | 0.2 | 1.3 | 1.4 | -4.35% |
| Class E |  |  |  |  |
| All | 0.2 | 1.3 | 1.1 | -3.45% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  |
|  | Low delay |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | % sig_flag count |
| Class A |  |  |  |  |
| Class B | 0.5 | 4.2 | 3.8 | -5.32% |
| Class C | 0.7 | 4.0 | 3.9 | -8.11% |
| Class D | 1.0 | 2.8 | 3.4 | -10.05% |
| Class E | 0.9 | 4.8 | 4.5 | -11.62% |
| All | 0.7 | 3.9 | 3.8 | -8.78% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  |

# Cross-check

The experimental results in above section were compared with the results in JCTVC-D190, and they completely match in terms of bitrate and PSNR.

# Conclusion

The simulation result of the context selection of coeff_level_greater_one and coeff_abs_level_minus1 in HHI_TRANSFORM_CODING in the context of CE11 was reported. The experimental results show that the method introduces average gain (Y BD-rate for each condition) ranging from 0.2% to 0.7%, and differences of the sig_flag count ranging from -2.06% to -8.78%.

# Reference

- Thomas Wiegand, et al., “Description of video coding technology proposal by Fraunhofer HHI”, JCTVC-A116, 1st JCT-VC Meeting, Dresden, April 2010

- Vivienne Sze, et al., “CE11: Coefficient scanning and coding”, JCTVC-C511, 3rd JCT-VC Meeting, Guangzhou, October 2010.