| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D114 WG11 Number: m18865 |
| --- | --- |

| Title: | CE8: Cross-check result of MQT_ALF - MediaTek, Qualcomm and Toshiba’s joint proposal (JCT-VC D-119) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | Tel: +81-43-299-8526 ikai.tomohiro@sharp.co.jp, |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution provides a cross-check result of the ALF algorithm by MediaTek / Qualcomm / Toshiba[1] in the context of CE8 subtest 2[2] (CE8: In-loop filtering, Subtest 2: Wiener-based in-loop filters). The encoding was conducted on our Linux-based grid system and the decoding was done on one win32 PC. Our results perfectly match with those of proponents except for one case (Intra Only, High Complexity QP 37, BasketballDrive). The coding efficiency is the same as the proponents results. Although there are some differences in terms of time ratio, it is considered due to the difference of the platform and compiler. In High Complexity cases, the encoder time ratio and the decoder time ratio are 96 % and 80 % on average. In Low Complexity cases, the time ratios are 115% and 171%.

# Performance experiments

The experimental results are summarized in Table 1. Detailed results are included in the attachment (JCTVC-D114.xls).

Table 1: Experimental results

|  | Intra |  |  |  |  | Intra LoCo |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -0.3 |  | 0.0 | 0.3 |  | -6.5 |  | -2.5 | -2.2 |  |
| Class B | -0.3 |  | -0.6 | -0.3 |  | -3.3 |  | -3.0 | -2.4 |  |
| Class C | -0.1 |  | -1.1 | -0.9 |  | -2.6 |  | -3.0 | -4.2 |  |
| Class D | -0.4 |  | -1.1 | -0.9 |  | -1.7 |  | -2.9 | -3.5 |  |
| Class E | 0.3 |  | 0.2 | -0.2 |  | -5.2 |  | -5.6 | -6.0 |  |
| All | -0.2 |  | -0.6 | -0.5 |  | -3.5 |  | -3.4 | -3.6 |  |
| Enc Time[%] | 89 % |  |  |  |  | 134 % |  |  |  |  |
| Dec Time[%] | 78 % |  |  |  |  | 179 % |  |  |  |  |
|  | Random access |  |  |  |  | Random access LoCo |  |  |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | 0.0 |  | 0.3 | 0.3 |  | -4.2 |  | -2.0 | -1.6 |  |
| Class B | -0.3 |  | 0.1 | -0.2 |  | -5.0 |  | -5.8 | -3.4 |  |
| Class C | -0.6 |  | -0.2 | -0.2 |  | -4.8 |  | -2.9 | -4.8 |  |
| Class D | -0.6 |  | -0.4 | -0.4 |  | -6.6 |  | -3.6 | -3.9 |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| All | -0.4 |  | -0.1 | -0.2 |  | -5.3 |  | -3.9 | -3.7 |  |
| Enc Time[%] | 99 % |  |  |  |  | 107 % |  |  |  |  |
| Dec Time[%] | 82 % |  |  |  |  | 165 % |  |  |  |  |
|  | Low delay |  |  |  |  | Low delay LoCo |  |  |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.3 |  | -0.4 | -0.3 |  | -5.6 |  | -3.4 | -2.8 |  |
| Class C | -1.9 |  | -0.3 | -0.5 |  | -6.7 |  | -4.4 | -6.0 |  |
| Class D | -1.3 |  | -0.4 | 0.1 |  | -5.6 |  | -4.9 | -5.7 |  |
| Class E | -3.5 |  | -1.2 | -1.1 |  | -6.4 |  | -1.0 | 1.7 |  |
| All | -1.9 |  | -0.5 | -0.4 |  | -6.0 |  | -3.6 | -3.5 |  |
| Enc Time[%] | 98 % |  |  |  |  | 106 % |  |  |  |  |
| Dec Time[%] | 79 % |  |  |  |  | 169 % |  |  |  |  |

# Conclusion

In this contribution, the CE8 subtest 2 cross-check result of the ALF algorithm by MediaTek/Qualcomm/Toshiba is reported. The coding efficiency is the same as the proponents results.

Although there are some differences in terms of time ratio, it is considered due to the difference of the platform and compiler. In High Complexity cases, the encoder time ratio and the decoder time ratio are 96 % and 80 % on average. In Low Complexity cases, the time ratios are 115% and 171%.

# Reference

- Ching-Yeh Chen, Chih-Ming Fu, Chia-Yang Tsai, Yu-Wen Huang, Shawmin Lei, Marta Karczewicz, In Suk Chong, Tomoo Yamakage, Takeshi Chujoh, Takashi Watanabe, “A Joint Proposal on Improving the Adaptive Loop Filter in TMuC0.9 by MediaTek, Qualcomm, and Toshiba ”, JCTVC-D119, 4th JCT-VC Meeting, Daegu, January 2011.

- Tomoo Yamakage, Keiichi Chono, et al., “Description of CE8: In-loop filtering” , JCTVC-C508, 3rd JCT-VC Meeting, Guangzhou, October 2010.