| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 4th Meeting: Daegu, KR, 20-28 January, 2011 | Document: JCTVC-D115 WG11 Number: m18866 |
| --- | --- |

| Title: | CE8: DF-combined adaptive loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai, Takanori Yamazaki 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution is a CE8 subtest 2[1] (Wiener-based in-loop filters) proposal. In this contribution, the DF (Deblocking Filter) combined adaptive loop filter [2], which was implemented into TMuC0.9, was evaluated according to the CE8 subtest 2 test conditions. The proposed technique provides the functionality to process the two inputs in parallel as well as coding efficiency improvement. The experimental results show 0.4 % and 4.9% bitrate reduction in High Complexity and Low Complecity respectively. The decoding time ratio is 85% to 93% (win32) or 110 % to 116% (linux). The encoding time ratio is 91 % to 98 %. Additional experimental results show the DF-combined improvement, which is the bdrate difference between two input result and one input result is 0.9%, 0.7%, 0.7% in Intra Only, Random Access and Low Delay respectively.

# DF combined adaptive loop filter

## Algorithm

In the proposed technique, Wiener based in-loop filter is applied using both pre-DF signal and post-DF signal as inputs. (cf. the traditional ALF uses only post-DF signal.) The ALF in the proposed system is defined by the following formula.

Figure 1: Parallel adaptive loop filter system

Where sout is ALF output, si is pre-DF signal, and spost is post-DF signal. And wiand w are ALF coefficients and c is dc-offset. (wi, w, and c are signaled in the bitstream.)  Note that, although the proposed system utilizes both pre-DF and post-DF signals, it is parallel-processing friendly due to the following feature.

- pre-DF signal is mainly used (spatial filtered)

- post-DF signal is merely weighted.

The two filters DF and ALF-spatial filter are independent. DF and ALF-spatial filter can be processed in parallel.

## filtering shapes and coefficients

For pre-DF signal, the luminance filter is a 7x7, 9x9, and 11x11 tap point-symmetrical diamond shape filterand the chrominance filter is a 7x7 tap point-symmetrical diamond shape filter. For post-DF signal the filter shape is one tap filter. The filter shape and the number of filter coefficients are shown in Table 1 and Table 2.

Table 1: Filter Shape

|  | filter shape |  |  |
| --- | --- | --- | --- |
|  | pre-DF | post-DF | DC offset |
| Luma | 7x7, 9x9, 11x11 diamond | 1 | 1 |
| Chroma | 7x7 diamond | 1 | 1 |

Table 2: The number of filter coefficients

|  | number of filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | Pre-DF | Post DF | DC offset | Sum |
| Luma | 13 | 1 | 1 | 15 |
|  | 22 | 1 | 1 | 23 |
|  | 30 | 1 | 1 | 32 |
| Chroma | 15 | 1 | 1 | 15 |

## Encoder process

The number of encoding pass and corresponding operations is shown Table 3. Encoding process consists of repetitions of three operations: coefficients design (Design), filtering on/off control (Control), and perform filtering (Filtering).  Control can be processed parallel with Filtering because encoder can calculate distortion of the each CU related block at the same time of filtering the corresponding block. Design can be processed parallel with Control because the correlation matrix can be calculated by merging enable block’s correlations for Design when the enable block is decided in Control.

Table 3: Encoding pass and its description

| pass | operations | access |  |  |  |  | note | function |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | pre-DF | post-DF | orig | filter | corr |  | xEncALFLuma |
| 0 | Design | yes | yes | yes | no | yes | decoding 7x7 diamond +2 |  |
| 1 | Filtering | yes | yes | no | yes | no |  |  |
|  | Control | no | no | yes | yes | no | Depth=0, RD=0 | xCUAdaptiveControl |
|  | Control#1 | no | no | yes | yes | no | Depth=1, RD=0 |  |
|  | Control#2 | no | no | yes | yes | no | Depth=2, RD=0 |  |
|  | Control#3 | no | no | yes | yes | no | Depth=3, RD=0 |  |
|  | Design | no | no | no | no | yes | Depth=0, RD=1 |  |
| 2 | Filtering |  |  |  |  |  | Depth=0, RD=1 |  |
|  | Control |  |  |  |  |  | Depth=0, RD=1 |  |
|  | Design |  |  |  |  |  | Depth=0, RD=2 |  |
| 3 | Filtering |  |  |  |  |  | Depth=0, RD=2 |  |
|  | Control |  |  |  |  |  | Depth=0, RD=2 |  |
|  | (Control#1) |  |  |  |  |  | Depth=1, RD=0 |  |
|  | Design |  |  |  |  |  | Depth=1, RD=1 |  |
| 4 | Filtering |  |  |  |  |  | Depth=1, RD=1 |  |
|  | Control |  |  |  |  |  | Depth=1, RD=1 |  |
|  | Design |  |  |  |  |  | Depth=1, RD=2 |  |
| 5 | Filtering |  |  |  |  |  | Depth=1, RD=2 |  |
|  | Control |  |  |  |  |  | Depth=1, RD=2 |  |
|  | (Control#2) |  |  |  |  |  | Depth=2, RD=0 |  |
|  | Design |  |  |  |  |  | Depth=2, RD=1 |  |
| 6 | Filtering |  |  |  |  |  | Depth=2, RD=1 |  |
|  | Control |  |  |  |  |  | Depth=2, RD=1 |  |
|  | Design |  |  |  |  |  | Depth=2, RD=2 |  |
| 7 | Filtering |  |  |  |  |  | Depth=2, RD=2 |  |
|  | Control |  |  |  |  |  | Depth=2, RD=2 |  |
|  | (Control#3) |  |  |  |  |  | Depth=3, RD=0 |  |
|  | Design |  |  |  |  |  | Depth=3, RD=1 |  |
| 8 | Filtering |  |  |  |  |  | Depth=3, RD=1 |  |
|  | Control |  |  |  |  |  | Depth=3, RD=1 |  |
|  | Design |  |  |  |  |  | Depth=3, RD=2 |  |
| 9 | Filtering |  |  |  |  |  | Depth=3, RD=2 |  |
|  | Control |  |  |  |  |  | Depth=3, RD=2 |  |
|  | Design |  |  |  |  |  | 7x7 diamond +2 redesign | xFilterTapDecision |
| 10 | Filtering |  |  |  |  |  |  |  |
|  | Control |  |  |  |  |  |  |  |
|  | Design |  |  |  |  |  | 9x9 diamond + 2 |  |
| 11 | Filtering |  |  |  |  |  |  |  |
|  | Control |  |  |  |  |  |  |  |
|  | Design |  |  |  |  |  | 11x11 diamond + 2 |  |
| 12 | Filtering |  |  |  |  |  |  |  |
|  | Control |  |  |  |  |  |  |  |
| 13 | Reconstruct | no | no | no | yes | no | best case |  |

note: highlighted cell means the access is yes.

## Syntax description

The syntax is shown in Table 4.

Table 4: Syntax of the proposed system

adaptive_loop_filter_flag specifies whether adaptive loop filter applies or not for the current slice

alf_parallel_mode specifies the pixels used in adaptive loop filter.

If alf_parallel_mode is 0, post-DF pixels are used, otherwise pre-DF pixels and a post-DF pixel are used.

alf_length_luma_minus7_div2 specifies the filter length for luma component used in the adaptive loop filter process. From this value, the filter length used to decide the number of coded filter coefficients shall be computed as

AlfLengthLuma = ( alf_length_luma_minus7_div2 << 1 ) + 7

From this value and alf_parallel_mode, the number of coded filter coefficients for luma shall be computed as follows.

If alf_parallel_mode is enabled

AlfNumCoeffLuma = ( AlfLengthLuma * AlfLengthLuma / 4 ) + 3

Otherwise

AlfNumCoeffLuma = ( AlfLengthLuma * AlfLengthLuma / 4 ) + 2

alf_coeff_luma[i] specifies the ith filter coefficient for luma component.

alf_chroma_idc specifies which chroma components are to be filtered.

alf_length_chroma_minus7_div2 specifies the filter length for chroma components used in the adaptive loop filter process. From this value, the filter length used to decide the number of coded filter cofficients shall be computed as

AlfLengthChroma = ( alf_length_chroma_minus7_div2 << 1 ) + 7

From this value and alf_parallel_mode, the number of coded filter coefficients for chroma shall be computed as follows.

If alf_parallel_mode is enabled

AlfNumCoeffChroma = ( AlfLengthChroma * AlfLengthChroma / 4 ) + 3

Otherwise

AlfNumCoeffChroma = ( AlfLengthChroma * AlfLengthChroma / 4 ) + 2

alf_coeff_chroma[i] specifies the ith filter coefficient for chroma components.

# Experimental results

## CE8 subtest 2 results

The experimental results are shown in Table 5.

In the Table 5, there are two values in Dec Time[%]. The left-hand(*1) values were measured in 32bit windows VISTA platform (Visual Studio 2005, on single system) while the right-hand value(*2) were measured in 64 bit linux platform(gcc 4.4.3, on a multi PC grid system). Enc Time was measured on 64 bit linux. Details of 32bit windows decoder are in JCTVC-D115.xls and Details of 64 bit linux decoder was JCTVC-D115_linux_dec.xls.

Table 5: Experimental results

|  | Intra |  |  |  |  | Intra LoCo |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -0.3 |  | 0.0 | 0.3 |  | -7.0 |  | -2.7 | -2.0 |  |
| Class B | -0.3 |  | -0.6 | -0.3 |  | -4.1 |  | -3.8 | -2.9 |  |
| Class C | -0.1 |  | -1.1 | -0.9 |  | -2.9 |  | -4.2 | -5.0 |  |
| Class D | -0.4 |  | -1.1 | -0.9 |  | -2.2 |  | -3.7 | -4.1 |  |
| Class E | 0.3 |  | 0.2 | -0.2 |  | -5.3 |  | -5.5 | -6.1 |  |
| All | -0.2 |  | -0.6 | -0.5 |  | -3.9 |  | -4.0 | -4.1 |  |
| Enc Time[%] | 91% |  |  |  |  | 135% |  |  |  |  |
| Dec Time[%] | 85% * 1 ¦ 110% *2 |  |  |  |  | 198% ¦ 216% |  |  |  |  |
|  | Random access |  |  |  |  | Random access LoCo |  |  |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A | -0.4 |  | -0.1 | -0.1 |  | -4.7 |  | -1.7 | -1.3 |  |
| Class B | 0.3 |  | -1.2 | -1.1 |  | -5.3 |  | -6.7 | -4.0 |  |
| Class C | -0.2 |  | -1.2 | -0.9 |  | -4.7 |  | -3.9 | -5.5 |  |
| Class D | -0.6 |  | -1.1 | -0.9 |  | -6.6 |  | -3.7 | -3.7 |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| All | -0.2 | -1.0 |  |  | -0.9 | -5.4 |  | -4.5 | -4.0 |  |
| Enc Time[%] | 98% |  |  |  |  | 105% |  |  |  |  |
| Dec Time[%] | 93% ¦ 111% |  |  |  |  | 194% ¦ 201% |  |  |  |  |
|  | Low delay |  |  |  |  | Low delay LoCo |  |  |  |  |
|  | Y BD-rate |  | U BD-rate | V BD-rate |  | Y BD-rate |  | U BD-rate | V BD-rate |  |
| Class A |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.5 |  | -1.1 | -1.2 |  | -5.7 |  | -3.9 | -3.4 |  |
| Class C | -0.6 |  | -1.0 | -0.8 |  | -5.8 |  | -4.8 | -6.2 |  |
| Class D | -1.5 |  | -1.8 | -1.1 |  | -5.6 |  | -4.7 | -5.5 |  |
| Class E | -0.4 |  | 0.3 | 0.5 |  | -4.3 |  | -1.0 | 1.7 |  |
| All | -0.7 |  | -1.0 | -0.8 |  | -5.4 |  | -3.8 | -3.7 |  |
| Enc Time[%] | 98% |  |  |  |  | 106% |  |  |  |  |
| Dec Time[%] | 93% ¦ 116% |  |  |  |  | 196% ¦ 205% |  |  |  |  |
| All average | -0.4 |  | -0.9 | -0.7 |  | -4.9 |  | -4.1 | -3.9 |  |

*1: left-hand values were measured in 32bit Windows VISTA.

*2: right-hand values were measured in 64bit linux.

Thanks to Panasonic [3] and Toshiba [4] cross-checks, the above coding performance was confirmed.

The numumber of encoder passesis shown in Table 6. Details of each encoding process are described in section 1.3.

Table 6: Number of encoding pass

|  | Random Access | Low Delay |
| --- | --- | --- |
| Anchor | 17 | 17 |
| Proposal | 13 | 13 |

DF-combined improvement results

The additional experiment, in which one input (post-DF signal) is used as conventional system, is conducted.The experimental result of two input and one input compared with TMuC 0.9 anchor and the difference between 2 input bdrate and 1 input bdrate are shown Table 7. The difference of two cases which indicates DF-combined improvement, is shown in Figure 2.

As shown, DF-combined improvement of Intra Only, Randam Access, and LowDelay is 0.9%, 0.7%, 0.7% respectively. Maximum bitrate reduction by DF-combined is 2.0 %.

Table 7: The experimental results of two input and one input and the difference

|  | Intra Only |  |  | Random Access |  |  | Low Delay |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | two input | one input | difference | two input | one input | difference | two input | one input | difference |
| PeopleOnStreet_c4k | 0.2 | -1.5 | 1.7 | -0.8 | -1.0 | 0.2 | -0.8 | -1.0 | 0.2 |
| Traffic_c4k | 0.4 | -1.0 | 1.4 | 1.6 | -0.3 | 2.0 | 1.6 | -0.3 | 2.0 |
| BQTerrace | 0.6 | -0.4 | 1.0 | -0.8 | -1.6 | 0.7 | 1.2 | -0.1 | 1.2 |
| BasketballDrive | 0.0 | -0.9 | 1.0 | -0.6 | -0.8 | 0.3 | -0.5 | -0.8 | 0.3 |
| Cactus | 0.3 | -0.9 | 1.2 | -0.4 | -1.3 | 0.9 | 0.3 | -0.2 | 0.5 |
| Kimono1 | -0.1 | -1.0 | 0.9 | -0.3 | -0.6 | 0.3 | -0.5 | -0.6 | 0.1 |
| ParkScene | 0.9 | -0.8 | 1.6 | 0.8 | -0.5 | 1.4 | 2.0 | 0.2 | 1.7 |
| BQMall | 0.2 | -0.6 | 0.8 | 0.4 | -0.2 | 0.6 | 0.8 | 0.2 | 0.5 |
| BasketballDrill | 0.2 | -1.1 | 1.3 | 0.7 | -0.4 | 1.1 | 0.5 | -0.0 | 0.6 |
| PartyScene | 0.2 | -0.3 | 0.5 | 0.9 | 0.5 | 0.4 | 1.3 | 1.0 | 0.3 |
| RaceHorses | -0.3 | -0.9 | 0.7 | -1.0 | -1.1 | 0.1 | -0.0 | -0.3 | 0.3 |
| BQSquare | 0.3 | -0.0 | 0.3 | 1.4 | 0.8 | 0.6 | 4.5 | 3.9 | 0.6 |
| BasketballPass | 0.5 | -0.4 | 0.9 | 0.3 | -0.2 | 0.4 | 0.1 | -0.2 | 0.3 |
| BlowingBubbles | 0.5 | -0.1 | 0.6 | 0.6 | 0.1 | 0.5 | 0.5 | 0.3 | 0.2 |
| RaceHorses_wqvga | 0.5 | -0.6 | 1.1 | 0.1 | -0.3 | 0.3 | 0.9 | 0.2 | 0.7 |
| vidyo1 | -1.0 | -1.6 | 0.6 | 0.3 | -0.5 | 0.8 | -0.1 | -0.6 | 0.5 |
| vidyo3 | 0.5 | -0.4 | 0.9 | 1.3 | -0.1 | 1.3 | -0.4 | -1.3 | 0.9 |
| vidyo4 | -0.4 | -1.2 | 0.8 | 0.7 | -0.6 | 1.3 | 1.5 | 0.2 | 1.3 |
| Class A | 0.3 | -1.3 | 1.6 | 0.4 | -0.7 | 1.1 | 0.4 | -0.7 | 1.1 |
| Class B | 0.3 | -0.8 | 1.1 | -0.3 | -1.0 | 0.7 | 0.5 | -0.3 | 0.8 |
| Class C | 0.1 | -0.7 | 0.8 | 0.2 | -0.3 | 0.5 | 0.6 | 0.2 | 0.4 |
| Class D | 0.4 | -0.3 | 0.7 | 0.6 | 0.1 | 0.5 | 1.5 | 1.1 | 0.5 |
| Class E | -0.3 | -1.1 | 0.8 | 0.8 | -0.4 | 1.1 | 0.4 | -0.6 | 0.9 |
| All | 0.2 | -0.8 | 0.9 | 0.3 | -0.5 | 0.7 | 0.7 | 0.0 | 0.7 |

The differences of the cases which are above 1 % are highlighted.

Figure 2: DF-combined improvement

# Conclusion

In this contribution, the DF-combined adaptive loop filter was evaluated as the part of CE8 subtest 2. The proposed approach, which uses both pre-DF signal and post-DF signal as inputs, provides parallel processing capability of in-loop filtering process and coding efficiency improvement. The experimental results show 0.4 % and 4.9 % bitrate reduction in High Complexity and Low Complexity respectively. The decoding time ratio is 85% to 93%  (win32) or 110 % to 116% (linux). The encoding time ratio is 91% to 98%. Additional experimental results show the DF-combined improvement, which is the bdrate difference between two input result and one input result is 0.9%, 0.7%, 0.7% in Intra Only, Random Access and Low Delay respectively. Thanks to Panasonic and Toshiba’s cross-checks coding performance was confirmed.

# Reference

- Tomoo Yamakage, Keiichi Chono, et al., “Description of CE8: In-loop filtering”, JCTVC-C508, 3rd JCT-VC Meeting, Guangzhou, October 2010.

- Tomohiro Ikai and Tomoyuki Yamamoto, “Parallel adaptive loop filter”, JCTVC-C173, 3rd JCT-VC Meeting, Guangzhou, October 2010.

- Matthias Narroschke, Semih Esenlik, “CE8 Subset2: Verification results of Sharp's Proposal JCTVC-D115” , D215, 4th JCT-VC Meeting, Daegu, January 2011.

- T. Yamakage. JCTVC-D211, “CE8 Subset2: Verification results of Sharp's Proposal JCTVC-D115” , 4th JCT-VC Meeting, Daegu, January 2011.

# Patent rights declaration(s)

SHARP Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).